303(d) Fact Sheet Region 9 Water Quality Control Board

(Santa Margarita River (lower) (902.10) – listing)

# Summary of Proposed Action

Based on the available data submitted, it is recommended that the lower Santa Margarita River be added to the 303d list of impaired waterbodies for TDS.  It should also be documented as threatened for sediment, nutrients, manganese, iron and sulfate in the 305b assessment.

303(d) Listing / TMDL Information

- Waterbody Name – Santa Margarita River (lower)

- Hydrologic Unit – 902.10

- Total Waterbody Size –  10.4 miles

- Potential Pollutants / Stressors – Mn, Fe, TDS, sulfate, phosphorous, sediment

- Suspected Sources – development/urban runoff? agricultural activities? natural sources?

imported water supply?

- Extent of Impairment –  10.4miles

- TMDL Priority – medium

#### Watershed Characteristics

Santa Margarita Hydrologic Unit is a rectangular area of approximately 750 square miles.  It includes portions of US Marine Corps’ Camp Pendleton, as well as the civilian populations of Murrieta, Temecula, and part of Fallbrook. The Santa Margarita Hydrologic Unit is comprised of the following nine hydrologic areas: the Ysidora, Deluz, Murrieta, Auld, Pechanga, Wilson, Cave Rocks, Aguanga and Oak Grove Hydrologic Areas. Annual precipitation ranges from less that 12 inches near the coast to more than 45 inches inland near Palomar Mountain. The major surface water storage areas are Vail Lake and O'Neil Lake.1

Santa Margarita Hydrologic Unit is drained largely by the Santa Margarita River, Murrieta Creek and the Temecula Creek.  Murrieta Creek flows southeasterly from the northern slope of the Santa Rosa Plateau to the confluence with the Temecula Creek to form the Santa Margarita River The Santa Margarita then flows approximately 27 miles to the Pacific Ocean. The coastal Santa Margarita Lagoon is at the mouth and lies within the Camp Pendleton Naval Reservation of the US Marine Corps.  The slough at the mouth of the river is normally closed off from the ocean by a sandbar.1  The Santa Margarita River provides groundwater recharge to Camp Pendleton’s only domestic water supply. 1,2

The Santa Margarita is divided into lower and upper reaches as defined by the confluence of the DeLuz Creek.

Water Quality Objectives Not Attained (or Objectives being Attained for Delisting)

All of the designated beneficial uses for the Santa Margarita River include MUN, AGR, IND, REC1, REC2, WARM, COLD, WILD, GWR, RARE and PROC.1 Since this waterbody is designated as MUN, drinking water standards were used as criteria.   Data shows exceedances of secondary drinking water MCLs and for Basin Plan objectives.

# Evidence of Impairment

Based on quarterly surface water quality data collected, compiled and analyzed by Camp Pendleton from 1997-2000, the lower Santa Margarita River has consistently exceeded water quality standards for iron, total phosphorus, manganese and TDS at 44%, 57% 33% and 33% of the time, respectively.  Additionally and to a lesser degree, there were exceedances for sulfate at 11%.2

Though the proceeding documents exceedances of Basin Plan and drinking water standards for various parameters, review of historical trend data for the lower Santa Margarita River (also compiled by Camp Pendleton) from 1965-2000, illustrates that for these other parameters including iron, manganese and sulfate there is no evidence of increasing trends and may represent ambient conditions. Based on these trends and since these parameters are associated with secondary MCLS for aesthetics it is not recommended that these be listed as impairments at this time.  Trend data shows a slight increase in TDS concentrations potentially indicating a decrease in water quality.

Grab sampling conducted by the SDRWQCB in June 1998 confirmed the high TDS levels in the this portion of the Santa Margarita River and also showed high concentrations of other constituents including As, Cd, Cr, Cu, Ni, Pb, Th, and nutrients.  However since these were only single grab samples, they did not serve as the basis for listing, but only support recommendations for the listings.3

TSMP fish tissue samples were collected and analyzed in 1997 and 1999.  The 1997 analysis showed an exceedance of total DDT EPA screening values  (14 ppb) for subsistence fishers at 91 ppb.4 However, there is not applicable to this waterbody as it does not support this subsistent fishing activity and the concentration is well below the other criteria. criteria used to evaluate fish tissue concentrations of organics including the MTRLs in Inland Surface Waters (32 ppb), NAS (500 ppb), FDA Action Levels (5000 ppb) and EPA Screening Values for recreational fishers (370 ppb).

The 1999 analysis showed an exceedance of Arsenic MTRL (0.2 ppm) at 0.221 ppm. 5  However it is not recommended that this waterbody be listed as impaired by arsenic since, there is difficulty in correlating tissue concentrations with water quality.

USACOE sponsored a sedimentation study for the Santa Margarita Watershed with results published in July 2000.8  This study included developing a hydrologic model, hydraulic model and ultimately sediment transport model for the Santa Margarita River.  Based on the hydrologic and hydraulic models and a sediment yield analysis, the sediment transport modeling of the river was conducted.  Though a very comprehensive study, the conclusions were to improve the model by collecting more data for calibration, “Sediment inflow to the Santa Margarita River is perhaps the biggest unknown factor influencing results.  To improve estimates of sediment entering the river, sediment-gauging stations need to be established.  Ata minimum a suspended sediment  gauging program should be established at the gorge jus below the confluence of Murrieta and Temecula Creek….” (p.6-16, 6-17).  Inconclusive study to determine impairment and/or extent of impairment due to sediment.  Also during conversations with L. Carlson & L. McKinney at Camp Pendleton both indicated nee for data to calibrate and need to look at habitat health related to sediment issues.8,10

Extent of Impairment (or Extent of Attainment)

??10.4 miles

#### Potential Sources

Unknown - development/urban runoff? agricultural activities? natural sources?  imported water supply?

#### TMDL Priority

Medium – There are current and extensive efforts underway to address water quality issues in the Santa Margarita Watershed.6  Various stakeholders have recognized and identified some existing and potential water quality issues and are attempting to gain a better understanding, conduct more monitoring, target pollutant sources and develop comprehensive management strategies.  TMDLs would provide the legal framework necessary to address some of these problems and could assist in this coordinated effort and be a major component of this work.8, 9

###### NOTES

TDS may consist of carbonates, bicarbonates, chlorides, sulfates, phosphates, nitrates, magnesium, sodium, iron and manganese.  The most frequent constituents are usually salts (sodium, chloride, boron, etc.)  Most of the problem can be traced to human impacts, and therefore, can be cleaned up. Geologic conditions help to define the natural levels of many of these constituents. High TDS concentrations may be expected to impair the MUN beneficial use. High concentrations of TDS are also expected to impact the AGR beneficial use directly through irrigation waters or indirectly through adverse effects on soil permeability.  TDS values between 450 to 2000 mg/L are expected to have a slight to moderate restriction on use of waters for irrigation of crops1.  Three of nine samples exceeded the Basin Plan Standard of 750 mg/L.1  Further downstream, the Basin Plan Standard of 500 mg/L for the Santa Margarita Estuary was exceeded in seven of eight samples.1

###### References

1 Water Quality Water Quality Control Plan for the San Diego Basin – Region 9. CA RWQCB, San Diego Region. September 1994, amended Oct. 1994 and May 1998.

2 Final Report of Water Quality Studies & Proposed Watershed Monitoring Program for Portions of San Mateo & Santa Margarita River Watershed. Marine Corps Base, Camp Pendleton, CA. Contract No. N68711-95-D-7573, D.O. 0021.

3 Santa Margarita Watershed Sampling. 	San Diego Regional Water Quality Control Board, L. Pardy. June 1998.

4 Preliminary Summary of 1997 Data Organic Chemicals in Fish & Clams. Toxic Substance Monitoring Program, SWRCB DWQ, D. Rassmussen. 1997.

5 Preliminary Summary of 1999 Data Organic Chemicals in Fish & Clams. Toxic Substance Monitoring Program, SWRCB DWQ, D. Rassmussen. 1999.

6 DRAFT Framework Monitoring Plan for the Santa Margarita Watershed California. US Bureau of Reclamation. CDM Federal Corp., Boyle Engr. Corp. RECON.  Feb 2001.

7 CA Code of Regulations, Title 22, MCLs & secondary MCLs.

8 Final Report Santa Margarita River Hydrology, Hydraulics and Sedimentation Study. Prepared for USACOE, LA District Contract DACW09-97-D-0022, prepared by WEST Consultants, Inc. July 2000.

9 Personal communications K. Cole with L. McKinney of Camp Pendleton 5/7/0, 5/9/01. Personal communication K. Cole with L. Carlson of Camp Pendleton 5/15/01.

10 Water Resources Management in the Santa Margarita River, Water Quality Monitoring and Water Management. Camp Pendleton & Camp Dresser & McKee, SDRWQCB presentation made on 5/9/01. April 2001