Long Canyon Creek (911.600) – 303(d) Fact Sheet

City of San Diego Data

This data does not lead to a listing recommendation.

## Watershed Characteristics

Long Canyon Creek is a 6.60-mile water body in the Tijuana Watershed of Region 9.  It is classified in land water with the following beneficial uses: MUN, AGR, IND, PROC, FRESH, REC1, REC2, WARM and WILD1.

# Water Quality Objectives not Obtained

The Region 9 Basin Plan Standard1 for total dissolved solids (TDS) was the only objective not obtained.

# Evidence of Impairment

One of four sampling events had high levels of TDS for all 6 samples.  See Table 1 for standards and exceedances.

# Extent of Impairment

Station LCC2 was sampled four times from 12 Mar 97 to 29 Jan 98.  The extent of impairment for the entire creek cannot be adequately assessed from one sampling station.

# Potential Sources

The high levels of TDS only occurred in the January sampling, possible indicative of a storm that increased natural erosion processes.

# TMDL Priority

No TMDL is required at this time.

# Notes

No standard could be found for Dissolved Oxygen percent saturation.  Values never dipped below 75% and the corresponding measure of Dissolved Oxygen as milligrams per liter was never below the standard.  No standard could be found for temperature, but the stream did seem to follow a natural, seasonal pattern.  No standard could be found for Oxidative Reduction Potential (ORP).

# Information Sources

1 Water Quality Control Plan for the San Diego Basin (9), 1994