San Diego River

Hydrologic Subareas 9073.11 and 907.12

Potential 303(d) Listing: Total Nitrogen

(The original analysis of Padre Dam data compared total nitrogen concentrations only against the drinking water standard of 10mg/L.  Based on that analysis, total nitrogen was not recommended for addition to the 303(d) list during the 2002 update.)  At this time, a recommendation for or against listing is not being made.  The 2002 list has been accepted by the USEPA and listing guidance is expected to be available before the next update to section 303(d).

# Water Quality Objectives Not Attained

Total Nitrogen		The Basin Plan states that “Inland surface waters…shall not contain biostimulatory substances in concentrations that promote aquatic growth to the extent that such growths cause nuisance or adversely affect beneficial uses.“  Additionally, threshold phosphorus levels shall not exceed 0.025 mg/L in any standing body of water.1  Phosphorus concentrations shall not exceed 0.01 mg/L in any flowing body of water.  Analogous threshold values for nitrogen compounds have not been set, however; it is stated that a ratio of N:P=10:1 shall be used.  In the case of a standing body of water, the threshold nitrogen level is therefore set at 0.25 mg/L.  For flowing water, the nitrogen level is set at 1.0g/L.  These objectives are not to be exceeded more than 10% of the time during any one-year period.

# EVIDENCE OF IMPAIRMENT

Total Nitrogen		Data from Padre Dam spanning from June 1997 to September 2002 indicates potential impairment at multiple sites along the lower San Diego River (see table below).  It should be noted that most of the stations showed a general decrease in total nitrogen concentrations over this time.  See associated tables at: S:\WQS\303dlist\Electronic data files\Padre Dam\San Diego River N&P.xls

Water Quality Control Plan for the San Diego Basin (9), California Regional Water Quality Control Board, San Diego Region, September 1994.

1