Prepared by Brennan Ott

7/26/01

Rattlesnake Creek

906.200

It is recommended that Rattlesnake Creek not be listed.

# Watershed Characteristics

Rattlesnake Creek is a tributary of the Los Penasquitos Creek, located within Los Penasquitos Creek Watershed. Rattlesnake Creek is 6 miles long.  Uses for the creek include water supply for agricultural and industrial service.  It is used for contact and non-contact recreation, wildlife habitat, and a warm and cold freshwater habitat as well.

Water Quality Objectives not Obtained

Rattlesnake Creek was found to be in violation of inland surface water quality standards for total dissolved solids (TDS).

Evidence of Impairment

Rattlesnake Creek was sampled on 6/3/98.  It had a TDS concentration of 1412 mg/l, clearly surpassing the standard of 500 mg/l for TDS.

## Biological assessment data is also available.  The area sampled consisted of 5 riffles adjacent to Hillary Park.  Bio-assessments were conducted in May, September and November 1998 and May 1999.  Results from these bio-assessments indicate that the distribution (richness) of taxa between September and November is relatively the same, while May showed a decrease.  However, in May, the site displayed a greater abundance than the September and November samples.  November and September have similar levels of functional feeding groups whereas May exhibited a decrease in functional feeders from these two months.  At all sampling times, the site was dominated by a few disturbance tolerant insect and worm taxa and the functional feeding groups were composed mainly of collectors, filterers and grazers.  Shredders were rare.  The physical habitat quality at all three sampling periods was fair to good.

Extent of Impairment

Rattlesnake Creek was sampled at Hilleary Park, off Community Road.  Since there is only one water quality data point for this Creek, not much can be said about its impairment.

Potential Sources

There is not sufficient data to support any hard claims as to the source(s) of the pollution, but there is enough to warrant good assumptions.

Since the sample site is near a park and a road, possible high TDS concentrations may due to urban runoff from the road.

TMDL Priority

It is recommended that Rattlesnake Creek not be placed on the TMDL list.

# Source References

Water quality standards were taken from the Water Quality Control Plan for the San Diego Basin.  Bio-assessment data taken from San Diego Regional Water Quality Control Board: 1999 Biological Assessment Annual Report.  TDS data came from Linda Pardy sheet 1.