Prepared by Brennan Ott

7/27/01

Buena Vista Creek

904.320

It is recommended that Buena Vista Creek not be listed.

# Watershed Characteristics

Buena Vista Creek is a coastal stream, 5 miles long.  Uses for the creek include water supply for agricultural and industrial process.  It is used for contact and non-contact recreation, wildlife habitat, and a warm freshwater habitat as well.

Water Quality Objectives not Obtained

Buena Vista Creek was found to be in violation of inland surface water quality standards for total dissolved solids (TDS), chloride and sulfate.

Evidence of Impairment

All dates and locations that the above said violations took place and the concentrations of the pollutants are summarized in the following table.  The standard set by the Basin Plan for inland surface waters for TDS is 500 mg/l and 250 mg/l for both sulfate and chloride.  The water in the Buena Vista Creek is in clear violation of these standards.

| Date | Station ID | Location | Constituent | Value (mg/l) |
| --- | --- | --- | --- | --- |
| 5/20/98 | DFG-978-301 | S. Vista Way | TDS | 1378 |
| 6/10/98 | DFG-978-329 | Wildwook park | TDS | 1360 |
| 6/29/98 | LLP-978-405-BUV | B.V. Creek | TDS | 1133 |
| 6/29/98 | LLP-978-405-BUV | B.V. Creek | Chloride | 454 |
| 6/29/98 | LLP-978-405-BUV | B.V. Creek | Sulfate | 281 |

## Bio-assessments were conducted in May, September, and November 1998 and May 1999.  Results from these bio-assessments indicate that the distribution (richness) of taxa between September and November is relatively the same, while May showed a decrease.  However, in May, the site displayed a greater abundance than the September and November samples.  November and September have similar levels of functional feeding groups whereas May exhibited a decrease in functional feeders from these two months.  At all sampling times, the site was dominated by a few disturbance tolerant insect and worm taxa and the functional feeding groups were composed mainly of collectors, filterers and grazers.  Shredders were rare.  The physical habitat quality at all three sampling periods was fair to good.  Bio-assessment data for Buena Vista Creek was sampled 5 riffles upstream of South Vista Way

Extent of Impairment

Since there is only three data points for this Creek all occurring within a month of each other at different sites, not much can be said about its impairment.  TDS was found to be high every time on these three days.  Chloride and sulfate were only sampled for once, both being in exceedance on that day.

Potential Sources

There is not sufficient data to support any hard claims as to the source(s) of the pollution, but there is enough to warrant good assumptions.

Possible high TDS, chloride, and sulfate concentrations may due to urban runoff or illegal dumping.

TMDL Priority

It is recommended that Buena Vista Creek not be placed on the TMDL list.

# Source References

Water quality standards were taken from the Water Quality Control Plan for the San Diego Basin.   Water Quality data from Linda Pardy Sheet 1.  Biological assessment data taken from San Diego Regional Water Quality Control Board: 1999 Biological Assessment Annual Report.