WATER AND SEDIMENT SAMPLES

COLLECTED AT

ORANGE COUNTY NPDES MONITORING STATIONS

Table 1.  EPA_OCPFRD Sampling  &  NPDES Station information

| Station Location | Station | Date | Time | Comments: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Costa Mesa Channel | CMCG02* | 08/28/01 | 1244 | water split-sample for dissolved metals only |  |  |  |
| Upper Newport Bay | UNBNSB | 08/30/01 | 919 | water split-sample for dissolved metals only |  |  |  |
| San Diego Creek | SDMF05 | 2/10/02 | 950 | water split-sample for dissolved metals only |  |  |  |
| Upper Newport Bay | UNBNSB | 2/10/02 | 1030 | water split-sample for dissolved metals only |  |  |  |
| Upper Newport Bay | UNBSDC | 10/29/02 | 1045 | Split sample, water = metals, sediment = organics |  |  |  |
| Upper Newport Bay | UNBNSB | 10/29/02 | 1120 | Split sample, water = metals, sediment = organics |  |  |  |
| Lower Newport Bay | LNBHIR | 10/29/02 | 1230 | Split sample for sediment = organics; only EPA for water = metals |  |  |  |
| Lower Newport Bay | UNBCHB | 10/29/02 | 1315 | only EPA for water = metals and sediment = organics |  |  |  |
| San Diego Creek | BARSED | 10/29/02 | 1455 | Split sample, water = metals, sediment = organics |  |  |  |
| San Diego Creek | WYLSED | 10/29/02 | 1515 | EPA only for water = metals |  |  |  |
| San Diego Creek | CICF25 | 10/29/02 | 1530 | Split sample, water = metals, sediment = organics |  |  |  |
|  | # Salinity | type |  | Analyses requested |  |  |  |
| QC Reference--A | 35 ppt | Coastal seawater |  | Dissolved metals only, QC evaluation |  |  |  |
| QC Reference--B | 15 ppt | Estuarine water |  | Dissolved metals only, QC evaluation |  |  |  |

*analytical results from this CMCG02 sample reported only for QC purposes

| COMPARISON OF DISSOLVED METAL RESULTS FOR AMBIENT FRESHWATER SPLIT- SAMPLES |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SAMPLE | SAMPLE | SAMPLE | DATE | Cd | Cr | Cu | Pb | Ni | Ag | Zn |
| SITE | I.D. | MATRIX | ANALYZED | ug/L | ug/L | ug/L | ug/L | ug/L | ug/L | ug/L |
| CMCG02--PFRD | WR41385 | ambient FW | 8/28/2001 | <1.0 | <8.0 | 12.0 | <2.0 | <4.0 | <2.0 | 21.0 |
| CMCG02--EPA | WR41385 | ambient FW | 8/28/2001 | 0.100 | 2.000 | 9.30 | 0.800 | 5.20 | <0.005 | 22.5 |
| SDMF05-PFRD | WR45281 | ambient FW | 2/10/2002 | <1.0 | <8 | 8.1 | <2.0 | <4.0 | <2.0 | 13.0 |
| SDMF05-EPA | WR45281 | ambient FW | 2/10/2002 | <0.005 | 3.030 | 12.10 | <0.005 | 3.90 | <0.005 | 10.6 |
| WYLSED-PFRD | WR51446 | ambient FW | 10/29/2002 | <1.0 | <8 | 2.7 | <2.0 | <4.0 | <2.0 | <20 |
| WYLSED-EPA | 84861 | ambient FW | 10/29/2002 | <0.05 | 1.97 | 1.66 | <0.5 | 3.19 | <0.1 | 4.42 |
| OPFRD LAB: WECK PREP: EPA 200.2 METHOD: EPA 200.8 DILUTION FACTOR: 1 |  |  |  |  |  |  |  |  |  |  |
| EPA laboratory: CRG Labs for ambient water samples METHOD: EPA 1640 |  |  |  |  |  |  |  |  |  |  |

| COMPARISON OF DISSOLVED METAL RESULTS FOR AMBIENT SALTWATER SPLIT- SAMPLES |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SAMPLE | SAMPLE | SAMPLE | DATE | Cd | Cr | Cu | Pb | Ni | Ag | Zn |
| SITE | I.D. | MATRIX | ANALYZED | ug/L | ug/L | ug/L | ug/L | ug/L | ug/L | ug/L |
| UNBNSB--PFRD | WR41439 | ambient SW | 8/30/2001 | <1.0 | 16.0 | 4.9 | <2.0 | 28.0 | <2.0 | 19.0 |
| UNBNSB--EPA | WR41439 | ambient SW | 8/30/2001 | 0.284 | <0.005 | 3.44 | 0.123 | 1.25 | <0.005 | 9.2 |
| UNBNSB-PFRD | WR45280 | ambient SW | 2/10/2002 | <1.0 | <8 | <2 | <2.0 | <4 | <2.0 | 23.0 |
| UNBNSB-EPA | WR45280 | ambient SW | 2/10/2002 | 0.690 | 0.500 | 2.74 | 0.025 | 1.01 | <0.005 | 11.9 |
| UNBSDC-PFRD | WR51445 | ambient SW | 10/29/2002 | <1.0 | <8 | 5.6 | <2.0 | 11 | <2.0 | 18 |
| UNBSDC-EPA | 8482 | ambient SW | 10/29/2002 | 0.095 | 0.46 | 3.16 | 0.161 | 0.81 | <0.005 | 6.8 |
| OPFRD LAB: WECK PREP: EPA 200.2 METHOD: EPA 200.8 DILUTION FACTOR: 1 |  |  | EPA laboratory: CRG Labs for ambient water samples METHOD: EPA 1640 |  |  |  |  |  |  |  |

| ## COMPARISON OF DISSOLVED METAL RESULTS FOR STANDARD REFERENCE MATERIALS |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SAMPLE | SAMPLE | SAMPLE | DATE | Cd | Cr | Cu | Pb | Ni | Ag | Zn |
| SITE | I.D. | MATRIX | ANALYZED | ug/L | ug/L | ug/L | ug/L | ug/L | ug/L | ug/L |
| PFRD result (Sample A) | WR41447 | SW | 9/10/2001 | <1.0 | 13.0 | <2.0 | <2.0 | 25.0 | <2.0 | <10.0 |
| EPA SRM coastal seawater |  | SW | 9/10/2001 | 0.030 | 0.092 | 0.517 | 0.012 | 0.386 | n/a | 1.240 |
| PFRD result (Sample B) | WR41448 | EW | 9/10/2001 | <1.0 | 9.9 | 5.0 | <2.0 | 14.0 | <2.0 | <10.0 |
| EPA SRM SF Bay estuarine |  | EW | 9/10/2001 | 0.048 | 0.183 | 1.550 | 0.009 | 1.230 | 0.003 | 0.201 |
| EPA SRM = Standard Reference Material supplied by National Reseach Council Canada (NRCC) |  |  |  |  |  |  |  |  |  |  |
| Sample A is CASS-3 = Coastal Atlantic nearshore seawater |  |  |  |  |  |  |  |  |  |  |
| Sample B is SLEW-3 = San Francisco Bay estuarine water |  |  |  |  |  |  |  |  |  |  |
| SW = sea water; EW = estuarine water |  |  |  |  |  |  |  |  |  |  |
| OPFRD LAB: WECK PREP: EPA 200.2 METHOD: EPA 200.8 DILUTION FACTOR: 1 |  |  | EPA laboratory: CRG Labs for ambient water samples METHOD: EPA 1640 SRM value shown is actual value from NRCC not measured by CRG Labs. |  |  |  |  |  |  |  |