NEWPORT BAY SEDIMENT TOXICITY IDENTIFICATION/EVALUATION  STUDY

(INCLUDING SEAWATER SAMPLES)

### Newport Bay Sample Collection and Handling

Water column samples for toxicity testing, metals and organics analyses were collected using an ISCO pump from a depth of 2 to 3 meters, using EPA suggested clean techniques.  Organics analysis samples were collected unfiltered and were stored in 1 gallon amber bottles at 5 C until tested.  Samples for dissolved metals were passed through a 0.45 µm filter attached to the pump.  Samples for total metals and methyl mercury were collected unfiltered.  All water column samples were stored at 5 C and transported to the chemistry laboratory for analysis within 24 hrs of collection.

Sediment samples were collected using a Van Veen grab.  The top 2 cm from multiple grabs were homogenized together.  Subsamples for sediment chemistry and whole sediment toxicity were taken from the homogenized composite sample.  The samples for chemistry were frozen at –20 C until analyzed.  Samples for sediment toxicity and grain size were stored at 5 C until analyzed.  For the November 2001 sediment-water interface sample collection, core samples were taken from a grab by manually pressing a plastic core tube into the sediment so that an undisturbed sample was obtained.  The depth of sediment in the core tubes was at least 5 cm.  Ten cores were collected from each station.  The cores were stored at 15 C with overlying water.  In the laboratory, during the course of toxicity testing, overlying water (representing the sediment-water interface) was removed from sediment cores and transferred to clean plastic containers.  The overlying water samples were transported to the chemistry laboratory with 24 hrs for analysis of dissolved metals.

San Diego Creek Sampling

For wet weather, samples were collected from the Campus Drive bridge.  To take the samples, a metal davit was attached to the bridge.  From the davit a torpedo sampler was lowered to the creek.  Inside the sampler was a 1 gallon glass bottle.  At each sampling period, multiple drops were made and the water was transferred to a 5 gallon polycarbonate bottle until it was full.  Samples for all analyses were then aliquoted from this composite.  Samples for dissolved metals were taken from the composite by an ISCO sampler with a 0.45 micron filter attached using EPA suggested clean sampling techniques.  Samples for total and dissolved metals analysis were transferred to cleaned plastic 500 ml or 1 liter bottles.  Total suspended solids aliquots were put into 1 liter plastic bottles.

For dry weather, samples were collected directly from the creek bed at the base of the bridge using an ISCO sampler.  For each collection event, a sample was taken in the morning and then another was taken 4 to 5 hours later.  The samples for metals analysis went straight from the creek into their respective containers (either through the filter or not).  The remaining samples were taken from a composite made in a 5 gallon polycarbonate bottle.  All sample containers were the same as described above.

All samples were stored on ice until transported back to SCCWRP where they were stored at 5 C.  Samples for chemical analysis were delivered to the chemistry lab within 24 hrs of collection.

Figure 1.  Map of all sampling locations in Newport Bay and San Diego Creek.

Table 1.  Station information.

| Station Location | Station | Date | Time | Depth (m) | Lat Degree | Lat Min. | Long Degree | Long Min. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Upper Newport Bay | NB10 | 11/28/01 | 919 | 4.27 | 33 | 38.973 | -117 | 52.255 |
| Rhine Channel | NB3 | 11/28/01 | 1244 | 3.05 | 33 | 36.72 | -117 | 55.67 |
| Upper Newport Bay | NB10 | 3/12/02 | 845 | 4.27 | 33 | 38.974 | -117 | 52.262 |
| Upper Newport Bay | NB10-B | 3/12/02 | 1000 | 3.96 | 33 | 38.979 | -117 | 52.246 |
| Upper Newport Bay | NB10-C | 3/12/02 | 1040 | 4.27 | 33 | 38.959 | -117 | 52.301 |
| Rhine Channel | NB3 | 3/12/02 | 1205 | 3.96 | 33 | 36.73 | -117 | 55.675 |
| Rhine Channel | NB11 | 3/12/02 | 1347 | 3.96 | 33 | 36.625 | -117 | 55.609 |
| Rhine Channel | NB12 | 3/12/02 | 1318 | 4.27 | 33 | 36.782 | -117 | 55.673 |
| Lower Newport Bay | 2136 | 5/14/02 | 1239 | 6.40 | 33 | 37.136 | -117 | 55.633 |
| Lower Newport Bay | 2137 | 5/14/02 | 1207 | 3.90 | 33 | 36.779 | -117 | 55.438 |
| Lower Newport Bay | 2142 | 5/14/02 | 1224 | 3.50 | 33 | 36.46 | -117 | 54.603 |

Table 2.  Sample analyses completed by station number.

| Station Number | Date | Matrix | Metals Total | Metals Dissolved | AVS-SEM | Chlorinated Pesticides Total | Chlorinated Pesticides Particulate | OP Pesticides Total | OP Pesticides Particulate | PAH | PCB Total | PCB Particulate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NB3 | 11/21/01 | Sediment | x |  | x | x |  |  |  | x | x |  |
| NB10 | 11/21/01 | Sediment | x |  | x | x |  |  |  | x | x |  |
| NB3 | 11/21/01 | Sed/Water Interface |  | x |  |  |  |  |  |  |  |  |
| NB10 | 11/21/01 | Sed/Water Interface |  | x |  |  |  |  |  |  |  |  |
| NB3 | 11/21/01 | Water | x | x |  |  |  |  |  |  |  |  |
| NB10 | 11/21/01 | Water | x | x |  |  |  |  |  |  |  |  |
| NB10 | 3/12/02 | Water | x | x |  | x |  |  |  |  | x |  |
| NB3 | 3/12/02 | Water | x | x |  | x |  |  |  |  | x |  |
| NB10 | 3/12/02 | Sediment | x |  | x | x |  |  |  | x | x |  |
| NB10-B | 3/12/02 | Sediment |  |  | x | x |  |  |  | x | x |  |
| NB10-C | 3/12/02 | Sediment |  |  | x | x |  |  |  | x | x |  |
| NB3 | 3/12/02 | Sediment | x |  |  | x |  |  |  | x | x |  |
| NB11 | 3/12/02 | Sediment |  |  |  | x |  |  |  | x | x |  |
| NB12 | 3/12/02 | Sediment |  |  |  | x |  |  |  | x | x |  |
| 2136 | 5/14/02 | Sediment | x |  |  | x |  |  |  | x | x |  |
| 2137 | 5/14/02 | Sediment | x |  |  | x |  |  |  | x | x |  |
| 2142 | 5/14/02 | Sediment | x |  |  | x |  |  |  | x | x |  |
| SDC | 3/7/02 | Water | x | x |  | x |  | x |  |  | x |  |
| SDC | 5/2/02 | Water | x | x |  | x |  | x |  |  | x |  |
| SDC | 8/12/02 | Water | x | x |  | x |  | x |  |  | x |  |
| SDC | 11/8/02 | Water | x | x |  | x | x | x | x |  | x | x |

NEWPORT BAY SEDIMENT AND WATER November 2001

Table 3.  Newport Bay sediment metals concentrations from November 2001 sampling.  Concentrations are in mg/dry kg.

| Constituent | MDL | NB3 | NB10 |
| --- | --- | --- | --- |
| Aluminum (Al) | 1 | 30450 | 46400 |
| Antimony (Sb) | 0.05 | 0.755 | 0.820 |
| Arsenic (As) | 0.05 | 8.61 | 6.13 |
| Barium (Ba) | 0.05 | 80.9 | 160 |
| Beryllium (Be) | 0.01 | 0.560 | 0.785 |
| Cadmium (Cd) | 0.01 | 0.505 | 1.53 |
| Chromium (Cr) | 0.05 | 38.8 | 41.2 |
| Cobalt (Co) | 0.01 | 5.70 | 8.36 |
| Copper (Cu) | 0.01 | 540 | 38.5 |
| Iron (Fe) | 1 | 27950 | 33100 |
| Lead (Pb) | 0.01 | 57.0 | 15.8 |
| Manganese (Mn) | 0.05 | 200 | 326 |
| Mercury (Hg) | 0.005 | 4.95 | 0.24 |
| Molybdenum (Mo) | 0.05 | 4.71 | 2.90 |
| Nickel (Ni) | 0.01 | 15.1 | 19.6 |
| Selenium (Se) | 0.05 | 1.28 | 1.75 |
| Silver (Ag) | 0.01 | 0.30 | 0.35 |
| Strontium (Sr) | 0.05 | 90.2 | 86.0 |
| Thallium (Tl) | 0.01 | 0.27 | 0.40 |
| Tin (Sn) | 0.05 | 7.20 | 2.87 |
| Titanium (Ti) | 0.05 | 1505 | 2270 |
| Vanadium (V) | 0.05 | 71.1 | 95.7 |
| Zinc (Zn) | 0.05 | 238 | 160 |

Table 4.  Newport Bay sediment simultaneously extracted metals (SEM) and acid volatile sulfide (AVS) values from samples collected in November 2001.  Method detection limit for Cd, Cu, Pb and Ni is 0.01 mg/dry mg and is 0.05 mg/dry mg for Zn.

| Constituent | NB3 (moles/g) | NB3 (mg/ dry kg) | NB10 (moles/g) | NB10 (mg/ dry kg) |
| --- | --- | --- | --- | --- |
| SEM |  |  |  |  |
| Cadmium | 0.00126 | 0.142 | 0.00512 | 0.575 |
| Copper | 0.00464 | 0.295 | 0.000755 | 0.048 |
| Lead | 0.0516 | 10.7 | 0.0151 | 3.12 |
| Nickel | 0.0125 | 0.732 | 0.0273 | 1.6 |
| Zinc | 1.22 | 79.7 | 0.489 | 32.0 |
| Total SEM | 1.29 | 91.5 | 0.54 | 37.3 |
| AVS | 60.2 | 1930 | 54.9 | 1760 |

Table 5.  Newport Bay sediment PCB congener concentrations from November 2001 sampling.  Concentrations are in g /dry kg.

| Compound | NB3 | NB10 |
| --- | --- | --- |
| PCB018 | 1.11 | <1.0 |
| PCB028 | 3.36 | <1.0 |
| PCB031 | 5.55 | <1.0 |
| PCB033 | 5.66 | <1.0 |
| PCB037 | <1.0 | <1.0 |
| PCB044 | 8.53 | <1.0 |
| PCB049 | 12.6 | <1.0 |
| PCB052 | 8.99 | <1.0 |
| PCB066 | 10.8 | <1.0 |
| PCB070 | 8.68 | <1.0 |
| PCB074 | 5.1 | <1.0 |
| PCB077 | <1.0 | <1.0 |
| PCB081 | <1.0 | <1.0 |
| PCB087 | 4.04 | <1.0 |
| PCB095 | 5.53 | <1.0 |
| PCB097 | 2.33 | <1.0 |
| PCB099 | 7.07 | <1.0 |
| PCB101 | 9.98 | <1.0 |
| PCB105 | 11.2 | <1.0 |
| PCB110 | 11.8 | <1.0 |
| PCB114 | <1.0 | <1.0 |
| PCB118 | 12.3 | <1.0 |
| PCB119 | <1.0 | <1.0 |
| PCB123 | 1.73 | <1.0 |
| PCB126 | <1.0 | <1.0 |
| PCB128 | <1.0 | <1.0 |
| PCB138 | 3.58 | <1.0 |
| PCB141 | <1.0 | <1.0 |
| PCB149 | 4.45 | <1.0 |
| PCB151 | <1.0 | <1.0 |
| PCB153 | 8.29 | <1.0 |
| PCB156 | <1.0 | <1.0 |
| PCB157 | <1.0 | <1.0 |
| PCB158 | 3.51 | <1.0 |
| PCB167 | <1.0 | <1.0 |
| PCB168/132 | <1.0 | <1.0 |
| PCB169 | <1.0 | <1.0 |
| PCB170 | <1.0 | <1.0 |
| PCB177 | <1.0 | <1.0 |
| PCB180 | 1.12 | <1.0 |
| PCB183 | <1.0 | <1.0 |
| PCB187 | 1.24 | <1.0 |
| PCB189 | <1.0 | <1.0 |
| PCB194 | <1.0 | <1.0 |
| PCB200 | <1.0 | <1.0 |
| PCB201 | <1.0 | <1.0 |
| PCB206 | <1.0 | <1.0 |
| Total Detectable PCBs | 158 | <1.0 |

Table 6.  Newport Bay sediment PAH concentrations from November 2001 sampling.  Concentrations are in g/dry kg.  Method detection limit for all constituents is 1 g/dry kg.

| Compound | NB3 | NB10 |
| --- | --- | --- |
| Naphthalene | 9.2 | 8.6 |
| 2-Methylnaphthalene | 7.0 | 5.6 |
| 1-Methylnaphthalene | 4.2 | 2.8 |
| Biphenyl | 3.2 | 2.0 |
| 2,6-Dimethylnaphthalene | 4.8 | 6.8 |
| Acenaphthene | 7.4 | 1.8 |
| Acenaphthylene | 12.3 | 4.7 |
| 2,3,5-Trimethylnaphthalene | 1.0 | 1.8 |
| Fluorene | 8.4 | 6.0 |
| Phenanthrene | 120 | 42.5 |
| Anthracene | 42.1 | 9.0 |
| 1-Methylphenanthrene | 12.5 | 10.6 |
| Fluoranthene | 322 | 124 |
| Pyrene | 330 | 152 |
| Benz[a]anthracene | 125 | 45.4 |
| Chrysene | 211 | 93.6 |
| Benzo[b]fluoranthene | 183 | 60.0 |
| Benzo[k]fluoranthene | 34.7 | 13.8 |
| Benzo[e]pyrene | 132 | 61.6 |
| Benzo[a]pyrene | 153 | 48.3 |
| Perylene | 34 | 32.6 |
| Indeno[1,2,3-c,d]pyrene | 91.1 | 37.4 |
| Dibenz[a,h]anthracene | 30 | 11.6 |
| Benzo[g,h,i]perylene | 90.9 | 65.0 |
| Total Detectable PAHs | 1970 | 847 |

Table 7.  Newport Bay sediment pesticides from November 2001 sampling.  Concentrations are in ug/dry kg.  Method detection limits for all constituents are 1 g /dry kg, except toxaphene which is 10 g/dry kg.

| Compound | NB3 | NB10 |
| --- | --- | --- |
| Toxaphene | <10 | <10 |
| Aldrin | <1.0 | <1.0 |
| BHC-alpha | <1.0 | <1.0 |
| BHC-beta | <1.0 | <1.0 |
| BHC-delta | <1.0 | <1.0 |
| BHC-gamma | <1.0 | <1.0 |
| Chlordane-alpha | <1.0 | <1.0 |
| Chlordane-gamma | <1.0 | <1.0 |
| Dieldrin | <1.0 | <1.0 |
| Endosulfan Sulfate | <1.0 | <1.0 |
| Endosulfan-I | <1.0 | <1.0 |
| Endosulfan-II | <1.0 | <1.0 |
| Endrin | <1.0 | <1.0 |
| Endrin Aldehyde | <1.0 | <1.0 |
| Heptachlor | <1.0 | <1.0 |
| Heptachlor Epoxide | <1.0 | <1.0 |
| Methoxychlor | <1.0 | <1.0 |
| Mirex | <1.0 | <1.0 |
| trans-Nonachlor | <1.0 | <1.0 |
| 2,4'-DDD | 4.4 | 6.25 |
| 2,4'-DDE | <1.0 | <1.0 |
| 2,4'-DDT | <1.0 | <1.0 |
| 4,4'-DDD | <1.0 | 8.6 |
| 4,4'-DDE | 31.7 | 61.4 |
| 4,4'-DDT | <1.0 | <1.0 |
| Total Detectable DDTs | 36.1 | 76.3 |

Table 8.  Water metals concentrations from November 2001 sampling.  Values are from water column and sediment-water interface samples.  All concentrations are in g/L.

Water Column			SWI

NB3			NB10			Dissolved

| Metal | MDL | Total | Dissolved | Total | Dissolved | NB3 | NB10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aluminum | 0.01 | 71.0 | 10.0 | 533 | 9.87 | 4.51 | 4.15 |
| Antimony | 0.01 | 0.165 | 0.207 | 0.165 | 0.251 | 0.276 | 0.419 |
| Arsenic | 0.01 | 1.71 | 1.72 | 2.07 | 1.80 | 1.33 | 1.70 |
| Beryllium | 0.005 | <0.005 | <0.005 | <0.005 | <0.005 | <0.005 | <0.005 |
| Cadmium | 0.005 | 0.151 | 0.183 | 0.124 | 0.116 | 0.298 | 0.168 |
| Chromium | 0.005 | 0.460 | 0.448 | 1.15 | 0.272 | 0.197 | 0.225 |
| Cobalt | 0.005 | <0.005 | <0.005 | 0.123 | 0.005 | <0.005 | 0.141 |
| Copper | 0.005 | 12.4 | 10.8 | 2.02 | 1.31 | 5.78 | 0.519 |
| Iron | 0.01 | 42.0 | 3.41 | 237 | 9.15 | 16.6 | 47.6 |
| Lead | 0.005 | 0.168 | 0.053 | 0.266 | 0.051 | 0.162 | 0.040 |
| Manganese | 5 | 23.4 | 22.8 | 178 | 155 | 21.1 | 232 |
| Mercury | 0.005 | <0.005 | <0.005 | <0.005 | <0.005 | <0.005 | <0.005 |
| Molybdenum | 0.005 | 11.6 | 11.8 | 11.8 | 12.0 | 13.1 | 11.9 |
| Nickel | 0.005 | 1.42 | 1.35 | 1.95 | 1.76 | 1.20 | 1.64 |
| Selenium | 0.01 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | 0.336 |
| Silver | 0.005 | <0.005 | <0.005 | <0.005 | <0.005 | <0.005 | <0.005 |
| Strontium | 0.01 | NA | NA | NA | NA | NA | NA |
| Thallium | 0.005 | <0.005 | <0.005 | <0.005 | <0.005 | 0.018 | 0.009 |
| Tin | 0.005 | 0.011 | <0.005 | <0.005 | <0.005 | 0.022 | <0.005 |
| Titanium | 0.005 | 4.06 | 0.495 | 23.7 | 0.627 | 0.208 | 0.392 |
| Vanadium | 0.005 | 3.62 | 3.58 | 5.05 | 3.74 | 2.05 | 3.55 |
| Zinc | 0.005 | 34.6 | 33.8 | 12.9 | 10.0 | 50.7 | 9.65 |

NA=Not analyzed.

NEWPORT BAY SEDIMENT AND WATER March 2002

Table 9.  Concentrations of sediment constituents in samples from the March 2002 Newport Bay sampling.

|  |  | Upper Bay |  |  |  | Rhine Channel |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Constituent | MDL | NB10 | NB10B | NB10C |  | NB3 | NB11 | NB12 |
| Metals (mg/dry kg) |  |  |  |  |  |  |  |  |
| Aluminum | 1 | 50700 | NA | NA |  | 39700 | NA | NA |
| Antimony | 0.05 | 0.845 | NA | NA |  | 0.675 | NA | NA |
| Arsenic | 0.05 | 7.14 | NA | NA |  | 10.2 | NA | NA |
| Barium | 0.05 | 184 | NA | NA |  | 125 | NA | NA |
| Beryllium | 0.01 | 0.925 | NA | NA |  | 0.810 | NA | NA |
| Cadmium | 0.01 | 1.90 | NA | NA |  | 0.635 | NA | NA |
| Chromium | 0.05 | 50.0 | NA | NA |  | 53.0 | NA | NA |
| Cobalt | 0.01 | 9.54 | NA | NA |  | 7.54 | NA | NA |
| Copper | 0.01 | 60.0 | NA | NA |  | 532 | NA | NA |
| Iron | 1 | 38850 | NA | NA |  | 36050 | NA | NA |
| Lead | 0.01 | 21.4 | NA | NA |  | 85.0 | NA | NA |
| Manganese | 0.05 | 351 | NA | NA |  | 251 | NA | NA |
| Mercury | 0.005 | 0.295 | NA | NA |  | 6.69 | NA | NA |
| Molybdenum | 0.05 | 3.31 | NA | NA |  | 4.71 | NA | NA |
| Nickel | 0.01 | 23.4 | NA | NA |  | 19.8 | NA | NA |
| Selenium | 0.05 | 2.36 | NA | NA |  | 1.52 | NA | NA |
| Silver | 0.01 | 0.405 | NA | NA |  | 0.39 | NA | NA |
| Strontium | 0.05 | 124 | NA | NA |  | 82.0 | NA | NA |
| Thallium | 0.01 | 0.44 | NA | NA |  | 0.37 | NA | NA |
| Tin | 0.05 | 3.74 | NA | NA |  | 9.66 | NA | NA |
| Titanium | 0.05 | 2340 | NA | NA |  | 1770 | NA | NA |
| Vanadium | 0.05 | 112 | NA | NA |  | 98.2 | NA | NA |
| Zinc | 0.05 | 219 | NA | NA |  | 294 | NA | NA |
| TOC (%Dry Wt) |  | 1.1 | 2.1 | 2.3 |  | 1.6 | 1.6 | 1.7 |

Table 10.  Sediment acid volatile sulfide (AVS) and simultaneously extracted metals (SEM) data for three upper bay stations from the March 2002 sampling.  Method detection limits for Cd, Cu, Pb, and Ni are 0.01 mg/dry g and 0.05 mg/dry g for Zn.

moles/dry g			mg/dry g

| Constituent | NB10 | NB10B | NB10C |  |  | NB10 | NB10B | NB10C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEM |  |  |  |  |  |  |  |  |
| Cadmium | 0.00512 | 0.0066 | 0.0056 |  | 0.543 |  | 0.742 | 0.630 |
| Copper | 0.00076 | 0.0001 | 0.0004 |  | 0.102 |  | 0.062 | 0.028 |
| Lead | 0.0151 | 0.0197 | 0.0153 |  | 3.96 |  | 4.08 | 3.16 |
| Nickel | 0.0273 | 0.0315 | 0.0291 |  | 1.35 |  | 1.85 | 1.71 |
| Zinc | 0.489 | 1.02 | 0.674 |  | 46.0 |  | 67.0 | 44.1 |
| Total SEM | 0.54 | 1.08 | 0.72 |  | 52.0 |  | 73.7 | 49.6 |
| AVS | 54.9 | 106 | 81.4 |  | 2140 |  | 3400 | 2610 |

Table 11.  Newport Bay sediment PCB congener concentrations from March 2002 sampling.  Concentrations are in g/dry kg.

|  | Upper Bay |  |  |  | Rhine Channel |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Compound | NB10 | NB10B | NB10C |  | NB3 | NB11 | NB12 |
| PCB018 | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| PCB028 | <1.0 | <1.0 | <1.0 |  | 3.63 | 3.94 | 0.93 |
| PCB031 | <1.0 | <1.0 | <1.0 |  | 5.72 | 6.07 | 1.56 |
| PCB033 | <1.0 | <1.0 | <1.0 |  | 5.3 | 3.58 | <1.0 |
| PCB037 | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| PCB044 | <1.0 | 3.38 | 1.61 |  | 6.07 | 9.83 | 2.77 |
| PCB049 | <1.0 | 1.74 | 0.93 |  | 14.6 | 12.3 | 5.2 |
| PCB052 | <1.0 | 1.08 | <1.0 |  | 8.55 | 8.81 | 3.39 |
| PCB066 | <1.0 | <1.0 | <1.0 |  | 12.2 | 14.8 | 9.83 |
| PCB070 | <1.0 | <1.0 | <1.0 |  | 8.44 | 8.6 | 7.08 |
| PCB074 | <1.0 | <1.0 | <1.0 |  | 4.92 | 8.81 | 7.34 |
| PCB077 | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| PCB081 | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| PCB087 | <1.0 | <1.0 | <1.0 |  | 3.4 | 3.05 | 4.19 |
| PCB095 | <1.0 | <1.0 | <1.0 |  | 4.16 | 5.07 | 3.8 |
| PCB097 | <1.0 | <1.0 | <1.0 |  | 1.03 | 3.84 | 4.51 |
| PCB099 | <1.0 | <1.0 | <1.0 |  | 8.11 | 8.68 | 5.85 |
| PCB101 | <1.0 | <1.0 | <1.0 |  | 10.4 | 11.1 | 9.08 |
| PCB105 | <1.0 | <1.0 | <1.0 |  | 7.37 | 4.11 | 4.02 |
| PCB110 | <1.0 | 1.2 | 1.75 |  | 9.6 | 10.2 | 9.7 |
| PCB114 | <1.0 | <1.0 | <1.0 |  | 4.54 | 3.64 | <1.0 |
| PCB118 | <1.0 | <1.0 | <1.0 |  | 12.5 | 13.9 | 8.98 |
| PCB119 | <1.0 | <1.0 | <1.0 |  | <1.0 | 0.7 | 0.75 |
| PCB123 | <1.0 | <1.0 | <1.0 |  | 2.84 | 2.09 | 6.8 |
| PCB126 | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| PCB128 | <1.0 | <1.0 | <1.0 |  | 1.08 | 1.93 | <1.0 |
| PCB138 | <1.0 | <1.0 | <1.0 |  | 3.77 | 14.6 | 7.99 |
| PCB141 | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | 0.93 |
| PCB149 | <1.0 | <1.0 | <1.0 |  | 4.87 | 7.48 | 4.79 |
| PCB151 | <1.0 | <1.0 | <1.0 |  | <1.0 | 1.5 | 0.48 |
| PCB153 | <1.0 | <1.0 | <1.0 |  | 8.41 | 9.65 | 8.38 |
| PCB156 | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| PCB157 | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| PCB158 | <1.0 | <1.0 | 1.52 |  | 2.22 | <1.0 | 1.83 |
| PCB167 | <1.0 | <1.0 | <1.0 |  | <1.0 | 1.68 | <1.0 |
| PCB168/132 | <1.0 | <1.0 | <1.0 |  | <1.0 | 1.23 | 1.25 |
| PCB169 | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| PCB170 | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| PCB177 | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| PCB180 | <1.0 | <1.0 | <1.0 |  | 1.9 | 1.41 | 1.96 |
| PCB183 | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| PCB187 | <1.0 | <1.0 | <1.0 |  | 1.26 | <1.0 | 2.44 |
| PCB189 | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| PCB194 | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| PCB200 | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| PCB201 | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| PCB206 | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| Total Detected PCBs | <1.0 | 7 | 6 |  | 157 | 183 | 126 |

Table 12.  Newport Bay sediment PAH concentrations from March 2002 sampling.  Concentrations are in g/dry kg.  Method detection limit for all constituents is 1 g/dry kg.

|  | Upper Bay |  |  |  | Rhine Channel |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Compound | NB10 | NB10B | NB10C |  | NB3 | NB11 | NB12 |
| Naphthalene | 13.5 | 7.6 | 4.9 |  | 8.1 | 7.9 | 3.4 |
| 2-Methylnaphthalene | 10.2 | 4.6 | 2.1 |  | 5.5 | 7.3 | 2.2 |
| 1-Methylnaphthalene | 4.2 | 1.9 | 1.6 |  | 2.2 | 5.7 | 1.3 |
| Biphenyl | 2.9 | 2.1 | 1.3 |  | 1.7 | 2.7 | 0.85 |
| 2,6-Dimethylnaphthalene | 8.1 | 4 | 6.9 |  | 5 | 6.4 | 2.2 |
| Acenaphthene | 3.1 | 2.1 | 1.6 |  | 9.6 | 4.5 | 4.5 |
| Acenaphthylene | 6.2 | 4.9 | 3.5 |  | 5.1 | 71.6 | 4.3 |
| 2,3,5-Trimethylnaphthalene | 2.7 | 1.9 | 2.5 |  | 1.7 | 5.7 | 2.3 |
| Fluorene | 8 | 5 | 3.1 |  | 6.6 | 51 | 3.9 |
| Phenanthrene | 79.3 | 44.8 | 25.8 |  | 67.3 | 572 | 71.8 |
| Anthracene | 14.3 | 9.3 | 6.8 |  | 32.1 | 137 | 18.5 |
| 1-Methylphenanthrene | 16.6 | 7.3 | 5.3 |  | 14.2 | 87.7 | 11.7 |
| Fluoranthene | 197 | 130 | 87.2 |  | 167 | 810 | 210 |
| Pyrene | 228 | 141 | 102 |  | 202 | 763 | 197 |
| Benz[a]anthracene | 58.1 | 48.2 | 35.6 |  | 111 | 408 | 102 |
| Chrysene | 144 | 85.9 | 60.4 |  | 216 | 398 | 124 |
| Benzo[b]fluoranthene | 79.8 | 61.3 | 41.8 |  | 209 | 285 | 128 |
| Benzo[k]fluoranthene | 14 | 25 | 17 |  | 39.1 | 114 | 56.2 |
| Benzo[e]pyrene | 92.1 | 46.3 | 31.5 |  | 153 | 172 | 78.0 |
| Benzo[a]pyrene | 64.4 | 39.4 | 27 |  | 177 | 238 | 90.3 |
| Perylene | 31.9 | 21 | 15.3 |  | 56.3 | 48.3 | 23.3 |
| Indeno[1,2,3-c,d]pyrene | 47.9 | 39.7 | 33.1 |  | 145 | 147 | 112 |
| Dibenz[a,h]anthracene | 12.6 | 17.7 | 8.4 |  | 37.8 | 35.7 | 17.0 |
| Benzo[g,h,i]perylene | 83.8 | 39.7 | 33.7 |  | 138 | 86.4 | 95.7 |
| Total Detectable PAHs | 1220 | 791 | 558 |  | 1810 | 4460 | 1360 |

Table 13.  Newport Bay sediment pesticide concentrations from March 2002 sampling.  Concentrations are in g/dry kg.

|  | Upper Bay |  |  |  | Rhine Channel |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Compound | NB10 | NB10B | NB10C |  | NB3 | NB11 | NB12 |
| Toxaphene | <10 | <10 | <10 |  | <10 | <10 | <10 |
| 2,4'-DDD | 8.3 | 5.97 | 6 |  | <1.0 | 4.34 | <1.0 |
| 2,4'-DDE | <1.0 | 8.23 | 10.3 |  | <1.0 | 22.2 | 14.1 |
| 2,4'-DDT | <1.0 | 5.23 | 4.67 |  | <1.0 | 3.22 | 2.21 |
| 4,4'-DDD | <1.0 | 10 | 9.98 |  | <1.0 | 4.38 | 4.58 |
| 4,4'-DDE | 64.9 | 76.6 | 80.6 |  | 41.1 | 54.2 | 25.1 |
| 4,4'-DDT | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| Total Detectable DDTs | 73 | 106 | 112 |  | 41 | 88 | 46 |
| Aldrin | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| BHC-alpha | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| BHC-beta | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| BHC-delta | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| BHC-gamma | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| Chlordane-alpha | <1.0 | 1.84 | 3.39 |  | <1.0 | <1.0 | <1.0 |
| Chlordane-gamma | <1.0 | 1.88 | 2.59 |  | <1.0 | <1.0 | <1.0 |
| Dieldrin | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| Endosulfan Sulfate | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| Endosulfan-I | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| Endosulfan-II | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| Endrin | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| Endrin Aldehyde | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| Heptachlor | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| Heptachlor Epoxide | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| Methoxychlor | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| Mirex | <1.0 | <1.0 | <1.0 |  | <1.0 | <1.0 | <1.0 |
| trans-Nonachlor | <1.0 | 0.92 | 2.23 |  | <1.0 | <1.0 | <1.0 |

Table 14.  Water column metals concentrations from March 2002 sampling.  All concentrations are in g/L.

NB3			NB10

| Description | MDL | Total | Dissolved | Total | Dissolved |
| --- | --- | --- | --- | --- | --- |
| Aluminum | 0.01 | 48.7 | <0.01 | 318 | <0.01 |
| Antimony | 0.01 | 0.255 | 0.160 | 0.2 | 0.230 |
| Arsenic | 0.01 | 1.33 | 1.37 | 1.48 | 1.38 |
| Beryllium | 0.005 | <0.005 | <0.005 | <0.005 | <0.005 |
| Cadmium | 0.005 | 0.05 | 0.11 | 0.06 | 0.18 |
| Chromium | 0.005 | 0.39 | 0.28 | 0.63 | 0.25 |
| Cobalt | 0.005 | <0.005 | <0.005 | 0.155 | 0.11 |
| Copper | 0.005 | 8.24 | 7.94 | 1.98 | 1.39 |
| Iron | 0.01 | 24.4 | <0.01 | 177 | 0.05 |
| Lead | 0.005 | 0.130 | 0.040 | 0.26 | 0.010 |
| Manganese | 0.005 | 16.7 | 16.2 | 54.9 | 48.8 |
| Mercury | 0.005 | <0.005 | <0.005 | <0.005 | <0.005 |
| Molybdenum | 0.005 | 10.3 | 10.9 | 11.4 | 11.6 |
| Nickel | 0.005 | 0.475 | 0.500 | 0.945 | 0.800 |
| Selenium | 0.01 | 0.055 | 0.040 | 0.18 | 0.170 |
| Silver | 0.005 | <0.005 | <0.005 | <0.005 | <0.005 |
| Strontium | 0.01 | NA | NA | NA | NA |
| Thallium | 0.005 | 0.010 | 0.010 | 0.01 | 0.010 |
| Tin | 0.005 | 0.055 | 0.030 | 0.02 | 0.010 |
| Titanium | 0.005 | 3.87 | 0.060 | 19.3 | 0.050 |
| Vanadium | 0.005 | 2.96 | 2.74 | 4.59 | 3.66 |
| Zinc | 0.005 | 29.8 | 29.3 | 9.63 | 7.23 |

NA=Not analyzed.

Table 15.  Water column total PCB concentrations from March 2002 sampling.

|  | NB3 | NB10 |
| --- | --- | --- |
|  | (ng/L) | (ng/L) |
| PCB018 | <1.0 | <1.0 |
| PCB028 | <1.0 | <1.0 |
| PCB031 | <1.0 | <1.0 |
| PCB033 | <1.0 | <1.0 |
| PCB037 | <1.0 | <1.0 |
| PCB044 | <1.0 | <1.0 |
| PCB049 | <1.0 | <1.0 |
| PCB052 | <1.0 | <1.0 |
| PCB066 | <1.0 | <1.0 |
| PCB070 | <1.0 | <1.0 |
| PCB074 | <1.0 | <1.0 |
| PCB077 | <1.0 | <1.0 |
| PCB081 | <1.0 | <1.0 |
| PCB087 | <1.0 | <1.0 |
| PCB095 | <1.0 | <1.0 |
| PCB097 | <1.0 | <1.0 |
| PCB099 | <1.0 | <1.0 |
| PCB101 | <1.0 | <1.0 |
| PCB105 | <1.0 | <1.0 |
| PCB110 | <1.0 | <1.0 |
| PCB114 | <1.0 | <1.0 |
| PCB118 | <1.0 | <1.0 |
| PCB119 | <1.0 | <1.0 |
| PCB123 | <1.0 | <1.0 |
| PCB126 | <1.0 | <1.0 |
| PCB128 | <1.0 | <1.0 |
| PCB138 | <1.0 | <1.0 |
| PCB141 | <1.0 | <1.0 |
| PCB149 | <1.0 | <1.0 |
| PCB151 | <1.0 | <1.0 |
| PCB153 | <1.0 | <1.0 |
| PCB156 | <1.0 | <1.0 |
| PCB157 | <1.0 | <1.0 |
| PCB158 | <1.0 | <1.0 |
| PCB167 | <1.0 | <1.0 |
| PCB168/132 | <1.0 | <1.0 |
| PCB169 | <1.0 | <1.0 |
| PCB170 | <1.0 | <1.0 |
| PCB177 | <1.0 | <1.0 |
| PCB180 | <1.0 | <1.0 |
| PCB183 | <1.0 | <1.0 |
| PCB187 | <1.0 | <1.0 |
| PCB189 | <1.0 | <1.0 |
| PCB194 | <1.0 | <1.0 |
| PCB200 | <1.0 | <1.0 |
| PCB201 | <1.0 | <1.0 |
| PCB206 | <1.0 | <1.0 |
| Total Detectable PCB | <1.0 | <1.0 |

Table 16.  Water column total chlorinated pesticide concentrations from March 2002 sampling.

|  |  | NB3 | NB10 |
| --- | --- | --- | --- |
|  |  | (ng/L) | (ng/L) |
| Toxaphene | <10 |  | <10 |
| 2,4'-DDD | <1.0 |  | <1.0 |
| 2,4'-DDE | <1.0 |  | <1.0 |
| 2,4'-DDT | <1.0 |  | <1.0 |
| 4,4'-DDD | <1.0 |  | <1.0 |
| 4,4'-DDE | <1.0 |  | <1.0 |
| 4,4'-DDT | <1.0 |  | <1.0 |
| Total Detectable DDT | 0 |  | 0 |
| Aldrin | <1.0 |  | <1.0 |
| BHC-alpha | <1.0 |  | <1.0 |
| BHC-beta | <1.0 |  | <1.0 |
| BHC-delta | <1.0 |  | <1.0 |
| BHC-gamma | <1.0 |  | <1.0 |
| Chlordane-alpha | <1.0 |  | <1.0 |
| Chlordane-gamma | <1.0 |  | <1.0 |
| Dieldrin | <1.0 |  | <1.0 |
| Endosulfan Sulfate | <1.0 |  | <1.0 |
| Endosulfan-I | <1.0 |  | <1.0 |
| Endosulfan-II | <1.0 |  | <1.0 |
| Endrin | <1.0 |  | <1.0 |
| Endrin Aldehyde | <1.0 |  | <1.0 |
| Heptachlor | <1.0 |  | <1.0 |
| Heptachlor Epoxide | <1.0 |  | <1.0 |
| Methoxychlor | <1.0 |  | <1.0 |
| Mirex | <1.0 |  | <1.0 |
| trans-Nonachlor | <1.0 |  | <1.0 |

LOWER NEWPORT BAY MAY SEDIMENT SAMPLES

Table 17.  Lower Newport Bay sediment metals concentrations from May 2002 sampling.  Concentrations are in mg/dry kg

|  | MDL | 2137 | 2136 | 2142 |
| --- | --- | --- | --- | --- |
| Aluminum | 1 | 46800 | 51200 | 48900 |
| Antimony | 0.05 | 0.46 | 0.62 | 0.37 |
| Arsenic | 0.05 | 10.8 | 11.0 | 8.68 |
| Barium | 0.05 | 137 | 156 | 184 |
| Beryllium | 0.01 | 1.06 | 1.18 | 1.12 |
| Cadmium | 0.01 | 0.72 | 0.74 | 1.1 |
| Chromium | 0.05 | 60.8 | 64.4 | 56.8 |
| Cobalt | 0.01 | 9.24 | 9.18 | 9.28 |
| Copper | 0.01 | 160 | 92.6 | 40.9 |
| Iron | 1 | 39800 | 42850 | 39300 |
| Lead | 0.01 | 55.2 | 57.9 | 22.5 |
| Manganese | 0.05 | 314 | 315 | 347 |
| Mercury | 0.005 | 2.08 | 0.69 | 0.32 |
| Molybdenum | 0.05 | 2.06 | 1.96 | 1.69 |
| Nickel | 0.01 | 25.4 | 27.0 | 24.8 |
| Selenium | 0.05 | 2.26 | 2.10 | 2.04 |
| Silver | 0.01 | 0.16 | 0.16 | 0.12 |
| Strontium | 0.05 | 81.3 | 83.6 | 134 |
| Thallium | 0.01 | 0.52 | 0.50 | 0.53 |
| Tin | 0.05 | 4.08 | 4.46 | 2.74 |
| Titanium | 0.05 | 2090 | 1990 | 1940 |
| Vanadium | 0.05 | 112 | 120 | 110 |
| Zinc | 0.05 | 260 | 256 | 164 |

Table 18.  Lower Newport Bay sediment PCB concentrations from May 2002 sampling.  Concentrations are in g/dry kg.

|  | 2137 | 2136 | 2142 |
| --- | --- | --- | --- |
| PCB018 | <1.0 | <1.0 | <1.0 |
| PCB028 | <1.0 | 0.95 | <1.0 |
| PCB031 | <1.0 | <1.0 | <1.0 |
| PCB033 | <1.0 | <1.0 | <1.0 |
| PCB037 | <1.0 | <1.0 | <1.0 |
| PCB044 | <1.0 | 4.17 | <1.0 |
| PCB049 | 0.89 | 5.45 | <1.0 |
| PCB052 | 1.11 | 2.17 | <1.0 |
| PCB066 | 1.23 | 6.41 | <1.0 |
| PCB070 | 0.69 | 4.71 | <1.0 |
| PCB074 | <1.0 | 6 | <1.0 |
| PCB077 | <1.0 | <1.0 | <1.0 |
| PCB081 | <1.0 | <1.0 | <1.0 |
| PCB087 | <1.0 | 1.13 | <1.0 |
| PCB095 | <1.0 | 2.16 | <1.0 |
| PCB097 | <1.0 | 1.67 | <1.0 |
| PCB099 | <1.0 | 2.44 | <1.0 |
| PCB101 | 2.7 | 4.72 | <1.0 |
| PCB105 | 2.33 | <1.0 | 1.14 |
| PCB110 | 3.11 | 7.11 | <1.0 |
| PCB114 | 1.02 | <1.0 | 0.79 |
| PCB118 | 3.9 | 10.2 | 0.85 |
| PCB119 | <1.0 | 1.13 | <1.0 |
| PCB123 | 0.79 | 7.54 | <1.0 |
| PCB126 | <1.0 | <1.0 | 2.1 |
| PCB128 | <1.0 | <1.0 | 0.76 |
| PCB138 | 5.24 | 7.86 | <1.0 |
| PCB141 | <1.0 | 2.12 | <1.0 |
| PCB149 | 2.08 | 2.83 | <1.0 |
| PCB151 | <1.0 | <1.0 | <1.0 |
| PCB153 | 6.27 | 5.01 | <1.0 |
| PCB156 | 2.91 | <1.0 | <1.0 |
| PCB157 | <1.0 | <1.0 | <1.0 |
| PCB158 | <1.0 | 0.71 | <1.0 |
| PCB167 | <1.0 | <1.0 | 0.76 |
| PCB168/132 | <1.0 | 0.78 | <1.0 |
| PCB169 | <1.0 | 8.14 | <1.0 |
| PCB170 | <1.0 | <1.0 | <1.0 |
| PCB177 | <1.0 | <1.0 | <1.0 |
| PCB180 | <1.0 | 0.93 | <1.0 |
| PCB183 | <1.0 | <1.0 | <1.0 |
| PCB187 | <1.0 | 0.96 | <1.0 |
| PCB189 | <1.0 | <1.0 | <1.0 |
| PCB194 | <1.0 | <1.0 | <1.0 |
| PCB200 | <1.0 | <1.0 | <1.0 |
| PCB201 | <1.0 | <1.0 | <1.0 |
| PCB206 | <1.0 | <1.0 | <1.0 |
| Total Detectable PCB | 34.3 | 97.3 | 6.4 |

Table 19.  Lower Newport Bay sediment PAH concentrations from May 2002 sampling.  Concentrations are in g/dry kg.  Method detection limit for all PAH constituents is 1 g/dry kg.

|  | 2137 | 2136 | 2142 |
| --- | --- | --- | --- |
| Naphthalene | 3.8 | 6.7 | 2.6 |
| 2-Methylnaphthalene | 2.4 | 14.9 | 1.4 |
| 1-Methylnaphthalene | 2.3 | 10.6 | 1.2 |
| Biphenyl | <1.0 | 1.9 | 0.6 |
| 2,6-Dimethylnaphthalene | 2.2 | 19.3 | 1.3 |
| Acenaphthene | 2.3 | 1.3 | 0.5 |
| Acenaphthylene | 3.2 | 7.2 | 2.6 |
| 2,3,5-Trimethylnaphthalene | 3.5 | 7.6 | <1.0 |
| Fluorene | 1.1 | 7.5 | 0.9 |
| Phenanthrene | 17.8 | 95.5 | 13.2 |
| Anthracene | 8.3 | 18.5 | 4.1 |
| 1-Methylphenanthrene | 5.2 | 11 | 3.7 |
| Fluoranthene | 50.9 | 174 | 40.6 |
| Pyrene | 60.5 | 185 | 52.8 |
| Benz[a]anthracene | 32 | 64.7 | 26.7 |
| Chrysene | 48.7 | 85.6 | 35.8 |
| Benzo[b]fluoranthene | 64.6 | 60.9 | 27.1 |
| Benzo[k]fluoranthene | 30.9 | 33.4 | 14.2 |
| Benzo[e]pyrene | 49.1 | 44.9 | 22.7 |
| Benzo[a]pyrene | 57.4 | 48.4 | 25.6 |
| Perylene | 20.1 | 19.4 | 10.6 |
| Indeno[1,2,3-c,d]pyrene | 60.6 | 44.3 | 22.3 |
| Dibenz[a,h]anthracene | 16.3 | 9.7 | 7.5 |
| Benzo[g,h,i]perylene | 43.4 | 34.1 | 18.7 |
| Total Detectable PAH | 587 | 1010 | 337 |
| Total Organic Carbon (% Dry Wt) | 0.94 | 1.51 | 0.65 |
| Total Solids (% Dry Wt) | 37.8 | 33.4 | 44.6 |

Table 20.  Lower Newport Bay sediment pesticide concentrations from May 2002 sampling.  Concentrations are in g/dry kg.

|  | 2137 | 2136 | 2142 |
| --- | --- | --- | --- |
| Toxaphene | <10 | <10 | <10 |
| 2,4'-DDD | <1.0 | 5 | 5.69 |
| 2,4'-DDE | 5.99 | 23 | 16.8 |
| 2,4'-DDT | <1.0 | <1.0 | 1.99 |
| 4,4'-DDD | <1.0 | 13.1 | 6.8 |
| 4,4'-DDE | 32 | 64.5 | 51.5 |
| 4,4'-DDT | <1.0 | 6.85 | <1.0 |
| Total Detectable DDT | 38 | 112 | 82.8 |
| Aldrin | <1.0 | <1.0 | <1.0 |
| BHC-alpha | <1.0 | <1.0 | <1.0 |
| BHC-beta | <1.0 | <1.0 | <1.0 |
| BHC-delta | <1.0 | <1.0 | <1.0 |
| BHC-gamma | <1.0 | <1.0 | <1.0 |
| Chlordane-alpha | <1.0 | 2.64 | <1.0 |
| Chlordane-gamma | <1.0 | 3.86 | <1.0 |
| Dieldrin | <1.0 | <1.0 | <1.0 |
| Endosulfan Sulfate | <1.0 | <1.0 | <1.0 |
| Endosulfan-I | <1.0 | <1.0 | <1.0 |
| Endosulfan-II | <1.0 | <1.0 | <1.0 |
| Endrin | <1.0 | <1.0 | <1.0 |
| Endrin Aldehyde | <1.0 | <1.0 | <1.0 |
| Heptachlor | <1.0 | <1.0 | <1.0 |
| Heptachlor Epoxide | <1.0 | <1.0 | <1.0 |
| Methoxychlor | <1.0 | <1.0 | <1.0 |
| Mirex | <1.0 | <1.0 | <1.0 |
| trans-Nonachlor | <1.0 | 1.64 | <1.0 |