CALIFORNIA REGIONAL WATER QUALITY CONTROL BOARD

SAN DIEGO REGION

ORDER NO. 2001-08

NPDES NO. CA0107611

MONITORING AND REPORTING PROGRAM

FOR THE

ALISO WATER MANAGEMENT AGENCY

ORANGE COUNTY

DISCHARGE TO THE PACIFIC OCEAN

THROUGH THE ALISO WATER MANAGEMENT AGENCY

OCEAN OUTFALL

This Monitoring and Reporting Program supersedes Monitoring and Reporting Program 95-107 in its entirety.  This Monitoring and Reporting program shall become effective with the adoption of Order No. 2001-08.

A.	PURPOSE

This monitoring program is intended to:

- Document short-term and long-term effects of the discharge on receiving waters, sediments, biota, and beneficial uses of the receiving water.

- Determine compliance with NPDES permit terms and conditions.

- Assess the need for industrial pretreatment and toxic control programs.

The monitoring data will be used to determine compliance with water quality standards.

B.	MONITORING PROVISIONS

- Samples and measurements taken as required herein shall be representative of the volume and nature of the monitored discharge.  All samples shall be taken at the monitoring points specified in this monitoring program and, unless otherwise specified, before the effluent joins or is diluted by any other waste stream, body of water, or substance.  Monitoring points shall not be changed without notification to and the approval of the Executive Officer.  Samples shall be collected at times representative of “worse case” conditions with respect to compliance with the requirements of Order No. 2001-08.

- Appropriate flow measurement devices and methods consistent with accepted scientific practices shall be selected and used to ensure the accuracy and reliability of measurements of the volume of monitored discharges.  The devices shall be installed, calibrated, and maintained to ensure that the accuracy of the measurements are consistent with the accepted capability of that type of device.  Devices selected shall be capable of measuring flows with a maximum deviation of less than  10% from true discharge rates throughout the range of expected discharge volumes.

- Monitoring must be conducted according to United States Environmental Protection Agency (USEPA) test procedures approved under Title 40 of the Code of Federal Regulations Part 136 (40CFR 136), Guidelines Establishing Test Procedures for the Analysis of Pollutants, as amended, unless otherwise specified for sludge in 40CFR 503, or unless other test procedures have been specified in Order No. 2001-08 and/or in this monitoring and reporting program.

- If the discharger monitors any pollutants more frequently than required by Order No. 2001-08 or by this monitoring and reporting program, using test procedures approved under 40CFR Part 136, or as specified in Order No. 2001-08 and this monitoring and reporting program, the results of this monitoring shall be included in the calculation and reporting of the data submitted in the discharger’s monitoring report.  The increased frequency of monitoring shall also be reported.

- The discharger shall retain records of all monitoring information, including all calibration and maintenance records and all original strip chart recordings for continuous monitoring instrumentation, copies of all reports required by Order No. 2001-08 and this monitoring and reporting program, and records of all data used to complete the application for Order No. 2001-08.  Records shall be maintained for a minimum of 5 years from the date of the sample, measurement, report, or application.  This period may be extended during the course of any unresolved litigation regarding this discharge or when requested by the Regional Board Executive Officer or the USEPA.

- Records of monitoring information shall include:

- The date, exact location, and time of sampling or measurements;

- The name(s) of individual(s) who performed the sampling or measurements;

- The date(s) analyses were performed;

- The name(s) of the laboratory and individual(s) who performed the analyses;

- The analytical techniques or methods used; and

- The results of such analyses.

- Calculations for all limitations that require averaging of measurements shall utilize an arithmetic mean unless otherwise specified in Order 2001-08 or this monitoring and reporting program.

- All monitoring instruments and devices used by the discharger to fulfill the prescribed monitoring program shall be properly maintained and calibrated as necessary to ensure their continued accuracy.  All flow measurement devices shall be calibrated at least once per year, or more frequently, to ensure continued accuracy of the devices.

- All analyses shall be performed in a laboratory certified to perform such analyses by the California Department of Health Services or a laboratory approved by the Regional Board Executive Officer.

- The discharger shall have and implement an acceptable written quality assurance (QA) plan for laboratory analyses.  An annual report shall be submitted by February 1st of each year that summarizes the QA activities for the previous year.  Duplicate chemical analyses must be conducted on a minimum of ten percent of the samples, or at least one sample per month, whichever is more frequent.  A similar frequency shall be maintained for analyzing spiked samples.  When requested by the USEPA or the Regional Board, the discharger will participate in the NPDES discharge monitoring report QA performance study.  The discharger shall have a success rate equal to or greater than 80 percent.

- The discharger shall report all instances of noncompliance not reported under Reporting Requirement G.11 of Order No. 2001-08 at the time monitoring reports are submitted.  The reports shall contain the information listed in Reporting Requirement G.11 of Order No. 2001-08.

- By February 1st of each year, the discharger shall submit an annual report to the Regional Board and USEPA Region 9 that contains tabular and graphical summaries of the monitoring data obtained during the previous year.  The discharger shall discuss the compliance record and corrective actions taken, or which may be needed to bring the discharge into full compliance with the requirements of Order No. 2001-08 and this monitoring and reporting program.

- Laboratory method detection limits (MDLs) and practical quantitation levels (PQLs) shall be identified for each constituent in the matrix being analyzed with all reported analytical data.  Acceptance of data shall be based on demonstrated laboratory performance.

- Monitoring results shall be reported at intervals and in a manner specified in Order No. 2001-08 and in this monitoring and reporting program.

- Monitoring reports shall be submitted to the Regional Board and to USEPA Region 9 according to the following schedule:

Monitoring Frequency			Reporting Period		Report Due

Continuous, Daily, Weekly, or Monthly	All			By the first day of the second

month after the month of sampling.

Quarterly					Jan.-March			May 1

April-June			August 1

July-September		November 1

October-December		February 1

Semiannually					January-June			August 1

July-December		February 1

Annually					January-December		February 1

Once every 5 years				---				February 1

C.	INFLUENT MONITORING

- Influent monitoring is intended to:

- Determine compliance with NPDES permit conditions and water quality standards.

- Assess treatment plant performance.

- Assess the need for and/or performance of an Industrial Pretreatment Program and a Toxic Control Program.

- Sampling stations shall be established at each point of inflow to all treatment plants and shall be located upstream of any in-plant return flows, and where representative samples of the influent can be obtained.  Influent samples shall be collected on the same day as, and shortly before the collection of effluent samples.

- During periods when no effluent from a particular treatment plant is discharged to the Pacific Ocean, no influent monitoring, except for flowrate monitoring, is required at that treatment plant.

- The following shall constitute the influent monitoring program:

Parameter			Unit		Type of Sample1		Minimum Frequency

Flowrate			MGD		recorder/ totalizer		continuous

CBOD5 @ 20C		mg/L		24-hour composite		weekly

BOD5 @ 20C			mg/L		24-hour composite		monthly

Suspended Solids		mg/L		24-hour composite		weekly

D.	EFFLUENT MONITORING

For the purpose of this Monitoring and Reporting Program, effluent includes Aliso Creek flows diverted to the AWMA Ocean Outfall as well as treatment plant effluent.

- Effluent monitoring is intended to:

- Determine compliance with NPDES permit conditions and water quality standards.

- Identify operational problems in order to improve plant performance.

- Provide information on waste characteristics and flows for use in interpreting water quality and biological data.

- The effluent sampling station shall be located downstream of any in-plant return flows, and disinfection units, where representative samples of the effluent discharged through the ocean outfall can be obtained.

- During periods where no effluent from a particular treatment plant is discharged to the Pacific Ocean, no effluent monitoring, except for flowrate monitoring, is required at that treatment plant.

- The following shall constitute the effluent monitoring program:

Minimum

Parameter			Unit			Type of Sample1		Frequency

Flowrate			MGD			recorder/totalizer		continuous

CBOD5 @ 20C		mg/L			24-hour composite		daily

BOD5 @ 20C			mg/L			24-hour composite		monthly

Suspended Solids		mg/L			24-hour composite		daily2

pH				pH units		grab 				daily2

Oil and Grease			mg/L			grab				monthly

Settleable Solids		mL/L			grab				daily2

Minimum

Parameter			Unit			Type of Sample1		Frequency

Turbidity			NTU			24-hour composite		weekly3

Acute Toxicity		TUa			24-hour composite		monthly

Dissolved Oxygen		mg/L			grab				weekly

Temperature			F			--				weekly

Arsenic			ug/L			24-hour composite		quarterly3,

Cadmium			ug/L			24-hour composite		quarterly3,4

Chromium

(hexavalent) 			ug/L			24-hour composite		quarterly3,4,

Copper				ug/L			24-hour composite		quarterly3,4

Lead 				ug/L			24-hour composite		quarterly3,4

Mercury			ug/L			24-hour composite		quarterly3,4

Nickel				ug/L			24-hour composite		quarterly3,4

Selenium 			ug/L			24-hour composite		quarterly3,4

Silver				ug/L			24-hour composite		quarterly3,4

Zinc				ug/L			24-hour composite		quarterly3,4

Cyanide			mg/L			24-hour composite		quarterly3,4

Total Residual Chlorine	mg/L			grab				daily

Ammonia (as N)		mg/L			24-hour composite		monthly3

Chronic Toxicity		TUc			24-hour composite		monthly

Phenolic Compounds 		mg/L			24-hour composite		quarterly3,4

(nonchlorinated)

Phenolic Compounds 		mg/L			24-hour composite		quarterly3,4

(chlorinated)

Endosulfan			ug/L			24-hour composite		quarterly3,4

Endrin				ug/L			24-hour composite		quarterly3,4

HCH				ug/L			24-hour composite		quarterly3,4

Radioactivity			pCi/L			24-hour composite		quarterly3

Acrolein			ug/L			grab				semiannually3

Antimony			ug/L			24-hour composite		semiannually3

Bis(2-chloroethoxy)

Methane			ug/L			grab				semiannually3

Bis(2-chloroisopropyl)

Ether				ug/L			grab				semiannually3

Chlorobenzene		ug/L			grab				semiannually3

Chromium (III)		ug/L			24-hour composite		semiannually3

Di-n-butyl phthalate		ug/L			grab				semiannually3

Dichlorobenzenes		ug/L			grab				semiannually3

1,1-dichloroethylene		ug/L			grab				semiannually3

Diethyl phthalate		ug/L			grab				semiannually3

Dimethyl phthalate		ug/L			grab				semiannually3

4,6-dinitro-2-

Methylphenol			ug/L			grab				semiannually3

2,4 dinitrophenol		ug/L			grab				semiannually3

Minimum

Parameter			Unit			Type of Sample1		Frequency

Ethylbenzene			ug/L			grab				semiannually3

Fluoranthene			ug/L			grab				semiannually3

Hexacyclopentadiene		ug/L			grab				semiannually3

Isophorone			ug/L			grab				semiannually3

Nitrobenzene			ug/L			grab				semiannually3

Thallium			ug/L			24-hour composite		semiannually3

Toluene			ug/L			grab				semiannually3

1,1,2,2-tetrachloro-

ethane				ug/L			grab				semiannually3

Tributyltin			ug/L			24-hour composite		semiannually3

1,1,1-trichloroethane		ug/L 			grab				semiannually3

1,1,2-trichloroehtane		ug/L 			grab				semiannually3

Acrylonitrile			ug/L			grab				semiannually3

Aldrin				ug/L			grab				semiannually3

Benzene			ug/L			grab				semiannually3

Benzidine			ug/L			grab				semiannually3

Beryllium			ug/L			24-hour composite		semiannually3

Bis(2-chloroethyl)

Ether				ug/L			grab				semiannually3

Bis(2-ethylhexyl)

Phthalate			ug/L			grab				semiannually3

Carbon Tetrachloride		ug/L			grab				semiannually3

Chlordane			ug/L			grab				semiannually3

Chloroform			ug/L			grab				semiannually3

DDT				ug/L			grab				semiannually3

1,4-dichlorobenzene		ug/L			grab				semiannually3

3,3-dichlorobenzidine		ug/L			grab				semiannually3

1,2-dichloroethane		ug/L			grab				semiannually3

Dichloromethane		ug/L			grab				semiannually3

1,3-dichloropropene		ug/L			grab				semiannually3

Dieldrin			ug/L			grab				semiannually3

2,4-dinitrotoluene		ug/L 			grab				semiannually3

1,2-

diphenylhydrazine 		ug/L			grab				semiannually3

Halomethanes			ug/L			grab				semiannually3

Heptachlor			ug/L			grab				semiannually3

Hexachlorobenzene		ug/L			grab				semiannually3

Hexachlorobutadiene		ug/L			grab				semiannually3

Hexachloroethane		ug/L			grab				semiannually3

N-nitrosodimethyl-

amine				ug/L			grab				semiannually3

N-nitrosodiphenyl-

amine				ug/L			grab				semiannually3

Minimum

Parameter			Unit			Type of Sample1		Frequency

PAHs				ug/L			grab				semiannually3

PCBs				ng/L			grab				semiannually3

TCDD equivalents		pg/L			grab				semiannually,3

Tetrachloroethylene		ug/L			grab				semiannually3

Toxaphene			ug/L			grab				semiannually3

Trichloroethylene		ug/L			grab				semiannually3

2,4,6-trichlorophenol		ug/L			grab				semiannually3

Vinyl Chloride		ug/L			grab				semiannually3

E.	SOLIDS MONITORING

- Solids monitoring is intended to:

- Assess the need for a pretreatment program.

- Maintain a record of the volume of solids generated and disposal sites used.

- Evaluate the character of sludge to ensure that appropriate disposal methods are employed.

- A report identifying the volume of screenings, sludges, grit, and other solids removed from the wastewater and the point(s) at which these wastes were disposed of shall be submitted annually.  A copy of all annual reports required by 40CFR 503 shall be submitted to the Regional Board at the same time those reports are submitted to the USEPA.

F.	RECEIVING WATER MONITORING

- To determine compliance with water quality standards, the receiving water quality monitoring program must document conditions in the vicinity of the “Zone of Initial Dilution” (ZID) boundary, at reference stations, and at areas beyond the ZID where discharge impacts might reasonably be expected.  Monitoring must reflect conditions during all critical environmental periods.

- Receiving water and sediment monitoring in the vicinity of the AWMA Ocean Outfall shall be conducted as specified below.  Station location, sampling, sample preservation and analyses, when not specified, shall be by methods approved by the Executive Officer.  The monitoring program may be modified by the Executive Officer at any time.

- Receiving water and sediment monitoring stations shall be located and numbered as follows:

Monitoring Station Locations

Station		Description

Surf Zone Stations:

S1		Surf 20,000 feet south of outfall.

S2		Surf 15,000 feet south of outfall.

S3		Surf 10,000 feet south of outfall.

S4		Surf 5,000 feet south of outfall.

S5		Surf 4,000 feet south of outfall.

S6		Surf 3,000 feet south of outfall.

S7		Surf 2,000 feet south of outfall.

S8		Surf 1,000 feet south of outfall.

S9		Surf at outfall.

S10		Surf 1,000 feet north of outfall.

S11		Surf 2,000 feet north of outfall.

S12		Surf 3,000 feet north of outfall.

S13		Surf 4,000 feet north of outfall.

S14		Surf 5,000 feet north of outfall.

S15		Surf 10,000 feet north of outfall.

S16		Surf 15,000 feet north of outfall.

Nearshore Stations:

N1		1,000 feet offshore, 2,500 feet south of the outfall.

N2		1,000 feet offshore, 1,000 feet south of the outfall.

N3		1,000 feet offshore, 500 feet south of the outfall.

N4		1,000 feet offshore, at the outfall.

N5		1,000 feet offshore, 500 feet north of the outfall.

N6		1,000 feet offshore, 1,000 feet north of the outfall.

N7		1,000 feet offshore, 2,500 feet north of the outfall.

Offshore Stations:

A1-A4	At the corners of a 1,000 x 1,000-foot square having one side parallel to shore and the intersection of the diagonals located at the center of the outfall diffuser section.  Station A1 shall be located at the northeastern corner and Stations A2 through A4 at successive corners in a clockwise direction.

A5	At the intersection of the diagonals of the above square.

B1	One mile down-coast from the outfall and over the same depth contour as Station A5.

B2	One mile up-coast from the outfall and over the same depth contour as Station A5.

It is recommended that stations be located using a land-based microwave positioning system, such as Mini-Ranger or trisponder, or a satellite positioning system such as Global Positioning System (GPS).  The high levels of accuracy and precision afforded by this type of positioning system will ensure that stations are properly located with respect to the ZID.  If an alternate navigation system (e.g. Loran C) is proposed, its accuracy should be compared to that of the systems recommended herein, and any compromises in accuracy should be justified.

Monitoring station locations may be modified with the approval of the Executive Officer.

- SURF ZONE WATER QUALITY MONITORING

- Surf zone monitoring is intended to assess bacteriological conditions in areas used for body-contact activities (e.g., swimming); and to assess aesthetic conditions for general recreational uses (e.g., picnicking).

- All “surf zone stations” shall be monitored as follows:

- Grab samples shall be collected and analyzed for total and fecal coliforms, and enterococcus at a minimum frequency of twice weekly.

- At the same time samples are collected from “surf zone stations,” the following information shall be recorded: observation of wind (direction and speed), weather (e.g., cloudy, sunny, or rainy), current (e.g., direction), and tidal conditions; observations of water color, discoloration, oil and grease, turbidity, odor, and materials of sewage origin in the water or on the beach; and water temperature (F).

- In the event of stormy weather which makes sampling hazardous at certain surf zone stations, collection of samples at such stations can be omitted, provided that such omissions do not occur more than 5 days in any calendar year or occur at consecutive sampling times.  The observations listed in (2) above shall still be recorded and reported to the Regional Board for these stations at the time the sample was attempted to be collected.

- NEARSHORE WATER QUALITY MONITORING

- Nearshore monitoring is intended to assess bacteriological conditions in areas used for body-contact activities (e.g. scuba diving) and where shellfish and/or kelp may be harvested; and to assess aesthetic conditions for general boating and recreational uses.

- All Nearshore Stations shall be monitored as follows:

- Reduced Monitoring

If the Executive Officer determines that the effluent at all times complies with Discharge Specifications B.1, B.5, and B.6 of Order No. 2001-08, only the reduced nearshore water quality monitoring specified below is required.

Determination		Units		Type of Sample	Minimum Frequency

Visual Observations							Monthly

Total and Fecal Coliforms,	#/100mL	Grab			Monthly

Enterococcus		#/100mL	Grab9			Monthly

- Intensive Monitoring

The intensive nearshore water quality monitoring specified below is required during the 12-month period beginning July 1, 2004 through June 30, 2005, and must be submitted by September 1, 2005.  This monitoring data will assist Regional Board staff in the evaluation of the Report of Waste Discharge required by Reporting Requirement G.2 to be submitted by August 21, 2005, 180 days prior to the Order’s expiration date of February 21, 2006.  The intensive nearshore water quality monitoring specified below is also required if the Executive Officer determines that the effluent does not at all times comply with Discharge Specifications B.1, B.5, and B.6 of Order No. 2001-08.

Determination			Units		Type of Sample	Minimum Frequency

Visual Observations			--		--			Monthly

Total and Fecal Coliforms		#/100mL	Grab			Monthly

Enterococcus				#/100mL	Grab11			Monthly

- OFFSHORE WATER QUALITY MONITORING

Offshore monitoring is intended to determine compliance with the Ocean Plan; and to determine if the applicant’s discharge causes significant impacts on the water quality within the ZID and beyond the ZID as compared to reference areas.

- All “offshore stations” shall be monitored as follows:

- Reduced Monitoring

If the Executive Officer determines that the effluent at all times complies with Discharge Specifications B.1, B.5, and B.6 of Order No. 2001-08, only the reduced offshore water quality monitoring specified below is required.

Determination			Units		Type of Sample	Minimum Frequency

Visual Observations								Monthly

Total and Fecal Coliforms,		#/100mL	Grab11			Monthly

Enterococcus10			#/100mL	Grab11			Monthly

- Intensive Monitoring

The intensive offshore water quality monitoring specified below is required during the 12-month period beginning July 1, 2004 through June 30, 2005, and must be submitted by September 1, 2005.  This monitoring data will assist Regional Board staff in the evaluation of the Report of Waste Discharge required by Reporting Requirement G.2 to be submitted by August 21, 2005, 180 days prior to the Order’s expiration date of February 21, 2006.  The intensive offshore water quality monitoring specified below is also required if the Executive Officer determines that the effluent does not at all times comply with Discharge Specifications B.1, B.5, and B.6 of Order No. 2001-08.

Determination			Units		Type of Sample	Minimum Frequency

Visual Observations								Monthly

Total and Fecal Coliforms,		#/100mL	Grab11			Monthly

Enterococcus				#/100mL	Grab11			Monthly

Temperature				F		Grab12			Monthly

Dissolved Oxygen			mg/L		Grab			Monthly

Light Transmittance			%		Instrument12		Monthly

pH					pH units	Grab9			Annually

- BENTHIC MONITORING

Benthic monitoring is intended to assess the status of the benthic community, and to evaluate the physical and chemical quality of the sediments.

- The intensive sediment monitoring specified below is required during the 12-month period beginning July 1, 2004 through June 30, 2005, and must be submitted by September 1, 2005.  This monitoring data will assist Regional Board staff in the evaluation of the Report of Waste Discharge required by Reporting Requirement G.2 to be submitted by August 21, 2005, 180 days prior to the Order’s expiration date of February 21, 2006.  The intensive sediment monitoring specified below is also required if the Executive Officer determines that the effluent does not at all times comply with Discharge Specifications B.1, B.5, and B.6 of Order No. 2001-08.

- Sediment Characteristics

Analyses shall be performed on the upper two inches of core.

Determination		Units		Type of Sample	Minimum Frequency

Sulfides			mg/kg		Core			Semiannually

Total Chlorinated

Hydrocarbons			mg/kg		Core			Semiannually

BOD				mg/kg		Core			Semiannually

COD				mg/kg		Core			Semiannually

Particle Size

Distribution  					Core			Semiannually

Arsenic			mg/kg		Core			Annually

Cadmium			mg/kg		Core			Annually

Total Chromium		mg/kg		Core			Annually

Copper				mg/kg		Core			Annually

Lead				mg/kg		Core			Annually

Mercury			mg/kg		Core			Annually

Nickel				mg/kg		Core			Annually

Silver				mg/kg		Core			Annually

Zinc				mg/kg		Core			Annually

Cyanide			mg/kg		Core			Annually

Phenolic Compounds

(non-chlorinated)	        	mg/kg	     	Core			Annually

Chlorinated Phenolics		mg/kg		Core			Annually

Aldrin and Dieldrin		mg/kg		Core			Annually

Chlordane and

Related Compounds		mg/kg		Core			Annually

DDT and Derivatives		mg/kg		Core			Annually

Endrin				mg/kg		Core			Annually

HCH				mg/kg		Core			Annually

PCBs				mg/kg		Core			Annually

Toxaphene			mg/kg		Core			Annually

Radioactivity			pCi/kg		Core			Annually

- Infauna

Samples are to be collected with a Paterson, Smith-McIntyre, or orange-peel-type dredge, having an open sampling area of not less than 124 square inches and a sediment capacity of not less than 210 cubic inches.  The sediment shall be sifted through a one-millimeter mesh screen and all organisms shall be identified to as low a taxon as possible.

Five replicate samples of bottom sediments shall be taken semiannually [once during late winter (February/March)] and once during late summer (August/September)] from all offshore stations.  These samples shall be separate from those collected for sediment analyses.

The following data shall be reported for benthic infauna:

- Total Biomass of:

- Mollusks

- Echinoderms

- Polychaetes

- Crustaceans

- All other macroinvertebrates.

- Community structure analysis for each station and each replicate.  Community structure analysis consists of the wet weight of each taxonomic group in a. (1) above, number of species, number of individuals per species, total numerical abundance, species abundance per square meter per station, species richness, species diversity, similarity analyses, and cluster analyses (using unweighted pair-group method).

- Station mean, range, standard deviation, and 95% confidence limits, if appropriate, for values determined above in (2).  The discharger may be required to conduct additional statistical analyses to determine temporal and spatial trends in the marine environment.

- Biota Monitoring

Determination		Units			Type of Sample	Minimum Frequency

Benthic Biota			Identification and	3 Grabs		Semiannually

Enumeration

- ADDITIONAL BIOLOGICAL MONITORING

- Kelp Bed Monitoring

Kelp bed monitoring is intended to assess the extent to which the discharge of wastes may affect the areal extent and health of coastal kelp beds.

- The discharger shall participate with other ocean dischargers in the San Diego Region in an annual regional kelp bed photographic survey.  Kelp beds shall be monitored annually by means of vertical aerial infrared photography to determine the maximum areal extent of the region’s coastal kelp beds within the calendar year.  Surveys shall be conducted as close as possible to the time when kelp bed canopies cover the greatest area.  The entire San Diego Region coastline, from the international boundary to the San Diego Region / Santa Ana Region boundary, shall be photographed on the same day.

- The images produced by the surveys shall be presented in the form of a 1:24,000 scale photo-mosaic of the entire San Diego Region coastline.  Onshore reference points, locations of all ocean outfalls and diffusers, and the 30-foot (MLLW) and 60-foot (MLLW) depth contours shall be shown.

- The areal extent of the various kelp beds photographed in each survey shall be compared to that noted in surveys of previous years.  Any significant losses which persist for more than one year shall be investigated by divers to determine the probable reason for the loss.

# G. 	Aliso Creek Monitoring

- The stream flow diversion from Aliso Creek to the AWMA Ocean Outfall shall be monitored for the following:

Parameter			Unit			Type of Sample		Frequency

Flowrate			MGD			recorder/totalizer		continuous

CBOD5 @ 20C		mg/L			24-hour composite		daily2

Suspended Solids		mg/L			24-hour composite		daily2

pH				pH units		grab 				daily2

Total and Fecal Coliform	#/100mL		grab				weekly

Acute Toxicity		TUa			24-hour composite		once every

two months13

I, John H. Robertus, Executive Officer of the San Diego Regional Water Quality Control Board, do hereby certify the foregoing is a full, true, and correct copy of Order No. 2001-08 adopted by the California Regional Water Quality Control Board, San Diego Region, on February 21, 2001.

_        ______________

JOHN H. ROBERTUS

Executive Officer

MONITORING AND REPORTING PROGRAM ENDNOTES

For samples collected from the various treatment plants which are to be physically composited prior to analysis or for the results of analyses which are to be arithmetically composited, the basis for compositing shall be the rate of discharge from the various plants to the ocean, not the rate of inflow to the various plants.

Five days per week except seven days per week for at least one week in July or August of each year.

The minimum frequency of monitoring for this constituent shall be automatically increased to twice the minimum frequency specified here if any analysis for this constituent yields a result higher than the effluent limit specified in Order No. 2001-08 for this constituent.  The increased minimum frequency of monitoring shall remain in effect until the results of a minimum of four consecutive analyses for this constituent are below all effluent limits specified in Order No. 2001-08 for this constituent.

The minimum frequency of monitoring for this constituent is automatically reduced to semiannually if the results of twelve consecutive analyses, representing each month of the year, or the results of twenty-four consecutive analyses, representing each quarter of the year, are below the Ocean Plan 6-month median water quality objective for this constituent, or below the laboratory MDL for this constituent in the matrix being analyzed, whichever is higher.

The discharger may at its option monitor for total chromium.  If the measured total chromium concentration exceeds the hexavalent chromium limitation, it will be assumed that the hexavalent chromium limitation was exceeded unless the results of a hexavalent chromium analysis of a replicate sample indicate otherwise.  When analyzing for hexavalent chromium, the appropriate sampling and analytical method must be used (i.e., 24-hour composite cooled to 4 C and analyzed within 24 hours).

Monitoring of Total Chlorine Residual is not required on days when none of the treatment facilities that are subject to Order No. 2001-08 use chlorine for disinfection.  If only one sample is collected for total Chlorine residual analysis on a particular day, that samples must be collected at the time when the concentration of total chlorine residual in the discharge would be expected to be greatest.  The times of chlorine discharges on the days the samples are collected, and the time at which samples are collected shall be reported.

A screening period for chronic toxicity shall be conducted every other year for three months, using a minimum of three test species (one plant, one invertebrate, and one vertebrate) chosen from the list of approved chronic toxicity test protocols specified in the 1997 Ocean Plan.  After the screening period, the most sensitive species (i.e. the species exhibiting the lowest NOEL) shall be used for the monthly testing.  Repeat screening periods may be terminated after the first month if the most sensitive species is the same as the species previously found to be most sensitive.

EPA method 8280 shall be used to analyze for TCDD equivalents.

? 	At the surface.

If the discharger demonstrates to the satisfaction of the Executive Officer, by means of daily analyses, that the concentrations of total and fecal coliform bacteria in the effluent are consistently less than 1,000 per 100 mL, enterococcus monitoring may be suspended.  The discharger shall conduct the monitoring as specified unless the Executive Officer provides written authorization to suspend it.  If this monitoring is suspended, the discharger shall resume it at the request of the Executive Officer.

At surface and mid-depth.

At surface, mid-depth, and bottom.

MONITORING AND REPORTING PROGRAM ENDNOTES (cont.)

13	Sampling shall be conducted each year a diversion is initiated.  Sampling shall be conducted within the first three days following initiation of Aliso Creek diversion into the AWMA Ocean Outfall for the season and once every two months thereafter until the seasonal diversion ends.