# Attachment C

# 1994 WATER QUALITY CONTROL PLAN FOR THE SAN DIEGO BASIN

# (BASIN PLAN)

# WASTE DISCHARGE PROHIBITIONS

California Water Code Section 13243 provides that a Regional Board, in a water quality control plan, may specify certain conditions or areas where the discharge of waste, or certain types of waste is not permitted.  The following discharge prohibitions are applicable to any person as defined by Section 13050(c) of the California Water Code and to any person who is a citizen, domiciliary, or political agency or entity of California whose activities in California could affect the quality of waters of the state within the boundaries of the San Diego Region.

- The discharge of waste to waters of the state in a manner causing, or threatening to cause, a condition of pollution, contamination, or nuisance as defined in California Water Code Section 13050, is prohibited.

- The discharge of waste to land, except as authorized by waste discharge requirements or the terms described in California Water Code Section 13264, is prohibited.

- The discharge of pollutants or dredged or fill material to waters of the United States except as authorized by a NPDES permit or a dredged or fill material permit (subject to the exemption described in California Water Code §13376) is prohibited

4.	Discharges of recycled water to lakes or reservoirs used for municipal water supply or to inland surface water tributaries thereto are prohibited, unless this Regional Board issues a NPDES permit authorizing such a discharge; the proposed discharge has been approved by the State Department of Health Services and the operating agency of the impacted reservoir; and the discharger has an approved fail-safe long-term disposal alternative.

5.	The discharge of waste to inland surface waters, except in cases where the quality of the discharge complies with applicable receiving water quality objectives, is prohibited.  Allowances for dilution may be made at the discretion of the Regional Board.  Consideration would include streamflow data, the degree of treatment provided, and safety measures to ensure reliability of facility performance.  As an example, discharge of secondary effluent would probably be permitted if streamflow provided 100:1 dilution capability.

- The discharge of waste in a manner causing flow, ponding, or surfacing on lands not owned or under the control of the discharger is prohibited, unless the discharge is authorized by the Regional Board.

7.	The dumping, deposition, or discharge of waste directly into waters of the state, or adjacent to such waters in any manner which may permit its being transported into the waters, is prohibited unless authorized by the Regional Board.

8.	Any discharge to a storm water conveyance system that is not composed entirely of storm water is prohibited unless authorized by the Regional Board.  (The federal regulations, 40 CFR 122.26(b)(13), define storm water as storm water runoff, snow melt runoff, and surface runoff and drainage.  40 CFR 122.26(b)(2) defines an illicit discharge as any discharge to a storm water conveyance system that is not composed entirely of storm water except discharges pursuant to a NPDES permit and discharges resulting from fire fighting activities.) (§122.26 amended at 56 FR 56553, November 5, 1991; 57 FR 11412, April 2, 1992)

- The unauthorized discharge of treated or untreated sewage to waters of the state or to a storm water conveyance system is prohibited.

- The discharge of industrial wastes to conventional septic tank/subsurface disposal systems, except as authorized by the terms described in California Water Code Section 13264, is prohibited.

- The discharge of radioactive wastes amenable to alternative methods of disposal into the waters of the state is prohibited.

- The discharge of any radiological, chemical, or biological warfare agent into waters of the state is prohibited.

- The discharge of waste into a natural or excavated site below historic water levels is prohibited unless the discharge is authorized by the Regional Board.

- The discharge of sand, silt, clay, or other earthen materials from any activity, including land grading and construction, in quantities which cause deleterious bottom deposits, turbidity, or discoloration in waters of the state or which unreasonably affect, or threaten to affect, beneficial uses of such waters is prohibited.

- The discharge of treated or untreated sewage from vessels to Mission Bay, Oceanside Harbor, Dana Point Harbor, or other small boat harbors is prohibited.

- The discharge of untreated sewage from vessels to San Diego Bay is prohibited.

- The discharge of treated sewage from vessels to portions of San Diego Bay that are less than 30 feet deep at mean lower low water (MLLW) is prohibited.

- The discharge of treated sewage from vessels, which do not have a properly functioning US Coast Guard certified Type I or Type II marine sanitation device, to portions of San Diego Bay that are greater than 30 feet deep at mean lower low water (MLLW) is prohibited.