# CALIFORNIA REGIONAL WATER QUALITY CONTROL BOARD

##### SAN DIEGO REGION

TENTATIVE ORDER NO. 2000-211

NPDES PERMIT NO. CA0109100

WASTE DISCHARGE REQUIREMENTS

FOR

DISCHARGES FROM NIELSEN & BEAUMONT MARINE

SAN DIEGO COUNTY

### Table of Contents

Findings											2

# A.	Prohibitions										7

# B.	Discharge Specifications									7

C.	Receiving Water Limitations								7

D.	Storm Water Discharge Requirements							9

E.	Best Management Practices (BMP) Plan							10

F.	Provisions											10

G.	Reporting Requirements									10

H.	Notifications										11

# Attachment A (Storm Water Pollution Prevention Plan)					12

Attachment B (Definitions)									16

Attachment C (Basin Plan Waste Discharge Prohibitions)					18

Attachment D (Regional Board Standard Provisions)					20

Monitoring and Reporting Program No. 2000-211

#### A.	Monitoring Provisions								33

B.	Compliance Certifications							37

C	Spill / Illicit Discharge Log							38

D.	Chemical Utilization Audit							39

E.	Tributyltin Log									40

F.	Waste Hauling Log								41

G.	Storm Water and Non-Storm Water Monitoring					41

H.	Toxicity Reduction Evaluation							46

I.	Sediment Chemistry Monitoring							48

J.	Sediment Monitoring Station Locations						55

# CALIFORNIA REGIONAL WATER QUALITY CONTROL BOARD

SAN DIEGO REGION

TENTATIVE ORDER NO. 2000-211

NPDES PERMIT NO. CA0109100

WASTE DISCHARGE REQUIREMENTS

FOR

DISCHARGES FROM NIELSEN & BEAUMONT MARINE

## SAN DIEGO COUNTY

The California Regional Water Quality Control Board, San Diego Region, (hereinafter Regional Board) finds that:

1.	Boatyards and similar facilities that discharge or have the potential to discharge waste to waters of the State and pollutants from point sources to waters of the United States must be regulated by a National Pollutant Discharge Elimination System (NPDES) Permit.  Similar facilities are facilities that conduct maintenance and repair activities similar to boatyards and generate similar wastes.  40 CFR 122.26, Storm Water Discharges, requires dischargers to obtain a NPDES permit if their discharge is associated with industrial activity.  The same section further states that facilities classified as Standard Industrial Classification (SIC) 373 are categorized as facilities considered to be engaging in "industrial activity" for purposes of that section.  SIC 373 applies to the following industry group: Ship and Boat Building and Repairing.

2.	Nielsen & Beaumont Marine, (hereinafter discharger) generates industrial process water.  Industrial process water is defined as any water which, during manufacturing or processing, comes into direct contact with or results from the production or use of any raw material, intermediate product, finished product, by-product, or waste product.

3.	On May 16, 1974, the State Water Resources Control Board (State Board) adopted the Water Quality Control Policy for the Enclosed Bays and Estuaries of California.  This policy prohibits the discharge of industrial process water to the enclosed bays and estuaries of the State unless it can be shown that such discharge would enhance the quality of the receiving water above that which would occur in the absence of the discharge.  There is insufficient evidence to conclude that industrial process water discharges from boatyards and similar facilities would enhance the quality of receiving waters.  Therefore, this Order prohibits the discharge of industrial process water to surface waters.

4.	Storm water monitoring data submitted from boatyards in the San Diego Region indicate that high levels of toxic pollutants, particularly copper and zinc, have been / are being discharged to surface waters.  It is generally accepted that storm water runoff from rainfall will mobilize solid particles located on the surfaces receiving the precipitation.  Generally, most municipal sanitation agencies will accept a "first-flush" (as defined in Attachment B) of polluted water from the facility's maintenance and repair areas into the municipal sanitary sewer system under an industrial discharge permit.  Therefore, this Order requires the termination of a "first-flush" of polluted water to surface waters.

5.	Section 402(p) of the Clean Water Act as amended by the Water Quality Act of 1987 and the related regulations published by the USEPA on November 16, 1990 (40 CFR Parts 122, 123, and 124) require persons who own or operate facilities engaged in building and repairing ships or boats (Standard Industrial Classification Group No. 373) to obtain a NPDES permit if they have an actual or threatened storm water discharge.  The State Board adopted a General Industrial Storm Water Permit on November 19, 1991, Order No. 91-13-DWQ, to regulate the discharge of storm water from industrial activities.  Prior to being regulated under Order No. 95-41, the San Diego Regional boatyards were regulated under the provisions of the statewide general industrial storm water permit.  Order No. 95-41 included requirements for the discharge of storm water.  After the adoption of Order No. 95-41 on October 12, 1995, the boatyards were authorized to terminate their coverage under the statewide general industrial storm water permit.

6.	On October 12, 1995, this Regional Board adopted Order No. 95-41, NPDES permit No. CAG719001, General Waste Discharge Requirements for Discharges from Boatyards and Similar Facilities Located Within the San Diego Region, which superseded Order No. 85-03 (NPDES Permit No. CA0107719).  40 CFR 122.28 provides for the issuance of General NPDES permits to regulate discharges of waste which result from similar operations, discharge similar types of waste, require similar effluent limitations, require similar monitoring, and may be more appropriately regulated under a general NPDES permit rather than individual permits.  As the San Diego Regional Boatyards met the requirements of 40 CFR 122.28, and in order to conserve Regional Board staff resources, Order No. 95-41 was issued as a general permit for discharges from boatyards and similar facilities within the region.  Order No. 95-41 contained an expiration date of October 12, 2000.

7.	On April 8, 1998, this Regional Board adopted Technical Change Order (TCO) No. 1 to Monitoring and Reporting Program (MRP) No. 95-41.  The monitoring and reporting program that was adopted with Order No. 95-41 did not contain sediment monitoring requirements due to disagreement among the interested parties on how the monitoring should be conducted.  TCO No. 1 to MRP No. 95-41 resulted from extensive efforts from all parties between October 12, 1995 and April 8, 1998 to develop an effective and feasible sediment monitoring program.  A brief history of the development of the boatyard monitoring program is contained in Item No. 9, Supporting Document No. 2, of the April 8, 1998 Regional Board meeting agenda.

8.	On November 18, 1998, this Regional Board adopted Addendum No. 1 to Order No. 95-41.  Originally, Order No. 95-41, Section B, Performance Goal/Discharge Specification, contained an acute toxicity performance goal for discharges of storm water runoff to surface waters.  The performance goal was effective until October 12, 1998, after which it was to become a discharge specification of the Order (i.e., a requirement rather than a goal).  Subsequent storm water sampling and analyses conducted during the winter of 1997/1998 indicated that the acute toxicity performance goal had not been achieved.  By letter dated June 4, 1998, the San Diego Unified Port District requested a postponement of the effective date for the transition to the discharge specification.  Addendum No. 1 to Order No. 95-41 revised the effective date of the discharge specification from October 12, 1998 to October 12, 1999.  In order to comply with the discharge specification, the majority of the San Diego Regional boatyards have implemented some form of storm water containment system to assist in reducing the occurrence of storm water runoff discharges from their facilities.  As of the date of preparation of this Order, the effectiveness of the containment strategy to reduce the acute toxicity of storm water runoff discharges to surface waters has not been demonstrated by any of the regional boatyards.

9.	On June 5, 2000, the discharger submitted an application for the renewal of its NPDES permit pursuant to Order No. 95-41, Standard Provision No. A.2.a, Duty to Reapply.  Order No. 95-41 expires on October 12, 2000.  Upon adoption, this Order will supersede Order No. 95-41 and will serve as its renewal.

10.	Nielsen & Beaumont Marine is located at 2420 Shelter Island Drive, San Diego, adjacent to San Diego Bay.  The facility conducts various boat maintenance and repair activities, including exterior/interior surface coating application/removal (e.g., painting and sanding), hull cleaning, engine repair, and general mechanical/fixture repair.  The facility’s property is owned by the San Diego Unified Port District.

11.	As applicable, the requirements contained in this Order apply to the discharger’s entire facility/leasehold including pier, float, dock, or other areas where work may be conducted by the discharger’s personnel, contractors, tenants, or otherwise, and may occur directly over or on the receiving water.

12.	The Comprehensive Water Quality Control Plan for the San Diego Basin (9) (Basin Plan) was adopted by the Regional Board on March 17, 1975.  Subsequent revisions to the Basin Plan have also been adopted by this Regional Board and approved by the State Board.  The Basin Plan designates beneficial uses, narrative and numerical water quality objectives, and prohibitions which are applicable to the discharges regulated under this Order.  The beneficial uses for the regional bays and harbors are listed in Findings 13, 14, and 15 below.

13.	The beneficial uses of San Diego Bay are:

a.	Industrial Service Supply (IND)

b.	Navigation (NAV)

c.	Contact Water Recreation (REC-1)

d.	Non-contact Water Recreation (REC-2)

e.	Commercial and Sport Fishing (COMM)

f.	Preservation of Biological Habitats of Special Significance (BIOL)

g.	Estuarine Habitat (EST)

h.	Wildlife Habitat (WILD)

i.	Rare, Threatened, or Endangered Species (RARE)

j.	Marine Habitat (MAR)

k.	Migration of Aquatic Organisms (MIGR)

l.	Shellfish Harvesting (SHELL)

14.	The beneficial uses of Mission Bay are:

a.	Industrial Service Supply (IND)

b.	Contact Water Recreation (REC-1)

c.	Non-contact Water Recreation (REC-2)

d.	Commercial and Sport Fishing (COMM)

e.	Estuarine Habitat (EST)

f.	Wildlife Habitat (WILD)

g.	Rare, Threatened, or Endangered Species (RARE)

h.	Marine Habitat (MAR)

i.	Migration of Aquatic Organisms (MIGR)

j.	Shellfish Harvesting (SHELL)

15.	The beneficial uses of Dana Point Harbor and Oceanside Harbor are:

a.	Industrial Service Supply (IND)

b.	Navigation (NAV)

c.	Contact Water Recreation (REC-1)

d.	Non-contact Water Recreation (REC-2)

e.	Commercial and Sport Fishing (COMM)

f.	Wildlife Habitat (WILD)

g.	Rare, Threatened, or Endangered Species (RARE)

h.	Marine Habitat (MAR)

i.	Migration of Aquatic Organisms (MIGR)

j.	Spawning, Reproduction, and/or Early Development (SPWN)

k.	Shellfish Harvesting (SHELL)

16.	This Regional Board, in establishing the requirements contained herein, considered factors including, but not limited to, the following:

a.	Past, present, and probable future beneficial uses of 	water;

b.	Environmental characteristics of the hydrologic unit under consideration, including the quality of water available thereto;

c.	Water quality conditions that could reasonably be achieved through the coordinated control of all factors which affect water quality in the area;

d.	Economic considerations;

e.	The need for developing housing within the region;

f.	The need to develop and use recycled water;

g. 	Beneficial uses to be protected and the water quality objectives reasonably required for that purpose;

h.	The need to prevent nuisance; and

i.	Other waste discharges.

17.	This NPDES permit is consistent with the antidegradation provisions of 40 CFR 131.12 and State Board Resolution No. 68-16, Statement of Policy with Respect to Maintaining High Quality of Waters in California (known collectively as "antidegradation" policies).  Existing and potential beneficial uses will be maintained.

18.	On June 17, 1999, the State Water Resources Control Board adopted the Consolidated Toxic Hot Spot Cleanup Plan (Consolidated Plan) required under the Bay Protection and Toxic Cleanup Program (CWC Section 13395).  The Consolidated Plan listed known toxic hot spots, including several located in San Diego Bay.  The Consolidated Plan also requires regional boards to reevaluate waste discharge requirements for those discharges associated with each known toxic hot spot that can reasonably be expected to cause or contribute to the creation and maintenance of the known toxic hot spot.  The San Diego Regional Board finds that the discharges from Southbay Boat Yard and Knight & Carver Yachtcenter may contribute to the pollution present at a toxic hot spot listed in the Consolidated Plan (Seventh Street Channel/Paleta Creek, Naval Station).  More information on the Consolidated Plan can be accessed by visiting the San Diego Regional Water Quality Control Board’s internet site at http://www.swrcb.ca.gov/rwqcb9, selecting the “Select a Link” drop-down, and then selecting “Bay Cleanup”.

19.	The issuance of this NPDES permit is exempt from the requirements for the preparation of environmental documents under the provisions of the California Environmental Quality Act (Public Resources Code, Section 21000, et seq.) in accordance with California Water Code, Section 13389.

20.	Discharge criteria established pursuant to Sections 301, 302, 304, 307, 308, and 403 of the Clean Water Act are applicable to discharges regulated by this permit.

21.	This Order shall serve as a NDPES Permit pursuant to Section 402 of the Clean Water Act and amendments thereto.

22.	This Order regulates the discharge of pollutants associated with boatyards and similar facilities to surface waters.  It does not preempt or supersede the authority of municipalities, flood control agencies, or other local agencies to prohibit, restrict, or control discharges of storm water to storm drain systems, sanitary sewer systems, or other water courses subject to their jurisdiction.

23.	The Regional Board has notified interested agencies and persons of its intent to reissue waste discharge requirements for boatyards and similar facilities and has provided them with an opportunity to submit their written views and recommendations.

24.	The Regional Board in a public hearing, heard and considered all comments pertaining to waste discharge requirements for discharges from boatyards and similar facilities.

IT IS HEREBY ORDERED that Nielsen & Beaumont Marine, in order to meet the provisions contained in Division 7 of the California Water Code and regulations adopted thereunder and the provisions of the Clean Water Act as amended, and regulations and guidelines adopted thereunder, shall comply with the following:

A.	PROHIBITIONS

1.	Compliance with the Basin Plan prohibitions listed in Attachment C is required as a condition of this Order.  However, for Basin Plan prohibitions 15 through 18, a discharger will achieve compliance by providing written notification to the owner/operator of each vessel of the owner’s/operator’s obligation to comply with Basin Plan prohibitions 15 through 18.  Notification shall be made within 24 hours of the time a vessel is moved into the leasehold of a discharger and annually thereafter for as long as the vessel is located in the leasehold.  The written notification shall include a list of contractors that provide sewage disposal services if the discharger cannot provide on-site disposal services at its facilities.  See Monitoring and Reporting Program, Section B.1.e, Compliance Certification, for guidelines regarding compliance with this prohibition.

2.	The discharge of industrial process water, as defined in Finding No. 2, is prohibited upon adoption of this Order.

3.	The discharge of first-flush storm water runoff, as defined in Attachment B, from the discharger's maintenance and repair areas is prohibited subject to Section D of this Order.

B.	DISCHARGE SPECIFICATIONS

1.	The acute toxicity of storm water runoff to surface waters or to storm drains shall not be less than seventy (70) percent survival as determined by a 96-hour bioassay.

C.	RECEIVING WATER LIMITATIONS

Discharges from boatyard and similar facilities shall not cause or contribute to violations of the following water quality objectives:

1.	Waters shall be free of coloration that causes nuisances or adversely affects beneficial uses.

2.	Waters shall not contain floating material, including solids, liquids, foams, and scum, in concentrations that cause nuisance or adversely affect beneficial uses.

3.	The discharge of wastes shall not cause concentrations of un-ionized ammonia (NH3) to exceed 0.025 mg/L (as N).

4.	Waters shall not contain suspended and settleable solids in concentrations that cause nuisance or adversely affect beneficial uses.

5.	Waters shall not contain oils, greases, waxes, or other materials in concentrations that result in a visible film or coating on the surface of the water or on objects in the water, or that cause nuisance or otherwise adversely affect beneficial uses.

6.	The suspended sediment load and suspended sediment discharge rate to surface waters shall not be altered in such a manner as to cause nuisance or adversely affect beneficial uses.

7.	Water shall be free of changes in turbidity that cause nuisance or adversely affect beneficial uses.  In addition, within San Diego Bay, the transparency of bay waters, insofar as it may be influenced by any controllable factor, either directly or through induced conditions, shall not be less than 8 feet in more than 20 percent of the readings in any zone, as measured by a standard Secchi disk.  Wherever the water is less than 10 feet deep, the Secchi disk reading shall not be less than 80 percent of the depth in more than 20 percent of the readings in any zone.

8.	Waters shall not contain taste or odor-producing substances in concentrations that impart undesirable tastes or odors to fish flesh or other edible products of aquatic origin, or that cause nuisance or adversely affect beneficial uses.

9.	Waters shall not contain biostimulatory substances in concentrations that promote aquatic growth to the extent that such growths cause nuisance or adversely affect beneficial uses.

10.	The annual mean dissolved oxygen concentrations shall not be less than 7 mg/L more than 10 percent of the time.

11.	The pH shall not be depressed below 7.0 nor raised above 9.0 in bays and estuaries and shall not be depressed below 6.5 nor raised above 8.5 in inland surface waters.  Changes in normal ambient pH levels shall not exceed 0.2 units from that which occurs naturally.

12.	The fecal coliform concentration shall not exceed a log mean of 200 MPN/100 mL based on a minimum of not less than two samples for any 30-day period.  During any 30-day period, not more than 10 percent of the samples shall exceed 400 MPN/100 mL.  However, if the discharge is in the vicinity where shellfish may be harvested for human consumption, the limits are 70 MPN/100 mL and 230 MPN/100 mL respectively.

13.	In San Diego Bay, where bay waters are used for whole fish handling, the density of E Coli shall not exceed 7 per mL in more than 20 percent of any 20 daily consecutive samples of bay water.

14.	In waters designated for contact recreation, the enterococci limit is 35 colonies per 100 mL.

15.	Radionuclides shall not be present in concentrations that are deleterious to human, plant, animal, or aquatic life or that result in the accumulation of radionuclides in the food web to an extent that presents a hazard to human, plant, animal, or aquatic life.

16.	No individual pesticide or combination of pesticides shall be present in the water column, sediments, or biota in concentrations that adversely affect beneficial uses.  Pesticides shall not be present at levels which will bioaccumulate in aquatic organisms to levels which are harmful to human health, wildlife, or aquatic organisms in the water column, sediments, or biota.

17.	Waters shall be maintained free of toxic substances in concentrations that are toxic to human, plant, animal, or aquatic life.  The survival of aquatic life in the area of the discharge shall not be impacted by the discharge.  Compliance with this objective will be determined by use of indicator organisms, analysis of species diversity, population density, growth anomalies, bioassays of appropriate duration, or other appropriate methods as specified by the Regional Board.

D.	STORM WATER DISCHARGE REQUIREMENTS

1.	The discharger shall eliminate the discharge of the first-flush storm water runoff, as defined in Attachment B, from their facility maintenance and repair areas to surface waters or storm drains, and shall appropriately maintain all means by which this is accomplished.

2.	The discharger shall develop and implement a Storm Water Pollution Prevention Plan (SWPPP).  The requirements for developing a SWPPP are listed in Attachment A.  The SWPPP shall be incorporated into the dischargers Best Management Practices (BMP) Plan as directed in Section E below.

3.	Treatment systems and related collection and conveyance facilities shall be constructed and maintained to prevent the discharge of pollutants to surface waters during and after collection in those facilities.

4.	Appropriate means, such as but not limited to berms, shall be used to isolate the facility's maintenance/repair area(s) to prevent storm water run-on from commingling with the storm water discharge from the maintenance/repair area, and to prevent storm water runoff to offsite areas.

## E.	BEST MANAGEMENT PRACTICES (BMP) PLAN

1.	The discharger shall develop, implement, and maintain a BMP plan in accordance with 40 CFR 125.100 through 125.104, and that is consistent with the general guidance contained in the Guidance Manual for Developing Best Management Practices (BMPs), USEPA, October 1993, EPA 833-B-93-004, or any subsequent editions.  See Monitoring and Reporting Program No. 2000-211, Section H.1.c for publication ordering information.  The BMP plan shall incorporate the SWPPP provisions in Attachment A to prevent and/or minimize the potential for the release of toxic and hazardous pollutants to surface waters.  The discharger shall amend its BMP plan in accordance with 40 CFR 125.100 through 125.104 whenever there is a change in facility/leasehold/work area design, construction, operation, or maintenance which materially affects the potential for discharge of toxic and hazardous pollutant to surface waters.  The BMP Plan shall also address pier, float, dock, or other leasehold areas where work may occur directly over or on the receiving water.  The discharger shall submit its BMP plan and any amendment thereto to the Executive Officer.

## F.	PROVISIONS

1.	The discharger shall comply with all items of Attachment D, Standard Provisions,  which are part of this Order.

2.	A copy or copies of this Order and the discharger’s BMP Plan shall be kept at a prominent location and shall be readily accessible to on-site personnel at all times.

3.	40 CFR 122.5, 122.21, 122.22, 122.41, and 122.61-64 are applicable to this Order and are hereby incorporated.

## G.	REPORTING REQUIREMENTS

1.	The discharger shall comply with the attached Monitoring and Reporting Program (MRP) No. 2000-211.  Monitoring results shall be reported at the intervals specified in the MRP.

2.	Except for data determined to be confidential under Title 40, U.S. Code of Federal Regulations, Part 2, all reports prepared in accordance with the terms of this Order shall be available for public inspection at the offices of the Regional Board.  As required by the Clean Water Act, Reports of Waste Discharge (permit applications), this Order, and effluent data shall not be considered confidential.

3.	The discharger shall submit reports required under this Order to:

Executive Officer

California Regional Water Quality Control Board

San Diego Region

9771 Clairemont Mesa Boulevard, Suite A

San Diego, CA 92124-1324

Phone: (858) 467-2952

Fax: (858) 571-6972

## H.	NOTIFICATIONS

1.	This Order expires on October 11, 2005.  However, if all requirements of the federal NPDES regulations on the continuation of expired permits are complied with, the terms and conditions of this Order will continue in force and effect until a new permit is issued or the Regional Board rescinds this permit.  (40 CFR 122.6, 23 CCR 2235.4)

2.	Nothing in this Order shall be construed to preclude the institution of any legal action or relieve the discharger from any responsibilities, liabilities, or penalties to which the discharger is or may be subject under Section 311 of the Clean Water Act.

3.   	Nothing in this Order shall be construed to preclude the institution of any legal action or relieve the discharger from any responsibilities, liabilities, or penalties established pursuant to any applicable State law or regulation under authority preserved by Section 510 of the Clean Water Act.

4.	Except as provided for in 40 CFR 122.41(m) and (n), nothing in this Order shall be construed to relieve the discharger from civil or criminal penalties for non-compliance.

5.	Nothing in this Order shall be construed to protect the discharger from its liabilities under federal, state, or local laws.

6.	This Order shall become effective ten days after the date of its adoption provided the Regional Administrator, U.S. Environmental Protection Agency, has no objection.  If the Regional Administrator objects to its issuance, this Order shall not become effective until such objection is withdrawn.  Order No. 95-41 is rescinded when this Order becomes effective.

I, John H. Robertus, Executive Officer of the San Diego Regional Water Quality Control Board, do hereby certify the foregoing is a full, true, and correct copy of Order No. 2000-211 as adopted by the California Regional Water Quality Control Board, San Diego Region, on October 11, 2000.

TENTATIVE________________________

John H. Robertus

Executive Officer