CALIFORNIA REGIONAL WATER QUALITY CONTROL BOARD

SAN DIEGO REGION

ORDER NO. 98-60

NPDES PERMIT NO.  CA0107492

WASTE DISCHARGE REQUIREMENTS

FOR

PADRE DAM MUNICIPAL WATER DISTRICT

PADRE DAM WATER RECYCLING FACILITY

DISCHARGE TO

SYCAMORE CREEK AND THE SAN DIEGO RIVER

SAN DIEGO COUNTY

The California Regional Water Quality Control Board (hereinafter Regional Board), finds

that:

1.	On June 21, 1993, this Regional Board adopted Order No. 93-48, National Pollutant

Discharge Elimination System Permit No. CA0107492, Waste Discharge Requirements for

Padre Dam Municipal Water District, Santee Water Reclamation Plant Discharge to

Sycamore Creek and the San Diego River, San Diego County.  Order No. 93-48 established

requirements for the discharge of up to 2.0 million gallons per day (MGD) of treated

sewage from the Padre Dam Water Recycling Facility (PDWRF), which was formerly

called the Santee Water Reclamation Plant, through the Santee Lakes, to Sycamore Creek

and the San Diego River.  Order No. 93-48 contains an expiration date of June 21, 1998.

2. Pursuant to Reporting Requirement E.14 of Order No. 93-48, PDMWD was required to 			submit their report of waste discharge 180 days prior to the June 21, 1998 expiration date. 		On April 1, 1998 PDMWD submitted an application to renew its NPDES permit. The 			report of waste discharge included a request for changes to the NPDES permit.  Because 			the application was submitted late, this Regional Board could not consider the requested 			changes at the time of adoption of this permit renewal.

3.	PDMWD discharges effluent from the PDWRF into a series of seven man made lakes

known as the Santee Lakes, a recreational facility owned and operated by PDMWD.  These 	artificial lakes are not waters of the United States.  Reclaimed wastewater from the lakes

occasionally overflows from Lake No. 1 into Sycamore Creek, a tributary of the San Diego

River.  Sycamore Creek and the San Diego River are both waters of the United States.

4.	PDMWD owns and operates the PDWRF located in Sycamore Canyon in Santee, 				California (Latitude- 32 Deg. 50 Min. 30 Sec. North, Longitude- 117 Deg. 00 Min. 10 Sec. 		West).  Wastewater goes through primary sedimentation, Bardenpho biological nutrient 			removal activated sludge, secondary clarification, alum and polymer chemical addition,

flocculation and sedimentation, denitrifying filtration, chlorine contact and dechlorination,

Order No. 98-60                   			-2-

before being discharged to Lake No. 7.  The State Department of Health Services has

determined that this treatment train provides treatment and reliability equivalent to that specified for "filtered wastewater" as defined in Title 22, Division 4, of the California Code of Regulations.

5.	PDMWD can regulate the influent flow to the PDWRF . The remainder of the District's untreated sewage flow is diverted to the San Diego metropolitan sewerage system for treatment at the E.W. Blom Point Loma Metropolitan Wastewater Treatment Plant.  PDMWD has a contractual agreement with the City of San Diego to discharge up to 6.22 MGD to the metro system.

6.	The Regional Board, acting in accord with Section 13244 of the California Water Code, 	adopted the Water Quality Control Plan for the San Diego Basin (9), (hereinafter Basin Plan) 	on September 8, 1994.  The Basin Plan was subsequently approved by the State Water 	Resources Control Board (hereinafter State Board) on December 13, 1994.  Subsequent 	revisions to the Basin Plan have also been adopted by the Regional Board and approved by 	the State Board.  The Basin Plan contains beneficial uses and water quality objectives.  The 	requirements of this Order are consistent with the Basin Plan.

7.	The discharge point is located in the Sycamore Canyon portion of the San Diego River watershed.  The Basin Plan identifies the following beneficial uses of the San Diego River watershed downgradient of the discharge point:

a.	agricultural supply

b.	industrial service supply

c.		contact and non-contact water recreation

d.		warm freshwater habitat

e.		cold freshwater habitat

f .   wildlife habitat

g.	preservation of rare, threatened or endangered species

8. 	In order to protect designated beneficial uses, the Basin Plan establishes water quality objectives (for bacteriological, physical, chemical, and biological characteristics, and for radioactivity), general requirements for management of waste discharged to the inland surface waters, quality requirements for waste discharges (effluent quality requirements), discharge prohibitions, and general provisions.  The Basin Plan also contains prohibitions applicable to surface waters subject to tidal influence and for inland surface waters.  The applicable prohibitions and discharge provisions of the Basin Plan have been incorporated herein.

9.	This Order complies with Section 402(o) of the Federal Clean Water Act, and the 	implementing regulations of 40 CFR 122.44(l) which prohibit the establishment of effluent 	limits in a renewed, reissued or modified NPDES permits that are less stringent than the limits 	established in the previous permit.

Order No. 98-60                   			-3-

10.	On November 16, 1990, the United States Environmental Protection Agency promulgated

NPDES permit application requirements for stormwater discharges (40 CFR Parts 122, 123,

and 124) which are applicable to the PDMWD treatment facilities.  On April 17, 1997

the State Water Resources Control Board adopted Water Quality Order No. 97-03-DWQ

National Pollutant Discharge Elimination System (NPDES) General Permit No.

CASOOOOO1 Waste Discharge Requirements for Discharges of Storm Water

Associated Activities Excluding Construction Activities.  Stormwater discharges from

PDMWD's PDWRF are subject to the terms and conditions of Water Quality Order No. 97-

03-DWQ.

11.	Effluent limitations, industrial pretreatment standards, sludge use and disposal regulations, and criteria established under Sections 208(b), 301, 302, 303(d), 304, 306, 307, 403 and 405 of the Clean Water Act, as amended (33 U.S.C. 1251 et seq.), are applicable to the discharge.

12.	The Regional Board, in establishing the requirements contained herein, has taken into

consideration the requirements of the State and Federal antidegradation policies and has determined that:

a.		The terms and conditions of this Order require that the existing beneficial uses and

quality of  Sycamore Creek, the San Diego River, and contiguous waters be maintained

and protected;

b.    The discharge from the PDWRF to the San Diego River and its tributaries in accordance

with approved plans indicated in the findings above, and the requirements of this Order,

is necessary to accommodate social and economic development important to the people

of the communities of the San Diego region and is to the maximum benefit of the people

of the State;

c.	No surface waters covered under the terms and conditions of this Order have been

designated an outstanding national resource water by the Sate Water Resources Control

Board.  No surface waters covered under this Order have been designated as Areas of

Special Biological Significance by the State Water Resources Control Board.  This Order 		prohibits the discharge of wastes to the waters of any ASBS.

13.	This Order shall serve as an NPDES permit for the discharge of treated wastewater

from the PDMWD’s PDWRF to Sycamore Creek, the San Diego River, and contiguous waters pursuant to Section 402 of the CWA, and amendments thereto.

14.	The Regional Board, in establishing the requirements contained herein, considered factors

including, but not limited to, the following:

a.	Beneficial uses to be protected and the water quality objectives reasonably required for that 	purpose;

Order No. 98-60                  			-4-

b.	Other waste discharges;

c.	The need to prevent nuisance;

d.	Past, present, and probable future beneficial uses of the waters under consideration;

e.	Environmental characteristics of the hydrographic unit under consideration, including the 	quality of water available thereto;

f.	Water quality conditions that could reasonably be achieved through the coordinated control 	of all factors which affect water quality in the area;

g.	Economic considerations;

h.	The need for developing housing within the region; and

i.   The need to develop and use recycled water.

15.	The issuance of waste discharge requirements for this discharge is exempt from the

requirement for preparation of environmental documents under the CEQA (Public Resources

Code, Division 13, Chapter 3, Section 21000 et seq.) in accordance with the California Water

Code, Section 13389.

16.	The Regional Board has considered all water resource related environmental factors 	associated with the discharge of treated wastewater from PDMWD’s PDWRF to the San 	Diego River and contiguous waters.

17.	The Regional Board has notified PDMWD and all know interested parties of its intent to 	issue NPDES permit requirements for the proposed discharge of treated wastewater.

18.	The Regional Board has, at a public meeting, heard and considered all comments pertaining -

to the discharge of treated wastewater from the PDWRF to the San Diego River and

contiguous waters.

IT IS HEREBY ORDERED, that Padre Dam Municipal Water District (hereinafter discharger), in order to meet the provisions contained in Division 7 of the California Water Code and regulations adopted thereunder and the Provisions of the Clean Water Act and the regulations adopted thereunder, shall comply with the following for the discharge of wastewater from the PDWRF to Sycamore Creek, the San Diego River and contiguous waters:

A.	PROHIBITIONS

1.	The discharge of waste at point(s) other than the discharge point from Lake No. 1 to Sycamore Creek, which have not been specifically described in the report of waste discharge and for which valid waste discharge requirements are not in force, is prohibited.

2.	The discharge of wastes to areas designated as being of special biological significance by the

State Water Resources Control Board is prohibited.  Discharges shall be located a sufficient

distance from such designated areas to assure maintenance of natural water quality conditions

in these areas.

Order No. 98-60                  			-5-

3.	The discharge of oil, trash or other solids directly to a surface water or in any manner which may permit it to be washed into a surface water is prohibited.

4.	The discharge of waste shall not cause surface erosion or scouring of aquatic substrates.

5.	The discharge of municipal waste sludge directly to the San Diego River and/or its tributaries, 	or into a waste stream that discharges to the San Diego River and/or its tributaries is

prohibited.

6.	The by-passing of untreated wastes to the San Diego River and/or its tributaries, except as authorized in Provision D. 22 of this Order, is prohibited.

7.	Discharge to the San Diego River and contiguous waters from the PDWRF at average daily

flowrate in excess of 2.0 MGD is prohibited unless the discharger obtains revised waste

discharge requirements authorizing an increased flowrate.  This flowrate limitation may be

revised by the Regional Board at any time if it is shown that the discharge of freshwater from  	PDWRF adversely affect the estuary.  The Regional Board may revise this flowrate limitation 	to make it more or less stringent.

8.	The discharge shall not:

(a)	Cause the presence of coliform or pathogenic organisms in waters pumped from the

affected basins;

(b)	Cause the occurrence of objectionable tastes and odors in waters pumped from the

affected basin;

(c)	Cause waters pumped from the affected basins to foam;

(d)		Cause the presence of toxic materials in waters pumped from the affected basins;

(e)	Cause concentrations of any contaminant in excess of the maximum contaminant level

specified in Title 22, Chapter 15, Article 4, Section 64435, Tables 2 and 4 of the

California Code of Regulations in waters pumped from the affected basins;

(f)	Cause the pH of waters pumped from the affected basins to fall below 6.0 or rise above

9.0;

(g)	Cause this Regional Board's groundwater objectives as established in the Basin Plan, to

be exceeded;

(h)	Cause a nuisance or adversely affect beneficial uses for the surface waters, as

established in the Basin Plan;

Order No. 98-60                  			-6-

(i)	Cause odors, septicity, mosquitoes or other vectors, weed growth or other nuisance

conditions in the San Diego River and/or its tributaries;

(j)	Cause concentrations of unionized ammonia (NH3) to exceed 0.025 mg/l as (N) in

receiving waters.

9.	The discharge of any substances in concentrations toxic to human, animal, plant or aquatic life 	is prohibited.

10.	The discharger shall not cause a violation of the waste discharge prohibitions contained in the 	Basin Plan (listed in Attachment No. 1 to this Order).

B.  DISCHARGE SPECIFICATIONS

1.	The discharge of treated wastewater from the PDWRF and Lake No. I to Sycamore Creek, the San Diego River, and contiguous waters containing pollutants in excess of the following effluent limitations is prohibited:

Order No. 98-60                  			-7-

a.	BASIN PLAN-EFFLUENT LIMITATIONS FOR GENERAL WASTE CONSTITUENTS

Constituent		Units	30-Day		Weekly	Daily

Average		Average	Maximum

------------------------------------------------------------------------------------------------------------------------

settleable solids		ml/L	0.1		--		0.2

biological oxygen		mg/L	15		23		25

demand(5-d,20 deg. C)	lb/d	250		375		417

total suspended solids		mg/L	15		23		25

lb/d	250		375		417

percent sodium		%	--		--		60

total nitrogen

April 1-Sept. 30		mg/L	3.0		--		5.0

lb/d	50		--		83.2

Oct. 1- Mar. 31		mg/L	6.0		--		10.0

lb/d	100		--		166.4

total phosphorous

April 1-Sept. 30		mg/L	0.5		--		0.8

lb/d	8.3		--		13.3

Oct. 1 -Mar. 31		mg/L	1.0		--		1.6

lb/d	16.6		--		26.6

MBAS		mg/L	--		--		0.5

lb/d	--		--		8.3

fluoride		mg/L	--		--		1.0

lb/d	--		--		16.6

hydrogen sulfide		ug/L	2		4		10

lb/d	0.0334		0.0667		0.1668

Order No. 98-60                  			-8-

a.	BASIN PLAN-EFFLUENT LIMITATIONS FOR GENERAL WASTE CONSTITUENTS-Continued

Constituent		Units		30-Day		Weekly	Daily

Average		Average	Maximum

------------------------------------------------------------------------------------------------------------------------

chlorine residual		ug/L		2.0		8.0		20

lb/d		0.0334		0.1334		0.3336

boron		mg/L		--		--		1.0

lb/d		--		--		16.6

sulfate		mg/L		--		--		500

lb/d		--		--		8320

chloride		mg/L		--		--		400

lb/d		--		--		6672

manganese		mg/L		--		--		1.0

lb/d		--		--		16.6

odor		units		none		none		none

color		units		20		--		40

total dissolved solids	mg/L		--		--		1500

lb/d		--		--		24960

oil and grease		mg/L		5			--		7.5

lb/d		83.2		--		124.8

iron		mg/l		--		--		1.0

lb/d	                 --		--		16.6

Order No. 98-60                   		    	-9-

b.	EFFLUENT LIMITATIONS FOR PROTECTION OF FRESHWATER AQUATIC LIFE

Constituent		Units		4-day	Daily		1 -hour		Instant-

Average	Average	Average	aneous

Max.

------------------------------------------------------------------------------------------------------------------------

arsenic		ug/L		190	--		360		--

cadmium		ug/L		b	--		b		--

chlordane		ng/L		--	4.3		--		--

chromium (VI)a		ug/L		11	--		16		--

chromium    (III)		ug/L		509	--		4270		--

cyanide		ug/L		5.2	--		22		--

copper		ug/L		c	--		c		--

DDT		ng/L		--	1.0		--		--

dieldrin		ng/L		--	1.9		--		--

endosulfan		ng/L		--	56		--		220

endrin		ng/L		--	2.3		--		180

heptachlor		ng/L		--	3.8		--		--

hexachlorocyclo-

hexane-gamma		ng/L		--	80		--		--

lead		ug/L		d	--		d		--

mercury		ug/L		--	--		2.4		--

nickel		ug/L		e	--		e		--

PCBs		ng/L		--	14		--		--

Order No. 98-60                   		    	-10-

b.	EFFLUENT LIMITATIONS FOR PROTECTION OF FRESHWATER AQUATIC LIFE-Continued

Constituent		Units			4-day		Daily		1 -hour		Instant-

Average	Average	Average	aneous

Max.

------------------------------------------------------------------------------------------------------------------------

pentachlorophenol	ug/L			h		--		h		--

selenium		ug/L			5.0		--		20		--

silver			ug/L			--		--		--		f

toxaphene		ng/L			0.2		--		730       .	--

tributyltin		ng/L			20i		40		--		60

zinc			ug/L			9		--		9		--

mg/L = milligram(s) per liter; ug/L = microgram(s) per liter;

ng/L = nanogram(s) per liter; pg/L = picogram(s) per liter;

"--" = Not applicable

a)	The discharger may at its option, meet this limitation as total chromium.

b-g) 	Effluent limits for these metals are expressed by the following formulas, where H = In 			(effluent hardness) in mg/L as CaCO3.

b)	4-DAY AVERAGE CADMIUM = eO.7852H-3.490; 1-HOUR AVERAGE CADMIUM =

el.128H-3.828.

c)	4-DAY AVERAGE COPPER = eO.8545H- 1.465; 1-HOUR AVERAGE COPPER =

eO.9422H-1.460.

d)	4-DAY AVERAGE LEAD = e 1.273H-4.705; 1-HOUR AVERAGE LEAD =

el.273H-1.460.

e)	4-DAY AVERAGE NICKEL = eO.846H+1.1645; 1-HOUR AVERAGE NICKEL =

eO.846H+3.3612.

f)	INSTANTANEOUS MAXIMUM SILVER-R = e1.72H-6.52.

Order No. 98-60                   		    	-11-

b.	EFFLUENT LIMITATIONS FOR PROTECTION OF FRESHWATER AQUATIC LIFE-Continued

g)	4-DAY AVERAGE ZINC = eO.8473H+0.7614; 1 -HOUR AVERAGE ZINC =

eO.8473H+ 0.8604.

h)	4-DAY AVERAGE PENTACHLOROPHENOL = el.005(pH)-5.290; 1-HOUR AVERAGE PENTACHLOROPHENOL = el.005(pH) - 4.830.

i)   Six-month median

Order No. 98-60                			  -12-

c.	EFFLUENT LIMITATIONS FOR THE PROTECTION OF HUMAN HEALTH

Constituent              			Units  		 30-Day -

Average

----------------------------------------------------------------------------------------------------------------------

NONCARCINOGENS

1,2-dichlorobenzene			mg/L		18

1,3-dichlorobenzene			ug/L		2600

endosulfan			ug/L		2.0

endrin			ug/L		0.8

fluoranthene			ug/L		42

mercury			ng/L		12

toluene			mg/L		300

acrolein			ug/L		780

antimony			mg/L		4.3

bis(2-chloroiso-propyl)ether		mg/L		170

chlorobenzene			mg/L		4.5

dibutylphthalate			mg/L		12

diethylphthalate			mg/L		120

2,4-dimethylphenol			mg/L		2.3

dimethylphthalate			mg/L		2900

4,6-dinitro-2-methylphenol		ug/L		770

2,4-dinitrophenol			mg/L		14

ethylbenzene			mg/L		29

hexachlorocyclo-pentadiene		mg/L		17

nitrobenzene			mg/L		1.9

thallium			ug/L		370

1, 1, 1 -trichloroethane		mg/L		11

CARCINOGENS

aldrin			pg/L		140

benzene			ug/L		21

chlordane			pg/L		81

chloroform			ug/L		480

DDT			pg/L		600

1,4-dichlorobenzene			ug/L		64

dichloromethane			ug/L		1600

dieldrin			pg/L		140

halomethanes			ug/L		480

heptachlor			ng/L		0.17

heptachlor epoxide			ng/L		0.07

hexachlorobenzene			ng/L		690

Order No. 98-60				-13-

c.	EFFLUENT LIMITATIONS FOR THE PROTECTION OF HUMAN HEALTH-Continued

Constituent              			Units  		 30-Day -

Average

----------------------------------------------------------------------------------------------------------------------

CARCINOGENS-Continued

hexachlorocyclohexane

alpha		ng/L		13

beta		ng/L		46

gamma		ng/L		62

PAHs					ng/L		31

PCBs					pg/L		70

pentachlorophenol			ug/L		8.2

TCDD equivalents			pg/L		0.014

toxaphene				pg/L		690

2,4,6-trichlorophenol			ug/L		1.0

acrylonitrile				ug/L		0.36

benzidine				ng/L		0.2

beryllium				ug/L		0.13

bis(2-chloroethyl)ether		ug/L		0.62

bis(2-ethylhexyl)-phthalate		ug/L		9.9

carbon tetrachloride			ug/L		3.8

3,3-dichlorobenzidine			ug/L		0.03

1,2-dichloroethane			ug/L		130

1, 1 -dichloroethylene			ug/L		3.2

1,3-dichloropropene			ug/L		31

2,4-dinitrotoluene			ug/L		9.1

1,2-diphenylhydrazine		ug/L		0.54

hexachlorobutadiene			ug/L		50

hexachloroethane			ug/L		8.9

isophorone				ug/L		610

N-nitrosodimethylamine		ng/L		26

N-nitrosodiphenylamine		ug/L		8.8

1,1,2,2-tetrachloroethane		ug/L		11

tetrachloroethylene			ug/L		6.9

1,1,2-trichloroethane			ug/L		42

trichloroethylene			ug/L		92

vinyl chloride				ug/L		34

mg/L = milligram(s) per liter; ug/L = microgram(s) per liter; ng/L = nanogram(s) per liter;

pg/L = picogram(s) per liter; "--" = Not applicable

Order No. 98-60				-14-

2.	The Mass Emission Rate (MER) limits in this Order were calculated using a Q = 2.0 MGD 	and the indicated C values.  When the discharge flowrate is lower than 2.0 MGD, the MER 	limits shall be correspondingly lower.

3.	Compliance with the daily, 4-Day, 30-day, and the 12-month average limits specified in the Discharge Specifications shall be determined from the flow-weighted average of all samples taken during the specified periods.  Compliance with the instantaneous and one-hour average limits specified in Discharge Specifications shall be determined from the results of grab samples taken during the specified periods.

4.	The pH of the discharge shall be at all times within the range of 6.5 and 8.5 pH units.

5.	There shall be no visible oil or grease in the discharge.

6.	The discharge of treated wastewater from the PDWRF to the San Diego River or its tributaries shall be adequately disinfected, oxidized, coagulated, clarified, filtered wastewater (tertiary treated effluent) or equivalent, pursuant to Title 22, Division 4, Chapter 3, Article 5, Section 60315 of the California Code of Regulations.  The wastewater shall be considered adequately disinfected if in the effluent the median number of coliform organisms does not exceed 2.2 per 100 milliliters and the number of coliform organisms does not exceed 23 per 100 milliliters in more than one sample within any 30-day period.  The median value shall be determined from the bacteriological results of the last 7 days for which analyses have been completed.  Filtered wastewater means an oxidized, coagulated, and clarified wastewater which has been passed through natural undisturbed soils or filter media, such as sand or diatomaceous earth (or equivalent as determined by the State Department of Health Services), so that the turbidity as determined by an approved laboratory method does not exceed an average operating turbidity of 2 turbidity units and does not exceed 5 turbidity units more than 5 percent of the time during any 24-hour period.  The discharge of treated wastewater from the PDWRF to the San Diego River and/or its tributaries, in excess of 2.0 MGD or from the modified treatment facilities, shall not occur until the discharger obtains written authorization from the Regional Board Executive Officer.

7.	Treated wastewater from the PDWRF discharged to the San Diego River and contiguous waters must be essentially free of:

a.	Material that is floatable or will become floatable upon discharge.

b.		Settleable material or substances that form sediments which degrade benthic

communities or other aquatic life.

c.		Substances which will accumulate to toxic levels in aquatic sediments or biota.

Order No. 98-60				-15-

d.	Substances that significantly decrease the natural light to benthic communities and other

aquatic life.

e.	Materials that result in aesthetically undesirable discoloration of surface waters.

8.	Waste discharges from the PDWRF shall be discharged in such a manner as to provide maximum protection to aquatic environments.

9.	All waste treatment, containment and disposal facilities shall be protected against 100-year peak stream flows as defined by the San Diego County flood control agency.

10.	All waste treatment. containment and disposal facilities shall be protected against erosion,

overland runoff and other impacts resulting from 100-year frequency 24-hour storm.

11.	Collected screenings, sludges, and other solids removed from liquid wastes, shall be disposed of in a manner approved by the Executive Officer of the Regional Board.

12.	The discharge shall not contain biostimulatory substances in concentrations that promote

aquatic growth to the extent that such growths cause nuisance or adversely affect beneficial uses.

C.	RECEIVING WATER LIMITATIONS

The discharge from the PDWRF shall not, separately or jointly with any other discharge, 	cause violations of the following water quality objectives in surface waters:

1.	Bacterial Characteristics of Inland Surface Waters Including Bays and Estuaries

(a)	Water-Contact and Non-Contact Standards

In waters designated for contact recreation (RECl), the fecal coliform

concentration based on a minimum of not less than five samples for any 30-day

period, shall not exceed a log mean of 200 per 100 ml, nor shall more than 10 	percent of total samples taken during any 30-day period exceed 400 per 100 ml.

In waters designated for noncontact recreation (REC2), and not designated for 	contact recreation (RECl), the average fecal coliform concentration for any 30-	day period, shall not exceed 2,000 per 100 ml nor shall more than 10 percent of 	samples collected during any 30-day period exceed 4,000 per 100 ml.

Order No. 98-60				-16-

In bays and estuaries, the most probable number of coliform organisms in the 			upper 60 feet of the water column shall be less than 1,000 per 100 ml provided 			that not more than 20 percent of the samples at any sampling station, in any 30-			day period, may exceed 1,000 per 100 ml, and provided further that no single 			sample when verified by a repeat sample taken within 48 hours shall exceed 			10,000 per 100 ml.

(b)	Shellfish

At all areas where shellfish may be harvested for human consumption (SHELL), 				the median total coliform concentration for any 30-day period shall not exceed 70 				per 100 ml nor shall more than 10 percent of the samples collected during any 30-				day period exceed 230 per 100 ml for a five-tube decimal dilution test or 330 per

100 ml when a three-tube decimal dilution test is used.

2.		Physical Characteristics of Surface Waters

a.	Floating particulates and grease and oil shall not be visible.

b.	The discharge of waste shall not cause aesthetically undesirable discoloration of the 	surface water.

c.	Natural light shall not be significantly reduced as a result of the discharge of treated

wastewater.

d.	The rate of deposition of solids and the characteristics of solids in receiving water

sediments shall not be changed such that benthic communities are degraded.

e.	Waters shall be free of changes in turbidity that cause nuisance or adversely affect

beneficial uses.

3.	Chemical Characteristics of Surface Waters

a.	Dissolved oxygen levels shall not be less than 5.0 mg/l in inland surface waters with designated WARM beneficial use.  The annual mean dissolved oxygen concentration shall not be less than 7 mg/l more than 10 percent of the time.

b.	Changes in normal ambient pH levels shall not exceed 0.5 units in fresh waters with 		designated warm freshwater habitat (WARM) beneficial uses.  In bays and estuaries 		the pH shall not be depressed below 7.0 nor raised above 9.0  In inland surface 			waters the pH shall not be depressed below 6.5 nor raised above 8.5.

Order No. 98-60				-17-

c.		The dissolved sulfide concentration of waters in and near sediments and throughout

the water column shall not be significantly increased above that present under natural

conditions.

d.	The concentration of organic materials in receiving water sediments shall not be

increased to levels which would degrade aquatic life.

e.	Nutrient materials shall not cause objectionable aquatic growth or degrade

indigenous biota.

4.	Biological Characteristics of Surface Waters

a.	Inland surface water communities and populations, including vertebrate, invertebrate, and plant species, shall not be degraded.

b.		The natural taste, odor, and color of fish, shellfish, or other inland surface water

resources used for human consumption shall not be impaired.

c.		The concentration of organic materials in fish, shellfish or other aquatic resources used

for human consumption shall not bioaccumulate to levels that are harmful to human

health.

d.		The concentration of contaminants in waters which are existing or potential sources of

drinking water shall not occur at levels which are harmful to human health.

5.	The discharge shall not result in acute toxicity in receiving waters.

6.	The discharge shall not result in chronic toxicity in receiving waters.

7.	The discharge shall not cause the presence of radionuclides in concentrations that exceed the maximum permissible concentrations for radionuclides in water set forth in Chapter 5, Title 17 of the California Code of Regulations.

8.	The discharge shall not cause the dissolved oxygen concentration of the San Diego River or 	contiguous waters to be depressed below 5.0 mg/I.  If the ambient dissolved oxygen 	concentration is less than 5.0 mg/l. the discharge shall not cause a further depression.

9.	The discharge shall not cause the natural receiving water temperature to be altered.

Order No. 98-60				-18-

D.	PROVISIONS

1.	The discharger must comply with all conditions of this Order.  Any permit 				noncompliance constitutes a violation of the Clean Water Act and the California Water 		Code and is grounds for enforcement action; for permit termination, revocation and 			reissuance, or modification; or for denial of a renewal application.

2.	The discharger shall take all reasonable steps to minimize or correct any adverse impact

on the environment resulting from noncompliance with this Order, including such

accelerated or additional monitoring as may be necessary to determine the nature and

impact of the noncomplying discharge.

3.		This Order may be modified, revoked and reissued, or terminated for cause including,

but not limited to, the following:

a.	Violation of any terms or conditions of this Order;

b.	Obtaining this Order by misrepresentation or failure to disclose fully all relevant

facts; or

c.	A change in any condition that requires either a temporary or permanent reduction

or elimination of the authorized discharge.

The filing of a request by the discharger for modification, revocation and reissuance, or 	termination of this Order, or a notification of planned change in or anticipated 	noncompliance with this Order does not stay any condition of this Order.

4.	Notwithstanding Provision 3 above, if a toxic effluent standard or prohibition (including

any schedule of compliance specified in such effluent standard or prohibition) is

established under Section 307(a) of the Clean Water Act (CWA) for a toxic pollutant

which is present in the discharge and such standard or prohibition is more stringent than

any limitation for such pollutant in this Order, this Order shall be modified or revoked and

reissued to conform to the toxic effluent standard or prohibition and the discharger so 			notified.

5.	The discharge shall not cause a violation of any applicable water quality standard for

receiving waters adopted by the Regional Board or the State Water Resources Control

Board as required by the CWA and regulations adopted thereunder.  If more stringent

applicable water quality standards are promulgated or approved pursuant to Section 303 of

the CWA or amendments thereto, the Regional Board will revise and modify this Order in

accordance with the more stringent standards.

Order No. 98-60				-19-

6.	The discharger shall comply with effluent standards and prohibitions established under

Section 307(a) of the CWA for toxic pollutants and with standards for sludge use and

disposal established under Section 405(d) of the CWA within the time provided in the

regulations that establish those standards or prohibitions or standards for sludge use or

disposal, even if this Order has not yet been modified to incorporate the requirement.

7.		The discharger shall give prior written notice to the Executive Officer of any change(s) 	planned in the discharger's sludge use or disposal practice.

8.		This Order is not transferable to any person except after notice to the Executive Officer of

this Regional Board.  The Regional Board may require modification or revocation and 	reissuance of this Order to change the name of the discharger and incorporate such other 	requirements as may be necessary under the California Water Code and the CWA.  The 	discharger shall submit notice of any transfer of this Order's responsibility and coverage to

a new discharger as described under Reporting Requirement E.4.

9		This Order does not convey any property rights of any sort or any exclusive privileges.  	The requirements prescribed herein do not authorize the commission of any act causing 	injury to persons or property of another, nor protect the discharger from its liabilities under 	federal, state, or local laws, nor create a vested right for the discharger to continue its waste 	discharge.

10.	The discharger shall allow the Regional Board, or an authorized representative or any

representative of the United States Environmental Protection Agency upon the presentation 	of credentials and other documents as may be required by law, to:

a.	Enter upon the discharger's premises where a regulated facility or activity is located 		or conducted, including sludge use and disposal activities, or where records must be 		kept under the conditions of this Order;

b.	Have access to and copy, at reasonable times, any records that must be kept under

the conditions of this Order;

c.	Inspect at reasonable times any facilities, equipment (including monitoring and

control equipment), practices or operation regulated or required under this Order;

and

d.	Sample or monitor at reasonable times, for the purposes of assuring compliance 			with this Order or as otherwise authorized by the CWA or California Water Code, 		any substances or parameters at any location.

11.	The discharger shall, at all times, properly operate and maintain all facilities and systems 		 treatment and control (and related appurtenances) which are installed or used by the

Order No. 98-60				-20-

discharger to achieve compliance with the conditions of this Order.  Proper operation and maintenance includes effective performance, adequate funding, adequate operator staffing and training, and adequate laboratory and process controls including appropriate quality assurance procedures.  This provision requires the operation of backup or auxiliary facilities or similar systems only when necessary to achieve compliance with the conditions of this Order.

12.	In an enforcement action, it shall not be a defense for the discharger that it would have been necessary to halt or reduce the permitted activity in order to maintain compliance with this Order.  Upon reduction, loss, or failure of the treatment facility, the discharger shall, to the extent necessary to maintain compliance with this Order, control production or all discharges, or both, until the facility is restored or an alternative method of treatment is provided.  This provision applies, for example, when the primary source of power of the treatment facility fails, is reduced or is lost.

13.	The provisions of this Order are severable, and if any provision of this Order, or the 	application of any provision of this Order to any circumstances, is held invalid, the 	application of such provision to other circumstances, and the remainder of this Order, shall 	not be affected thereby.

14.	The discharger shall comply with any interim effluent limitations as established by addendum. 	enforcement action or revised waste discharge requirements which have been or may be 	adopted by this Regional Board.

15.	All proposed new treatment facilities and expansions of existing treatment facilities shall be completely constructed and operable prior to initiation of the discharge from the new or expanded facilities.  The discharger shall submit a certification report for each new treatment facility, expansion of an existing treatment facility, and re-rating of an existing treatment facility.  For new treatment facilities and expansions, the certification report shall be prepared by the design engineer.  For re-ratings, the certification report shall be prepared by the engineer who evaluated the treatment facility capacity.  The certification report shall:

a     Identify the design capacity of the treatment facility;

b.	Certify the adequacy of each component of the treatment facility; and

c.	Contain a requirement-by-requirement analysis, based on acceptable engineering

practices, of how the process and physical design of the facility will ensure compliance

with this Order.

The signature and engineering license number of the engineer preparing the certification 	report shall be affixed to the report.  The certification report, should, if possible, be 	submitted prior to beginning construction.  The discharger shall not initiate a discharge 	from a new treatment facility or 	initiate a discharge to the San Diego River and contiguous waters from an existing treatment

facility at a flowrate in excess of its previously approved design capacity until:

Order No. 98-60				-21-

a.	The certification report is received by the Executive Officer;

b.	The Executive Officer has received written notification of the completion of construction 			(new treatment facilities and expansions only);

c.	An inspection of the plant has been made by the Regional Board staff (new

treatment facilities and expansions only); and

d.	The Executive Officer has provided the discharger with written authorization to discharge

from the treatment facility to the San Diego River or its tributaries at a flowrate not to

exceed the revised treatment facility design capacity.

16.	The discharger shall comply with requirements for the control of pollutants in stormwater

runoff established by agencies having jurisdiction over stormwater conveyance systems

serving the discharger's facilities.

17.	The discharger's wastewater treatment facilities shall be supervised and operated by persons

possessing certificates of appropriate grade pursuant to Chapter 4, Subchapter 14, Title 23 of

the California Code of Regulations.

18.	The discharger shall implement a Nonindustrial Source Control Program consisting of a

public education program designed to minimize the entrance of nonindustrial toxic pollutants

and pesticides into the sanitary sewer system.  The Nonindustrial Source Control Program

shall be reviewed by the discharger and, if necessary, updated at least once during the life of

this Order.

19.	A copy of this Order shall be posted at a prominent location at or near the treatment and disposal facilities, and shall be available to operating personnel at all times.

20.	This Order may be modified in accordance with the requirements set forth in 40 CFR Parts 122 and 124, to include appropriate conditions or limits to address demonstrated effluent toxicity based upon newly available information or the results of the monitoring required in MRP 98-60, and/or to implement new or revised water quality standards for acute and chronic toxicity approved by this Regional Board, the State Water Resources Control Board, and/or the United States Environmental Protection Agency.

21.	Order No. 93-48 is rescinded when this Order becomes effective according to Notification No. H.7 of this Order.

22.	Bypass

a.	Definitions

Order No. 98-60				-22-

(1)	“Bypass” means the intentional diversion of waste streams from any portion of the 		treatment facility.

(2)	“Severe property damage” means substantial physical damage to property, damage 		to the treatment facilities which cause them to become inoperable, or substantial 			and permanent loss of natural resources which can reasonably be expected to occur 		in the absence of a bypass.  Severe property damage does not mean economic loss 		caused by delays in production.

b.	The discharger may allow any bypass to occur which does not cause effluent limitations 		to be exceeded, but only if it also is for essential maintenance to assure efficient 				operations.  These bypasses are not subject to the provisions of paragraphs (c) and (d) of 		this section.

c.	Notice

(1)	Anticipated bypass.  If the discharger knows in advance of the need for a bypass, 			they shall submit prior notice, if possible, at least ten days before the date of the 			bypass.

(2)	Unanticipated bypass.  The discharger shall submit notice of an unanticipated 			bypass as described under Reporting Requirement E.6.

d.	Prohibition of Bypass

(1)	Bypass is prohibited and the RWQCB may take enforcement action against the 			discharger for bypass, unless:

(2)	Bypass was unavoidable to prevent loss of life, personal injury, or severe property 		damage;

(3)	There were no feasible alternatives to the bypass, such as the use of auxiliary 			treatment facilities, retention of untreated waste or maintenance during normal 			periods of equipment downtime.  This condition is not satisfied if the discharger 			could have installed adequate backup equipment to prevent a bypass which occurs 		during normal periods of equipment downtime or preventive maintenance; and

(4)	The discharger submitted notices as required under paragraph (c) of this section.

e.	The Executive Officer may approve an anticipated bypass, after considering its adverse 			effect, it the Executive Officer determines that it will meet the three conditions listed 			above in paragraph (d) of this section.

Order No. 98-60				-23-

23.	Upset

a.	Definitions

“Upset” means an exceptional incident in which there is unintentional and 				temporary noncompliance with technology-based effluent limitations because of 			factors beyond the reasonable control of the discharger.  An upset does not include 		noncompliance to the extent caused by operational error, improperly designed 			treatment facilities, inadequate treatment facilities, lack of preventive maintenance, 		or careless or improper operation.

b.	Effect of an Upset

An upset constitutes an affirmative defense to an action brought for noncompliance 		with such technology-based permit effluent limitations if the requirements of 			paragraph (c) of this section are met.  No determination made during administrative 		review of claims that noncompliance was caused by upset, and before an action for 		noncompliance, is final administrative action subject to judicial review.

c.	Conditions Necessary for a Demonstration of Upset

A discharger who wishes to establish the affirmative defense of upset shall 			demonstrate, through properly signed, contemporaneous operating logs, or other 			relevant evidence that:

(1)	An upset occurred and that the discharger can identify the specific cause(s) 			of the upset;

(2)	The permitted facility was at the time being properly operated; and

(3)	The discharger submitted notice of the upset as required in Reporting 				Requirement E.6.

d.	Burden of Proof

In any enforcement proceeding the discharger seeking to establish the occurrence of 		an upset has the burden of proof.

24.	Pollution, Contamination, Nuisance

Neither the treatment nor the discharge shall create a condition of pollution, contamination 	or nuisance.

Order No. 98-60				-24-

E.	REPORTING REQUIREMENTS

1.	The discharger shall meet the design, operational, and reliability, requirements of Articles 		7, 8, 9 and 1 0 of the California Code of Regulations, Title 22, Division 4, Chapter 3. The

discharger shall develop an engineering report conforming to Section 60323, Article 7 of 			the California Code of Regulations, Title 22, Division 4, Chapter 3. The engineering report 		shall be submitted to the State Department of Health Services, the San Diego County 			Department of Environmental Health , and the Regional Board Executive Officer.  The 			discharge of treated wastewater from the PDWRF to the San Diego River and/or its 			tributaries, in 	excess of 2.0 MGD or from the modified treatment facilities, shall not occur 		until the engineering report is approved by the Regional Board Executive Officer.

2.	The discharger shall file a new Report of Waste Discharge not less than 180 days prior to 	the following:

a.	Addition of a major industrial waste discharge to a discharge of essentially domestic

sewage, or the addition of a new process or product by an industrial facility resulting

in a change in the character of the wastes.

b.	Significant change in disposal method (e.g., change in the method of treatment which

would significantly alter the nature of the waste).

c.	Significant change in disposal area (e.g., moving the discharge to a disposal area

significantly removed from the original area, potentially causing a different water

quality or nuisance problem).

d.	Increase in flow beyond that specified in this Order.

e.	Other circumstances which result in a material change in character, amount, or

location of the waste discharge.

f.	Any planned physical alterations or additions to the permitted facility.

3.	The discharger shall give advance notice to the Executive Officer of any planned changes

in the permitted facility or activity which may result in noncompliance with the 	requirements of this Order.

4.	The discharger must notify the Executive Officer, in writing, at least 30 days in advance of

any proposed transfer of this Order's responsibility and coverage to a new discharger.  The

notice must include a written agreement between the existing and new discharger 	containing a specific date for the transfer of this Order's responsibility and coverage 	between the current discharge and the new discharger.  This agreement shall include an

Order No. 98-60				-25-

acknowledgment that the existing discharger is liable for violations up to the transfer date 	and that the new discharger is liable from the transfer date on.

5.		The discharger shall comply with the attached Monitoring and Reporting Program No. 98-

60.  Monitoring results shall be reported at the intervals specified in Monitoring and 	Reporting Program No. 98-60.

6.		The discharger shall report any noncompliance which may endanger health or the 				environment.  Any such information shall be provided orally to the Executive Officer 			within 24 hours from the time the discharger becomes aware of the circumstances.  A 			written submission shall contain a description of the noncompliance and its cause; the 			period of noncompliance, including exact dates and times, and if the noncompliance has 			not been corrected, the anticipated time it is expected to continue; and steps taken or 			planned to reduce, eliminate, and prevent recurrence of the noncompliance.  The Executive 		Officer, or an authorized representative may waive the written report on a case-by-case 			basis if the oral report has been received within 24 hours.  The following occurrences must 		be reported to the Executive Officer within 24 hours:

a.	Any bypass from any portion of the treatment facility

b.	Any discharge of treated or untreated wastewater resulting from sewer line breaks,

obstruction, surcharge or any other circumstance.

c.	Any treatment plant upset which causes the effluent limitations of this Order to be

exceeded.

d.	Violation of a daily maximum or instantaneous maximum effluent limitation as

specified in Discharge Specification B. 1 of this Order.

e.	Any violation of the prohibitions of this Order.

7.		The discharger shall report sewer overflow events in accordance with Order No. 96-04, 	General Waste Discharge Requirements Prohibiting Sanitary Sewer Overflows by Sewage

Collection Agencies.

8.	The discharger shall furnish to the Executive Officer, within a reasonable time, any information which the Executive Officer may request to determine whether cause exists for modifying, revoking and reissuing, or terminating this Order, or to determine compliance with this Order.  The discharger shall also furnish to the Executive Officer, upon request, copies of records required to be kept by this Order.

9.   The discharger shall provide adequate notice to the Executive Officer of the following:

Order No. 98-60				-26-

a.		Any new introduction of pollutants into the discharger's treatment works from an

indirect discharger which would be subject to Sections 301 or 306 of the CWA if the- 		were directly discharging those pollutants.

b.		Any planned physical alterations or additions to the permitted facility, where such

alterations, additions, or changes may justify the application of permit conditions that 		are different from or absent in the existing permit, including notification of additional 		disposal sites not reported during the permit application process.

10.	Where the discharger becomes aware that it failed to submit any relevant facts in a Report of Waste Discharge, or submitted incorrect information in a Report of Waste Discharge, or in any report to the Regional Board, it shall promptly submit such facts or information.

11.	If a need for a discharge bypass is known in advance, the discharger shall submit prior notice and, if at all- possible, such notice shall be submitted at least ten days prior to the date of the bypass.

12.	The discharger shall develop and implement a BMP plan approved by the Executive Officer pursuant to Section 304(e) of the CWA.  The discharger shall implement the BMP plan to assist in the prevention of, or minimize the potential for, the release of hazardous and/or toxic pollutants from ancillary activities to the San Diego River or its tributaries through plant site runoff, spillage or leaks, sludge or waste disposal.  The proposed plan shall be developed and submitted to the Executive Officer for approval within six months of the issuance of this Order and shall be implemented as soon as practicable, but not later than one year after the effective date of this Order unless a later date is specified by the Executive Officer.  All revisions to the BMP plan shall be subject to the approval of the Executive Officer.  A copy of this BMP plan shall be kept at the PDWRF and be available to operating personnel at all times.

13.	The discharger shall file a written report with this Regional Board within 90 days after the 	average dry weather waste flowrate for any 30-day period equals or exceeds 75 percent of the 	design capacity of the waste treatment and/or disposal facilities, which is 75% of the 	combined capacity of the PDWRF and PDMWD's contractual capacity in the San Diego 	metro system.  The discharger's senior administrative officer shall sign a letter which 	transmits that report and certifies that the policymaking body is adequately informed about it.  	The report shall include:

a.	Average daily flow for the 30-day period, the date on which the instantaneous peak flow 	occurred, the rate of that peak flow, and the total flow for that day.

b.	The discharger's best estimate of when the average daily dry-weather flowrate will equal or 	exceed the design capacity of the facilities.

Order No. 98-60				-27-

c.	The discharger's intended schedule for studies, design, and other steps needed to provide 	additional capacity for the waste treatment and/or disposal facilities and/or control the 	flowrate before the waste flowrate equals the capacity of present units.

14.	This Order expires on June 21, 2003.  If the discharger wishes to continue an activity 	regulated by this Order after the expiration date of this Order, the discharger must apply for 	and obtain new waste discharge requirements.  The discharger must file a Report of Waste 	Discharge in accordance with Title 23, California Code of Regulations not later than 180 days 	in advance of the expiration date, as application for issuance of new waste discharge 	requirement.  The terms and conditions of an expired permit are automatically continued 	pending issuance of a new permit if all requirements of the federal NPDES regulations on 	continuation of expired permits are complied with.

15.	All applications, reports, or information submitted to the Executive Officer of this Regional

Board shall be signed and certified.

a.	The Report of Waste Discharge shall be signed as follows:

1.	For a corporation - by a principal executive officer of at least the level of vice-

president.

2.	For a partnership or sole proprietorship - by a general partner or the proprietor,

respectively.

3.	For a municipality, state federal or other public agency - by either a principal

executive officer or ranking elected official.

b.	All other reports required by this Order and other information requested by the Executive

Officer shall be signed by a person designated in paragraph (a) of this provision, or by a 	duly authorized representative of that person.  An individual is a duly authorized 	representative only if:

1.	The authorization is made in writing by a person described in paragraph (a) of this

provision:

2.	The authorization specified either an individual or a position having responsibility for

the overall operation of the regulated facility or activity, such as the position of plant

manager, operator of a well or well field, superintendent. or position of equivalent

responsibility (a duly authorized representative may thus be either a named individual

or any individual occupying a named position); and

3.	The written authorization is submitted to the Executive Officer.

c.	Any person signing a document under this Section shall make the following certification:

Order No. 98-60				-28-

"I certify under penalty of law that I have personally examined and am familiar with the 	information submitted in this document and all attachments and that, based on my inquiry 	of those individuals immediately responsible for obtaining the information, I believe that 	the information is true, accurate, and complete.  I am aware that there are significant 	penalties for submitting false information, including the possibility of fine and 	imprisonment.”

16.	Except for data determined to be confidential under Title 40, Code of Federal Regulations Part 2 (40 CFR Part 2), all reports prepared in accordance with the terms of this Order shall be available for public inspection at the offices of the California Regional Water Quality Control Board, San Diego Region.  As required by the CWA, Reports of Waste Discharge, this Order, and effluent data shall not be considered confidential.

17.	The discharger shall submit reports to all appropriate agencies as required under this Order.  These agencies include the Regional Board and EPA.  Reports shall be submitted to the Regional Board and EPA at the following addresses:

a.	Executive Officer

California Regional Water Quality Control Board San Diego Region

9771 Clairemont Mesa Blvd., Suite B

San Diego, California 92124-1331

b.	Regional Administrator

U.S. Environmental Protection Agency Region 9

75 Hawthorne Street

San Francisco, CA 94105

18.	If the discharger’s wastewater treatment plant is publicly owned or subject to regulation by 	the California Public Utilities Commission, it shall be supervised and operated by persons 	possessing certificates of appropriate grade.

19.	Reports of compliance or noncompliance with, or any progress reports on, interim and final 	requirements contained in any compliance schedule of this permit shall be submitted no later 	than 14 days following each schedule date.

F.	SLUDGE REQUIREMENTS

1.	The discharger shall comply with all federal and state regulations pertaining to sludge use and disposal.  Current federal regulations regarding the application of sludge containing cadmium and PCBs to land are contained in 40 CFR 257.3-5.

2.		The discharger shall comply with the standards for sludge use and disposal promulgated

under Section 405(d) of the Clean Water Act in 40 CFR 503, that are applicable to the

Order No, 98-60				-29-

discharger's sludge treatment, use, or disposal practices.

3.		The discharger is encouraged to comply with the State of California guidance manual

issued by the Department of Health Services entitled "Manual of Good Practice for

Landspreading of Sewage Sludge".

G.	NOTIFICATIONS

1.	California Water Code Section 13263(g) states:

No discharge of waste into the waters of the state, whether or not such discharge is 	made pursuant to waste discharge requirements, shall create a vested right to 	continue 	such discharge.  All discharges of waste into waters of the state are privileges, not rights.

2.	The CWA provides that any person who violates a condition of this Order implementing

Sections 301, 302, 306, 307, 308, 318 or 405 of the CWA is subject to a civil penalty not

to exceed $10,000 Per day of such violations.  Any person who willfully or negligently

violates conditions of this Order implementing Section 301, 302, 306, 307 or 308 of the 	CWA is subject to a fine of not less than $2,500 nor more than $25.000 per day of 	violation, or by imprisonment for not more than one year, or both.

3.		The CWA provides that any person who knowingly makes any false statement,

representation, or certification in any record or other document submitted or required to

be maintained under this Order, including monitoring reports or reports of compliance or

noncompliance shall, upon conviction, be punished by a fine of not more than $10,000

per violation, or by imprisonment for not more than six months per violation, or by both.

4.		Nothing in this Order shall be construed to relieve the discharger from civil or criminal

penalties for noncompliance.

5.		Nothing in this Order shall be construed to preclude the institution of any legal action or

relieve the discharger from any responsibilities, liabilities, or penalties to which the

discharger is or may be subject to under Section 311 of the CWA.

6.		Nothing in this Order shall be construed to preclude institution of any legal action or

relieve the discharger from any responsibilities , liabilities, or penalties established

pursuant to any applicable State law or regulation under authority preserved by Section

510 of the CWA.

7.		This Order shall become effective 30 days after the date of its adoption, provided the

Regional Administrator, United States Environmental Protection Agency, has no

objection.  If the Regional Administrator objects to its issuance, this Order shall

not become effective until such objection is withdrawn.

Order No. 98-60				-30-

I, John H. Robertus, Executive Officer, do hereby certify the foregoing is a full, true, and correct copy of an Order adopted by the Regional Water Quality Control Board, San Diego Region on June 10, 1998.

________________________

JOHN H. ROBERTUS

Executive Officer