CALIFORNIA REGIONAL WATER QUALITY CONTROL BOARD

SAN DIEGO REGION

MONITORING AND REPORTING PROGRAM NO. 98-60

NPDES PERMIT NO.  CA0107492

FOR

PADRE DAM MUNICIPAL WATER DISTRICT

SANTEE WATER RECLAMATION PLANT

DISCHARGE TO

SYCAMORE CREEK AND THE SAN DIEGO RIVER

SAN DIEGO COUNTY

A .         MONITORING PROVISIONS

1.	Samples and measurements taken as required herein shall be representative of the volume and nature of the monitored discharge.  All samples shall be taken at the monitoring points specified in this Order and, unless otherwise specified, before the effluent joins or is diluted by any other waste stream, body of water, or substance.  Monitoring points shall not be changed without notification to and the approval of the Executive Officer.

2.	Appropriate flow measurement devices and methods consistent with accepted scientific practices shall be selected and used to ensure the accuracy aid reliability of measurements of the volume of monitored discharges.  The devices shall be installed, calibrated and maintained to ensure that the accuracy of the measurements are consistent with the accepted capability of that type of device.  Devices selected shall be capable of measuring flows with a maximum deviation of less than +5 percent from true discharge rates throughout the range of expected discharge volumes.  Guidance in selection, installation, calibration and operation of acceptable flow measurement devices can be obtained from the following references:

a.	"Guide to Methods and Standards for the Measurement of Water Flow," U.S. Department

of Commerce, National Bureau of Standards, NBS Special Publication 421, May 1975, 97

pp. (Available from the U.S. Government Printing Office, Washington, D.C. 20402.  	Order by SD Catalog No. C13.10:421.)

b.	“Water Measurement Manual," U.S. Department of Interior, Bureau of Reclamation,

Second Edition, Revised Reprint, 1974, 327 pp. (Available from the U.S. Government 	Printing Office, Washington D. C. 20402.  Order by Catalog No. 127, 19/2:W29/2,

Stock No. S/N 24003-0027.)

c.   "Flow, Measurement in Open Channels and Closed Conduits," U.S.

Department of Commerce, National Bureau of Standards, NBS Special Publication

484, October 1977, 982 pp. (Available in paper copy of microfiche from

National Technical Information Service (NTIS) Springfield, VA 22151.  Order by

NTIS No.  PB273-535/5ST.)

d.	"NPDES Compliance Sampling Manual," U.S. Environmental Protection Agency, Office  		of  Water Enforcement.  Publication MCD-51, 1977, 140 pp. (Available from the

MRP No. 98-60				-2-

General Services Administration (8FFS), Centralized Mailing Lists Services, Building 41,

Denver Federal Center, Denver, CO 80225).

3.	Monitoring must be conducted according to United States Environmental Protection Agency test procedures approved under Title 40, Code of Federal Regulations (CFR), Part 136, "Guidelines Establishing Test Procedures for Analysis of Pollutants Under the Clean Water Act" as amended, unless other test procedures have been specified this Order.

4.	Published values for MDLs (defined below) and PQLs (defined below) should be used except where revised MDLs and PQLs are available from recent laboratory performance evaluations, in which case the revised MDLs and PQLs should be used.  Where published values are not available, the Executive Officer will determine appropriate values based on available information, including information submitted by the discharger upon request by the Executive Officer.

a)	The Method Detection Limit (MDL) is the minimum concentration of a substance that can

be measured and reported with 99% confidence that the analyte concentration is greater 	than zero, as defined in 40 CFR 136, Appendix B.

b)	The Practical Quantitation Level (PQL) is the lowest concentration of a substance which 	can be consistently determined within +20% of the true concentration by 75% of the labs 	tested in a performance evaluation study.  Alternatively, if performance data are not 	available, the PQL for carcinogens is the MDL x 5, and for noncarcinogens is the MDL x 	10.

5.	All analyses shall be performed in a laboratory certified to perform such analyses by the California Department of Health Service or a laboratory approved by the Executive Officer.

6.	Monitoring results must be reported on discharge monitoring report forms approved by the Executive Officer.

7.   If the discharger monitors any pollutants more frequently than required by this Order, using                         test procedures approved under 40 (CFR), Part 136, or as specified in this Order, the results of this monitoring shall be included in the calculation and reporting of the data submitted in the discharger's monitoring report.  The increased frequency of monitoring shall also be reported.

8.	The discharger shall retain records of all monitoring information, including all calibration and 	maintenance records and all original strip chart recordings for continuous monitoring 	instrumentation, copies of all reports required by this Order, and records of all data used to 	complete the application for this Order.  Records shall be maintained for a minimum of five 	years from the date of the sample, measurement, report, or application.  This period may be 	extended during the course of any unresolved litigation regarding this discharge of when 	requested by the Regional Board Executive Officer or the United States Environmental 	Protection Agency.

9.    Records of monitoring information shall include:

MRP No. 98-60				-3-

a.  	The date, exact place, and time of sampling or measurements;

b.	The individuals who performed the sampling or measurements;

c.	The date(s) analyses were performed;

d.	The laboratory and individual(s) who performed the analyses;

e.	The analytical techniques or methods used; and

f.    The results of all such analyses.

10.	Calculations for all limitations which require averaging of measurements shall utilize an arithmetic mean unless otherwise specified by the Executive Officer or in this Order.

11.	All monitoring instruments and devices used by the discharger to fulfill the prescribed monitoring program shall be properly maintained and calibrated as necessary to ensure their continued accuracy.  All flow measurement devices shall be calibrated at least once per year,

or more frequently, to ensure continued accuracy of the devices.  Annually, the discharger shall submit to the Executive Officer a written statement signed by a registered professional

engineer certifying that all flow measurement devices have been calibrated and will reliably achieve the accuracy required by Monitoring Provision A.2.

12.  All analytical data shall be reported uncensored with method detection limits (MDLs) and

either practical quantitation levels (PQLs) or limits of quantitation (LOQs) identified.

Acceptance of data should be based on demonstrated laboratory performance.

13.	The discharger shall report all instances of noncompliance not reported under Reporting

Requirement No. E.6 of Order No. 98-60 at the time monitoring reports arc submitted.  The

reports shall contain the information listed in Reporting Requirement No. E.6.

14.  The monitoring reports shall be signed by an authorized person as required by Reporting

Requirement No. E. 15.

15.	A composite sample is defined as a combination of at least 8 sample aliquots of at least 100

milliliters, collected at periodic intervals during the operating hours of a facility over a 24-

hour period.  For volatile pollutants, aliquots must be combined in the laboratory -	immediately before analysis.  The composite must be flow proportional; either the time interval between

each aliquot or the volume of each aliquot must be proportional to either the stream flow

at the time of sampling or the total stream flow since the collection of the precious aliquot.

Aliquots may be collected manually or automatically.

16.	A grab sample is an individual sample of at least 100 milliliters collected at a randomly 	selected time over a period not exceeding 15 minutes.

17.  Reports of monitoring surveys conducted to meet receiving water monitoring  requirements of

this monitoring and reporting program shall include, as a minimum, the following

information:

a.	A description of climatic and receiving water characteristics at the time of sampling 			(weather observations, floating debris, discoloration, time of sampling, etc.).

MRP No. 98-60				-4-

b.	A description of sampling stations, including differences unique to each station (e.g.,

station location, sediment grain size, distribution of bottom sediments, rocks, etc.)

c.	A description of the sample collection and preservation procedures used in the survey.

d.	A description of the specific method used for laboratory analysis.

e. 	An in-depth discussion of the results of the survey.  The discussion shall compare data

from the reference station(s)  with data from the stations located in the area of the

discharge.  All tabulations and computations shall be explained.

18.  For all bacterial analyses, sample dilutions should be performed so the range of values extends

from 2 to 16,000 MPN/100 ml.  The detection method used for each analysis shall be reported 	with the results of the analysis.

19.	Detection methods used for coliforms (total and fecal) shall be those presented in the most recent edition of Standard Methods for the Examination of Water and Wastewater or any improved method approved by the Executive Officer.

20.	The purpose of this monitoring program is to:

a.  Determine compliance with the terms and conditions of Order No. 98-60.

b.	Determine that the applicable State and federal water quality standards are met.

21.  Revisions to this MRP may be made by the Executive Officer at any time during the term

Order No. 98-60, and may include a reduction or increase in the number of parameters to be monitored, the frequency of monitoring, or the number and size of samples collected.

22.	The discharger shall have, and implement, an acceptable written quality assurance (QA) plan for laboratory analyses.  An annual report shall be submitted by January 30 of each year which summarizes the QA activities for the previous year.  Duplicate chemical analyses must be conducted on a minimum of ten percent of the samples or at least one sample per month,

whichever is greater.  A similar frequency shall be maintained for analyzing spiked samples.  The discharger should have a success rate equal to or greater than 80 percent.

B.  INFLUENT MONITORING

Influent monitoring is required to determine the effectiveness of a pretreatment program and assess treatment plant performance.  The sampling station shall be located upstream of any in-plant return flows and where a representative sample of the influent to the treatment plant can be obtained.  The date and time of sampling shall be reported with the analytical values determined.

The following table shall constitute the influent monitoring program. The Executive Officer may

reduce the monitoring frequency for those constituents required to be sampled/analyzed quarterly

to an annual sampling/analyses requirement, for those constituents found to be below acceptable

detection limits (40 CFR 136) after four quarters of monitoring.

MRP No. 98-60		  	    -5-

1.	INFLUENT SAMPLING AND ANALYSIS REQUIREMENTS

Parameter		Units	Type of Sample  		Sample/Analysis

Frequency

_______________________________________________________________________________

flowrate		mgd	Record/Totalizer		Continuous

specific conductance	umhos/cm	Recorder		Continuous

pH		pH Units	Recorder		Continuous

BOD(5-d,20-C)	mg/L	24 Hr. 	Composite	Daily

COD		mg/L	24 Hr. 	Composite	Daily

total suspended solids	mg/L	24 Hr.   Composite	Daily

ammonia nitrogen	mg/L	Grab		Monthly

total nitrogen	mg/L	24 Hr.   Composite	Monthly

total phosphorous	mg/L	24 Hr.   Composite	Monthly

total dissolved solids	mg/L	24 Hr.   Composite	Monthly

total hardness	mg/L	24 Hr.   Composite	Quarterly

fluoride		mg/L	24 Hr.   Composite	Quarterly

boron		mg/L	24 Hr.   Composite	Quarterly

sulfate		mg/L	24 Hr.   Composite	Quarterly

chloride		mg/L	24 Hr.   Composite	Quarterly

manganese		mg/L	24 Hr.   Composite	Quarterly

arsenic		ug/L	24 Hr.   Composite	Quarterly

cadmium		ug/L	24 Hr.   Composite	Quarterly

chlordane		ng/L	24 Hr.   Composite	Quarterly

chromium (VI)	ug/L	24 Hr.   Composite	Quarterly

chromium (III)	ug/L	24 Hr.   Composite	Quarterly

cyanide		ug/L	24 Hr.   Composite	Quarterly

copper		ug/L	24 Hr.   Composite	Quarterly

DDT		ng/L	24 Hr.   Composite	Quarterly

dieldrin		ng/L	24 Hr.   Composite	Quarterly

endosulfan		ng/L	24 Hr.   Composite	Quarterly

endrin		ng/L	24 Hr.   Composite	Quarterly

heptachlor		ng/L	24 Hr.   Composite	Quarterly

hexachlorocyclo-

hexane-gamma	ng/L	24 Hr.   Composite	Quarterly

lead		ug/L	24 Hr.   Composite	Quarterly

mercury		ug/L	24 Hr.   Composite	Quarterly

nickel		ug/L	24 Hr.   Composite	Quarterly

PCBs		ng/L	24 Hr.   Composite	Quarterly

pentachlorophenol	ug/L	24 Hr.   Composite	Quarterly

selenium		ug/L	24 Hr.   Composite	Quarterly

silver		ug/L	24 Hr.   Composite	Quarterly

toxaphene		ng/L	24 Hr.   Composite	Quarterly

tributyltin		ng/L	24 Hr.   Composite	Quarterly

zinc		ug/L	24 Hr.   Composite	Quarterly

MRP No. 98-60                   			-6-

1.	INFLUENT SAMPLING AND ANALYSIS REQUIREMENTS-Continued

Parameter	Units	Type of Sample			Sample/Analysis

Frequency

_______________________________________________________________________________

1,2-dichlorobenzene	mg/L	       24 Hr. Composite 		Annual

1,3-dichlorobenzene	ug/L	24 Hr. Composite			Annual

endosulfan	ug/L	24 Hr. Composite			Annual

endrin	ug/L	24 Hr.  Composite		Annual

fluoranthene	ug/L	24 Hr.  Composite		Annual

mercury	ng/L	24 Hr.  Composite		Annual

toluene	mg/L	Grab			Annual

acrolein	ug/L	Grab			Annual

antimony	mg/L	24 Hr.  Composite		Annual

bis(2-chloroiso-

propyl)ether	mg/L	24 Hr.  Composite		Annual

chlorobenzene	mg/L	Grab			Annual

dibutylphthalate	mg/L	24 Hr.  Composite		Annual

diethylphthalate	mg/L	24 Hr.  Composite		Annual

2,4-dimethylphenol	mg/L	24 Hr. Composite			Annual

dimethylphthalate	mg/L	24 Hr.  Composite		Annual

4,6-dinitro-2-

methylphenol	ug/L	24 Hr.  Composite		Annual

2,4-dinitrophenol	mg/L	24 Hr.  Composite		Annual

ethylbenzene	mg/L	Grab			Annual

hexachlorocyclo-

pentadiene	mg/L	24 Hr.  Composite		Annual

nitrobenzene	mg/L	24 Hr.  Composite		Annual

thallium	ug/L	24 Hr. Composite			Annual

1, 1, 1 -trichloroethane	mg/L	Grab			Annual

aldrin       	pg/L	24 Hr. Composite		Annual

benzene	    ug/L	          	Grab			Annual

chlordane	    pg/L	           	24 Hr. Composite		Annual

chloroform	    ug/L	           	Grab			Annual

DDT	    pg/L	            24 Hr. Composite		Annual

1,4-dichlorobenzene	    ug/L	            24 Hr. Composite		Annual

dichloromethane	    ug/L	            Grab			Annual

dieldrin	    pg/L	            24 Hr. Composite		Annual

halomethanes	    ug/L	            Grab			Annual

heptachlor	    ng/L	             24 Hr. Composite		Annual

heptachlor epoxide	    ng/L	             24 Hr. Composite		Annual

hexachlorobenzene	    ng/L	             24 Hr. Composite		Annual

hexachlorocyclohexane

alpha	    ng/L	             24 Hr. Composite		Annual

beta	    ng/L	             24 Hr. Composite		Annual

gamma	    ng/L	             24 Hr. Composite		Annual

MRP No. 98-60				-7-

1.	INFLUENT SAMPLING AND ANALYSIS REQUIREMENTS-Continued

Parameter                    	 Units    	   Type of Sample            	Sample/Analysis

Frequency

_______________________________________________________________________________

PAHs		ng/L                24 Hr. Composite		Annual

PCBs		pg/L                24 Hr.   Composite		Annual

pentachlorophenol		ug/L                24 Hr.   Composite		Annual

TCDD equivalents		pg/L                24 Hr.   Composite		Annual

toxapliene		pg/L                24 Hr.   Composite		Annual

2,4,6-trichlorophenol		uglL                24 Hr.   Composite		Annual

acrylonitrile		ug/L                24 Hr.   Composite		Annual

benzidine		ng/L                24 Hr.   Composite		Annual

beryllium		ug/L                24 Hr.   Composite		Annual

bis(2-chloroethyl)ether		ug/L                24 Hr.   Composite		Annual

bis(2-ethylhexyl)-

phthalate		ug/L                24 Hr.   Composite		Annual

carbon tetrachloride		ug/L                24 Hr.   Composite		Annual

3,3-dichlorobenzidine		ug/L                24 Hr.   Composite		Annual

1,2-dichloroethane		ug/L                Grab				Annual

1, 1 -dichloroethylene		ug/L                24 Hr.   Composite		Annual

1,3-dicl-iloropropene		ug/L                24 Hr.   Composite		Annual

2,4-dinitrotoluene		ug/L                24 Hr.   Composite		Annual

1,2-diphenylhydrazine		ug/L                24 Hr.   Composite		Annual

hexachlorobutadiene		ug/L                24 Hr.   Composite		Annual

hexachloroethane		ug/L                24 Hr.   Composite		Annual

isophorone		ug/L                24 Hr.   Composite		Annual

N-nitrosodimethylamine	ng/L                24 Hr.   Composite		Annual

N-nitrosodiphenylamine	ug/L                24 Hr.   Composite		Annual

1, 1,2.2-tetrachloro-

ethane		ug/L                Grab				Annual

tetrachloroethylene		ug/L                Grab				Annual

1, 1,2-trichloroethane		ug/L                Grab				Annual

trichloroethylene		ug/L                Grab				Annual

vinyl chloride		ug/L                Grab				Annual

MRP No. 98-60				-8-

C. EFFLUENT MONITORING

Effluent monitoring is required to determine compliance with the permit conditions and to identify operational problems and improve plant' performance.  Effluent monitoring also provides information on wastewater characteristics and flows for use in interpreting water quality and biological data.

Sample stations are a) at the point leaving the chlorine contact basin (after dechlorination), b) at the point where the effluent enters Lake No. 7 after alum addition and c) at the point where the effluent enters Lake No. 1.  The date and time of sampling shall be reported with the analytical values determined.

If the discharge is intermittent rather than continuous, then on the first day of each such intermittent discharge, the discharger shall monitor and record data for all of the parameters listed in the effluent monitoring schedule, after which the frequencies of analyses listed in the schedule shall apply for the duration of each such intermittent discharge.  In no event shall the discharger be required to monitor and record data more often than twice the frequencies listed in the schedule.

The following table shall constitute the effluent monitoring program.  The Executive Officer may reduce the monitoring frequency for those constituents required to be sampled/analyzed quarterly and semi-annually to an annual sampling/analyses requirement, for those constituents found to be below acceptable detection-limits (40 CFR 136) after four quarters of monitoring.

In conformance with federal regulations (40 CFR 122.45(c)), analyses to determine compliance with the effluent concentration limitations for heavy metals shall be conducted using the total recoverable method.  For these constituents, if the discharger satisfactorily demonstrates to the Executive Officer an acid soluble/total recoverable method relationship, determination of compliance will be based on a comparison of the adjusted total recoverable method results to permit limits.

MRP No. 98-60                		          -9-

1.	EFFLUENT SAMPLING AND ANALYSIS REQUIREMENTS

Parameter		Units	Type of Sample		Sample/Analysis

Frequency

_______________________________________________________________________________

flowrate		mgd	Record/Totalizer		Continuous

specific conductance	umhos/cm	Recorder		Continuous

pH		pH Units	Recorder		Continuous

chlorine residual	ug/L	Recorder		Continuous

turbidity		NTU	Recorder		Continuous

BOD(5-d,20 deg. C)	mg/L	24-Hr. Composite	Daily

COD		mg/L	24-Hr. Composite	Daily

total suspended solids	mg/L	24-Hr. Composite	Daily

total/fecal Coliform	MPN/	Grab		Daily

100 ml

ammonia nitrogen	mg/L	Grab		Weekly

total nitrogen	mg/L	24-Hr. Composite	Weekly

total phosphorous	mg/L	24 Hr. Composite		Weekly

oil and grease	mg/L	24 Hr. Composite		Monthly

color		units	Grab		Monthly

MBAS		mg/L	Grab		Monthly

percent sodium	%	Grab		Monthly

total dissolved solids	mg/L	24 Hr.   Composite	Monthly

total hardness	mg/L	24 Hr.   Composite	Quarterly

total organic carbon	mg/L	24 Hr.   Composite	Quarterly

fluoride		mg/L	24 Hr.   Composite	Quarterly

boron		mg/L	24 Hr.   Composite	Quarterly

sulfate		mg/L	24 Hr.   Composite	Quarterly

chloride		mg/L	24 Hr.   Composite	Quarterly

manganese	mg/L	24 Hr.   Composite	Quarterly

arsenic		ug/L	24 Hr.   Composite	Quarterly

cadmium		ug/L	24 Hr.   Composite	Quarterly

chromium   (VI)	ug/L	24 Hr.   Composite	Quarterly

chromium   (III)	ug/L	24 Hr.   Composite	Quarterly

cyanide 		ug/L	24 Hr.   Composite	Quarterly

copper		ug/L	24 Hr.   Composite	Quarterly

DDT		ng/L	24 Hr.   Composite	Quarterly

dieldrin		ng/L	24 Hr.   Composite	Quarterly

endosulfan	ng/L	24 Hr.   Composite	Quarterly

endrin		ng/L	24 Hr.   Composite	Quarterly

heptachlor	ng/L	24 Hr.   Composite	Quarterly

hexachlorocvclo-

hexane-gamma	ng/L	24 Hr.   Composite	Quarterly

lead		ug/L	24 Hr.   Composite	Quarterly

mercury		ug/L	24 Hr.   Composite	Quarterly

nickel		ug,/L	24 Hr.   Composite	Quarterly

PCBs		ng/L	24 Hr.   Composite	Quarterly

MRP No. 98-60                          		 -10-

1.	EFFLUENT SAMPLING AND ANALYSIS REQUIREMENTS-Continued

Parameter		Units	Type of Sample		Sample/Analysis

Frequency

_______________________________________________________________________________

pentachlorophenol		ug/L	24 Hr.  Composite		Quarterly

selenium		ug/L	24 Hr.  Composite		Quarterly

silver		ug/L	24 Hr.  Composite		Quarterly

toxaphene		ng/L	24 Hr.  Composite		Quarterly

tributyltin		ng/L	24 Hr.  Composite		Quarterly

zinc		ug/L	24 Hr.  Composite		Quarterly

1,2-dichlorobenzene		mg/L	24 Hr.  Composite		Annual

1,3-dichlorobenzene		ug/L	24 Hr.  Composite		Annual

endosulfan		ug/L	24 Hr.  Composite		Annual

endrin		ug/L	24 Hr.  Composite		Annual

fluoranthene		ug/L	24 Hr.  Composite		Annual

mercury		ng/L	24 Hr.  Composite		Annual

toluene		mg/L	Grab			Annual

acrolein		ug/L	Grab			Annual

antimony		mg/L	24 Hr. Composite		Annual

bis(2-chloroiso-

propyl)ether		mg/L	24 Hr.  Composite		Annual

chlorobenzene		mg/L	24 Hr.  Composite		Annual

dibutylphthalate		mg/L	24 Hr.  Composite		Annual

diethylphthalate		mg/L	24 Hr.  Composite		Annual

2,4-dimethylphenol		mg/L	24 Hr.  Composite		Annual

dimethylphthalate		mg/L	24 Hr.  Composite		Annual

4,6-dinitro-2-

methylphenol		ug/L	24 Hr.  Composite		Annual

2,4-dinitrophenol		mg/L	24 Hr.  Composite		Annual

ethylbenzene		mg/L	Grab			Annual

hexachlorocyclo-

pentadiene		mg/L	24 Hr.  Composite		Annual

nitrobenzene		mg/L	24 Hr.  Composite		Annual

thallium		ug/L	24 Hr.  Composite		Annual

1, 1, I -trichloroethane	mg/L	Grab			Annual

aldrin		pg/L	24 Hr.  Composite		Annual

benzene		ug/L	Grab			Annual

chlordane		pg/L	24 Hr.  Composite		Annual

chloroform		ug/L	Grab			Annual

DDT		pg/L	24 Hr.  Composite		Annual

1.4-dichlorobenzene		ug/L	24 Hr.  Composite		Annual

dichloromethane		ug/L	Grab			Annual

dieldrin		pg/L	24 Hr.  Composite		Annual

MRP No. 98-60				 -11-

1.	EFFLUENT SAMPLING AND ANALYSIS REQUIREMENTS-Continued

Parameter                  		 Units       	Type of Sample	 	Sample/Analysis

Frequency

_______________________________________________________________________________

halomethanes		  ug/L	            Grab	               Annual

heptachlor		  ng/L	            24 Hr. Composite	               Annual

heptachlor epode		nag/L	       24 Hr. Composite		Annual

hexachlorobenzene		ng/L	       24 Hr. Composite		Annual

hexachlorocyclohexane

alpha		ng/L	       24 Hr.	Composite		Annual

beta		ng/L	       24 Hr.	Composite		Annual

gamma		ng/L	       24 Hr.	Composite		Annual

PAHs		ng/L	       24 Hr.	Composite		Annual

PCBs		pg/L	       24 Hr.	Composite		Annual

pentachlorophenol		ug/L	       24 Hr.	Composite		Annual

TCDD equivalents		pg/L	       24 Hr.	Composite		Annual

toxaphene		pg/L	       24 Hr. Composite		Annual

2,4,6-trichlorophenol		ug/l	       24 Hr. Composite		Annual

acrylonitrile		ug/L	       Grab				Annual

benzidine		ng/L	       24 Hr. Composite		Annual

beryllium		ug/L	       24 Hr. Composite		Annual

bis(2-chloroethyl)ether		ug/L	       24 Hr. Composite		Annual

bis(2-ethylhexyl)-

phthalate		ug/L	       24 Hr.	Composite		Annual

carbon tetrachloride		ug/L	       Grab				Annual

3,3-dichlorobenzidine		ug/L	        24 Hr.	Composite		Annual

1,2-dichloroethane		ug/L	        Grab				Annual

1, 1 -dichloroethylene		ug/L	        Grab				Annual

1,3-dichloropropene		ug/L	        Grab				Annual

2,4-dinitrotoluene		ug/L	        Grab				Annual

1,2-diphenylhydrazine		ug/L	        24 Hr.	Composite		Annual

hexachlorobutadiene		ug/L	        24 Hr.	Composite		Annual

hexachloroethane		ug/L	        24 Hr.	Composite		Annual

isophorone		ug/L	        24 Hr.	Composite		Annual

N-nitrosodimethvlamine	ng/L	        24 Hr.	Composite		Annual

N-nitrosodiphenylamine	ug/L	        24 Hr.	Composite		Annual

1, 1,2,2-tetrachloro-

ethane		ug/L	        Grab				Annual

tetrachloroethylene		ug/L	        Grab				Annual

1. 1,2-trichloroethane		ug/L	        Grab				Annual

trichloroethvlene		ug/L	        Grab				Annual

vinyl chloride		ug/L	        Grab				Annual

MRP No. 98-60				-12-

D.	BIOMONITORING

1.	Static Acute Toxicity Monitoring

Static acute toxicity testing toxicity bioassay) of 24-hour composite samples of one hundred

percent (100%) effluent, and a control containing no effluent, shall be conducted on a

quarterly basis in accordance with Methods for Measuring the Acute Toxicity of Effluents to

Freshwater and Marine Organisms (EPA 600/4-90-027, September, 1991 or subsequent

editions), using the Fathead Minno (Pimephales promelas) at 20N C - 23N C for 96 hours.  The

test results shall be reported as percent survival of the test organisms or 96-hour LC-50.  This

biomonitoring requirement may be revised based on the results of the interim toxicity

monitoring required in Biomonitoring Requirement No. 3 below.

2.	Chronic Toxicity Monitoring

Chronic toxicity testing of 24-hour composite samples of one hundred percent (100%) 	effluent, and a control containing no effluent, shall be conducted on a quarterly basis in 	accordance with Short-Term Methods for Estimating the Chronic Toxicity of Effluents and 	Receiving Waters to Freshwater Organisms (Second Edition, EPA/600/4-89/001, March, 1989 	or subsequent editions) and Supplement to Short-Term Methods for Estimating the Chronic 	Toxicity of Effluents and Receiving Waters to Freshwater Organisms (Revision 1, 	EPA/600/4-89/001a, September, 1989), using the most sensitive test organism and the most 	sensitive test deter-mined by the interim biomonitoring required in No. 3 below.  Chronic 	toxicity testing of one hundred percent (100%) effluent shall also be conducted once annually 	using the two other test species.  These selected test species shall be submitted for approval of 	the Executive Officer of the Regional Board prior to being used for conducting chronic 	toxicity testing.  The chronic toxicity testing shall be conducted and reported in accordance 	with Items 3.b., c., d. and e., 4, and 5, outlined below.

3.	Interim Toxicity Monitoring of Dechlorinated and pH Adjusted Effluent

a.   Static Acute Toxicity Test

The discharger shall conduct 96-hour static acute toxicity tests monthly for a period 			of one year- (total number of tests = 12) using 24-hour composite samples of one hundred 		percent (100%), dechlorinated, and pH adjusted effluent.  All tests shall be conducted in 			accordance with Methods for Measuring the Acute Toxicity  Effluents to Freshwater

and Marine Organisms  (EPA 600/4-90-027, September, 1991 or subsequent editions)

using the Fathead Minnow (Pimephales promelas) at 20NC - 23NC for 96 hours.  The test 	results shall be reported as percent survival of the test organisms or 96-hour LC-50.

b.	Chronic Toxicity Test

The discharger shall conduct chronic toxicity monitoring on a monthly basis for a period of 	one year using 24-hour composite samples of one hundred percent (100%). dechlorinated, 	and pH adjusted effluent.  Effluent concentrations shall be 100%, 75%, 50%, 25%, 12.5%, 	and a control containing no effluent.  Chronic toxicity testing shall be conducted in

accordance with Short-Term Methods for Estimating the Chronic Toxicity of Effluents and

MRP No. 98-60               			 -13-

Receiving Waters to Freshwater Organisms (Second Edition, EPA/600/4-89/001, March, 			1989 or subsequent editions) and Supplement to Short-Term Methods for Estimating the 			Chronic Toxicity of Effluents and  Receiving Waters to Freshwater Organisms (Revision 			1, EPA/600/4-89/001a, September, 1989) These tests shall be initiated immediately 			upon the adoption of this order using at least 3 test species (one fish, one

invertebrate, and one alga).  The following tests shall be used unless an alternative is 			approved by the Executive Officer:

(1)	7-Day Fathead Minnow (Pimephales promelas) Larval Survival and Growth Test.

(2)	7-Day Ceriodaphnia Survival and Reproduction Test.

(3)	4-Day Selenastrum Growth Test.

c.	In the event that recommended species is/are not available or the prescribed tests are not 	successful due to no fault of the discharger, other species/tests may be substituted with the

prior approval of the Executive Officer of the Regional Board.

d.	Results of the interim toxicity monitoring program shall be summarized in a final report 	and shall be submitted to the Executive Officer of the Regional Board within 60 days of 	completion of all the tests discussed in a. and b., above.  The final report shall include the

following:

(1)	For the acute toxicity tests, the results shall be reported as 96-hour LC-10 or LC-50, 		whichever is appropriate.

(2)   	For the chronic toxicity tests, the no-observable effect concentrations (NOEC) shall 		be reported.  Additionally, the effect of the effluent stream on the growth, 				reproduction, and survival rate of each test species shall be discussed.

(3)	Any other statistically significant difference in response between the control and

test organisms shall be reported.

(4)	From the test results, the most sensitive test organism and the most sensitive test

shall be determined and reported.

(5)	Any other significant finding related to the biomonitoring program shall also be

included in the final report.

4.	A quality assurance/quality control program shall be instituted for independent verification of the results generated by the effluent toxicity monitoring program. Twice during the interim toxicity monitoring program, and once annually thereafter, the discharger shall split samples with an independent laboratory for conducting tests specified under Item l.a. and b., above.  Prior approval of the Executive Officer shall be obtained for the selection of the independent laboratory.  Results from the independent laboratory shall be submitted to the Regional Board and the discharger for evaluation.

5.	After analyzing the data generated from the interim toxicity monitoring program, this monitoring and reporting program may be revised to include revised biomonitoring and

MRP No. 98-60                   			  -14-

reporting programs.  This may include a reduction or increase in the frequency of the sampling and testing.

6.	If, in any acute toxicity test of 100% effluent, the effluent test result is statistically different from the control test result with a T-test, then the discharger will perform followup acute tests with a dillution series of effluent.  The effluent concentrations will be 100%, 750/o, 50%, 25%, 12.5%. and a control containing no effluent.  For chronic toxicity testing effluent concentrations shall be 100%, 75%, 50%, 25%, 12.5%, and a control containing no effluent.  For both acute and chronic toxicity testing, a minimum of four replicates are required per concentration.  The effluent tests must be conducted with concurrent reference toxicant tests.  Both the reference toxicant and the effluent test must meet all test acceptability criteria specified in the acute and/or chronic toxicity testing manual.  If the test acceptability criteria is not achieved, then the discharger shall re-test within 14 days.  The test results must be reported according to the acute and/or toxicity testing manual chapter on report preparation, and shall be attached to the regular monitoring report.  The discharger shall also submit the chronic toxicity data on a computer disk as specified in Suggested Standardized Reporting Requirements for Monitoring

Chronic Toxicity  SWRCB (February, 1993).

7.	Toxic effects will be demonstrated if there is a statistically significant difference in response between the control and test organisms for any of the tests.

E.	RECEIVING WATER MONITORING

1.	The following table constitutes the receiving water monitoring program. The monitoring 	outlined in the table is to be conducted at the following 6 monitoring stations, except for those 	constituents with a sample/analyses frequency of quarterly and annually, which shall be 	conducted only at Sample Stations a and e.  The Executive Officer may reduce the monitoring 	frequency for those constituents required to be sampled/analyzed quarterly and semi-annually 	to annual sampling/analyses requirement, for those constituents found to be below acceptable 	detection limits (40 CFR 136) after four quarters of monitoring.

a.     San Diego River @ Carlton Hills Boulevard in Santee-Upstream Station

b.		Forrester Creek @ 50 feet upstream of the confluence with the San Diego River-

Upstream -Station (The discharger may propose one upstream sampling station for

approval by the Executive Officer, if one Can found, that determines the upstream

concentrations of pollutants, including the contribution from Forrester Creek.)

c.		San Diego River @ 50 feet downstream of the confluence with Sycamore Creek

d.		San Diego River @ the pond just downstream of Old Mission Dam

MRP No. 98-60			     -15-

e.	San Diego River @ San Diego Mission ponds just south of Friars Road bridge

f.	San Diego River @ Interstate Highway 5-beginning of the estuary

Note:	The exact locations of these monitoring stations shall be coordinated with the Regional Board staff prior to initiating monitoring, and are subject to the approval of the Executive Officer.

MRP No. 98-60                      			     -16-

1.    RECEIVING WATER SAMPLING AND ANALYSIS REQUIREMENTS

Parameter		Units	Type of Sample		Sample/Analysis

Frequency

_______________________________________________________________________________

flowrate		mgd/cfs			Biweekly*

specific conductance	umhos/cm	Grab		Biweekly*

pH		pH Units	Grab		Biweekly*

chlorine residual	ug/L	Grab		Biweekly*

turbidity		NTU	Grab		Biweekly*

Total/Fecal Coliform	MPN	Grab		Biweekly*

ammonia nitrogen	mg/L	Grab		Biweekly*

nitrate		mg/L	Grab		Biweekly*

total nitrogen	mg/L	Grab		Biweekly*

ortho phosphate	mg/L	Grab		Biweekly*

phosphorous

total phosphate	mg/L	Grab		Biweekly*

phosphorous

general plankton**		Grab		Biweekly*

transparency		Grab		Biweekly*

total dissolved solids	mg/L	Grab		Biweekly*

dissolved oxygen***	mg/L	Grab		Biweekly*

temperature		deg.	Grab		Biweekly*

total hardness	mg/L	Grab		Quarterly

total organic carbon	mg/L	Grab		Quarterly

fluoride		mg/L	Grab		Quarterly

boron		mg/L	Grab		Quarterly

sulfate		mg/L	Grab		Quarterly

chloride		mg/L	Grab		Quarterly

arsenic		ug/L	Grab		Quarterly

cadmium		ug/L	Grab		Quarterly

chromium     (VI)	ug/L	Grab		Quarterly

chromium     (III)	ug/L	Grab		Quarterly

cyanide		ug/L	Grab		Quarterly

copper		ug/L	Grab		Quarterly

DDT		ng/L	Grab		Quarterly

dieldrin		ng/L	Grab		Quarterly

endosulfan		ng/L	Grab		Quarterly

endrin		ng/L	Grab		Quarterly

heptachlor		ng/L	Grab		Quarterly

MRP NO. 98-60		-17-

1.	RECEIVING WATER SAMPLING AND ANALYSIS REQUIREMENTS-

Continued

Parameter		Units	Type of Sample	Sample/Analysis

Frequency

_______________________________________________________________________________

hexachlorocyclo-

hexane-gamma		ng/L	Grab			Quarterly

lead		ug/L	Grab			Quarterly

mercury		ug/L	Grab			Quarterly

nickel		ug/L	Grab			Quarterly

PCBs		ng/L	Grab			Quarterly

pentachlorophenol		ug/L	Grab			Quarterly

selenium		ug/L	Grab			Quarterly

silver		ug/L	Grab			Quarterly

toxaphene		ng/L	Grab			Quarterly

tributyltin		ng/L	Grab			Quarterly

zinc		ug/L	Grab			Quarterly

1.2-dichlorobenzene		mg/L	Grab			Annual

1,3-dichlorobenzene		ug/L	Grab			Annual

endosulfan		ug/L	Grab			Annual

endrin		ug/L	Grab			Annual

fluoranthene		ug/L	Grab			Annual

mercury		ng/L	Grab			Annual

toluene		mg/L	Grab			Annual

acrolein		ug/L	Grab			Annual

antimony		mg/.L	Grab			Annual

bis(2-chloroiso-

propyl)ether		mg/L	Grab			Annual

chlorobenzene		mg/L	Grab			Annual

dibutylphthalate		mg/L	Grab			Annual

diethylphthalate		mg/L	Grab			Annual

2,4-dimethylphenol		mg/L	Grab			Annual

dimethylphthalate		mg/L	Grab			Annual

4,6-dinitro-2-

methylphenol		ug/L	Grab			Annual

2,4-dinitrophenol		mg/L	Grab			Annual

ethylbenzene		mg/L	Grab			Annual

hexachlorocyclo-

pentadiene		mg/L	Grab			Annual

nitrobenzene		mg/L	Grab			Annual

thallium		ug/L	Grab			Annual

1, 1, 1 -trichloroethane	mg/L	Grab			Annual

MRP NO. 98-60		-18-

1.	RECEIVING WATER SAMPLING AND ANALYSIS REQUIREMENTS-

Continued

Parameter         	          Units     		   Type of Sample         Sample/Analysis

Frequency

_______________________________________________________________________________

aldrin		pg/L	Grab			Annual

benzene		ug/L	Grab			Annual

chlordane		pg/L	Grab			Annual

chloroform		ug/L	Grab			Annual

DDT		pg/L	Grab			Annual

1,4-dichlorobenzene		ug/L	Grab			Annual

dichloromethane		ug/L	Grab			Annual

dieldrin		pg/L	Grab			Annual

halomethanes		ug/L	Grab			Annual

heptachlor		ng/L	Grab			Annual

heptachlor epoxide		ng/L	Grab			Annual

hexachlorobenzene		ng/L	Grab			Annual

hexachlorocyclohexane

alpha		ng/L	Grab			Annual

beta		ng/L	Grab			Annual

gamma		ng/L	Grab			Annual

PAHs		ng/L	Grab			Annual

PCBs		pg/L	Grab			Annual

pentachlorophenol		ug/L	Grab			Annual

TCDD equivalents		pg/L	Grab			Annual

toxaphene		pg/L	Grab			Annual

2,4,6-trichlorophenol		ug/l	Grab			Annual

acrylonitrile		ug/L	Grab			Annual

benzidine		ng/L	Grab			Annual

beryllium		ug/L	Grab			Annual

bis(2-chloroethyl)ether	ug/L	Grab			Annual

bis(2-ethylhexyl)-

phthalate		ug/L	Grab			Annual

carbon tetrachloride		ug/L	Grab			Annual

3,3-dichlorobenzidine		ug/L	Grab			Annual

1,2-dichloroethane		ug/L	Grab			Annual

1, I -dichloroethylene		ug/L	Grab			Annual

1,3-dichloropropene		ug/L	Grab			Annual

2,4-dinitrotoluene		ug/L	Grab			Annual

1,2-diphenylhydrazine	ug/L	Grab			Annual

hexachlorobutadiene		ug/L	Grab			Annual

hexachloroethane		ug/L	Grab			Annual

MRP NO. 98-60		-19-

1.	RECEIVING WATER SAMPLING AND ANALYSIS REQUIREMENTS-

Continued

Parameter                 		 Units       		Type of Sample       Sample/A-nalysis

Frequency

_______________________________________________________________________________

isophorone		ug/L		Grab			Annual

N-nitrosodimethylamine	ng/L		Grab			Annual

N-nitrosodiphenylamine	ug/L		Grab			Annual

1, 1,2,2-tetrachloro-

ethane	       ug/L		Grab			Annual

tetrachloroethylene	       ug/L		Grab			Annual

1, 1,2-trichloroethane	       ug/L		Grab			Annual

trichloroethylene	       ug/L		Grab			Annual

vinyl chloride	       ug/L		Grab			Annual

acute toxicity****				Grab			Annual

chronic toxicity****				Grab			Annual

fish*****				Survey			Annual

benthic invertebrates*****			Survey			Annual

Note: Annual monitoring shall be conducted in October.

*Biweekly between April 1 and October 1, Monthly between October 2 and

March 31

**General Plankton shall be collected with a 25 mesh plankton net and classified by genus to determine the relative concentration of each major group.  Parameters to be assessed shall include: a) groups of planktonic taxa, b) frequency of phytoplankton and zooplankton, c) ratio of tolerant to intolerant taxa, and d) ratio of bluegreen algae to other taxa.  Phytoplankton pigments shall be assessed by measurement of chlorophyll a, pheophyton a, and corrected chlorophyll a.

***Near surface D.O.s shall be collected within 2 hours of sunrise.

**** Acute and chronic toxicity testing shall be conducted according to the

Biomonitoring Requirements above.

*****Benthic invertebrates and fish surveys shall be conducted in

accordance with USEPA's guidance document entitled "Rapid Bioassessment Protocols" (May, 1989).  Parameters to be assessed shall include: a) taxa, b) biotic index, ratio of scrapers to filtering species, c) ratio of EPT and chironomid, d) EPT index, e) taxa of fish, f) trophic levels,

g) fish reproduction, h) fish recruitment, i) population size of each taxa, and j) ratio of intolerant to tolerant species.

MRP No. 98-60				  -20-

F.	SOLIDS MONITORING

1.	The discharger shall maintain a permanent log of all the solids discharged to the San Diego metro system or hauled away from the treatment facility for use/disposal elsewhere and shall provide a monthly summary of the volume, type (screenings, grit, raw sludge, digested sludge), use (agricultural, composting, etc.), and the destination.

2.	The discharger shall monitor, on a daily basis, the volume (gallons per day) of sludge discharged to the San Diego metro system.  The discharger shall also collect a daily grab sample of the sludge discharged to the San Diego metro system, and analyze this sample for total solids, total suspended solids, and biological oxygen demand.  The results of this monitoring shall be submitted monthly.

G.  WATER SUPPLY   MONITORING

In August of each year, a sample of each source of the water supplied to the SWRP service area shall be obtained and analyzed for the following constituents:

Specific Conductance		Chloride

Total Dissolved Solids		Nitrate

Total Hardness				Fluoride

Boron				Magnesium

Sulfate				pH

Percent Sodium

All of the above constituents shall be expressed in "mg/l" except specific conductance and pH, which shall be expressed in "micromhos/cm" and "pH units," respectively.

H.  MONITORING REPORT SCHEDULE

Monitoring reports shall be submitted to the Executive Officer according to

the dates in the following schedule:

MRP No. 98-60                			  -21-

Monitoring Frequency		Report Period			 Report Due

Continuous, daily		January			By the 30th

weekly, or monthly		February, March..		of the

April, May, June.		following

July, August,			month

September, October,		(February 28

November, December		for January)

Quarterly		January - March		April 30th

April - June			July 30th

July - September		October 30th				October - December		January 30th

Semiannually		January - June			August 30th

July - December		February 28th

Annually					January - December		February 28th

I. ANNUAL SUMMARY OF MONITORING DATA

By February 28th of each year the discharger shall submit an annual report to the Executive

Officer.  The report shall contain both tabular and graphical summaries of the monitoring data obtained during the previous year.  In addition, the discharger shall discuss the compliance record and the corrective actions taken or planned which may be needed to bring the discharger into full compliance with the waste discharge requirements of this Order.

________________________

John H. Robertus

Executive Officer

Date:  June 10, 1998