CALIFORNIA REGIONAL WATER QUALITY CONTROL BOARD

SAN DIEGO REGION

#### MONITORING AND REPORTING PROGRAM NO. 99-71

NPDES PERMIT NO. CA0107999

FOR THE

SAN ELIJO JOINT POWERS AUTHORITY

SAN ELIJO WATER RECLAMATION FACILITY DISCHARGE

TO THE PACIFIC OCEAN

VIA THE SAN ELIJO OCEAN OUTFALL

This Monitoring and Reporting Program Supersedes Technical Change Order No. 1 to Order 94-105 in its entirety.  This Monitoring and Reporting program shall become effective with the adoption of Order No. 99-71.

##### I.  Purpose

This monitoring program is intended to:

- Document short-term and long-term effects of the discharge on receiving waters, sediments, biota, and beneficial uses of the receiving water.

- Determine compliance with NPDES permit terms and conditions.

- Assess the need for industrial pretreatment and toxic control programs.

The monitoring data will be used to determine compliance with water quality standards.

# II.  Monitoring Provisions

- Samples and measurements taken as required herein shall be representative of the volume and nature of the monitored discharge.  All samples shall be taken at the monitoring points specified in this monitoring program and, unless otherwise specified, before the effluent joins or is diluted by any other waste stream, body of water, or substance.  Monitoring points shall not be changed without notification to and the approval of the Executive Officer.  Samples shall be collected at times representative of “worse case” conditions with respect to compliance with the requirements of Order No. 99-71.

- Appropriate flow measurement devices and methods consistent with accepted scientific practices shall be selected and used to ensure the accuracy and reliability of measurements of the volume of monitored discharges.  The devices shall be installed, calibrated, and maintained to ensure that the accuracy of the measurements are consistent with the accepted capability of that type of device.  Devices selected shall be capable of measuring flows with a maximum deviation of less than 10% from true discharge rates throughout the range of expected discharge volumes.

- Monitoring must be conducted according to United States Environmental Protection Agency (USEPA) test procedures approved under Title 40 of the Code of Federal Regulations Part 136 (40 CFR 136), Guidelines Establishing Test Procedures for the Analysis of Pollutants, as amended, unless otherwise specified for sludge in 40 CFR 503, or unless other test procedures have been specified in Order No. 99-71 and/ or in this monitoring and reporting program.

- If the discharger monitors any pollutants more frequently than required by Order No. 99-71 or by this monitoring and reporting program, using test procedures approved under 40 CFR Part 136, or as specified in Order No. 99-71 and this monitoring and reporting program, the results of this monitoring shall be included in the calculation and reporting of the data submitted in the discharger’s monitoring report.  The increased frequency of monitoring shall also be reported.

- The discharger shall retain records of all monitoring information, including all calibration and maintenance records and all original strip chart recordings for continuous monitoring instrumentation, copies of all reports required by Order No. 99-71 and this monitoring and reporting program, and records of all data used to complete the application for Order No. 99-71.  Records shall be maintained for a minimum of 5 years from the date of the sample, measurement, report, or application.  This period may be extended during the course of any unresolved litigation regarding this discharge or when requested by the Regional Board Executive Officer or the USEPA.

- Records of monitoring information shall include:

- The date, exact location, and time of sampling or measurements;

- The name(s) of individual(s) who performed the sampling or measurements;

- The date(s) analyses were performed;

- The name(s) of the laboratory and individual(s) who performed the analyses;

- The analytical techniques or methods used; and

- The results of such analyses.

- Calculations for all limitations that receive averaging of measurements shall utilize an arithmetic mean unless otherwise specified in Order 99-71 or this monitoring and reporting program.

- All monitoring instruments and devices used by the discharger to fulfill the prescribed monitoring program shall be properly maintained and calibrated as necessary to ensure their continued accuracy.  All flow measurement devices shall be calibrated at least once per year, or more frequently, to ensure continued accuracy of the devices.

- All analyses shall be performed in a laboratory certified to perform such analyses by the California Department of Health Services or a laboratory approved by the Regional Board Executive Officer.

- The discharger shall have, and implement, an acceptable written quality assurance (QA) plan for laboratory analyses.  An annual report shall be submitted by February 1st of each year that summarizes the QA activities for the previous year.  Duplicate chemical analyses must be conducted on a minimum of ten percent of the samples, or at least one sample per month, whichever is more frequent.  A similar frequency shall be maintained for analyzing spiked samples.  When requested by the USEPA or the Regional Board, the discharger will participate in the NPDES discharge monitoring report QA performance study.  The discharger shall have a success rate equal to or greater than 80 percent.

- The discharger shall report all instances of noncompliance not reported under Reporting Requirement G.4 of Order No. 99-71 at the time monitoring reports are submitted.  The reports shall contain the information listed in Reporting Requirement G.4 of Order No. 99-71.

- By February 1st of each year, the discharger shall submit an annual report to the Regional Board and USEPA Region 9 that contains tabular and graphical summaries of the monitoring data obtained during the previous year.  The discharger shall discuss the compliance record and corrective actions taken, or which may be needed to bring the discharge into full compliance with the requirements of Order No. 99-71 and this monitoring and reporting program.

- Laboratory method detection limits (MDLs) and practical quantitation levels (PQLs) shall be identified for each constituent in the matrix being analyzed with all reported analytical data.  Acceptance of data shall be based on demonstrated laboratory performance.

- Monitoring results shall be reported at intervals and in a manner specified in Order No. 99-71 and in this monitoring and reporting program.  Monitoring reports shall be submitted to the Regional Board and to USEPA Region 9 according to the following schedule:

Monitoring Frequency			Reporting Period	Report Due

Continuous, Daily, Weekly, or Monthly		All		By the first											day of the second

month after the				month of sampling.

Quarterly			Jan.-March		May 1

April-June		August 1

July-September	November 1

October-December	February 1

Semiannually			January-June		August 1

July-December	February 1

Annually			January-December	February 1

Once every 5 years			---			February 1

## III.  Influent Monitoring

Influent monitoring is intended to:

- Determine compliance with NPDES permit conditions and water quality standards.

- Assess treatment plant performance.

- Assess the need for an Industrial Pretreatment Program and a Toxic Control Program.

Sampling stations shall be established at each point of inflow to all treatment plants and shall be located upstream of any in-plant return flows, and where representative samples of the influent can be obtained.  Influent samples shall be collected on the same day as, and shortly before the collection of effluent samples.

During periods when no effluent from a particular treatment plant is discharged to the Pacific Ocean, no influent monitoring, except for flowrate monitoring, is required at that treatment plant.

The following shall constitute the influent monitoring program:

Parameter		Unit		  Type of Sample1	Minimum Frequency

Flowrate		MGallons/Day	  recorder/ totalizer		continuous

CBOD5 @ 20C	mg/L		  24-hour composite		weekly

Suspended Solids	mg/L		  24-hour composite		weekly

IV.  Effluent Monitoring

Effluent monitoring is intended to:

- Determine compliance with NPDES permit conditions and water quality standards.

- Identify operational problems in order to improve plant performance.

- Provide information on waste characteristics and flows for use in interpreting water quality and biological data.

The effluent sampling station shall be located downstream of any in-plant return flows, and disinfection units, where representative samples of the effluent discharged through the ocean outfall can be obtained.

During periods where no effluent from a particular treatment plant is discharged to the Pacific Ocean, no effluent monitoring, except for flowrate monitoring, is required at that treatment plant.

The following shall constitute the effluent monitoring program:

Parameter		Unit		Type of Sample1		Minimum Frequency

CBOD5 @ 20C	mg/L		24-hour composite		daily

BOD5			mg/L		24-hour composite		monthly

Suspended Solids	mg/L		24-hour composite		daily2

pH			pH units	grab 				daily2

Oil and Grease	mg/L		grab				monthly

Settleable Solids	mL/L		grab				daily2

Turbidity		NTU		24-hour composite		weekly3

Acute Toxicity		TUa		24-hour composite		quarterly

Dissolved Oxygen	mg/L		grab				weekly

Temperature		F		--				weekly

Arsenic		ug/L		24-hour composite		semiannually3,

Cadmium		ug/L		24-hour composite		semiannually 3,4

Chromium(hexavalent)ug/L		24-hour composite		semiannually3,4,

Copper		ug/L		24-hour composite		semiannually 3,4

Lead 			ug/L		24-hour composite		semiannually 3,4

Mercury		ug/L		24-hour composite		semiannually 3,4

Nickel			ug/L		24-hour composite		semiannually 3,4

Selenium 		ug/L		24-hour composite		semiannually 3,4

Silver			ug/L		24-hour composite		semiannually 3,4

Zinc			ug/L		24-hour composite		semiannually 3,4

Cyanide		mg/L		24-hour composite		semiannually 3,4

Total Residual Cl	mg/L		grab				daily

Ammonia (as N)	mg/L		24-hour composite		monthly3

Chronic Toxicity	TUc		24-hour composite		quarterly

Phenolic Compounds 	mg/L		24-hour composite		semiannually 3,4

(nonchlorinated)

Phenolic Compounds 	mg/L		24-hour composite		semiannually 3,4

(chlorinated)

Endosulfan		ug/L		24-hour composite		semiannually 3,4

Endrin			ug/L		24-hour composite		semiannually 3,4

HCH			ug/L		24-hour composite		semiannually 3,4

Radioactivity		pCi/L		24-hour composite		semiannually 3

Acrolein		ug/L		grab				annually3

Antimony		ug/L		24-hour composite		annually3

bis(2-chloroethoxy)

methane		ug/L		grab				annually3

bis(2-chloroisopropyl)

ether			ug/L		grab				annually3

chlorobenzene	ug/L		grab				annually3

chromium (III)		ug/L		24-hour composite		annually3

di-n-butyl phthalate	ug/L		grab				annually3

dichlorobenzenes	ug/L		grab				annually3

1,1-dichloroethylene	ug/L		grab				annually3

diethyl phthalate	ug/L		grab				annually3

dimethyl phthalate	ug/L		grab				annually3

4,6-dinitro-2-

methylphenol		ug/L		grab				annually3

2,4 dinitrophenol	ug/L		grab				annually3

Parameter		Unit		Type of Sample1		Minimum Frequency

ethylbenzene		ug/L		grab				annually3

fluoranthene		ug/L		grab				annually3

hexacyclopentadiene	ug/L		grab				annually3

isophorone		ug/L		grab				annually3

nitrobenzene		ug/L		grab				annually3

thallium		ug/L		24-hour composite		annually3

toluene		ug/L		grab				annually3

1,1,2,2-tetrachloro-

ethane			ug/L		grab				annually3

tributyltin		ug/L		24-hour composite		annually3

1,1,1-trichloroethane	ug/L 		grab				annually3

1,1,2-trichloroehtane	ug/L 		grab				annually3

acrylonitrile		ug/L		grab				annually3

aldrin			ug/L		grab				annually3

benzene		ug/L		grab				annually3

benzidine		ug/L		grab				annually3

beryllium		ug/L		24-hour composite		annually3

bis(2-chloroethyl)

ether			ug/L		grab				annually3

bis(2-ethylhexyl)

phthalate		ug/L		grab				annually3

carbon tetrachloride	ug/L		grab				annually3

chlordane		ug/L		grab				annually3

chloroform		ug/L		grab				annually3

DDT			ug/L		grab				annually3

1,4-dichlorobenzene	ug/L		grab				annually3

3,3-dichlorobenzidine	ug/L		grab				annually3

1,2-dichloroethane	ug/L		grab				annually3

dichloromethane	ug/L		grab				annually3

1,3-dichloropropene	ug/L		grab				annually3

dieldrin			ug/L		grab				annually3

2,4-dinitrotoluene	ug/L 		grab				annually3

1,2-diphenylhydrazine ug/L		grab				annually3

halomethanes		ug/L		grab				annually3

heptachlor		ug/L		grab				annually3

hexachlorobenzene	ug/L		grab				annually3

hexachlorobutadiene	ug/L		grab				annually3

hexachloroethane	ug/L		grab				annually3

N-nitrosodimethyl-

amine			ug/L		grab				annually3

N-nitrosodiphenyl-

amine			ug/L		grab				annually3

PAHs			ug/L		grab				annually3

PCBs			ng/L		grab				annually3

TCDD equivalents	pg/L		grab				annually,3

Tetrachloroethylene	ug/L		grab				annually3

Toxaphene		ug/L		grab				annually3

Trichloroethylene	ug/L		grab				annually3

2,4,6-trichlorophenol	ug/L		grab				annually3

vinyl chloride		ug/L		grab				annually3

V.  Solids Monitoring

Solids monitoring is intended to:

- Assess the need for a pretreatment program.

- Maintain a record of the volume of solids generated and disposal sites used.

- Evaluate the character of sludge to ensure that appropriate disposal methods are employed.

A report identifying the volume of screenings, sludges, grit, and other solids removed from the wastewater and the point(s) at which these wastes were disposed of shall be submitted annually.  A copy of all annual reports required by 40 CFR 503 shall be submitted to the Regional Board at the same time those reports are submitted to the USEPA.

VI.  Receiving Water Monitoring

To determine compliance with water quality standards, the receiving water quality monitoring program must document conditions in the vicinity of the “Zone of Initial Dilution” (ZID) boundary, at reference stations, and at areas beyond the ZID where discharge impacts might reasonably be expected.  Monitoring must reflect conditions during all critical environmental periods.

Receiving water and sediment monitoring in the vicinity of the San Elijo Ocean Outfall (SEOO) shall be conducted as specified below.  Station location, sampling, sample preservation and analyses, when not specified, shall be by methods approved by the Executive Officer.  The monitoring program may be modified by the Executive Officer at any time.

The receiving water and sediment monitoring program for the SEOO may be conducted jointly with other dischargers to the SEOO if the discharger so chooses.  Receiving water and sediment monitoring stations shall be located and numbered as follows:

Monitoring Station Locations

Station		Description

Surf Zone Stations

S1		Surf zone, 8,000’ south of the outfall.

S2		Surf zone, 4,500’ south of the outfall.

S3		Surf zone, 2,500’ south of the outfall.

S4		Surf zone, 500’ south of the outfall.

S5		Surf zone, 500’ north of the outfall.

S6		Surf zone, 2,200’ north of the outfall.

S7		Surf zone, 4,000’ north of the outfall.

Nearshore Stations

N1		Opposite S1, 3,000 feet seaward, MLLW.

N2		Opposite S2, 3,000 feet seaward, MLLW.

N3		Opposite S3, 3,000 feet seaward, MLLW.

N4		Opposite S4, 3,000 feet seaward, MLLW.

N5		Opposite S5, 3,000 feet seaward, MLLW.

N6		Opposite S6, 3,000 feet seaward, MLLW.

N7		Opposite S7, 3,000 feet seaward, MLLW.

Offshore Stations

A14S		At the 120’ depth contour, 14,000’ south of the outfall.

A4S		At the 120’ depth contour, 4,000’ south of the outfall.

A2S		At the 120’ depth contour, 2,000’ south of the outfall.

A1S		At the 120’ depth contour, 1,000’ south of the outfall.

A0.5S		At the 120’ depth contour, 500’ south of the outfall.

A1N		At the 120’ depth contour, 1,000’ north of the outfall.

A2N		At the 120’ depth contour, 2,000’ north of the outfall.

Biological Transects

T0.5S	At the 20, 40, 60,and 80 foot depth contours along the transect located 500 feet downcoast of and parallel to the outfall.

T4S	At the 20, 40, 60,and 80 foot depth contours along the transect located 4,000 feet downcoast of and parallel to the outfall.

T14S	At the 20, 40, 60,and 80 foot depth contours along the transect located 14,000 feet downcoast of and parallel to the outfall.

It is recommended that stations be located using a land-based microwave positioning system, such as Mini-Ranger or trisponder, or a satellite positioning system such as Global Positioning System (GPS).  The high levels of accuracy and precision afforded by this type of positioning system will ensure that stations are properly located with respect to the ZID.  If an alternate navigation system (e.g. Loran C) is proposed, its accuracy should be compared to that of the systems recommended herein, and any compromises in accuracy should be justified.

Monitoring station locations may be modified with the approval of the Executive Officer.

- SURF ZONE WATER QUALITY MONITORING

Surf zone monitoring is intended to assess bacteriological conditions in areas used for body-contact activities (e.g., swimming); and to assess aesthetic conditions for general recreational uses (e.g., picnicking).

All “surf zone stations” shall be monitored as follows:

- Grab samples shall be collected and analyzed for total and fecal coliforms, and enterococcus at a minimum frequency of once per week from May 1 through October 31, and at a minimum frequency of once every other week from November 1, through April 30 of each year.

- At the same time samples are collected from “surf zone stations” the following information shall be recorded: observation of wind (direction and speed), weather (e.g., cloudy, sunny, or rainy), current (e.g., direction), and tidal conditions; observations of water color, discoloration, oil and grease, turbidity, odor, and materials of sewage origin in the water or on the beach; water temperature (F); and status of San Elijo Lagoon mouth (e.g. open, closed, flow).

- NEARSHORE WATER QUALITY MONITORING

Nearshore monitoring is intended to assess bacteriological conditions in areas used for body-contact activities (e.g. scuba diving) and where shellfish and/or kelp may be harvested; and to assess aesthetic conditions for general boating and recreational uses.

All “nearshore stations” shall be monitored as follows:

1.	Reduced Monitoring

If the Executive Officer determines that the effluent at all times complies with Discharge Specifications B.1 and B.3 of Order No. 99-71, only the reduced nearshore water quality monitoring specified below is required.

Determination		Units	Type of Sample	Minimum Frequency

Visual Observations	--		--			Monthly

Total and Fecal

Coliforms,		#/100mL	Grab			Monthly

Enterococcus	#/100mL	Grab9			Monthly

2.	Intensive Monitoring

The intensive nearshore water quality monitoring specified below is required during the 12-month period beginning May 1, 2003 through April 30, 2004, and must be submitted by May 31, 2004.  This monitoring data will assist Regional Board staff in the evaluation of the Report of Waste Discharge required by Reporting Requirement G.2 to be submitted by May 10, 2004, 180 days prior to the Order’s expiration date of November 10, 2004.  The intensive nearshore water quality monitoring specified below is also required if the Executive Officer determines that the effluent does not at all times comply with Discharge Specifications B.1 and B.3 of Order No. 99-71.

Determination		Units	Type of Sample	Minimum Frequency

Visual Observations	--		--			Monthly

Total and Fecal

Coliforms,		#/100mL	Grab			Monthly

Enterococcus		#/100mL	Grab11			Monthly

- OFFSHORE WATER QUALITY MONITORING

Offshore monitoring is intended to determine compliance with the Ocean Plan; and to determine if the applicant’s discharge causes significant impacts on the water quality within the ZID and beyond the ZID as compared to reference areas.

All “offshore stations” shall be monitored as follows:

1.	Reduced Monitoring

If the Executive Officer determines that the effluent at all times complies with Discharge Specifications B.1 and B.3 of Order No. 99-71, only the reduced offshore water quality monitoring specified below is required.

Determination		Units	Type of Sample	Minimum Frequency

Visual Observations	--		--			Monthly

Total and Fecal

Coliforms,		#/100mL	Grab11		Monthly

Enterococcus10	#/100mL	Grab11		Monthly

2.	Intensive Monitoring

The intensive water quality monitoring specified below is required during the 12-month period beginning May 1, 2003 through April 30, 2004, and must be submitted by May 31, 2004.  This monitoring data will assist Regional Board staff in the evaluation of the Report of Waste Discharge required by Reporting Requirement G.2 to be submitted by May 10, 2004, 180 days prior to the Order’s expiration date of November 10, 2004.  The intensive offshore water quality monitoring specified below is also required if the Executive Officer determines that the effluent does not at all times comply with Discharge Specifications B.1 and B.3 of Order No. 99-71.

Determination		Units	Type of Sample	Minimum Frequency

Visual Observations	--		--			Monthly

Total and Fecal

Coliforms,		#/100mL	Grab11			Monthly

Enterococcus		#/100mL	Grab11			Monthly

Temperature		F		Grab12			Monthly

Dissolved Oxygen	mg/L		Grab			Monthly

Light Transmittance	%		Instrument12		Monthly

pH			pH units	Grab9			Annually

- BENTHIC MONITORING

Benthic monitoring is intended to assess the status of the benthic community, and to evaluate the physical and chemical quality of the sediments.

The intensive monitoring specified below is required during the 12-month period beginning May 1, 2003 through April 30, 2004, and must be submitted by May 31, 2004.  This monitoring data will assist Regional Board staff in the evaluation of the Report of Waste Discharge required by Reporting Requirement G.2 to be submitted by May 10, 2004, 180 days prior to the Order’s expiration date of November 10, 2004.  The sediment monitoring specified below is also required if the Executive Officer determines that the effluent does not at all times comply with Discharge Specifications B.1 and B.3 of Order No. 99-71.  Sediment monitoring shall be conducted at all “offshore stations”.

1.	Sediment Characteristics

Analyses shall be performed on the upper two inches of core.

Determination		Units	Type of Sample	Minimum Frequency

Sulfides		mg/kg		Core			Semiannually

Total Chlorinated

Hydrocarbons		mg/kg		Core			Semiannually

BOD			mg/kg		Core			Semiannually

COD			mg/kg		Core			Semiannually

Particle Size Distribution  --		Core			Semiannually

Arsenic		mg/kg		Core			Annually

Cadmium		mg/kg		Core			Annually

Total Chromium	mg/kg		Core			Annually

Copper		mg/kg		Core			Annually

Lead			mg/kg		Core			Annually

Mercury		mg/kg		Core			Annually

Nickel			mg/kg		Core			Annually

Silver			mg/kg		Core			Annually

Zinc			mg/kg		Core			Annually

Cyanide		mg/kg		Core			Annually

Phenolic Compounds	mg/kg		Core			Annually

Radioactivity		pCi/kg		Core			Annually

2.	Infauna

Samples are to be collected with a Paterson, Smith-McIntyre, or orange-peel-type dredge, having an open sampling area of not less than 124 square inches and a sediment capacity of not less than 210 cubic inches.  The sediment shall be sifted through a one-millimeter mesh screen and all organisms shall be identified to as low a taxon as possible.

Determination		Units			Minimum Frequency

Benthic Biota		Identification and		3 Grabs					Enumeration			Semiannually

- ADDITIONAL BIOLOGICAL MONITORING

- Demersal Fish and Macroinvertebrates

The intensive monitoring specified below is required during the 12-month period beginning May 1, 2003 through April 30, 2004, and must be submitted by May 31, 2004.  This monitoring data will assist Regional Board staff in the evaluation of the Report of Waste Discharge required by Reporting Requirement G.2 to be submitted by May 10, 2004, 180 days prior to the Order’s expiration date of November 10, 2004.  The biological transect monitoring specified below is also required if the Executive Officer determines that the effluent does not at all times comply with Discharge Specification B.1, and B.3 of Order No. 99-71.

Determination	Units		Type of Sample	Minimum Frequency

Biological	Identification and	***		Annually

Transects	Enumeration

In rocky or cobble areas, a 30-meter band transect, one meter wide shall established on the ocean bottom.  Operations at each underwater station shall include: (1) Water temperature (may be measured from a boat), estimated visibility and pelagic macrobiota at each 10-foot depth increment throughout the water column and at the bottom recorded; (2) general bottom description recorded; (3) enumeration by estimate of the larger plants and animals in the band transect area recorded; (4) representative photographic record of sampled area taken; and (5) within each band, three one-quarter meter square areas shall be randomly selected and all macroscopic plant and animal life shall be identified to as low a taxon as possible and measured.

For both epifauna and infauna, size frequency and distribution shall be shown for at least the three numerically largest populations identified to the lowest possible taxon and appropriate graphs showing the relationship between species frequency and population shall be plotted from each sample.

- Kelp Bed Monitoring

Kelp bed monitoring is intended to assess the extent to which the discharge of wastes may affect the areal extent and health of coastal kelp beds.

The discharger shall participate with other ocean dischargers in the San Diego Region in an annual regional kelp bed photographic survey.  Kelp beds shall be monitored annually by means of vertical aerial infrared photography to determine the maximum areal extent of the region’s coastal kelp beds within the calendar year.  Surveys shall be conducted as close as possible to the time when kelp bed canopies cover the greatest area.  The entire San Diego Region coastline, from the international boundary to the San Diego Region/ Santa Ana Region boundary, shall be photographed on the same day.

The images produced by the surveys shall be presented in the form of a 1:24,000 scale photo-mosaic of the entire San Diego Region coastline.  Onshore reference points, locations of all ocean outfalls and diffusers, and the 30-foot (MLLW) and 60-foot (MLLW) depth contours shall be shown.

The areal extent of the various kelp beds photographed in each survey shall be compared to that noted in surveys of previous years.  Any significant losses which persist for more than one year shall be investigated by divers to determine the probable reason for the loss.

I, John H. Robertus, Executive Officer of the San Diego Regional Water Quality Control Board, do hereby certify the foregoing is a full, true, and correct copy of Order No. 99-71 adopted by the California Regional Water Quality Control Board, San Diego Region, on November 10, 1999.

--------------------

JOHN H. ROBERTUS

Executive Officer

MONITORING AND REPORTING PROGRAM ENDNOTES

For samples collected from the various treatment plants which are to be physically composited prior to analysis or for the results of analyses which are to be arithmetically composited, the basis for compositing shall be the rate of discharge from the various plants to the ocean, not the rate of inflow to the various plants.

Five days per week except seven days per week for at least one week in July or August of each year.

The minimum frequency of monitoring for this constituent shall be automatically increased to twice the minimum frequency specified here if any analysis for this constituent yields a result higher than the effluent limit specified in Order No. 99-71 for this constituent.  The increased minimum frequency of monitoring shall remain in effect until the results of a minimum of four consecutive analyses for this constituent are below all effluent limits specified in Order No. 99-71 for this constituent.

The minimum frequency of monitoring for this constituent is automatically reduced to annually if the results of twelve consecutive analyses, representing each month of the year, or the results of twenty-four consecutive analyses, representing each quarter of the year, are below the Ocean Plan 6-month median water quality objective for this constituent, or below the laboratory MDL for this constituent in the matrix being analyzed, whichever is higher.

The discharger may at its option monitor for total chromium.  If the measured total chromium concentration exceeds the hexavalent chromium limitation, it will be assumed that the hexavalent chromium limitation was exceeded unless the results of a hexavalent chromium analysis of a replicate sample indicate otherwise.  When analyzing for hexavalent chromium, the appropriate sampling and analytical method must be used (i.e., 24-hour composite cooled to 4 C and analyzed within 24 hours.

Monitoring of Total Chlorine Residual is not required on days when none of the treatment facilities that are subject to Order No. 99-71 use chlorine for disinfection.  If only one sample is collected for total Chlorine residual analysis on a particular day, that samples must be collected at the time when the concentration of total chlorine residual in the discharge would be expected to be greatest.  The times of chlorine discharges on the days the samples are collected, and the time at which samples are collected shall be reported.

A screening period for chronic toxicity shall be conducted every other year for three quarters, using a minimum of three test species (one plant, one invertebrate, and one vertebrate) chosen from the list of approved chronic toxicity test protocols specified in the 1997 Ocean Plan.  After the screening period, the most sensitive species (i.e. the species exhibiting the lowest NOEL) shall be used for the quarterly testing.  Repeat screening periods may be terminated after the first month if the most sensitive species is the same as the species previously found to be most sensitive.

Results for chronic toxicity shall be submitted, electronically, in the TOXIS version 2.4-database format.  After one year, the data will be evaluated by regional board staff to determine if a reduction in the minimum monitoring frequency is appropriate.  If the Executive Officer determines that a reduction in the minimum monitoring frequency is appropriate, the minimum monitoring frequency will be specified by the Executive Officer.

EPA method 8280 shall be used to analyze for TCDD equivalents.

? at the surface.

If the discharger demonstrates to the satisfaction of the Executive Officer, by means of daily analyses, that the concentrations of total and fecal coliform bacteria in the effluent are consistently less than 1,000 per 100 mL, enterococcus monitoring may be suspended.  The discharger shall conduct the monitoring as specified unless the Executive Officer provides written authorization to suspend it.  If this monitoring is suspended, the discharger shall resume it at the request of the Executive Officer.

At surface and mid-depth.

At surface, mid-depth, and bottom.

***Sampling techniques will follow those employed by biologist divers of the California State Department of Fish and Game.  In sandy areas, a 30 meter band transect, one meter wide, shall be established on the ocean bottom.  Operations at each underwater station shall include: (1) Water temperature (may be measured from a boat), estimated visibility and pelagic macrobiota at each 10-foot depth increment throughout the water column and at the bottom recorded; (2) general bottom description recorded; (3) height, period, and crest direction of ripple marks recorded; (4) amount, description, and location of detritus on bottom recorded; (5) representative photographic record of sampled area taken; and (6) within each band, three cores of at least 42.5 cm2 in area shall be randomly taken to a depth of 15 cm where possible, (the three cores may be taken from a boat) and the material removed sifted through at least a 1 millimeter mesh screen, and all organisms identified to as low a taxon as possible, enumerated, measured, and reproductive conditions assessed where feasible.