# CALIFORNIA REGIONAL WATER QUALITY CONTROL BOARD

SAN DIEGO REGION

### ORDER NO. 2000-25

NPDES PERMIT NO. CA 0107336

AMENDED

FACT SHEET

FOR SEAWORLD SAN DIEGO

SAN DIEGO COUNTY

DISCHARGE TO MISSION BAY

- Contact information:

## Regional Water Quality Control Board Contact Person:

## Chiara Clemente

(858) 467-2359

9771 Clairemont Mesa Blvd., Suite A

##### San Diego, CA 92124-1324

# SeaWorld Contact Person:

Kevin Carr

Environmental Director

(619) 226-3934

500 Sea World Drive

San Diego, CA 92109-7995

- Facility Description

SeaWorld San Diego is a privately owned amusement park, belonging to Anheuser-Busch Adventure Parks.  SeaWorld presents aquatic life to the public in an informational and entertaining way.  The discharger currently pumps seawater through two separate intake structures from Mission Bay for use in its mammal pools, aquaria, and other exhibits.  The influent water is filtered, treated and continuously monitored to ensure proper water quality for the mammal and fish exhibits.  This seawater is then collected, disinfected, dechlorinated, and discharged through two separate submerged outfalls to Mission Bay.

- Discharge Description

SeaWorld’s discharge consists of the following:

- continuous overflows from the exhibit pools,

- intermittent flows during pool draining and cleaning operations,

- landscape irrigation and facility wash down water,

- and stormwater discharges during rain events.

The influent from Mission Bay is filtered and chlorinated to maintain a suitable aquaria environment and then circulated through the various aquaria, mammal pools and other exhibits.  Alternative disinfection processes for certain pools include ozonation or UV-treatment for exhibits sensitive to chlorination.

Stormwater flow is collected, directed through, treated, and discharged to Mission Bay in the same pipeline network used to drain the various mammal and bird pools.  Each of the two treatment facilities is designed with a diversion weir to collect and treat stormwater flows until the facilities reach their maximum discharge capacity.

Prior to discharge from the East and West outfalls, wastewater treatment facilities filter and chlorinate the effluent by metered injections of sodium hypochlorite into the waste stream.  After an appropriate contact time, the treated water is dechlorinated by injecting liquid sodium bisulfite into the waste stream.  This treated effluent is then discharged through two separate submerged outfalls to Mission Bay.

| The locations and permitted discharge volumes of the two outfalls are as follows: |  |  |  |
| --- | --- | --- | --- |
| ### Outfall | ###### Maximum Discharge | North Latitude | West Longitude |
| East | 3.24 MGallons/Day | 32 Deg 46 Min 03 Sec | 117 Deg 13 Min 33 Sec |
| West | 6.12 MGallons/Day | 32 Deg 46 Min 04 Sec | 117 Deg 13 Min 40 Sec |

SeaWorld’s average daily flow rate from the facility in 1999 was reported to be approximately 2.155 MGallons/Day from the West discharge and 1.559 MGallons/Day from the East discharge.  Their Report of Waste Discharge for permit reissuance, submitted January 7, 2000, proposes an increase in the average daily discharge for the East outfall from 1.5 to 2.3 MGallons/Day.  NPDES Order No. 95-36 establishes a maximum permitted flow of up to 3.24 MGallons/Day from the East discharge point.

Figure 1 of the discharger’s Report of Waste Discharge demonstrates a spatial representation of the facility and discharge area.

- Receiving Water

SeaWorld discharges continuously through two submerged outfalls to Mission Bay.  On May 12, 1999 the USEPA approved a list of impaired water bodies, unable to meet the “fishable, swimmable” objective, under section 303 (d) of the Clean Water Act.  Mission Bay was listed as impaired on the count of elevated coliform concentrations, eutrophication, and elevated lead concentrations.  The receiving waters of the SeaWorld effluent are impaired for elevated coliform concentrations.

The Comprehensive Water Quality Control Plan, San Diego Basin (9) (Basin Plan) identifies the following beneficial uses of Mission Bay waters to be protected:

a.	Industrial service supply

b.	Contact water recreation

c.	Non-contact water recreation

d.	Commercial and sport fishing

e.	Estuarine habitat

f.	Wildlife habitat

g.	Rare, threatened, or endangered species

h.	Marine habitat

i.	Migration of aquatic organisms

j.	Shellfish harvesting

The Water Quality Control Policy for the Enclosed Bays and Estuaries of California (Bays and Estuaries Policy) was adopted by the SWRCB on May 16, 1974.  This policy established water quality principles, guidelines, effluent quality requirements and prohibitions to govern the disposal of waste in the enclosed bays and estuaries of California.  The requirements of this Order do not conflict with the Bays and Estuaries Policies.

The SWRCB adopted a revised Water Quality Control Plan for Ocean Waters of California (California Ocean Plan) on July 23, 1997.  The Ocean Plan establishes water quality objectives (for bacterial, physical, chemical, and biological characteristics, and for radioactivity), general requirements for management of waste discharge to the ocean, quality requirements for waste discharges (effluent water quality requirements), discharge prohibitions, and general provisions.

The Ocean Plan identifies the following beneficial uses of state ocean waters to be protected:

a.	Industrial water supply

b.	Navigation

c.	Water contact recreation

d.	Non-contact water recreation

e.	Ocean commercial and sport fishing

f.	Preservation and enhancement of Areas of Special Biological Significance

g.	Preservation of rare and endangered species

h.	Marine habitat

i.	Mariculture

j.	Fish migration

k.	Fish spawning

l.	Shellfish harvesting

m.	Aesthetic enjoyment

- Basis for Draft Waste Discharge Requirements

In order to protect the beneficial uses of Mission Bay, the Basin Plan establishes general requirements for the management of waste discharges to the bay, quality requirements for bay waters (receiving water quality requirements), discharge prohibitions, and general provisions.  In addition, the Bays and Estuaries Policy, provides additional prohibitions, specific to California bays and estuaries.  Beneficial uses to Mission Bay are similar to those of the ocean waters of the state.  In order to protect these beneficial uses, limits to those constituents not established in the Basin Plan, have been developed from the 1997 California Ocean Plan.

Effluent limits for pH, enterococcus, and total and fecal coliform were obtained from the Basin Plan.  Limits for acute toxicity, oil and grease, settleable solids, and turbidity were transferred directly from the 1997 Ocean Plan, Table A.  Suspended solids and chlorine residual limitations remain unchanged from the previous Order.  However, additional language has been added to Discharge Specification B.4 pertaining to suspended solids, making the limitation consistent with Ocean Plan standards.  Effluent limits for ammonia were calculated using the Basin Plan receiving water objective, and the equation shown below.  And, effluent limits for halomethanes, chronic toxicity, copper, and silver were calculated using receiving water objectives from the 1997 Ocean Plan, Table B, and the following equation:

Ce = Co + Dm (Co – Cs)

Where:

Ce = the effluent concentration limit obtained

Co = the concentration (Ocean Plan water quality objective) to be met at the completion of initial dilution.

Cs = the background seawater concentration (from Ocean Plan Table C)

Dm = minimum probable initial dilution expressed as parts seawater per part wastewater.

State Water Resources Control Board (SWRCB) staff has determined the minimum initial dilution (Dm) at each outfall to be 21.

Additionally, the SWRCB staff has recognized an error in the water quality objective for silver from the 1988 Ocean Plan to the 1997 version.  The SWRCB will be reverting to the more stringent standard from the 1983 draft, in the upcoming Ocean Plan.  In light of this upcoming change, the more stringent limit was implemented in this permit renewal.

The mass emission rate (MER) limitations, in pounds per day, were obtained from the following calculation:

mass emission rate limitation (lb/Day) = 8.34 x Q x C

where Q is the permitted flow rate for each outfall, C is the constituent effluent limitation (in mg/L), and 8.34 is a conversion factor.

A receiving water monitoring program is currently not required under the provisions of this order.  To ensure that the discharger is not adversely impacting the receiving waters, for non-conservative constituents such as pH, Enterococcus, and coliform, the discharger is held to the objectives of the receiving waters, prior to discharge with no dilution rate.  To ensure no adverse effects to the receiving waters occur as a result of other constituents or synergistic effects, whole effluent chronic and acute toxicity testing is also required prior to discharge.

- Additional Provisions

On May 12, 1999 the USEPA approved a list of impaired water bodies, unable to meet the “fishable, swimmable” objective, under section 303 (d) of the Clean Water Act.  Portions of Mission Bay were listed as impaired on the count of elevated coliform concentrations, eutrophication, and/or elevated lead concentrations.  Although the receiving waters of Mission Bay immediately adjacent to SeaWorld are not considered impaired for eutrophication, no thorough studies have been performed to assess any potential impacts of eutrophication from the SeaWorld discharge on Mission Bay.  Consequently Provision E.33 of this Order, requiring the discharger to perform a study to assess any potential eutrophic effects on the receiving waters, has been added to address this issue.

Concerning biological pollutants, the discharger has been required to conduct a survey of its facilities and submit to this Regional Board a technical report that identifies the source of any chemical or biological pollutant that may be discharged to Mission Bay and that could alter the natural water conditions or adversely affect the biological community.

Discharge Specification B.7 pertaining to the effluent coliform requirements has been removed from the Tentative Order.  Discharge Specification B.7 would allow SeaWorld to discharge coliform concentrations equal to the influent coliform concentrations, when the influent concentrations exceed the receiving water objectives.  A number of commentors have asked for this requirement to be removed, and the discharger, in their March 10 comment letter, has agreed to the removal of this specification.

In the event that effluent coliform concentrations (total or fecal) exceed the discharge specifications outlined in this Order, the discharger is required to report such exceedances to the County of San Diego, Environmental Health Department.

In addition to provisions that are standard to this Regional Board, 40 CFR 122.5, 122.21-.22, 122.41, 122.61-64 incorporates additional conditions that are to be applied to all NPDES permits, either expressly or by reference.

Regional Board staff has consulted with U.S. EPA staff on the proposed tentative order and the responses to comments.  U.S. EPA staff concurs with the proposed tentative order, the proposed changes to the order, and staff’s draft of responses to public comments.  Specifically, U.S. EPA staff determined that the tentative order, with the errata sheet, are in accordance with U.S. EPA’s latest guidance document, the “EPA Region 9 Draft Guidance for Permitting Discharges into Impaired Waterbodies in Absence of a TMDL” issued March 12, 2000.  This guidance document deals with many of the issues raised by the commentors—antidegradation analyses, dilution, effluent limitations, and Total Maximum Daily Load (TMDL) studies.

- Monitoring and Reporting Requirements

The frequency of monitoring for a particular constituent was determined based on the dynamics of that constituent and the nature of the discharger’s effluent.

Due to the discharger’s compliance record with copper throughout the past permit, this Monitoring and Reporting Program reverts to semiannual sampling for the life of this permit.  Although the discharger has also demonstrated compliance with the settleable and suspended solids limitations, quarterly monitoring is deemed necessary to continue to assess compliance.  Weekly Enterococcus and fecal coliform sampling has been added to determine compliance with Discharge Specifications B.2, 3, 5 and 6 of this Order.  Semiannual ammonia sampling is required to determine compliance with the Basin Plan ammonia objective.

Semiannual effluent silver and halomethane sampling have also been added in order to better characterize the effluent.  The discharger uses a copper/silver ionization process to treat water used for the Beluga whale exhibit, and the Report of Waste Discharge submitted indicated silver to be present in the effluent.  The discharger chlorinates and dechlorinates their effluent.  There is evidence that dechlorination does not effectively remove toxic chlorination by-products (measured as halomethanes) so limitations have been added in accordance with the State Ocean Plan Table B.  Monthly temperature monitoring has been added to ensure that the discharger is not significantly altering the background temperature of the receiving waters.  All other monitoring constituents and frequencies remain unchanged.

In an effort to standardize permit monitoring and reporting requirements in order to support electronic data submittal of discharger self-monitoring reports, reporting units, definitions, and deadlines in this permit have been modified in accordance with recommendations contained in the SWRCB’s Water Quality Permit Standards Team Final Report.  The discharger will be required to submit monitoring reports in accordance with the recommendations of this report.

- Written Comments

The final date for submitting written comments to the Regional Board office was extended through March 10, 2000.  Comments will not be accepted thereafter.

- Public Hearing

In accordance with 40 CFR 124.10, the RWQCB must issue a public notice whenever tentative NPDES permits have been prepared, and that the tentative permits will be brought before the RWQCB at a public hearing.  A notice of public hearing for the reissuance of Order No. 2000-25 was published in two local newspapers prior to the March 8 and April 12, Regional Board meeting.

The 2000-25 draft waste discharge requirements will be considered by the Regional Board at a public hearing to be held April 12, 2000, starting at 9:00 am, at the following address:

San Diego Wastewater Mgmt.

District Board Room

600 B Street, 3rd Floor

San Diego, California

- Waste Discharge Requirement Appeals

After the close of the public hearing, the RWQCB may adopt a final NPDES permit.  The final permit will become effective ten (10) days after adoption by the RWQCB, unless a later date is specified by the RWQCB.  The expiration date for Tentative Order No. 2000-25 is March 8, 2005.

Any person may petition the State Board to review the decision of the Regional Board regarding the final Waste Discharge Requirements.  A petition must be sent to the State Water Resources Control Board, P.O. Box 100, Sacramento, CA 95801 within 30 days of the Regional Board public hearing.

- Additional Information

For additional information, interested persons may write the following address or contact Chiara Clemente of the Regional Board staff at (858) 467-2359.

# Attn: 	Chiara Clemente

California Regional Water Quality Control Board, Region 9

9771 Clairemont Mesa Blvd., Suite A

San Diego, CA 92124-1324.

Fax: (858) 571-6972

Copies of the applications, tentative NPDES waste discharge requirements, and other documents (other than those that the Executive Officer maintains as confidential) are available at the RWQCB office for inspection and copying according to the following schedule (excluding holidays):

Monday and Thursday:	1:30 P.M. to 4:30 P.M.

Tuesday and Wednesday:	8:30 A.M. to 11:30 A.M.,

and 1:30 P.M. to 4:30 P.M.

Friday:				8:30 A.M. to 11:30 A.M.

- Basis for Waste Discharge Requirements

The following documents provide the necessary references for the basis of these NPDES permits:

- This RWQCB's Order No. 95-36, Waste Discharge Requirements for  SeaWorld of California.

B.	NPDES Report of Waste Discharge (permit application) submitted by SeaWorld, on January 7, 2000.

C.	The Comprehensive Water Quality Control Plan, San Diego Basin (9) (Basin Plan) 1994.

D.	SWRCB’s Water Quality Permit Standards Team Final Report, April 1999.

- The Water Quality Control Policy for the Enclosed Bays and Estuaries of California (Bays and Estuaries Policy), adopted by the SWRCB on May 16, 1974.

- The Water Quality Control Plan for Ocean Waters of California (California Ocean Plan), July 23, 1997 (1997).

G.	The Code of Federal Regulations 40 CFR; Sections 122.21, .22, .41, .5, .61-.64, 131.12 and 131.36.

H.	The “EPA Region 9 Draft Guidance for Permitting Discharges into Impaired Waterbodies in Absence of a TMDL” issued March 12, 2000.