# CALIFORNIA REGIONAL WATER QUALITY CONTROL BOARD

SAN DIEGO REGION

ORDER NO. 2000-25

NPDES PERMIT NO. CA 0107336

WASTE DISCHARGE REQUIREMENTS

FOR SEAWORLD SAN DIEGO

SAN DIEGO COUNTY

DISCHARGE TO MISSION BAY

## Table of Contents									Page

Order No. 2000-25

Findings.	.	.	.	.	.	.	.	.	.	2

# A.	Prohibitions.	.	.	.	.	.	.	.	.	6

B.	Discharge Specifications.	.	.	.	.	.	.	7

#### C.	Receiving Water Limitations.	.	.	.	.	.	.	10

D.	Best Management Practices.	.	.	.	.	.	.	12

E.	Provisions.	.	.	.	.	.	.	.	.	12

###### F.	Reporting Requirements.	.	.	.	.	.	.	19

G.	Notifications.	.	.	.	.	.	.	.	.	24

H.	Endnotes for Order No. 2000-25.	.	.	.	.	.	25

#### Attachment No. 1 (Enclosed Bays and Estuaries Policy Prohibitions).	26

Attachment No. 2 (Basin Plan Prohibitions).	.	.	.	.	27

Attachment No. 3 (Region 9 Standard Provisions).	.	.	.	29

Attachment No. 4 (CFR Standard Provisions).	.	.	.	34

Monitoring and Reporting Program No. 2000-25.	.	.	.	.	78

A.	Purpose.	.	.	.	.	.	.	.	.	78

B.	Monitoring Provisions.	.	.	.	.	.	.	78

C.	Reporting Schedule.	.	.	.	.	.	.	.	79

D.	Influent Monitoring.	.	.	.	.	.	.	.	79

#### E.	Effluent Monitoring.	.	.	.	.	.	.	.	80

#### Biomonitoring.	.	.	.	.	.	.	.	81

#### Toxicity Reduction Evaluation.	.	.	.	.	.	81

H.	Provisions.	.	.	.	.	.	.	.	.	82

The California Regional Water Quality Control Board, San Diego Region (hereinafter referred to as Regional Board) finds that:

1.	On March 24, 1986, this Regional Board adopted Order No. 86-12, National Pollutant Discharge Elimination System (NPDES) Permit No. CA0107336, “Waste Discharge Requirements for SeaWorld Enterprises, Inc., San Diego County.”  Order No. 86-12 established the requirements for the discharge of up to 3.75 million gallons per day (MGallons/Day) of aquaria wastewater from mammal pools, aquaria, and other exhibits to Mission Bay through three outfalls.  Order No. 86-12 contained an expiration date of March 24, 1991.

2.	On October 3, 1988, this Regional Board adopted Cease and Desist Order (CDO) No. 88-95 requiring SeaWorld San Diego to come into compliance with the chlorine residual and bacteriological requirements contained in Order No. 86-12.  On October 23, 1989, Addendum No. 1 to CDO No. 88-95 was adopted which ordered the discharger to complete construction of all treatment facilities necessary to achieve compliance with Order No. 86-12 by May 31, 1990.

3.	In November 1989, SeaWorld Enterprises, Inc. was acquired by Anheuser-Busch Adventure Parks, which changed the name to SeaWorld San Diego (hereinafter referred to as discharger).

4.	In July 1990, the discharger determined that the installed wastewater treatment facilities would not consistently achieve full compliance with the applicable pH and coliform limitations and requested additional time to design and install a treatment system to comply with those limitations.  Subsequently, on December 17, 1990, the Regional Board adopted Addendum No. 2 to CDO No. 88-95, requiring complete construction of the East and West treatment facilities and full compliance with Order No. 86-12 by November 1, 1991.

5.	On October 29, 1991, the discharger reported that it had achieved full compliance with the requirements of Order No. 86-12 and CDO No. 88-95.  On January 26, 1994, the discharger requested rescission of CDO No. 88-95.  Monitoring data taken in 1993 and early 1994 showed consistent compliance with the discharge requirements.

6.	On March 9, 1995 this Regional Board adopted Order No. 95-36, “Waste Discharge Requirements for SeaWorld of California, San Diego County,” superseding Order No. 86-12, and rescinding CDO No. 88-95.  Order No. 95-36 established the requirements for a maximum total discharge of 9.36 MGallons/Day of aquaria wastewater from two discharge points a maximum flow of 3.24 MGallons/Day from the East discharge point, and a maximum flow of 6.12 MGallons/Day from the West discharge point.  Order No. 95-36 expires March 9, 2000.

7.	On November 23, 1999 the Regional Board sent a staff enforcement letter, notifying the discharger that the required Report of Waste Discharge for permit reissuance was overdue.  On December 20, 1999, staff issued Notice of Violation No. 99-209 for non-submittal of the Report of Waste Discharge, and on January 7, 2000, staff received the discharger’s Report of Waste Discharge for permit reissuance.  The Report of Waste Discharge proposed an increase in the average daily discharge for the East outfall from 1.5 to 2.3 MGallons/Day.  Order No. 95-36 authorizes the maximum discharge of up to 3.24 MGallons/Day from the East discharge point.

8.	The discharger is located at 500 Sea World Drive, San Diego, adjacent to Mission Bay.  The discharger currently pumps seawater from two separate intake structures in Mission Bay for use in its mammal pools, aquaria, and other exhibits.  The influent water is filtered, treated, and continuously monitored to ensure proper water quality for the mammal and fish collections.  This seawater is then screened, chlorinated, dechlorinated, and discharged through two outfall pipes to Mission Bay.  Filter backwash water is discharged to the municipal sewer system, and solids are hauled away for disposal.

9.	The discharges to Mission Bay consist of the following:

- continuous overflows from the pools,

- intermittent flows from pool draining,

- facility irrigation and wash down water,

- and stormwater discharges.

| The locations and permitted discharge volumes of the two outfalls are as follows: |  |  |  |
| --- | --- | --- | --- |
| ### Outfall | Maximum Discharge | North Latitude | West Longitude |
| East | 3.24 MGallons/Day | 32 Deg 46 Min 03 Sec | 117 Deg 13 Min 33 Sec |
| West | 6.12 MGallons/Day | 32 Deg 46 Min 04 Sec | 117 Deg 13 Min 40 Sec |

Seawater taken from Mission Bay is initially filtered and chlorinated to maintain a suitable aquaria environment and then circulated through the various aquaria, mammal pools, and other exhibits.  Alternative treatment processes for certain pools include disinfection by ozonation or UV-treatment for species sensitive to chlorination.

Some stormwater flow generated from the park grounds is collected, directed, treated, and discharged to Mission Bay in the same pipeline network used to drain the various mammal and bird pools.  Each of the two treatment facilities are designed with a diversion weir to collect and treat stormwater flows until the facilities reach their maximum discharge capacity.

Prior to discharge from the East and West outfalls, the two wastewater treatment facilities screen out solids larger than 1” diameter, and chlorinate the effluent by metered injections of sodium hypochlorite into the waste stream.  After an appropriate contact time, the treated water is dechlorinated by injecting liquid sodium bisulfite into the waste stream.

10.	On May 5, 1995, the discharger sent notification to this Regional Board that they had completed the installation of prefabricated pipe reducers at each of the two submerged discharge outfall pipes, in accordance with requirements established in Order No. 95-36.  The reducers increased the initial zone of dilution to a factor of 21 for each of the two discharge outfalls.  The dilution factor was calculated by the State Water Resources Control Board (SWRCB) and was based on data supplied by the discharger.

11.	The Comprehensive Water Quality Control Plan, San Diego Basin (9), (Basin Plan) was adopted by this Regional Board on September 8, 1994 and approved by the SWRCB.

The Basin Plan identifies the following existing beneficial uses of Mission Bay waters to be protected:

a.	Industrial service supply

b.	Contact water recreation

c.	Non-contact water recreation

d.	Commercial and sport fishing

e.	Estuarine habitat

f.	Wildlife habitat

g.	Rare, threatened, or endangered species

h.	Marine habitat

i.	Migration of aquatic organisms

j.	Shellfish harvesting

12.	The SWRCB adopted a revised Water Quality Control Plan for Ocean Waters of California (California Ocean Plan) on July 23, 1997.  The Ocean Plan establishes water quality objectives (for bacterial, physical, chemical, and biological characteristics, and for radioactivity), general requirements for management of waste discharge to the ocean, quality requirements for waste discharges (effluent water quality requirements), discharge prohibitions, and general provisions.

The Ocean Plan identifies the following beneficial uses of state ocean waters to be protected:

a.	Industrial water supply

b.	Navigation

c.	Water contact recreation

d.	Non-contact water recreation

e.	Ocean commercial and sport fishing

f.	Preservation and enhancement of Areas of Special Biological Significance

g.	Preservation of rare and endangered species

h.	Marine habitat

i.	Mariculture

j.	Fish migration

k.	Fish spawning

l.	Shellfish harvesting

m.	Aesthetic enjoyment

13.	The Water Quality Control Policy for the Enclosed Bays and Estuaries of California (Bays and Estuaries Policy) was adopted by the SWRCB on May 16, 1974.  This policy established water quality principles, guidelines, effluent quality requirements, and prohibitions to govern the disposal of waste in the enclosed bays and estuaries of California.  The requirements of this Order do not conflict with the Bays and Estuaries Policies.

14.	Permit prohibitions and objectives found in Order No. 2000-25 are derived from the Basin Plan and the Bays and Estuaries Policy.  Beneficial uses to Mission Bay are similar to those of the ocean waters of the State.  In order to protect the beneficial uses of Mission Bay, discharge specifications for some parameters in this Order were derived from the 1997 Ocean Plan.

15.	On May 12, 1999 the USEPA approved a list of impaired water bodies, unable to meet the “fishable, swimmable” objective, under section 303 (d) of the Clean Water Act.  Portions of Mission Bay were listed as impaired due to elevated coliform concentrations, eutrophication, and/or elevated lead concentrations.  Although SeaWorld receiving waters are not considered impaired for eutrophication, no studies have been performed to assess any potential impacts of eutrophication from the SeaWorld discharge on Mission Bay.  Consequently Provision E.33 of this Order has been added to address this issue.

16.	Effluent limitations, toxic effluent standards, and ocean discharge criteria established pursuant to Sections 301, 302, 304, 307, 308 and 403 of the Clean Water Act and amendments thereto are applicable to this discharge.

17.	The issuance of waste discharge requirements for this discharge is exempt from the requirements for preparation of environmental documents under the California Environmental Quality Act (Public Resources Code Section 21100, et seq.) in accordance with California Water Code Section 13389.

18.	This NPDES Permit is consistent with the antidegradation provisions of 40 Code of Federal Regulations (CFR) 131.12 and SWRCB Resolution No. 68-16.  Existing and potential beneficial uses in Mission Bay will be maintained.

19.	This Regional Board, in establishing the requirements contained herein, considered factors including, but not limited to the following:

a.	The beneficial uses to be protected and the water quality objectives required to meet these beneficial uses.

b.	Past, present and probable future beneficial uses of water.

c.	Environmental characteristics of the hydrologic unit under consideration, including the quality of water available thereto.

d.	Water quality conditions that could reasonably be achieved through the coordinated control of all factors which affect water quality in the area.

e.	Economic considerations.

- The need for developing housing within the region.

- The need to develop and use recycled water.

20.	This Order shall serve as a National Pollutant Discharge Elimination System Permit pursuant to Section 402 of the Clean Water Act and amendments thereto.

21.	This Regional Board has notified the discharger and all known interested parties of its intent to prescribe waste discharge requirements for the existing discharge.

22.	This Regional Board, in a public meeting, heard and considered all comments pertaining to the existing discharge.

IT IS HEREBY ORDERED, that SeaWorld San Diego, in order to meet the provisions contained in Division 7 of the California Water Code and regulations adopted thereunder and the provisions of the Clean Water Act and the regulations adopted thereunder, shall comply with the following:

### A.	PROHIBITIONS

1.	Compliance with Discharge Prohibitions as stated in the 1974 Bays and Estuaries Plan (Attachment No. 1) is required as a condition of this Order.

2.	Compliance with Discharge Prohibitions contained in the Basin Plan (Attachment No. 2) is also required as a condition of this Order.

3.	Discharges of wastes in a manner or to a location which have not been specifically authorized by this Order and for which valid waste discharge requirements are not in force are prohibited.

4.	Aquaria and pool draining operations are prohibited upon the commencement of a storm event in order to minimize the use of the stormwater bypasses at the East and West outfalls.

5.	The Discharge of wastewater and stormwater in excess of 3.24 MGallons/Day through the East outfall or 6.12 MGallons/Day through the West outfall is prohibited unless the discharger obtains revised waste discharge requirements authorizing an increased discharge.

B.	DISCHARGE SPECIFICATIONS

- Effluent discharged to Mission Bay must be essentially free of:

- Material that is floatable or will become floatable upon discharge.

- Settleable material or substances that may form sediments that degrade benthic communities or other aquatic/marine life.

- Substances that will accumulate to toxic levels in waters, sediments or biota.

- Substances that significantly decrease the natural light to benthic communities and other aquatic/marine life.

- Materials that result in aesthetically undesirable discoloration of the water surface.

- The discharge of aquaria wastewater through the East outfall into Mission Bay containing pollutants in excess of the following limitations is prohibited.

3.24 MGALLONS/DAY PEAK DRY WEATHER FLOW

21.0 DILUTION FACTOR

| CONSTITUENT | Unit | 6 Month Median | Monthly Average | Daily Maximum | Maximum at any Time |
| --- | --- | --- | --- | --- | --- |
| Acute Toxicity | TUa | -- | 1.5 | -- | 2.5 |
| Chronic Toxicity | TUc | -- | -- | 22 | -- |
| Ammonia | mg/L lb/Day | -- | -- | -- | 0.55 15 |
| Chlorine Residual | mg/L lb/Day | -- | 0.21 5.7 | -- | 0.42 11.3 |
| Halomethanes | g/L lb/Day | -- | 2,900 77 | -- | -- |
| Copper | g/L lb/Day | 24 0.65 | -- | 220 6.0 | 620 17 |
| Silver | g/L lb/Day | 6.5 0.18 | -- | 36 1.0 | 96 2.6 |
| Oil & Grease | mg/L lb/Day | -- | 25 680 | -- | 75 2,000 |
| pH | pH units | Within the limits of 7.0 and 9.0 at all times. |  |  |  |
| Settleable Solids | mL/L | -- | 1.0 | -- | 3.0 |
| Suspended Solids (Refer to B.4) | mg/L lb/Day | -- | 10 270 | 15 405 | -- |
| Enterococcus | CFU/100 mL | -- | -- | -- | 104 |
| Fecal Coliform | MPN/100 mL | Refer to Discharge Specification B.5 |  |  |  |
| Total Coliform | MPN/100 mL | Refer to Discharge Specification B.6 |  |  |  |
| Turbidity | NTU | -- | 75 | -- | 225 |

3.	The discharge of aquaria wastewater through the West outfall into Mission Bay containing pollutants in excess of the following limitations is prohibited.

6.12 MGALLONS/DAY PEAK DRY WEATHER FLOW

21.0 DILUTION FACTOR1

| CONSTITUENT | Unit2 | 6 Month Median | Monthly Average3 | Daily Maximum4 | Maximum at any Time5 |
| --- | --- | --- | --- | --- | --- |
| Acute Toxicity | TUa | -- | 1.5 | -- | 2.5 |
| ##### Chronic Toxicity | TUc | -- | -- | 22 | -- |
| Ammonia | mg/L lb/Day | -- | -- | -- | 0.55 28 |
| Chlorine Residual | mg/L lb/Day | -- | 0.21 10.7 | -- | 0.42 21.4 |
| ##### Halomethanes6 | g/L lb/Day | -- | 2,900 150 | -- | -- |
| Copper | g/L lb/Day | 24 1.2 | -- | 220 11 | 620 32 |
| # Silver | g/L lb/Day | 6.5 0.33 | -- | 36 1.8 | 96 4.9 |
| Oil & Grease | mg/L lb/Day | -- | 25 1,300 | -- | 75 3,800 |
| pH | pH units | Within the limits of 7.0 and 9.0 at all times. |  |  |  |
| Settleable Solids | mL/L | -- | 1.0 | -- | 3.0 |
| Suspended Solids (Refer to B.4) | mg/L lb/Day | -- | 10 510 | 15 765 | -- |
| # Enterococcus | CFU/100 mL | -- | -- | -- | 104 |
| Fecal Coliform | MPN/100 mL | Refer to Discharge Specification B.5 |  |  |  |
| Total Coliform | # MPN/100 mL | Refer to Discharge Specification B.6 |  |  |  |
| Turbidity | # NTU | -- | 75 | -- | 225 |

- The discharge of aquaria wastewater through the East and West outfalls shall contain no significant increase in the concentration of total suspended solids when compared to the intake water.  Significant increase is defined as an increase in excess of 10 mg/L for a monthly average or 15 mg/L for a daily maximum.  The discharger shall also remove 75% of the suspended solids from the influent stream any time that the influent suspended solids concentration exceeds four times the effluent limits specified in Discharge Specifications B.2 and B.3.

5.	The fecal coliform concentration based on a minimum of not less than five samples for any 30-day period, shall not exceed a log mean of 200/100 mL, nor shall more than 10 percent of total samples during any 30-day period exceed 400/100 mL7.

6.	The median total coliform concentration throughout the water column for any 30-day period shall not exceed 70/100 mL nor shall more than 10 percent of the samples collected during any 30-day period exceed 230/100 mL8 for a five-tube decimal dilution test or 330/100 mL8 when a three tube dilution test is used.

### C.	RECEIVING WATER LIMITATIONS

Discharges shall not cause violations of the following water quality objectives in Mission Bay:

1.	In waters designated for contact recreation (REC-1), the fecal coliform concentration based on a minimum of not less than five samples for any 30-day period, shall not exceed a log mean of 200/100 mL7, nor shall more than 10 percent of total samples during any 30-day period exceed 400/100 mL7.

2.	In waters where shellfish harvesting for human consumption, commercial or sports purposes is designated (SHELL), the median total coliform concentration throughout the water column for any 30-day period shall not exceed 70/100 mL8 nor shall more than 10 percent of the samples collected during any 30-day period exceed 230/100 mL8 for a five-tube decimal dilution test or 330/100 mL8 when a three tube dilution test is used.

- In bays and estuaries, the most probable number of coliform organisms in the upper 60 feet of the water column shall be less than 1,000/100 mL7; provided that not more than 20 percent of the samples at any sampling station, in any 30-day period, may exceed 1,000 per 100 mL7, and provided further that no single sample when verified by a repeat sample taken within 4 hours shall exceed 10,000 per 100 mL7.

4.	The discharge of wastes shall not cause concentrations of un-ionized ammonia (NH3) to exceed 0.025 mg/L (as N) in inland waters, enclosed bays and estuaries, and coastal lagoons.

5.	Inland surface waters, bays and estuaries and coastal lagoon waters shall not contain biostimulatory substances in concentrations that promote aquatic growth to the extent that such growths cause nuisance or adversely affect beneficial uses.

6.	Waters shall be free of coloration that causes nuisance or adversely affects beneficial uses.

7.	The natural color of fish, shellfish, or other resources in inland surface waters, coastal lagoon, or bay and estuary shall not be impaired.

8.	Dissolved oxygen levels shall not be less that 5.0 mg/L in inland surface waters with designated Marine Habitat (MAR) or Warm Freshwater habitat (WARM) beneficial uses.  The annual mean dissolved oxygen concentration shall not be less than 7.0 mg/L more than 10% of the time.

9.	Waters shall not contain floating material, including solids liquids, foams, and scum in concentrations which cause nuisance or adversely affect beneficial uses.

10.	Changes in normal ambient pH levels shall not exceed 0.2 units in waters with designated MAR, or estuarine (EST), or saline (SAL) beneficial uses.

11.	Waters shall not contain oils, greases, waxes or other materials in concentrations which result in a visible film or coating on the surface of the water or on objects in the water, or which cause nuisance or which otherwise adversely affect beneficial uses.

12.	No individual pesticide or combination of pesticides shall be present in the water column, sediments or biota at concentration(s) that adversely affect beneficial uses.  Pesticides shall not be present at levels which bioaccumulate in aquatic organisms to levels that are harmful to human health, wildlife, or aquatic organisms.

13.	Radionuclides shall not be present in concentrations that are deleterious to human, plant, animal, or aquatic life nor that result in the accumulation of radionuclides in the food web to an extent that presents a hazard to human, plant, animal, or aquatic life.

14.	Waters shall not contain suspended and settable solids in concentrations of solids that cause nuisance or adversely affect beneficial uses.

15.	Waters shall not contain taste or odor producing substances at concentrations that cause a nuisance or adversely affect beneficial uses.

16.	The natural taste and odor of fish, shellfish, or other regional water resources used for human consumption shall not be impaired in inland surface waters and bays and estuaries.

17.	All waters shall be maintained free of toxic substances in concentrations that are toxic to, or that produce detrimental physiological responses in human, plant, animal, or aquatic life.

The survival of aquatic life in surface waters subjected to a waste discharge or other controllable water quality factors, shall not be less than that for the same water body in areas unaffected by the waste discharge or, when necessary, for other control water that is consistent with the requirements specified in the US EPA, SWRCB, or other protocol authorized by the Regional Board.

18.	Waters shall be free of changes in turbidity that cause nuisance or adversely affect beneficial uses.

### D.	BEST MANAGEMENT PRACTICES

The following best management practices shall be employed to maximize capture and treatment of any wastewater, and reduce or eliminate any mixing with stormwater:

1.	Aquaria and pool draining activities shall be halted upon commencement of a storm event.

2.	All paved areas shall be swept down periodically to minimize stormwater pollutant loading into Mission Bay.

3.	A periodic wash down following the periodic sweep is authorized.  Care shall be taken to direct as much of the runoff as possible into the treatment system.

### E.	PROVISIONS

1.	The discharger must comply with all regional standard provisions, where applicable, as stated in Attachment No. 3.

2.	The following sections of 40 CFR (Attachment No. 4) are incorporated into this permit by reference, and the discharger must comply with these provisions:

a)122.5		Effect of a permit

b)122.21		Application for a permit

c)122.22		Signatories to permit applications and reports

d)122.41		Conditions applicable to all permits

d)122.61		Transfer of permits

e)122.62		Modification or revocation of permits

f)122.63		Minor modifications of permits

g)122.64		Termination of permits

3.	It shall not be a defense for the discharger in an enforcement action that it would have been necessary to halt or reduce the permitted activity in order to maintain compliance with the conditions of this Order.  Upon reduction, loss, or failure of a treatment facility, the discharger shall, to the extent necessary to maintain compliance with this Order, control all discharges until the facility is restored or an alternative method of treatment is provided.  This provision applies, for example, when the primary source of power of a treatment facility fails, is reduced, or is lost.

4.	If only one sample is collected during the time period associated with the effluent limitations (e.g., 30-day average or 6-month median), the single measurement shall be used to determine compliance with the effluent limitation for the entire time period.

5.	All analytical data shall be reported uncensored with detection limits and quantitation limits identified.  For any effluent limitation, compliance shall be determined using appropriate statistical methods to evaluate multiple samples.  Sufficient sampling and analysis shall be conducted to determine compliance.

6.	The 6-month median effluent concentration limitation shall apply as a moving median of daily values for any 180-day period in which daily values represent flow-weighted average concentrations within a 24-hour period.  For intermittent discharges, the daily value shall be considered to equal zero for days on which no discharge occurred.  The 6-month median receiving water limitation shall apply as a moving median of all values collected for any 180-day period.

- The monthly average effluent limitation shall be the moving arithmetic mean using the results of analyses of all samples collected during any 30 consecutive calendar day period.

8.	The maximum at any time shall be determined from the results of a single grab sample or from the results of a single composite sample collected over a period of 24 hours.

9.	The daily maximum effluent limitation shall apply to flow-weighted 24-hour composite samples.  The daily maximum receiving water limitation shall apply to grab sample determinations.

10.	The mass emission rate (MER), in pounds per day, shall be obtained from the following calculation for any calendar day:

mass emission rate (lb/Day) = 8.34 x Q x C

in which Q and C are the flow rate in MGallons/ Day and the constituent concentration in mg/L, respectively, and 8.34 is a conversion factor.  If a composite sample is taken, then C is the concentration measured in the composite sample and Q is the average flow rate occurring during the period over which the samples are composited.

11.	Compliance with the Acute Toxicity limitation in Discharge Specifications B.2 and B.3 of this Order shall be determined using an established protocol, e.g., American Society for Testing Materials (ASTM), USEPA, American Public Health Association, or State Board.  Acute Toxicity (TUa) shall be expressed in Toxic Units Acute (TUa), where:

TUa =         100

96-hour LC50

Where LC50 is the Lethal Concentration 50% and the percent waste giving 50% survival of test organisms.  LC50 shall be determined by static or continuous flow bioassay techniques using standard test species.  If specific identifiable substances in wastewater can be demonstrated by the discharger as being rapidly rendered harmless upon discharge to the marine environment, but not as a result of dilution, the LC 50 may be determined after the test samples are adjusted to remove the influence of those substances.

When it is not possible to measure the 96-hour LC50 due to greater than 50% survival of the test species in 100% waste, the toxicity concentration shall be calculated by the following:

TUa = log (100 - S)

1.7

where S is the percentage survival in 100% waste. If S > 99, TUa shall be reported as zero.

12.	Compliance with the Chronic Toxicity effluent limitations established in Discharge Specifications No. B.2 and B.3 of this Order shall be determined using critical life stage toxicity tests.  Chronic Toxicity (TUc) shall be expressed as Toxic Units Chronic (TUc), where:

TUc = 100

NOEL

where NOEL is the No Observed Effect Level and is expressed as the maximum percent effluent that causes no observable effect on a test organism, as determined by the result of a critical life stage toxicity test listed below.

A minimum of three test species with approved test protocols shall be used to measure compliance with the chronic toxicity objective.  The test species shall include a fish, an invertebrate, and an aquatic plant.  After an initial screening, monitoring may be reduced to the most sensitive species.  Dilution and control water should be obtained from an unaffected area of the receiving waters.  The sensitivity of the test organisms to a reference toxicant shall be determined concurrently with each bioassay test and reported with the test results.

The tests specified in the July 1997 Ocean Plan shall be used to measure TUc.  Other tests may be added to the list when approved by the SWRCB.

13.	If toxicity testing results show a violation of any acute or chronic toxicity limitation identified in Discharge Specifications B.2 and B.3 of this Order, the discharger shall:

a.	Take all reasonable measures necessary to immediately minimize toxicity; and

b.	Increase the frequency of the toxicity test(s) that showed a violation to monthly until the results of at least two consecutive toxicity tests do not show violations.

14.	If the Executive Officer determines that toxicity testing shows consistent violation of any acute or chronic toxicity limitations identified in Discharge Specifications B.2 and B.3 of this Order, the discharger shall conduct a Toxicity Reduction Evaluation (TRE) which includes all reasonable steps to identify the source of toxicity.  Once the source of toxicity is identified, the discharger shall take all reasonable steps to reduce the toxicity to meet the toxicity limitations identified in Discharge Specifications B.2 and B.3 of this Order.

Within fourteen days of completion of the TRE, the discharger shall submit the results of the TRE, including a summary of the findings, data generated, a list of corrective actions necessary to achieve consistent compliance with all the toxicity limitations of this Order and prevent recurrence of violations of those limitations, and a time schedule for implementation of such corrective actions.  The corrective actions and time schedule shall be modified at the direction of the Executive Officer.

15.	The discharger must submit to this RWQCB a copy of its revised TRE workplan, incorporating chronic toxicity, within 6 months of the issuance of this Order.  Toxicity Reduction Evaluation Methods Manuals (Phases I, II, and III) are available to assist the discharger in developing and implementing the TRE workplan.

16.	For all bacterial analyses, sample dilutions should be performed so the range of values extends from 2 to 16,000 MPN/100 mL (most probable number). The detection methods used for each analysis shall be reported with the results of the analysis.  Detection methods used for coliforms (total and fecal) shall be those presented in the most recent edition of Standard Methods for the Examination of Water and Wastewater or any improved method determined by the Regional Board (and approved by USEPA) to be appropriate.

17.	The geometric mean used for determining compliance with bacterial standards is calculated with the following equation:

Geometric Mean = (C1 x C2 x ... x Cn) 1/n

where n is the number of days samples were collected during the period and C is the concentration of bacteria (MPN/100 mL) found on each day of sampling.

18.	Pursuant to the California Water Code Section 13267 (b) and 13383, the discharger shall comply with Monitoring and Reporting Program No. 2000-25 as specified by this Regional Board.  The discharger shall comply with all items of the "Standard Provisions" that are part of this Order (including Attachment No. 3 and Attachment No. 4).

19.	Neither the treatment nor the discharge of pollutants shall create a condition of pollution, contamination, or nuisance as defined by Section 13050 of the California Water Code.

20.	The discharger must comply with all conditions of this Order.  Any permit noncompliance constitutes a violation of the Clean Water Act and the California Water Code and is grounds for enforcement action; for permit termination, revocation and reissuance, or modification; or for denial of a Report of Waste Discharge application.

21.	The discharger shall take all reasonable steps to minimize or correct any adverse impact on the environment resulting from noncompliance with this Order, including such accelerated or additional monitoring as may be necessary to determine the nature and impact of the noncomplying discharge.

22.	This Order may be modified, revoked and reissued, or terminated for cause including, but not limited to, the following:

a.	Violation of any terms or conditions of this Order;

b.	Obtaining this Order by misrepresentation or failure to disclose fully all relevant facts; or

c.	A change in any condition that requires either a temporary or permanent reduction or elimination of the authorized discharge.

The filing of a request by the discharger for modification, revocation and reissuance, or termination of this Order, or a notification of planned change in anticipated noncompliance with this Order does not stay any condition of this Order.

23.	Notwithstanding Provision E.22 above, if a toxic effluent standard or prohibition (including any schedule of compliance specified in such effluent standard or prohibition) is established under Section 307(a) of the Clean Water Act for a toxic pollutant which is present in the discharge and such standard or prohibition is more stringent than any limitation for such pollutant in this Order, this Order shall be modified or revoked and reissued to conform to the toxic effluent standard or prohibition and the discharger so notified.

24.	In addition to any other grounds specified herein, this permit shall be modified or revoked at any time if, on the basis of any new data, this Regional Board determines that continued discharges may cause unreasonable degradation of the aquatic/marine environment of Mission Bay.

25.	The discharge shall not cause a violation of any applicable water quality standard for receiving waters adopted by the Regional Board or the State Water Resources Control Board as required by the Clean Water Act and regulations adopted thereunder.  If more stringent applicable water quality standards are promulgated or approved pursuant to Section 303 of the Clean Water Act or amendments thereto, the Regional Board will revise and modify this Order in accordance with the more stringent standards.

26.	The discharger shall comply with effluent standards or prohibitions established under Section 307(a) of the Clean Water Act for toxic pollutants within the time provided in the regulations that establish those standards or prohibitions, even if this Order has not yet been modified to incorporate the requirement.

27.	This Order is not transferable to any person except after notice to this Regional Board.  This Regional Board may require modification or revocation and reissuance of this Order to change the name of the discharger and incorporate such other requirements as may be necessary under the California Water Code and the Clean Water Act.  The discharger shall submit notice of any transfer of this Order's responsibility and coverage to a new discharger as described in Attachment No. 4.

28.	This Order does not convey any property rights of any sort or any exclusive privileges.  The requirements prescribed herein do not authorize the commission of any act causing injury to persons or property of another, nor protect the discharger from its liabilities under federal, state, or local laws, nor create a vested right for the discharger to continue its waste discharge.

29.	The discharger shall allow this Regional Board, or an authorized representative, or any representative of the United States Environmental Protection Agency upon the presentation of credentials and other documents as may be required by law, to:

a.	Enter upon the discharger’s premises where a regulated facility or activity is located or conducted, or where records must be kept under the conditions of this Order;

b.	Have access to and copy, at reasonable times, any records that must be kept under the conditions of this Order;

c.	Inspect at reasonable times any facilities, equipment (including monitoring and control equipment), practices or operation regulated or required under this Order; and

d.	Sample or monitor at reasonable times, for the purposes of assuring compliance with this Order or as otherwise authorized by the Clean Water Act or California Water Code, any substances or parameters at any location.

30.	The discharger shall, at all times, properly operate and maintain all facilities and systems of treatment and control (and related appurtenances) which are installed or used by the discharger to achieve compliance with the conditions of this Order.  Proper operation and maintenance includes effective performance, adequate funding, adequate operator staffing and training, and adequate laboratory and process controls including appropriate quality assurance procedures. This provision requires the operation of backup or auxiliary facilities or similar systems only when necessary to achieve compliance with the conditions of this Order.

31.	A copy of this Order shall be posted at a prominent location at SeaWorld San Diego, and shall be available to operating personnel at all times.

32.	The provisions of this Order are severable, and if any provision of this Order, or the application of any provision of this Order to any circumstances, is held invalid, the application of such provision to other circumstances, and the remainder of this Order, shall not be affected thereby.

- The discharger shall, as a provision of this order, develop and implement a 12-month study addressing potential impacts of eutrophication from the SeaWorld discharge on Mission Bay.  The objective of this study will be to determine if the SeaWorld discharge results in any eutrophication to the receiving waters of Mission Bay.  The proposed study shall, at a minimum:

- Include sampling of oxygen levels (D.O., B.O.D.), nitrates, and phosphates in the effluent and the receiving water area.

- Include a tentative schedule of sampling events.

- Evaluate and compare data obtained to any historical data that the discharger may be able to access.

- Cover all the seasons in the span of a continuous 12-month period.

- Compare these values to control stations in the Bay, and to the Basin Plan objectives.

The study plan shall be submitted to the Regional Board office, and initiated within six months of the adoption of this order.  The discharger will commence with the proposed study, and submit the results to the Regional Board within 20 months after the adoption of this Order.

### F. 	REPORTING REQUIREMENTS

- The discharger shall comply with the attached Monitoring and Reporting Program.  Monitoring results shall be reported at the intervals specified in the Monitoring and Reporting Program.

2.	The discharger must comply with standard monitoring and reporting requirements, where applicable, as stated in 40 CFR 122.41 (Attachment No. 4) and Attachment No. 3.

3.	Except for data determined to be confidential under Title 40, United States Code of Federal Regulations, Part 2, all reports prepared in accordance with the terms of this Order shall be available for public inspection at the offices of this Regional Board.  As required by the Clean Water Act, Reports of Waste Discharge, this Order, and effluent data shall not be considered confidential.

4.	This Order expires on March 8, 2005.  If the discharger wishes to continue an activity regulated by this Order after the expiration date of this Order, the discharger must apply for and obtain new waste discharge requirements.  The discharger must file a complete Report of Waste Discharge in accordance with Title 23, California Code of Regulations, not later than 180 days in advance of such date, as an application for issuance of new waste discharge requirements.

5.	The discharger shall file a new Report of Waste Discharge not less than 180 days prior to the following:

a.	Addition of any industrial waste to the discharge or the addition of a new process or product resulting in a change in the character of the wastes.

b.	Significant change in disposal method (e.g., change in the method of treatment which would significantly alter the nature of the waste).

c.	Significant change in disposal area (e.g., moving the discharge to a disposal area significantly removed from the original area, potentially causing different water quality or nuisance problems).

d.	Increase in flow beyond that specified in this Order.

e.	Other circumstances which result in a material change in character, amount, or location of the waste discharge.

f.	Any planned physical alterations or additions to the permitted facility.

6.	The discharger shall give advance notice to this Regional Board of any planned changes in the permitted facility or activity that may result in noncompliance with the requirements of this Order.

7.	The discharger must notify this Regional Board, in writing, at least 30 days in advance of any proposed transfer of this Order's responsibility and coverage to a new discharger.  The notice must include a written agreement between the existing and new discharger containing a specific date for the transfer of this Order's responsibility and coverage between the current discharger and the new discharger.  This agreement shall include an acknowledgment that the existing discharger is liable for violations up to the transfer date and that the new discharger is liable after the transfer date.

8.	The discharger shall report any noncompliance that may endanger health or the environment.  Any information shall be provided orally to this Regional Board within 24 hours from the time the discharger becomes aware of the circumstances.  A written submission shall contain a description of the noncompliance and its cause; the period of noncompliance, including exact dates and times, and if the noncompliance has not been corrected, the anticipated time it is expected to continue; and steps taken or planned to reduce, eliminate, and prevent recurrence of the noncompliance.  This Regional Board, or an authorized representative may waive the written report on a case-by-case basis if the oral report has been received within 24 hours.  The following occurrences must be reported to this Regional Board within 24 hours:

a.	Any upset which causes the effluent limitations of this order to be exceeded.

b.	Any violation of any prohibition of this Order.

9.	The discharger shall notify this Regional Board as soon as it knows or has reason to believe:

- That any activity has occurred or will occur which would result in the discharge, on a routine or frequent basis, of any toxic or non-toxic pollutant which is not limited in this Order, if that discharge will exceed the highest of the following “notification levels”:

(1)	One hundred micrograms per liter (100 µg/l);

(2)	Two hundred micrograms per liter (200 µg/l) for acrolein and acrylonitrile; five hundred micrograms per liter (500 µg/l) for 2,4-dinitrophenol and for 2-methyl-4,6-dinitrophenol; and one milligram per liter (1 mg/L) for antimony;

(3)	Five (5) times the maximum concentration value reported for that pollutant in the Report of Waste Discharge submitted in application for this Order in accordance with 40 CFR 122.21(g)(7); or

- The level established by this Regional Board in accordance with 40 CFR 122.44(f).

- That any activity has occurred or will occur which would result in any discharge, on a non-routine or infrequent basis, of a toxic or non-toxic pollutant which is not limited in the permit, if that discharge will exceed the highest of the following "notification levels:”

(1)	Five hundred micrograms per liter (500 µg/l);

(2)	One milligram per liter (1 mg/L) for antimony;

(3)	Ten (10) times the maximum concentration value reported for that pollutant in the Report of Waste Discharge submitted in application for this Order in accordance with 40 CFR 122.21(g)(7); or,

(4)	The level established by this Regional Board in accordance with 40 CFR 122.44(f).

10.	In the event that an effluent sample is found to contain coliform concentrations in excess of the effluent limits established in this permit, the discharger shall also notify the County of San Diego, Department of Environmental Health within 24 hours.

11.	The discharger shall furnish to this Regional Board, within a reasonable time, any information which this Regional Board may request to determine whether cause exists for modifying, revoking and reissuing, or terminating this Order, or to determine compliance with this Order.  The discharger shall also furnish to this Regional Board, upon request, copies of records required to be kept by this Order.

- The discharger shall provide adequate notice to this Regional Board of the following:

a.	Any new introduction of pollutants to the discharge.

b.	Any substantial change in the volume or character of pollutants being introduced into the discharge.

c.	For the purpose of this requirement, adequate notice shall include information on (1) the quality and quantity of waste introduced into the discharge, and (2) any anticipated impact of the change on the quantity or quality of effluent discharged to the Pacific Ocean.

13.	Where the discharger becomes aware that they failed to submit any relevant facts in a Report of Waste Discharge, or submitted incorrect information in a Report of Waste Discharge, or in any report to this Regional Board, they shall promptly submit such facts or information.

14.	All applications, reports, or information submitted to this Regional Board shall be signed and certified.

- All Reports of Waste Discharge shall be signed as follows:

(1)	For a corporation: by a responsible corporate officer.  For the purpose of this section, a responsible corporate officer means: (a) A president, secretary, treasurer, or vice-president of the corporation in charge of a principal business function, or any other person who performs similar policy- or decision-making functions for the corporation, or (b) the manager of one or more manufacturing, production, or operating facilities employing more than 250 persons or having gross annual sales or expenditures exceeding $25 million (in second-quarter 1980 dollars), if authority to sign documents has been assigned or delegated to the manager in accordance with corporate procedures.

(2)	For a partnership or sole proprietorship: by a general partner or the proprietor, respectively; or

- For a municipality, State, Federal or other public agency: by either a principal executive officer or ranking elected official.  For purposes of this section, a principal executive officer of a Federal agency includes: (a) the chief executive officer of the agency, or (b) a senior executive officer having responsibility for the overall operations of a principal geographic unit of the agency (e.g., Regional Administrators of U.S. EPA).

- All reports required by this Order, and other information requested by this Regional Board shall be signed by a person described in paragraph a. of this reporting requirement, or by a duly authorized representative of that person.  A person is a duly authorized representative only if:

(1)	The authorization is made in writing by a person described in paragraph a. of this reporting requirement;

(2)	The authorization specifies either an individual or a position having responsibility for the overall operation of the regulated facility or activity, such as the position of plant manager, operator of a well or a well field, superintendent, position of equivalent responsibility, or an individual or position having overall responsibility for environmental matters for the company. (A duly authorized representative may thus be either a named individual or any individual occupying a named position.); and,

- The written authorization is submitted to this Regional Board.

- If an authorization under paragraph b. of this reporting requirement is no longer accurate because a different individual or position has responsibility for the overall operation of the facility, a new authorization satisfying the requirements of paragraph b. of this reporting requirement must be submitted to this Regional Board prior to or together with any reports, information, or applications to be signed by an authorized representative.

d.	Any person signing a document under paragraph a. or b. of this reporting requirement shall make the following certification:

"I certify under penalty of law that this document and all attachments were prepared under my direction or supervision in accordance with a system designed to assure that qualified personnel properly gather and evaluate the information submitted.  Based on my inquiry of the person or persons who manage the system, or those persons directly responsible for gathering the information, the information submitted is, to the best of my knowledge and belief, true, accurate, and complete.  I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment for knowing violations."

15.	The discharger shall conduct a survey of its facilities and submit to this Regional Board a technical report that identifies the source of any chemical or biological pollutant that may be accidentally or intentionally discharged to Mission Bay and that could alter the natural water conditions or adversely affect the biological community.  As a minimum, the report shall contain the following information:

a.	Identification of chemicals stored where, if spillage should occur, they would be washed into the water circulation system or storm water drainage system.

b.	Assessment of the possibility of biological pollutants being accidentally or intentionally discharged into the water circulation system. Biological pollutants include any aquatic organism not indigenous to Mission Bay. The evaluation should consider a discharge that could be caused by human error, mechanical failure and malicious intent.

c.	Evaluation of the likelihood of chemical and biological pollutants being discharged to Mission Bay. If it is deemed reasonably possible that such a discharge might occur, the report shall recommend facilities and procedures that could be instituted to minimize both the possibility and effects of the discharge.

This report shall be submitted to the Executive Officer by September 1, 2000. After reviewing the report, the Executive Officer may require the discharger to establish additional safeguards to protect against the accidental discharge of chemical and biological pollutants to Mission Bay.

16.	The discharger shall submit reports and provide notifications to the Regional Board and other agencies as specified in this Order.  Reports shall be submitted and notifications shall be made to the following addresses:

a.	Executive Officer

California Regional Water Quality Control Board

San Diego Region

9771 Clairemont Mesa Boulevard, Suite A

San Diego, California  92124-1324

Phone - (858) 467-2952

Fax - (858) 571-6972

b		Environmental Health Services Division

Department of Health Services

County of San Diego

P.O. Box 85261

San Diego, California  92138-5261

Phone - (619) 338-2222

Fax - (619) 338-2174

# G.	NOTIFICATIONS

1.	Nothing in this Order shall be construed to preclude the institution of any legal action or relieve the discharger from any responsibilities, liabilities, or penalties to which the discharger is or may be subject to under Sections 311 and 510 of the Clean Water Act.

2.	The discharger is held accountable for responsibilities, liabilities, legal actions, and penalties as stated in Attachment No. 3 and Attachment No. 4 of this Order.

3.	This Order shall become effective ten days after the date of its adoption provided the Regional Administrator, United States Environmental Protection Agency, has no objection. If the Regional Administrator objects to its issuance, this Order shall not become effective until such objection is withdrawn.

4.	This Order supersedes Order No. 95-36 when this Order becomes effective.

I, John H. Robertus, Executive Officer, do hereby certify that the foregoing is a full, true and correct copy of an Order adopted by the California Regional Water Quality Control Board, San Diego Region, on April 12, 2000.

___________________________________

JOHN H. ROBERTUS

Executive Officer

H.	ORDER NO. 2000-25 ENDNOTES

A dilution factor of 21:1 was used to calculate the effluent limitations established for the following constituents: ammonia, chlorine residual, chronic toxicity, copper, halomethanes, and silver.

Definitions of constituent units are as follows:

TUa = Toxic units, acute

TUc = Toxic units, chronic

lb/Day = pounds per day

mg/L = milligrams per liter

g/L = micrograms per liter

mL/L = milliliters per liter

MPN/100mL = most probable number in 100 milliliters

CFU/100 mL = colony forming units in 100 milliliters

MGallons/Day = million gallons per day

NTU = nephelometric turbidity units

The monthly average effluent limitation shall be the moving arithmetic mean using the results of analyses of all samples collected during any 30 consecutive calendar day period.

The daily maximum effluent limitation shall apply to flow-weighted 24-hour composite samples.  The daily maximum receiving water limitation shall apply to grab sample determinations.

The maximum at any time shall be determined from the results of a single grab sample or from the results of a single composite sample collected over a period of 24 hours.

Halomethanes shall mean the sum of bromoform, bromomethane (methyl bromide), chloromethane (methyl chloride), chlorodibromomethane, and dichlorobromomehtane.

As reported in either colony forming units (CFU) or most probable number (MPN) per 100 mL.  The Basin Plan does not specify any laboratory method that must be performed, therefore, either multiple tube fermentation or millipore filtration methods are applicable.

Most Probable Number (MPN) per 100 mL, as obtained by the multiple tube fermentation process.