# CALIFORNIA REGIONAL WATER QUALITY CONTROL BOARD

SAN DIEGO REGION

ORDER NO. 2000-25

NPDES PERMIT NO. CA 0107336

MONITORING AND REPORTING PROGRAM

FOR SEAWORLD SAN DIEGO

SAN DIEGO COUNTY

DISCHARGE TO MISSION BAY

This Monitoring and Reporting Program supersedes Monitoring and Reporting Program No. 95-36 in its entirety.  This Monitoring and Reporting program shall become effective with the adoption of Order No. 2000-25.

### A.	PURPOSE

This monitoring program is intended to:

- Document potential short-term and long-term effects of the discharge to Mission Bay waters, sediments, biota, and beneficial uses.

- Assess treatment efficiency.

- Characterize the effluent.

- Determine compliance with NPDES permit terms and conditions.

## B.	MONITORING PROVISIONS

1.	Samples and measurements taken as required herein shall be representative of the volume and nature of the monitored discharge.  All samples shall be taken at the monitoring points specified in this Order and, unless otherwise specified, before the effluent joins or is diluted by any other waste stream, body of water, or substance.  Monitoring points shall not be changed without notification to and the approval of the Executive Officer.

2.	Appropriate flow measurement devices and methods consistent with accepted scientific practices shall be selected and used to ensure the accuracy and reliability of measurements of the volume of monitored discharges.  The devices shall be installed, calibrated and maintained to ensure that the accuracy of the measurements are consistent with the accepted capability of that type of device.  Devices selected shall be capable of measuring flows with a maximum deviation of less than + 5 percent from true discharge rates throughout the range of expected discharge volumes.

3.	All analyses shall be performed in a laboratory certified to perform such analyses by the California Department of Health Services or a laboratory approved by the Executive Officer.

C.	REPORTING SCHEDULE

1 .	Monitoring results shall be reported at intervals and in a manner specified in Order No. 2000-25 and in this monitoring and reporting program.  Monitoring reports shall be submitted to the Regional Board according to the following schedule:

| Monitoring Frequency | Reporting Period | Report Due |
| --- | --- | --- |
| Continuous, Daily, Weekly, or Monthly | All | By the first day of the second month after the month of sampling. |
| Quarterly | Jan.-March | May 1 |
|  | April-June | August 1 |
|  | July-September | November 1 |
|  | October-December | February 1 |
| Semiannually | January-June | August 1 |
|  | # July-December | February 1 |
| Annually | January-December | February 1 |
| Once during the life of the permit |  | No later than 6 months prior to the date of expiration of this Order. |

## D.	INFLUENT MONITORING

1.	Sampling stations shall be established at each intake point where representative samples of the influent can be obtained.  Influent samples shall be collected on the same day as, and shortly before the collection of effluent samples.

2.	The following shall constitute the influent monitoring program for each of the two discharge points.

| Parameter | Unit | Type of Sample | Minimum Frequency |
| --- | --- | --- | --- |
| Total Coliform | MPN/ 100mL | Grab | weekly |
| Fecal Coliform | MPN/ 100mL | Grab | weekly |
| Suspended Solids | mg/L | 24-hr. composite | quarterly |

E.	EFFLUENT MONITORING

- A sampling station shall be established at each discharge point where representative samples of the discharge to Mission Bay can be obtained.

2.	Samples shall not be collected within three days following a storm event.

3.	The following shall constitute the effluent monitoring program for the two discharge points.

| Parameter | Reporting Unit | Type of Sample | Minimum Frequency |
| --- | --- | --- | --- |
| Chronic Toxicity | TUc | 24-hr. composite | Every 5 years |
| Acute Toxicity | TUa | 24-hr. composite | annual |
| Ammonia | mg/L lb/Day | 24-hr. composite | semiannual |
| Copper | g/L lb/Day | 24-hr. composite | semiannual |
| Silver | g/L lb/Day | 24-hr. composite | semiannual |
| Turbidity | NTU | 24-hr. composite | semiannual |
| Grease and Oil | mg/L lb/Day | grab | semiannual |
| Halomethanes | g/L lb/Day | grab | semiannual |
| Settleable Solids | mL/L | grab | quarterly |
| Suspended Solids | mg/L | 24-hr. composite | quarterly |
| Temperature | C | grab/ continuous | monthly |
| Total Chlorine Residual | mg/L lb/Day | grab/ continuous | weekly |
| Enterococcus | CFU/100 mL | grab | weekly |
| Fecal Coliform | MPN/100 mL | grab | weekly |
| Total Coliform | MPN/100 mL | grab | weekly |
| pH | pH units | grab/ continuous | weekly |
| Flowrate | MGallons/Day | recorder/ totalizer | continuous |

4.	By February 1 of each year, the discharger shall submit an annual report to the Regional Board’s Executive Officer.  The report shall contain both tabular and graphical summaries of the monitoring data obtained during the previous year.  In addition, the discharger shall discuss their compliance record and the corrective actions taken or planned that may be needed to bring the discharge into full compliance with this Order.

F.	BIOMONITORING

- The presence of acute toxicity will be determined as specified in Methods for Measuring the Acute Toxicity of Effluents to Freshwater and Marine Organisms (EPA 600/4-90-027, August, 1993 or subsequent editions).  The discharger shall conduct annual toxicity tests on 24-hour composite effluent samples.  The discharger shall conduct 96-hour static-renewal tests with the topsmelt, Atherinops affinis.  The effluent concentrations will be 100%, and a laboratory control.  The effluent tests must be conducted with concurrent reference toxicant tests.  Both the reference toxicant and the effluent test must meet all test acceptability criteria as specified in the acute manual.  If the test acceptability criteria are not achieved, then the permittee must re-sample and re-test within 14 days. If acceptable test results are not achieved on the retest, a toxicity reduction evaluation (TRE) must be implemented.

Acute toxicity test results will be expressed as TUa, which equals 100/ NOAEC.  The NOAEC (No Observable Adverse Effects Concentration) is the highest concentration at which survival is not significantly different from the control in a 96-hour renewal test.

- TOXICITY REDUCTION EVALUATION

1.	The discharger shall develop a revised Toxicity Reduction Evaluation (TRE) workplan.  The workplan shall be subject to the approval of the Executive Officer and shall be modified as directed by the Executive Officer.  Dischargers shall submit the TRE workplan to the Executive Officer upon request of the Executive Officer.  The TRE workplan shall be developed no later than six months after the adoption of this Order in accordance with the following manuals:

a.	Generalized Methodology for Conducting Industrial Toxicity Reduction Evaluations (EPA/600/2-88/070).

b.	Toxicity Identification Evaluation, Phase I (EPA/600/6-91/005F).

c.	Methods for Aquatic Toxicity Identification Evaluations, Phase II (EPA/600/R-92/080).

d.	Methods for Aquatic Toxicity Identification Evaluations, Phase III (EPA/600/R-92/081).

2.	If toxicity testing results show a violation of any chronic or acute toxicity limitation identified in Discharge Specifications B.2 and B.3 of this Order, the discharger shall:

a.	Take all reasonable measures necessary to immediately minimize toxicity; and

b.	Increase the frequency of the toxicity test(s) that showed a violation to monthly until the results of at least two consecutive toxicity tests do not show violations.

3.	If the Executive Officer determines that toxicity testing shows consistent violation of any acute or chronic toxicity limitation identified in Discharge Specifications B.2 and B.3 of this Order, the discharger shall conduct a TRE which includes all reasonable steps to identify the source of toxicity.  Once the source of toxicity is identified, the discharger shall take all reasonable steps to reduce the toxicity to meet the toxicity limitations identified in Discharge Specifications B.2 and B.3 of this Order.

Within fourteen days of completion of the TRE, the discharger shall submit the results of the TRE, including a summary of the findings, data generated, a list of corrective actions necessary to achieve consistent compliance with all the toxicity limitations of this Order and prevent recurrence of violations of those limitations, and a time schedule for implementation of such corrective actions.  The corrective actions and time schedule shall be modified at the direction of the Executive Officer.

H.	 PROVISIONS

- All reports submitted in response to this Order shall comply with signatory requirements specified in Reporting Requirement F.14.

2.	The discharger shall implement the above monitoring program on the first day of the month following the effective date of this Order.

I, John H. Robertus, Executive Officer, do hereby certify that the foregoing is a full, true and correct copy of an Order adopted by the California Regional Water Quality Control Board, San Diego Region, on April 12, 2000.

___________________________________

JOHN H. ROBERTUS

Executive Officer