Bight 98 Summary

Sediment Toxicity (13 July – 16 Sep, 1998)

303 sites sampled once per week

Point Conception, CA to USA / Mexico Intl Border

Stratified Random Sampling

5 off shore	- river mouths

- large POTWs

- small POTWs

- remaining shallow areas (5-30m)

- remaining mid depth areas (30 – 120m)

3 within bays and harbors

(including marinas, ports/industrial areas, other harbor areas)

Testing

10 day amphipod survival on 241 bulk sediments

P450 Human Reporter Gene System (HRGS) on 268 sed extracts

(PAHs, dioxins, furans, some PCBs)

Qwiksed on 173 Sediment elutriates

Sublethal tox =  biolum of phytoplankton

Microtox on 88 Interstitial Waters

Data not valid due to handling problems

Results

Amphipod Tox Test

Toxicity found in 7 strata

Bays and Harbors:		13-37% of area

POTW Outfalls:		6% of area

Shallow Coastal Shelf:	3% of area

Quick Sed (beware of ammonia effects)

Bays and Harbors toxic

POTW Outfalls toxic

HRGS

Positive for 30 stations (mostly ports / industrial / marinas)

Temperature, pH, dissolved oxygen, salinity, and total ammonia were also measured.

No data found for Sediment Chemistry or Benthic Community Analysis

Summer Microbiology 1998 (2 Aug – 5 Sep)

307 sites sampled once per week

(5 sample min for calculation of 30 day geometric mean under the

California Ocean Plan and AB 411)

Point Conception, CA to USA / Mexico Intl Border

Methods

Coliforms

Membrane filtration, Multiple Tube Fermentation, Colilert and

Enterolert

Totals, Fecal, Total/Fecal and Fecal/Total

Indicator Thresholds from Cal Ocean Plan and AB 411

Viruses

Enterviruses, Adenoviruses and Coliphage

Results

Coliforms

> 90% of shoreline miles met all bacterial thresholds

Only 10.6% of 880 samples exceeded at least one indicator

Freshwater outfalls!

60% of shoreline miles failed state standards (monthly analysis) (multiple indicators and repetitively)

Viruses

7 out of 15 stations tested had positive results

Winter Microbiology (1 Feb – 5 Mar, 1999)

240 sites sampled once per week

Results

Coliforms

90% of stations did not exceed indicator thresholds

Freshwater Outfalls!

½ the shoreline miles in these areas failed state stds

Viruses

7 of 18 stations sampled exceeded indicator values

Storm Event (22 February 2000)

Sampled 36 hours after at least 1” of rain fell bight-wide

254 sites (1.1 – 3.0”)

Rain Background

So Cal Health Agencies post beach warnings if >0.1”

LAX Rainfall

0 – 0.1”	21 days /yr		0.50 < 1.00	4 days/yr

0.1 < 0.25	7			1.0 < 2.0 	3

0.25 < 0.50	5			2.0 <		1

Results

Up to 87% of shoreline miles exceeded water quality for 1 indicator

58% for 2 indicators, 38% for all 3 indicators

two thirds of exceedences were for multiple indicators in which one

was twice the allowable standard

BUT:
			even if 2” fell, some storm drains still had limited flow

If not over 0.25” the pollution was not widespread