Identification_Information:

Citation:

Citation_Information:

Originator:

Southern California Coastal Water Research Project

(SCCWRP)

Publication_Date: 20000701

Publication_Time: Unknown

Title:

Bight'98 Dry WInter Shoreline Microbiology Project

1999

Edition: Version 1

Publication_Information:

Publication_Place: Westminster, CA

Publisher: SCCWRP

Online_Linkage:

ftp.sccwrp.org/pub/download/MICROBIOLOGY_SURVEYS/S

UMMER_1998/Winter1999.zip

Description:

Abstract:

The Southern California Bight (SCB; Figure I-1),

an open embayment in the coast between Point

Conception and Cabo Colnett (south of Ensenada),

Baja California, is an important and unique

recreational resource.  World renowned for its

recreational waters, more than 100 million people

visit Southern California beaches and coastal

areas annually to sunbathe, surf, swim, skin- and

SCUBA-dive.  The average number of visitors to

Santa Monica Bay beaches on a summer weekend alone

is more than 600,000 (Economic Resources Data,

1993).   Southern California is also one of the

most densely populated coastal regions in the

country, which creates stress upon these

recreational resources.  Nearly 20 million people

inhabit coastal Southern California, a number that

is expected to increase another 20% by 2010 (NRC

1990).  Population growth generally results in

conversion of open land into non-permeable

surfaces.  This "hardening of the coast" increases

the rate of runoff and can impact water quality

through addition of sediment, toxic chemicals,

microbial pathogens and nutrients to the ocean.

Besides the impacts of land conversion, the SCB is

home to fifteen municipal wastewater treatment

facilities, eight power generating stations, 10

industrial treatment facilities, and 18 oil

platforms that discharge to the open coast.  An

important part of ensuring public safety in

shoreline recreational areas is bacteriological

monitoring.  Monitoring of bacteriological water

quality along the shoreline of the SCB is

currently performed by 20 agencies taking

measurements at 510 stations.  These agencies

include health departments and municipal

dischargers in compliance with National Pollution

Discharge Elimination System (NPDES) permits.

Together, these organizations collect and analyze

77,730 samples annually to assess beach quality

(Schiff et al. 1998).  Although this represents an

impressive amount of bacteriological monitoring,

the data cannot be easily integrated to provide a

regional assessment of condition.  Most monitoring

programs are spatially focused on a small set of

high-use beaches or other areas of concern and the

data from these programs cannot be easily

extrapolated to assess the condition of the coast

as a whole.  Moreover, many of the organizations

involved in beach monitoring use different

indicators to assess beach quality, and even those

that measure the same indicators often use

different sampling or analysis methodologies;

interlaboratory exercises to assess data

comparability are rare.   The inconsistencies with

bacteriological monitoring in the recreational

waters of coastal California have been recognized

by the State of California,  which recently passed

legislation, Assembly Bill 411, requiring the

State Health Services to adopt procedures that

increase consistency in data collection and

reporting.

Purpose:

Recognizing the need for greater consistency and

communication, all of the agencies that routinely

monitor bacteriological water quality of Southern

California beaches have agreed to conduct an

integrated survey to assess the overall condition

of the southern California shoreline in the summer

of 1998.   The study will also include an element

in which volunteer monitoring organizations will

enhance sampling effort within selected portions

of the Bight.   The study will be coordinated by

the Southern California Coastal Water Research

Project (SCCWRP) as one component of the Southern

California Bight 1998 Regional Monitoring Program

(Bight’98), in which 55 organizations (Table I-1)

have agreed to cooperate in assessing the overall

condition of the SCB ecosystem.   Bight’98 builds

upon the success of a similar SCCWRP-coordinated

regional monitoring effort conducted in 1994 to

assess the condition of offshore ecological

habitats (SCBPP 1998).  This data set compares

conditions in the summer of 1998 with dry winter

conditions in 1999.

Time_Period_of_Content:

Time_Period_Information:

Range_of_Dates/Times:

Beginning_Date: 19990201

Ending_Date: 19990305

Currentness_Reference: ground condition

Status:

Progress: Complete

Maintenance_and_Update_Frequency: None planned

Spatial_Domain:

Description_of_Geographic_Extent: Southern California Bight

Bounding_Coordinates:

West_Bounding_Coordinate: -120.2269

East_Bounding_Coordinate: -117.1251

North_Bounding_Coordinate: 34.4709

South_Bounding_Coordinate: 32.541

Keywords:

Theme:

Theme_Keyword_Thesaurus: None

Theme_Keyword: Microbiology

Access_Constraints: None

Use_Constraints: Citation Required

Point_of_Contact:

Contact_Information:

Contact_Person_Primary:

Contact_Person: Rachel Noble

Contact_Position: manager

Contact_Address:

Address_Type: mailing address

Address: 7171 Fenwick

City: Westminster

State_or_Province: California

Postal_Code: 92683

Country: USA

Contact_Voice_Telephone: (714) 894-2222

Hours_of_Service: 8:00 a.m. to 4:00 p.m. Monday Through Friday

Contact_Instructions: unavailable

Native_Data_Set_Environment: Microsoft Access Version 97

Cross_Reference:

Citation_Information:

Originator:

Southern California Coastal Water Reseach Project

(SCCWRP)

Originator:

Southern California Coastal Water Reseach Project

(SCCWRP)

Originator:

Southern California Coastal Water Reseach Project

(SCCWRP)

Publication_Date: 20000609

Publication_Time: 12000000

Title: Bight'98 Summer Shorleline Microbiology Project

Edition: Version 1

Publication_Information:

Publication_Place: Westminster, CA

Publisher: SCCWRP

Online_Linkage:

ftp.sccwrp.org/pub/download/MICROBIOLOGY_SURVEYS/S

UMMER_1998/Winter1999.zip

Data_Quality_Information:

Attribute_Accuracy:

Attribute_Accuracy_Report: Coordinates accurate to within 100 ft

Logical_Consistency_Report:

The tables may be related by the StationID field

or the SampleID field where applicable

Completeness_Report:

This data set includes where and when all samples

for shoreline microbiology analysis were taken,

and the results for each of the bacteriological

analyses.  Lab results data is missing for

stations 3189 19990201 for Enterococcus, Fecal

Coliforms, and Total Coliforms.  Station 3265

19990302 for Enterococcus.  Station 3361 19999215

for Enterococcus.  Station 3366 19990201 for

Enterococcus.  Station 3383 19990201 for

Enterococcus.  Station 3384 19990201 for

Enterococcus.  Station 3429 19990218 for

Enterococcus, Total Coliforms, and Fecal

Coliforms.  Station 3434 19990218 for

Enterococcus, Total Coliforms, and Fecal

Coliforms.

Positional_Accuracy:

Horizontal_Positional_Accuracy:

Horizontal_Positional_Accuracy_Report:

The station locations were determined using a hand

held Magellan GPS and are accurate with in a

radius of 100 ft

Vertical_Positional_Accuracy:

Vertical_Positional_Accuracy_Report: There is no vertical component in this study

Lineage:

Methodology:

Methodology_Type: Field

Methodology_Description:

Samples were collected in sterile sample bottles

or whirl-paks from ankle-deep waters on an

incoming wave just prior to receding, with the

sampler positioned downstream from the bottle and

the mouth of the bottle facing into the current.

After the sample was taken, the bottle was tipped

to decant enough sample to ensure 1 to 2 inches of

airspace in the sample bottle.  The bottle was

tightly capped and stored on ice in the dark.  All

samples were returned to the laboratory in time to

begin analysis within 6 hours of sample

collection.

Methodology_Type: Lab

Methodology_Description:

n addition to the baseline sampling design, the

study will also include an adaptive component in

which the level of sampling is increased in areas

where elevated indicator bacteria levels are

found.  Criteria for increased sampling are based

on exceeding a single sample maximum of:  Total

coliforms   >10,000 cfu or MPN/100 ml; or Fecal

coliforms   > 400 cfu or MPN/100 ml; or

Enterococcus  >104 cfu/100 ml; or Coliform Index

(fecal/total coliform x 100) > 20, if total

coliform > 5,000 cfu or MPN/100 mL  The adaptive

component will involve sampling additional sites

on either side of the elevated indicator site

within a week following the observation.   For

sites located on open shoreline, the adaptive

sites will be located 100 yards on either side of

the elevated site; for water outlet sites, the

adaptive sites will be located 25 yards on either

side.   C. Indicators  All three bacterial

indicators currently used for routine monitoring

of recreational water in Southern California,

total coliforms, fecal coliforms, and enterococcus

will be measured for the shoreline microbiology

component of Bight'98.  Samples will be collected

in ankle-deep waters, when the wave is coming

toward the sampler.  Sample bottles will be stored

on ice and in the dark during transportation, and

laboratory analysis will begin within 6 hours of

sampling.   Several analytical methods will be

used for laboratory analysis of the samples.  The

study is conducted primarily through the donation

of in-kind services and not all facilities that

monitor bacteriological water quality in Southern

California use the same methods. A performance

based approach will be used to ensure data

comparability among labs; intercalibration tests

using common samples will be performed and only

laboratories that meet the performance criteria

will be permitted to participate.  Further details

on the intercalibration process are provided in

Section IV.   The methods to be used by each

participant are outlined in Table II-2 and

described below; more detailed information on the

methods can be found in Standard Methods for the

Examination of Water and Wastewater, 19th Edition,

1995.  Total and Fecal Coliform: Total and fecal

coliforms are a group of bacteria typically found

in fecal contamination.  The total coliform group

consists of several genera of bacteria belonging

to the family Enterobacteriaceae.  Fecal coliform

bacteria are the component of total coliforms that

are found in the feces of warm-blooded animals.

Detection of this subset of organisms is based on

incubation of the sample at an elevated

temperature above that of the total coliform

procedure.    Three methods will be used to

measure coliform bacteria.   The first is membrane

filtration (MF).   MF is a direct plating method

for the detection and enumeration of bacteria in

water.  The second method will be multiple tube

fermentation (MTF).  MTF involves inoculating

multiple tubes of broth with dilutions of sample.

Organism density is based on the number of tubes

with acid and gas production at the various

dilutions and reported in terms of the Most

Probable Number (MPN).  This number is based on

certain probability formulas and is an estimate of

the mean density of organisms in the sample.

Methodology_Type: Field

Methodology_Description:

Samples were collected in sterile sample bottles

or whirl-paks from ankle-deep waters on an

incoming wave just prior to receding, with the

sampler positioned downstream from the bottle and

the mouth of the bottle facing into the current.

After the sample was taken, the bottle was tipped

to decant enough sample to ensure 1 to 2 inches of

airspace in the sample bottle.  The bottle was

tightly capped and stored on ice in the dark.  All

samples were returned to the laboratory in time to

begin analysis within 6 hours of sample

collection.

Process_Step:

Process_Description:

Bacterial samples collected at the beach and

incubated in the lab

Process_Date: 19999395

Spatial_Data_Organization_Information:

Spatial_Reference_Information:

Horizontal_Coordinate_System_Definition:

Geographic:

Latitude_Resolution: .00001

Longitude_Resolution: .00001

Geographic_Coordinate_Units: Decimal degrees

Geodetic_Model:

Horizontal_Datum_Name: North American Datum of 1983

Ellipsoid_Name: Geodedic Reference System 80

Semi-major_Axis: 6378137

Denominator_of_Flattening_Ratio:

298.257

4.2.2.2 Depth Resolution:

.1

Entity_and_Attribute_Information:

Detailed_Description:

Entity_Type:

Entity_Type_Label: Stations

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: NominalLatitude

Attribute_Definition: the latitude of the station

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 32.541

Range_Domain_Maximum: 34.4709

Entity_Type:

Entity_Type_Label: Stations

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: NominalLongitude

Attribute_Definition: the longitude of the station

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: -120.2269

Range_Domain_Maximum: -117.1251

Entity_Type:

Entity_Type_Label: Stations

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: Station Description

Attribute_Definition: a physical description of the station

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Unrepresentable_Domain: A physical description of the stations

Entity_Type:

Entity_Type_Label: Stations

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: AreaWeight

Attribute_Definition: the statistical area weight

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 1

Range_Domain_Maximum: 50

Entity_Type:

Entity_Type_Label: Stations

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: InclusionProbability

Attribute_Definition: the statistical inclusion probability

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: .02

Range_Domain_Maximum: 1

Entity_Type:

Entity_Type_Label: Stations

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information management plan

Attribute:

Attribute_Label: Comments

Attribute_Definition: additional comments

Attribute_Definition_Source: Bight'98 Information management plan

Attribute_Domain_Values:

Unrepresentable_Domain: Any additional Comments

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: SampleDate

Attribute_Definition: The date the sample was collected

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 19990201

Range_Domain_Maximum: 19990305

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: SampleTime

Attribute_Definition:

The time the sample was collected expressed in 24

hour time format

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 05:15

Range_Domain_Maximum: 14:55

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: WeatherCode

Attribute_Definition: the weather at the time the sample was taken

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: luWeatherCodes

Enumerated_Domain_Value_Definition: look up list 8 Weather Codes

Enumerated_Domain_Value_Definition_Source: Bight'98 Information Management Plan

Entity_Type:

Entity_Type_Label: Results

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information management plan

Attribute:

Attribute_Label: StationID

Attribute_Definition: a geographic label

Attribute_Definition_Source: Bight'98 Information management plan

Attribute_Domain_Values:

Codeset_Domain:

Codeset_Name: tblStations

Codeset_Source: Bight'98 Information Management Plan

Entity_Type:

Entity_Type_Label: Results

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information management plan

Attribute:

Attribute_Label: SampleDate

Attribute_Definition: the date the sample was processed in the lab

Attribute_Definition_Source: Bight'98 Information management plan

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 19990201

Range_Domain_Maximum: 19990305

Entity_Type:

Entity_Type_Label: Results

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information management plan

Attribute:

Attribute_Label: ParameterCode

Attribute_Definition: the parameter that was measured

Attribute_Definition_Source: Bight'98 Information management plan

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: luParameters

Enumerated_Domain_Value_Definition: the parameters measured in this project

Enumerated_Domain_Value_Definition_Source: Bight'98 Information management plan

Entity_Type:

Entity_Type_Label: Results

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information management plan

Attribute:

Attribute_Label: Qualifier

Attribute_Definition: a modifier for the numerical results

Attribute_Definition_Source: Bight'98 Information management plan

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: luQualifiers

Enumerated_Domain_Value_Definition: look up list 13 qualifier codes

Enumerated_Domain_Value_Definition_Source: Bight'98 Information Management Plan

Entity_Type:

Entity_Type_Label: Results

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information management plan

Attribute:

Attribute_Label: Result

Attribute_Definition: the numerical result of the colony count

Attribute_Definition_Source: Bight'98 Information management plan

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 0

Range_Domain_Maximum: 1600000

Entity_Type:

Entity_Type_Label: Results

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information management plan

Attribute:

Attribute_Label: Units

Attribute_Definition: the units associated with the numerical result

Attribute_Definition_Source: Bight'98 Information management plan

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: luUnits

Enumerated_Domain_Value_Definition: the units associated with the numerical result

Enumerated_Domain_Value_Definition_Source: Bight'98 Information management plan

Entity_Type:

Entity_Type_Label: Results

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information management plan

Attribute:

Attribute_Label: LabCode

Attribute_Definition: the code for the agency preforming the analysis

Attribute_Definition_Source: Bight'98 Information management plan

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: luAgencyCodes

Enumerated_Domain_Value_Definition: a list of two letter agency codes

Enumerated_Domain_Value_Definition_Source: Bight'98 Information Management Plan

Entity_Type:

Entity_Type_Label: Results

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information management plan

Attribute:

Attribute_Label: AnalysisMethod

Attribute_Definition: The laboratory method used to detect the parameter

Attribute_Definition_Source: Bight'98 Information management plan

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: AnalysisMethod

Enumerated_Domain_Value_Definition: The method used to detect the parameter

Enumerated_Domain_Value_Definition_Source: Bight'98 Information management plan

Entity_Type:

Entity_Type_Label: Results

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information management plan

Attribute:

Attribute_Label: StartTime

Attribute_Definition: The time the incubation started

Attribute_Definition_Source: Bight'98 Information management plan

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 13:08

Range_Domain_Maximum: 18:40

Entity_Type:

Entity_Type_Label: Results

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information management plan

Attribute:

Attribute_Label: Comments

Attribute_Definition: additional comments

Attribute_Definition_Source: Bight'98 Information management plan

Attribute_Domain_Values:

Unrepresentable_Domain: Any additional Comments

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information management plan

Attribute:

Attribute_Label: StationID

Attribute_Definition: a geographic location label

Attribute_Definition_Source: Bight'98 Information management plan

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: tblStations

Enumerated_Domain_Value_Definition: a geographic station label

Enumerated_Domain_Value_Definition_Source: Bight'98 Information management plan

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information management plan

Attribute:

Attribute_Label: SampleDate

Attribute_Definition: the date the sample was taken

Attribute_Definition_Source: Bight'98 Information management plan

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 19990201

Range_Domain_Maximum: 19990305

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information management plan

Attribute:

Attribute_Label: SampleTime

Attribute_Definition: the time the sample was taken

Attribute_Definition_Source: Bight'98 Information management plan

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 04:30

Range_Domain_Maximum: 14:35

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information management plan

Attribute:

Attribute_Label: AgencyCode

Attribute_Definition:

the two digit code for the acency taking the

sample

Attribute_Definition_Source: Bight'98 Information management plan

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: luAgencyCodes

Enumerated_Domain_Value_Definition: a list of two digit agency codes

Enumerated_Domain_Value_Definition_Source: Bight'98 Information Management Plan

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: Surf

Attribute_Definition:

the height of the surf when the sample was

collected

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: luSurf

Enumerated_Domain_Value_Definition:

one of three values for the surf condition Low,

Mid, High

Enumerated_Domain_Value_Definition_Source: Bight'98 Information Management Plan

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: SeaState

Attribute_Definition:

the condition of the sea at the time the sample

was collected

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: luSeaState

Enumerated_Domain_Value_Definition: calm, rough, or choppy

Enumerated_Domain_Value_Definition_Source: Bight'98 Information Management Plan

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: EvidenceOfSewage

Attribute_Definition: was there any evidence of sewage?

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: No

Range_Domain_Maximum: Yes

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: WaterOutletFl

Attribute_Definition:

if the station was located at or near a stormwater

outlet, was the water flowing?

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: No

Range_Domain_Maximum: Yes

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: PeopleInWater

Attribute_Definition:

the number of people in the water when the sample

was collected

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 0

Range_Domain_Maximum: 150

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: StationFailCode

Attribute_Definition: was the sample uncollectable

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: luList 9 fail codes

Enumerated_Domain_Value_Definition: look up list 9

Enumerated_Domain_Value_Definition_Source: Bight'98 Information Management Plan

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: Comments

Attribute_Definition: Additional Comments

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Unrepresentable_Domain: Any Additional Comments

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: Dogs

Attribute_Definition:

the number of dogs on the beach at the time the

sample was collected

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 0

Range_Domain_Maximum: 5

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: Birds

Attribute_Definition:

the number of birds on the beach at the time the

sample was collected

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 0

Range_Domain_Maximum: 2500

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: SeaLions

Attribute_Definition:

the number of sea lions on the beach at the time

the sample was collected

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Range_Domain:

Range_Domain_Minimum: 0

Range_Domain_Maximum: 2

Entity_Type:

Entity_Type_Label: StationOccupation

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: Current

Attribute_Definition:

the direction the long-shore current was moving at

the time the sample was collected

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: luCurrent

Enumerated_Domain_Value_Definition: upcoast, downcost, no current, or indeterminate

Enumerated_Domain_Value_Definition_Source: Bight'98 Information Management Plan

Entity_Type:

Entity_Type_Label: Stations

Entity_Type_Definition: Table

Entity_Type_Definition_Source: Bight'98 Information Management Plan

Attribute:

Attribute_Label: StationID

Attribute_Definition: A geographic location label

Attribute_Definition_Source: Bight'98 Information Management Plan

Attribute_Domain_Values:

Enumerated_Domain:

Enumerated_Domain_Value: tblStations

Enumerated_Domain_Value_Definition: the station list

Enumerated_Domain_Value_Definition_Source: Bight'98 Information Management Plan

Distribution_Information:

Distributor:

Contact_Information:

Contact_Person_Primary:

Contact_Person: Rachel Noble

Contact_Position: manager

Contact_Address:

Address_Type: mailing address

Address: 7171 Fenwick

City: Westminster

State_or_Province: California

Postal_Code: 92683

Country: USA

Contact_Voice_Telephone: (714) 894-2222

Hours_of_Service: 8:00 a.m. to 4:00 p.m. Monday Through Friday

Contact_Instructions: unavailable

Resource_Description: 1999 Winter Shoreline Microbiology data

Distribution_Liability:

Although these data have been processed

successfully on a computer system at the SCCWRP,

no warranty expressed or implied is made regarding

the accuracy or utility of the data on any other

system or for general or scientific purposes, nor

shall the act of distribution constitute any such

warranty. This disclaimer applies both to

individual use of the data and aggregate use with

other data.  It is strongly recommended that these

data are directly acquired from a SCCWRP server,

and not indirectly through other sources which may

have changed the data in some way.  It is also

strongly recommended that careful attention be

paid to the contents of the metadata file

associated with these data.  SCCWRP shall not be

held liable for improper or incorrect use of the

data described and/or contained herein.

Standard_Order_Process:

Digital_Form:

Digital_Transfer_Information:

Format_Name: ASCII

Digital_Transfer_Option:

Online_Option:

Computer_Contact_Information:

Network_Address:

Network_Resource_Name: ftp.sccwrp.org/pub/download/MICROBIOLOGY_SURVEYS

Fees: None

Metadata_Reference_Information:

Metadata_Date: 20000701

Metadata_Contact:

Contact_Information:

Contact_Person_Primary:

Contact_Person: Rachel Noble

Contact_Position: manager

Contact_Address:

Address_Type: mailing address

Address: 7171 Fenwick

City: Westminster

State_or_Province: California

Postal_Code: 92683

Country: USA

Contact_Voice_Telephone: (714) 894-2222

Hours_of_Service: 8:00 a.m. to 4:00 p.m. Monday Through Friday

Contact_Instructions: unavailable

Metadata_Standard_Name:

NBS Content Standard for Nation Biological

Information Infrastructure Metadata

Metadata_Standard_Version: FGDC-STD-001-1998

Metadata_Time_Convention: local time with time differential factor