Prepared by Brennan Ott

8/2/01

Encinitas Creek

904.510

It is recommended that Encinitas Creek not be listed.

# Watershed Characteristics

Encinitas Creek is located within the Santa Marcos Creek Watershed. Encinitas Creek is 5.6 miles long.  Uses for the creek include agricultural water supply, contact and non-contact recreation, wildlife habitat, and a warm freshwater habitat as well.

Water Quality Objectives not Obtained

Encinitas Creek was found to be in violation of inland surface water quality standards for total dissolved solids (TDS).

Evidence of Impairment

The water quality data used for Encinitas Creek was taken on June 3, 1998.  The TDS concentration on this day was 2082 mg/l.  The standard set by the Basin Plan for inland surface waters for TDS is 500 mg/l.  The water in the Encinitas Creek is in clear violation of this standard.

## Bio-assessments were conducted in May, September and November 1998 and May 1999.  Results from these bio-assessments indicate that the distribution (richness) of taxa between September and November is relatively the same, while May showed a decrease.  However, in May, the site displayed a greater abundance than the September and November samples.  November and September have similar levels of functional feeding groups whereas May exhibited a decrease in functional feeders from these two months.  At all sampling times, the site was dominated by a few disturbance tolerant insect and worm taxa and the functional feeding groups were composed mainly of collectors, filterers and grazers.  Shredders were rare.  The physical habitat quality at all three sampling periods was fair to good. The area sampled consisted of 5 riffles downstream of Green Valley Road.

Extent of Impairment

Encinitas Creek was sampled at station EC-LCA, located Green Valley Road.  Since there is only one water quality data point for this Creek, not much can be said about its impairment.

Potential Sources

There is not sufficient data to support any hard claims as to the source(s) of the pollution, but there is enough to warrant good assumptions.

Since the sample site is near a road, possible high TDS concentrations may due to urban runoff.

TMDL Priority

It is recommended that Encinitas Creek not be placed on the TMDL list.

# Source References

Water quality standards were taken from the Water Quality Control Plan for the San Diego Basin.  Bio-assessment data taken from San Diego Regional Water Quality Control Board: 1999 Biological Assessment Annual Report.  Water quality data taken from Linda Pardy sheet 1.