SANTA MARGARITA RIVER, Lower

Hydrologic Subareas # 902.11 and 902.12

NEW 303(d) LISTINGS

Total Dissolved Solids (TDS)

# PREVIOUS 303(d) LISTINGS

None

WATERSHED CHARACTERISTICS

Santa Margarita Hydrologic Unit is a rectangular area of approximately 750 square miles.  It includes portions of US Marine Corps’ Camp Pendleton, as well as the civilian populations of Murrieta, Temecula, and part of Fallbrook. The Santa Margarita Hydrologic Unit is comprised of the following nine hydrologic areas: the Ysidora, Deluz, Murrieta, Auld, Pechanga, Wilson, Cave Rocks, Aguanga and Oak Grove Hydrologic Areas. Annual precipitation ranges from less that 12 inches near the coast to more than 45 inches inland near Palomar Mountain. The major surface water storage areas are Vail Lake and O'Neil Lake.1

Santa Margarita Hydrologic Unit is drained largely by the Santa Margarita River, Murrieta Creek and the Temecula Creek.  Murrieta Creek flows southeasterly from the northern slope of the Santa Rosa Plateau to the confluence with the Temecula Creek to form the Santa Margarita River The Santa Margarita then flows approximately 27 miles to the Pacific Ocean. The coastal Santa Margarita Lagoon is at the mouth and lies within the Camp Pendleton Naval Reservation of the US Marine Corps.  The slough at the mouth of the river is normally closed off from the ocean by a sandbar.1  The Santa Margarita River provides groundwater recharge to Camp Pendleton’s only domestic water supply. 1,2

The Santa Margarita is divided into lower and upper reaches as defined by the confluence of the DeLuz Creek.

Designated beneficial uses for the Santa Margarita River include MUN, AGR, IND, REC1, REC2, WARM, COLD, WILD, GWR, RARE and PROC.1

WATER QUALITY OBJECTIVE NOT ATTAINED

TDS	The Basin Plan objective for TDS is 750 mg/L.

EVIDENCE OF IMPAIRMENT

TDS	Quarterly sampling collected, compiled and analyzed by Camp Pendleton2 from 1997 to 2000 shows the river to have elevated concentrations of TDS. In the lower Santa Margarita River at Ysidora, 3 of 9 samples (33%) exceeded the Basin Plan objective.  The average value was 704 mg/L and the median value was 717 mg/L.  Trend analysis from 1987 to 20002 indicates a slight increase in TDS concentrations, indicating a decrease in water quality (see figure below).

TDS may consist of carbonates, bicarbonates, chlorides, sulfates, phosphates, nitrates, magnesium, sodium, iron and manganese.  The most frequent constituents are usually salts (sodium, chloride, boron, etc.)  Most of the problem can be traced to human impacts, and therefore, can be mitigated. Geologic conditions help to define the natural levels of many of these constituents.  High TDS concentrations may be expected to impair the MUN beneficial use1. High concentrations of TDS are also expected to impact the AGR beneficial use directly through irrigation waters or indirectly through adverse effects on soil permeability.  TDS values between 450 to 2000 mg/L are expected to have a slight to moderate restriction on use of waters for irrigation of crops1.

EXTENT OF IMPAIRMENT

TDS	TDS concentrations were in excess of the Basin Plan objective at the one station it was measured in the lower Santa Margarita River.  Combining this knowledge with the evidence of elevated concentrations of TDS in the upper Santa Margarita River (see previous Fact Sheet ) leads to listing the entire lower reach (10.4 miles).

POTENTIAL SOURCES

TDS 	Urban runoff, other point sources and non-point sources.

TMDL PRIORITY
TDS	Medium (There are current and extensive efforts underway to address water quality issues in the Santa Margarita Watershed.3  Various stakeholders have recognized and identified some existing and potential water quality issues and are attempting to gain a better understanding, conduct more monitoring, target pollutant sources and develop comprehensive management strategies.  TMDLs would provide the legal framework necessary to address some of these problems and could assist in this coordinated effort and be a major component of this work.)

INFORMATION SOURCES

Water Quality Objectives

1 Water Quality Control Plan for the San Diego Basin (9), 1994.  California Regional

Water Quality Control Board, San Diego Region.

### Data Sources

2 Final Report of Water Quality Studies & Proposed Watershed Monitoring Program for

Portions of San Mateo & Santa Margarita River Watershed. Marine Corps Base, Camp Pendleton, CA. Contract No. N68711-95-D-7573, D.O. 0021.

3 DRAFT Framework Monitoring Plan for the Santa Margarita Watershed California. US

Bureau of Reclamation. CDM Federal Corp., Boyle Engr. Corp. RECON.  Feb 2001.