San Luis Rey River

903.12 Upstream of Old Highway 395 and Interstate 15

It is recommended that the San Luis Rey River 3 riffles upstream of Old Highway 395 and Interstate 15 not be added to 303(d) listing for 2002 for benthic community degradation.

Watershed Characteristics

The San Luis Rey River is located in the San Luis Rey Watershed in north end of San Diego County, California. The area examined for this summary was described as 3 riffles upstream of Old Highway 395 and Interstate 15.

The San Luis Rey River is designated as Inland Surface Water.  It is used for municipal and domestic, agricultural, and industrial water supply, contact and non-contact recreation, warm freshwater habitat, and wildlife habitat.

# Water Quality Objectives not Obtained

The San Luis Rey river was sampled for aquatic insects and compared to other locations in San Diego County in the San Diego Regional Water Quality Control Board: 1999 Biological Assessment Annual Report

# Evidence of Impairment

None

# Extent of Impairment

The San Luis Rey River originates from Lake Henshaw in its upper reaches and runs parallel to Highway 76 in the lower segment all the way to the pacific coastline. The area examined were 3 riffles upstream of Old Highway 395 and Interstate 15, in May, September, and November 1998, and May of 1999.

The Benthic Macroinvertebrate Index (BMI) ranking for the San Luis Rey River at 3 riffles upstream of Old Highway 395 and Interstate 15, designated SLRR-395 in the study, were at or above the overall study average (BMI > 0) for this location when compared against other sites in San Diego County. In May of 1998 and Sept 1998, SLRR-395 ranked above average in the site comparison while the samples collected in November 1998 and May of 1999 ranked slightly above average. It was noted in the “Recommendations” section that this initial study indicates the San Luis Rey watershed is a potential candidate for reference conditions for this region (Harrington 1999).

Since the sampling events occurred only 4 times in a 12-month period, it is difficult to come to any conclusions about the watershed.  The preliminary studies are encouraging but further bioassessment and water quality data will be needed before any management decisions can be made.

# Potential Sources

The San Luis Rey Watershed Guidelines 2000 indicates that land use of the watershed is 50% undeveloped, 37% agriculture, 5% Residential, 5% Parks and Recreation, Water 2% and Industrial, Transportation, Commercial and Military comprising the remaining 1%.

# TMDL Priority

From the data presented in the San Diego County in the “San Diego Regional Water Quality Control Board: 1999 Biological Assessment Annual Report” it appears that the San Luis Rey River 3 riffles upstream of Old Highway 395 and Interstate 15 has average to above average aquatic benthic communities when compared to other locations in San Diego County. It is recommended that these stretches of the San Luis Rey River not have a 303(d) listing for benthic communities and be considered a low priority.  Furthermore benthic communities studies are needed before supportable conclusions can be reached about the health of the aquatic insects in this watershed.

# Source References

All water quality standards were taken from the Water Quality Control Plan for the San Diego Basin.

San Luis Rey River Watershed Guidelines, 2000

Harrington, James. “San Diego Regional Water Quality Control Board: 1999 Biological Assessment Report.” California Department Fish and Game. 1999.

.