Project Title:  San Diego County Municipal Copermittees Bight 2008 Sediment Chemistry, Toxicity and Benthic Infauna Data

Originator:  County of San Diego Municipal Copermittees

Contact:  Jo Ann Weber; e-mail:  joann.weber@sdcounty.ca.gov; tel. 858-495-5317

Publication Info.:  Published as part of the 2008/2009 Urban Runoff Monitoring Final Report  under the San Diego County Municipal National Pollutant Discharge Elimination System (NPDES) Permit (RWQCB-Order NO. R9-2007-0001).

Coordinate System:  NAD83

Project Background and Objectives:  The Bight program is a regional assessment of the aquatic health of the San Diego estuaries and lagoons along the Southern California Bight (Bight) and is organized every 5 years by the Southern California Coastal Water Research Project (SCCWRP). The study area extends from Point Conception to the Mexican border.

The objective of this monitoring program is to provide data needed to answer questions related to the Southern California Bight, the San Diego Region, and the individual lagoons of study. The data will assist in determining the spatial extent of the impacts in the bight and identify the trends in sediment conditions. Additionally, monitoring of sediment conditions using a spatially-balanced design will directly determine if the conditions in receiving waters are protective, or likely to be protective, of beneficial uses, and determine the extent and magnitude of the current or potential receiving water problems. Lastly it will determine urban runoff sources and contribution to the receiving water problems and determine if the conditions are getting better or worse.  A detailed project description together with background, objectives and the basic elements of the sampling design may be found in the Southern California Bight 2008 Regional Marine Monitoring Survey (Bight’08) Coastal Ecology Workplan (included with this submittal).

Geographical Setting/ Sampling Locations:  Monitoring locations (Table 1) included eight lagoons/estuaries in the San Diego Region.

Table 1: Bight 2008 Locations

| Lagoon | Station Code | Longitude | Latitude |
| --- | --- | --- | --- |
| Santa Margarita | 902_6303 | -117.4312 | 33.232267 |
|  | 902_6308 | -117.415167 | 33.2334167 |
|  | 902_6311 | -117.409 | 33.2344 |
|  | 902_6314 | -117.40745 | 33.2354 |
|  | 902_6317 | -117.40555 | 33.235567 |
| Agua Hedionda Lagoon | 904_6269 | -117.33775 | 33.13925 |
|  | 904_6270 | -117.3186167 | 33.139625 |
|  | 904_6271 | -117.3251 | 33.14016667 |
|  | 904_6280 | -117.3281167 | 33.14456667 |
|  | 904_6282 | -117.33565 | 33.1451 |
| Los Penasquitos Lagoon | 906_6228 | -117.2485677 | 32.93045247 |
|  | 906_6229 | -117.2509766 | 32.9316569 |
|  | 906_6230 | -117.2533667 | 32.9321 |
|  | 906_6232 | -117.2579 | 32.9328 |
|  | 906_6236 | -117.2568848 | 32.9338501 |
| San Elijo Lagoon | 904_6239 | -117.2706833 | 33.008 |
|  | 904_6242 | -117.2635905 | 33.01061605 |
|  | 904_6243 | -117.2732833 | 33.01131667 |
|  | 904_6244 | -117.2796333 | 33.01413333 |
|  | 904_6245 | -117.2769833 | 33.01435 |
| San Diego River | 907_6181 | -117.235026 | 32.75680745 |
|  | 907_6189 | -117.242 | 32.75766 |
|  | 907_6192 | -117.2269368 | 32.75791016 |
|  | 907_6197 | -117.2208171 | 32.76009115 |
|  | 907_6200 | -117.21015 | 32.76053 |
| Sweetwater Estuary | 909_6052 | -117.1032389 | 32.65135905 |
|  | 909_6057 | -117.1005046 | 32.65211995 |
|  | 909_6060 | -117.0949219 | 32.65368652 |
|  | 909_6065 | -117.0912923 | 32.65504557 |
|  | 909_6069 | -117.0871257 | 32.65668132 |
| Tijuana River Estuary | 911_6001 | -117.1281 | 32.55658333 |
|  | 911_6004 | -117.1223958 | 32.55737712 |
|  | 911_6009 | -117.11604 | 32.55922038 |
|  | 911_6010 | -117.1111833 | 32.55935 |
|  | 911_6012 | -117.108431 | 32.5624471 |

Project/ Task Description:  Data were collected along longitudinal-transect to investigate changes in sediment conditions as the relative distance from freshwater-input areas of lagoons increases. Lagoons were partitioned into five segments and sampling stations were located using a Generalized Random Tessellated Stratified (GRTS) sampling design consistent with Bight protocols as described in the Southern California Bight 2008 Regional Marine Monitoring Survey (Bight’08) Field Operations Manual (included with this submittal).

Sediment grab samples were collected during the summer (dry season) and analyzed for chemistry, toxicity, and benthic community as described in the Bight’08 Coastal Ecology Field Operations Manual – Appendix G:  Bight’08 Sediment Chemistry Sampling Guide. Table 2 below provides a summary of the sediment parameters and methods.  The sediment samples were further tested for toxicity using Eohaustorius estuarius, and Mytilus galloprovincialis using methods and procedures described in the Southern California Bight 2008 Regional Marine Monitoring Survey (Bight’08) Toxicology Laboratory Manual (included with this submittal).

Benthic community assemblage samples were also collected and analyzed.  Benthic community sampling, storage and processing procedures are described in the Southern California Bight 2008Regional Marine Monitoring Survey (Bight’08)Macrobenthic (Infaunal) Sample Analysis Laboratory Manual (included with this submittal).  Table 3 below provides a list of all species found in the samples.  In addition to the sediment grab samples, water samples were collected for total suspended solids and bacteria (total coliform, fecal coliform, and Enterococci).

Table-2.  Bight ’08 Water and Sediment Sample Analytical List, Methods, and Detection Limits

| Group/Analyte | Analysis Method | Units | MDL | RL | No. Samples Analyzed |
| --- | --- | --- | --- | --- | --- |
| Water Sample Analysis |  |  |  |  |  |
| Total Suspended Solids | SM 2540 D | mg/L | NR | NR | 45 |
| Total Coliform | SM 9221 B | MPN/ 100 ml | NR | NR | 41 |
| Fecal Coliform | SM 9221 E | MPN/ 100 ml | NR | NR | 41 |
| Enterococci | SM 9230 B | MPN/ 100 ml | NR | NR | 41 |
| Sediment Sample Analysis |  |  |  |  |  |
| General Parameters |  |  |  |  |  |
| Percent Solids | EPA 160.3 | % Dry Weight | 0.1 | 0.1 | 43 |
| Particle Size Distribution | SM 2560 D | µm | 0.020 | 0.24 | 43 |
| Total Organic Carbon | EPA 9060A | % Dry Weight | 0.01 | 0.02 | 43 |
| Acid Volatile Sulfide | Plumb, 1981 and TERL | mg/kg dw | 0.5 | 0.1 | 43 |
| Total Nitrogen | SM4500-N | mg/kg dw | 2 | 4 | 43 |
| Metals |  |  |  |  |  |
| Aluminum, Total | EPA 6020 | mg/kg dw | 1 | 5 | 43 |
| Antimony, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Arsenic, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Barium, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Beryllium, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Cadmium, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Chromium, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Cobalt, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Copper, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Iron, Total | EPA 6020 | mg/kg dw | 1 | 5 | 43 |
| Lead, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Manganese, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Mercury, Total | EPA 245.7 | mg/kg dw | 0.01 | 0.02 | 43 |
| Molybdenum, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Nickel, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Selenium, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Silver, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Strontium, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Thallium, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Tin, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Titanium, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Vanadium, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Zinc, Total | EPA 6020 | mg/kg dw | 0.025 | 0.05 | 43 |
| Synthetic Pyrethroids |  |  |  |  |  |
| Allethrin | EPA8270C-NCI | μg/kg dw | 0.5 | 2 | 43 |
| Bifenthrin | EPA8270C-NCI | μg/kg dw | 0.5 | 2 | 43 |
| Cyfluthrin | EPA8270C-NCI | μg/kg dw | 0.5 | 2 | 43 |
| Cypermethrin | EPA8270C-NCI | μg/kg dw | 0.5 | 2 | 43 |
| Fenpropathrin | EPA8270C-NCI | μg/kg dw | 0.5 | 2 | 43 |
| Deltamethrin | EPA8270C-NCI | μg/kg dw | 0.5 | 2 | 43 |
| Esfenvalerate | EPA8270C-NCI | μg/kg dw | 0.5 | 2 | 43 |
| Fenvalerate | EPA8270C-NCI | μg/kg dw | 0.5 | 2 | 43 |
| Fluvalinate | EPA8270C-NCI | μg/kg dw | 0.5 | 2 | 43 |
| L-Cyhalothrin | EPA8270C-NCI | μg/kg dw | 0.5 | 2 | 43 |
| Permethrin-cis | EPA8270C-NCI | μg/kg dw | 5 | 25 | 43 |
| Permethrin-trans | EPA8270C-NCI | μg/kg dw | 5 | 25 | 43 |
| Prallethrin | EPA8270C-NCI | μg/kg dw | 0.5 | 2 | 43 |
| Resmethrin | EPA8270C-NCI | μg/kg dw | 5 | 25 | 43 |
| Organochlorine Pesticides & PCBs |  |  |  |  |  |
| 2,4'-DDD | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| 2,4'-DDE | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| 2,4'-DDT | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| 4,4'-DDD | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| 4,4'-DDE | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| 4,4'-DDT | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Aldrin | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| BHC-alpha | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| BHC-beta | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| BHC-delta | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| BHC-gamma (Lindane) | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Chlordane-cis | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Chlordane-trans | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Nonachlor-cis | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Nonachlor-trans | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Dieldrin | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Dacthal | EPA 8270 | μg/kg dw | 5 | 10 | 43 |
| Dicofol | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Endosulfan Sulfate | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Endosulfan-I | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Endosulfan-II | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Endrin | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Endrin Aldehyde | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Endrin Ketone | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Heptachlor | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Heptachlor Epoxide | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Methoxychlor | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Mirex | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Oxychlordane | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Perthane | EPA 8270 | μg/kg dw | 5 | 10 | 43 |
| Toxaphene | EPA 8270 | μg/kg dw | 10 | 50 | 43 |
| 4,4’-DDMU | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB003 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB008 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB018 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB028 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB031 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB033 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB037 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB044 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB049 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB052 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB056/060 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB066 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB070 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB074 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB077 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB081 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB087 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB095 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB097 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB099 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB101 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB105 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB110 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB114 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB118 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB105 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB110 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB114 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB118 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB119 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB123 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB126 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB128 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB138 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB141 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB149 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB151 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB153 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB156 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB157 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB158 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB167 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB168/132 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB169 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB170 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB174 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB177 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB180 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB183 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB187 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB189 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB194 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB195 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB200 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB201 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB203 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB206 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| PCB209 | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Polynuclear Aromatic Hydrocarbons |  |  |  |  |  |
| 1-Methylnaphthalene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| 1-Methylphenanthrene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| 1,6,7-Trimethylnaphthalene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| 2,6-Dimethylnaphthalene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| 2-Methylnaphthalene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Acenaphthene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Acenaphthylene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Anthracene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Benz[a]anthracene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Benzo[a]pyrene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Benzo[b]fluoranthene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Benzo[e]pyrene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Benzo[g,h,i]perylene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Benzo[k]fluoranthene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Biphenyl | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Chrysene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Dibenz[a,h]anthracene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Dibenzothiophene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Fluoranthene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Fluorene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Indeno[1,2,3-c,d]pyrene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Naphthalene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Perylene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Phenanthrene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Pyrene | EPA 8270 | μg/kg dw | 1 | 5 | 43 |
| Toxicity |  |  |  |  |  |
| Eohaustorius estuaries | 10 day amphipod test | % Survival | NA | NA | 40 |
| Mytillus galloprovinciali | 8 hour sediment pore water interface | % Survival | NA | NA | 40 |

NR – Not Recorded

Table 3.  Benthic organisms collected as part of the Bight’08 Monitoring Program.

| Species or Family | Pylum | Number of Locations with Occurances | Range in Number of Individuals Observed at Station |
| --- | --- | --- | --- |
| Acanthina sp | Mollusca | 1 | 1 |
| Acteocina culcitella | Mollusca | 3 | 2-51 |
| Acteocina inculta | Mollusca | 24 | 1-128 |
| Actiniaria | Cnidaria | 2 | 1-2 |
| Acuminodeutopus heteruropus | Arthropoda | 1 | 1 |
| Alderia willowi | Mollusca | 4 | 1-8 |
| Alia carinata | Mollusca | 4 | 1-50 |
| Alvania sp | Mollusca | 1 | 1 |
| Amathia sp | Ectoprocta | 1 | 1 |
| Americorophium salmonis | Arthropoda | 1 | 1 |
| Amphideutopus oculatus | Arthropoda | 1 | 9 |
| Amphilochidae | Arthropoda | 1 | 1 |
| Amphiodia digitata | Echinodermata | 1 | 2 |
| Amphipholis squamata | Echinodermata | 6 | 1-1021 |
| Amphiuridae | Echinodermata | 1 | 1 |
| Ampithoe longimana | Arthropoda | 3 | 13-100 |
| Ampithoe sp | Arthropoda | 4 | 1-22 |
| Anemonactis sp A | Cnidaria | 1 | 1 |
| Anoplodactylus erectus | Arthropoda | 2 | 1-2 |
| Aphelochaeta monilaris | Annelida | 1 | 1 |
| Aphelochaeta sp | Annelida | 1 | 1 |
| Aplousobranchia | Chordata | 1 | 1 |
| Aplysiopsis enteromorphae | Mollusca | 1 | 1 |
| Apoprionospio pygmaea | Annelida | 1 | 1 |
| Arcularia tiarula | Mollusca | 4 | 1-2 |
| Argopecten ventricosus | Mollusca | 2 | 1-8 |
| Armandia brevis | Annelida | 12 | 1-22 |
| Ascidiacea sp A | Chordata | 1 | 1 |
| Asthenothaerus diegensis | Mollusca | 2 | 1-2 |
| Axiothella sp | Annelida | 1 | 19 |
| Barentsia sp | Entoprocta | 1 | 1 |
| Barleeia sp | Mollusca | 12 | 1-221 |
| Bemlos macromanus | Arthropoda | 4 | 7-93 |
| Bivalvia | Mollusca | 2 | 1-6 |
| Boccardia proboscidea | Annelida | 2 | 2 |
| Boccardiella hamata | Annelida | 2 | 1-6 |
| Brachyura | Arthropoda | 2 | 1-2 |
| Brania californiensis | Annelida | 2 | 6-17 |
| Bugula neritina | Ectoprocta | 1 | 2 |
| Bugula sp | Ectoprocta | 1 | 1 |
| Bulla gouldiana | Mollusca | 2 | 1-4 |
| Caecum californicum | Mollusca | 3 | 1-16 |
| Callianax biplicata | Mollusca | 1 | 2 |
| Capitella capitata Cmplx | Annelida | 23 | 1-371 |
| Capitellidae | Annelida | 2 | 1-5 |
| Caprella californica | Arthropoda | 2 | 38-126 |
| Carinoma mutabilis | Nemertea | 6 | 1-6 |
| Ceriantharia | Cnidaria | 1 | 1 |
| Cerithidea californica | Mollusca | 19 | 1-240 |
| Chione californiensis | Mollusca | 1 | 1 |
| Chione sp | Mollusca | 1 | 1 |
| Cirriformia sp | Annelida | 1 | 3 |
| Cirriformia sp HYP1 | Annelida | 3 | 13-29 |
| Cleantioides occidentalis | Arthropoda | 1 | 1 |
| Clevelandia ios | Chordata | 3 | 1 |
| Conopeum sp | Ectoprocta | 1 | 1 |
| Conus californicus | Mollusca | 1 | 3 |
| Cooperella subdiaphana | Mollusca | 1 | 1 |
| Copepoda | Arthropoda | 1 | 1 |
| Crepidula sp | Mollusca | 1 | 6 |
| Crepipatella lingulata | Mollusca | 2 | 1-7 |
| Crucibulum spinosum | Mollusca | 2 | 1-9 |
| Cryptomya californica | Mollusca | 3 | 1 |
| Ctenostomata | Ectoprocta | 1 | 2 |
| Dendraster sp | Echinodermata | 1 | 1 |
| Diadumene sp | Cnidaria | 7 | 1-31 |
| Diopatra ornata | Annelida | 1 | 1 |
| Diplocirrus sp SD1 | Annelida | 1 | 6 |
| Diptera | Arthropoda | 1 | 1 |
| Dorvillea (Schistomeringos) annulata | Annelida | 2 | 1 |
| Dorvillea (Schistomeringos) longicornis | Annelida | 1 | 2 |
| Echinoidea | Echinodermata | 1 | 1 |
| Ectoprocta | Ectoprocta | 2 | 1 |
| Edwardsia californica | Cnidaria | 2 | 1-7 |
| Elasmopus bampo | Arthropoda | 2 | 4-6 |
| Entoprocta | Entoprocta | 1 | 1 |
| Eohaustorius barnardi | Arthropoda | 1 | 4 |
| Epitonium sawinae | Mollusca | 2 | 1 |
| Ericthonius brasiliensis | Arthropoda | 3 | 7-800 |
| Eteone lighti | Annelida | 2 | 1-2 |
| Eteone sp 11 | Annelida | 1 | 1 |
| Euchone limnicola | Annelida | 5 | 1-20 |
| Euclymeninae | Annelida | 2 | 3-4 |
| Euclymeninae sp A | Annelida | 1 | 7 |
| Eulithidium pulloides | Mollusca | 1 | 1 |
| Eulithidium rubrilineatum | Mollusca | 1 | 1 |
| Euphilomedes carcharodonta | Arthropoda | 2 | 2-3 |
| Excirolana chiltoni | Arthropoda | 2 | 1 |
| Exogone lourei | Annelida | 11 | 1-86 |
| Exogone sp A | Annelida | 2 | 1-50 |
| Exosphaeroma sp | Arthropoda | 1 | 1 |
| Fabricia limnicola | Annelida | 5 | 1-139 |
| Flabellifera | Arthropoda | 1 | 1 |
| Gammaridea | Arthropoda | 5 | 2-10 |
| Garveia sp | Cnidaria | 1 | 1 |
| Gastropoda | Mollusca | 1 | 1 |
| Gastropoda sp 1 | Mollusca | 1 | 7 |
| Glossaulax reclusianus | Mollusca | 1 | 1 |
| Glycera americana | Annelida | 4 | 1-3 |
| Glycera macrobranchia | Annelida | 1 | 3 |
| Gobiidae | Chordata | 1 | 1 |
| Goniada littorea | Annelida | 2 | 1-3 |
| Grandidierella japonica | Arthropoda | 27 | 1-981 |
| Granulina margaritula | Mollusca | 2 | 1-35 |
| Halosydna johnsoni | Annelida | 1 | 1 |
| Haminoea vesicula | Mollusca | 3 | 1-17 |
| Harmothoe imbricata Cmplx | Annelida | 2 | 1-3 |
| Harpacticoida | Arthropoda | 1 | 3 |
| Hartmanodes hartmanae | Arthropoda | 3 | 1-7 |
| Hemigrapsus oregonensis | Arthropoda | 7 | 1-3 |
| Hemipodus borealis | Annelida | 2 | 1 |
| Hippolyte californiensis | Arthropoda | 1 | 4 |
| Laevicardium substriatum | Mollusca | 11 | 1-28 |
| Laonice cirrata | Annelida | 1 | 2 |
| Latocestidae | Platyhelminthes | 1 | 1 |
| Leitoscoloplos pugettensis | Annelida | 10 | 1-71 |
| Leptochelia dubia | Arthropoda | 4 | 1-68 |
| Leptopecten latiauratus | Mollusca | 1 | 4 |
| Leptoplanidae | Platyhelminthes | 1 | 2 |
| Leptosynapta sp | Echinodermata | 2 | 1-2 |
| Leukoma laciniata | Mollusca | 3 | 1-8 |
| Leukoma sp | Mollusca | 1 | 1 |
| Lineidae | Nemertea | 23 | 1-10 |
| Littorina planaxis | Mollusca | 2 | 1-2 |
| Lyonsia californica | Mollusca | 3 | 1-11 |
| Macoma nasuta | Mollusca | 1 | 1 |
| Macoma sp | Mollusca | 1 | 2 |
| Mactrotoma californica | Mollusca | 4 | 1-2 |
| Malacoplax californiensis | Arthropoda | 2 | 1-3 |
| Marphysa angelensis | Annelida | 1 | 3 |
| Marphysa sp SR1 | Annelida | 1 | 1 |
| Mayerella acanthopoda | Arthropoda | 3 | 2-6 |
| Mediomastus sp | Annelida | 19 | 1-116 |
| Megalomma pigmentum | Annelida | 3 | 1-2 |
| Melinna oculata | Annelida | 1 | 2 |
| Microphthalmus hystrix | Annelida | 1 | 1 |
| Molgula sp | Chordata | 3 | 1-5 |
| Monocorophium acherusicum | Arthropoda | 4 | 1-34 |
| Monocorophium insidiosum | Arthropoda | 13 | 1-583 |
| Monocorophium sp | Arthropoda | 12 | 1-75 |
| Monocorophium uenoi | Arthropoda | 8 | 2-70 |
| Musculista senhousia | Mollusca | 10 | 1-81 |
| Naineris sp | Annelida | 1 | 114 |
| Naineris sp SW1 | Annelida | 1 | 47 |
| Neanthes acuminata Cmplx | Annelida | 10 | 1-64 |
| Nebalia kensleyi | Arthropoda | 1 | 5 |
| Nemertea | Nemertea | 9 | 1-26 |
| Neodexiospira pseudocorrugata | Annelida | 1 | 36 |
| Neotrypaea gigas | Arthropoda | 3 | 1-4 |
| Neotrypaea sp | Arthropoda | 2 | 1 |
| Nippoleucon sp WS1 | Arthropoda | 4 | 1-10 |
| Notomastus magnus | Annelida | 2 | 1-10 |
| Notomastus sp | Annelida | 3 | 1-3 |
| Notomastus tenuis | Annelida | 4 | 1-4 |
| Ocinebrina foveolata | Mollusca | 1 | 6 |
| Odontosyllis phosphorea | Annelida | 2 | 1-5 |
| Oedicerotidae | Arthropoda | 1 | 1 |
| Oligochaeta | Annelida | 21 | 1-544 |
| Ophiactis simplex | Echinodermata | 1 | 1 |
| Owenia fusiformis | Annelida | 1 | 1 |
| Oxyurostylis pacifica | Arthropoda | 2 | 12-101 |
| Palaemon macrodactylus | Arthropoda | 2 | 1-3 |
| Paleonemertea | Nemertea | 1 | 1 |
| Paracerceis sculpta | Arthropoda | 4 | 1-109 |
| Paradexamine sp A | Arthropoda | 1 | 1 |
| Paranemertes californica | Nemertea | 2 | 1-2 |
| Paranthura elegans | Arthropoda | 4 | 2-11 |
| Pectinaria californiensis | Annelida | 1 | 1 |
| Penilla sp | Arthropoda | 1 | 5 |
| Periploma discus | Mollusca | 1 | 1 |
| Petricola sp | Mollusca | 2 | 1-8 |
| Pherusa capulata | Annelida | 1 | 1 |
| Phoronida | Phorona | 17 | 1-192 |
| Pista brevibranchiata | Annelida | 2 | 2-14 |
| Platyhelminthes | Platyhelminthes | 2 | 1 |
| Podocerus fulanus | Arthropoda | 2 | 38-194 |
| Podocerus sp | Arthropoda | 1 | 1 |
| Polyandrocarpa zorritensis | Chordata | 1 | 1 |
| Polydora cornuta | Annelida | 2 | 1-3 |
| Polydora limicola | Annelida | 1 | 2 |
| Polydora nuchalis | Annelida | 19 | 1-3917 |
| Polydora sp | Annelida | 1 | 2 |
| Polyophthalmus pictus | Annelida | 2 | 3 |
| Postasterope barnesi | Arthropoda | 1 | 3 |
| Prionospio (Minuspio) lighti | Annelida | 2 | 1 |
| Prionospio (Prionospio) heterobranchia | Annelida | 12 | 1-19 |
| Protohyale sp | Arthropoda | 6 | 2-199 |
| Pseudodexiospira corrugata | Annelida | 1 | 9 |
| Pseudopolydora paucibranchiata | Annelida | 14 | 1-134 |
| Pugettia dalli | Arthropoda | 1 | 1 |
| Quietula y-cauda | Chordata | 1 | 2 |
| Rhabdocoela | Platyhelminthes | 2 | 1 |
| Roperia poulsoni | Mollusca | 1 | 1 |
| Rudilemboides stenopropodus | Arthropoda | 2 | 2-10 |
| Sabellaria nanella | Annelida | 1 | 4 |
| Saxidomus nuttalli | Mollusca | 1 | 1 |
| Scolelepis (Parascolelepis) sp SD1 | Annelida | 7 | 1 |
| Scoletoma erecta | Annelida | 2 | 1 |
| Scoletoma sp | Annelida | 1 | 1 |
| Scoletoma sp B | Annelida | 1 | 1 |
| Scoletoma sp C | Annelida | 8 | 1-22 |
| Scoletoma zonata | Annelida | 1 | 7 |
| Scoloplos acmeceps | Annelida | 1 | 1 |
| Solen rostriformis | Mollusca | 4 | 1-2 |
| Sphaeromatidae | Arthropoda | 2 | 1 |
| Sphaerosyllis californiensis | Annelida | 1 | 1 |
| Spio maculata | Annelida | 1 | 1 |
| Spiochaetopterus costarum Cmplx | Annelida | 1 | 1 |
| Spionidae | Annelida | 1 | 3 |
| Spiophanes duplex | Annelida | 7 | 1-3 |
| Spirorbidae | Annelida | 2 | 1-15 |
| Streblospio benedicti | Annelida | 17 | 1-168 |
| Styela sp | Chordata | 2 | 1 |
| Stylochus exiguus | Platyhelminthes | 1 | 1 |
| Synaptotanais notabilis | Arthropoda | 3 | 2-56 |
| Syngnathus sp | Chordata | 1 | 1 |
| Tagelus californianus | Mollusca | 5 | 1-6 |
| Tagelus sp | Mollusca | 10 | 1-6 |
| Tagelus subteres | Mollusca | 10 | 1-131 |
| Tanaidacea | Arthropoda | 1 | 2 |
| Tectura depicta | Mollusca | 3 | 1-27 |
| Tellina cadieni | Mollusca | 9 | 1-9 |
| Tellina meropsis | Mollusca | 8 | 1-12 |
| Tethygenia opata | Arthropoda | 7 | 1-25 |
| Tetrastemma sp | Nemertea | 1 | 1 |
| Thalamoporella californica | Ectoprocta | 1 | 1 |
| Theora lubrica | Mollusca | 5 | 1-3 |
| Thraciidae | Mollusca | 2 | 1 |
| Thysanocardia nigra | Sipuncula | 1 | 1 |
| Tresus nuttallii | Mollusca | 2 | 1-2 |
| Tryonia imitator | Mollusca | 21 | 1-836 |
| Tubulanus polymorphus | Nemertea | 3 | 1 |
| Turbonilla sp | Mollusca | 1 | 2 |
| Veneridae | Mollusca | 4 | 1-2 |
| Venerupis philippinarum | Mollusca | 13 | 1-35 |
| Zeuxo normani | Arthropoda | 3 | 1-32 |
| Zoobotryon verticillatum | Ectoprocta | 1 | 1 |
| Zygonemertes virescens | Nemertea | 1 | 1 |

Quality Control: The quality control requirements employed in this project are those described in Southern California Bight 2008 Regional Marine Monitoring Survey (Bight 08’) Quality Assurance Plan, July 2008 (included with this submittal).  These include methods used for sample collection and handling, data management, validation, and record keeping (including proper chain of custody procedures), and data quality objectives for laboratory measurements and analysis.

Enclosed Documents in Electronic Format:

Bight’08 Coastal Ecology Field Operations Manual – Appendix G:  Bight’08 Sediment Chemistry Sampling Guide. Electronic Filename: Bight08_FieldManual_AppendixG.pdf

Southern California Bight 2008 Regional Marine Monitoring Survey (Bight’08) Coastal Ecology Workplan.  Electronic Filename: Bight08_CEworkplan.pdf

Southern California Bight 2008 Regional Marine Monitoring Survey (Bight’08) Field Operations Manual.  Electronic Filename: Bight08_FieldManual.pdf

Southern California Bight 2008 Regional Marine Monitoring Survey (Bight’08) Macrobenthic (Infaunal) Sample Analysis Laboratory Manual. Electronic Filename: Bight08_BenthicLabManual.pdf.

Southern California Bight 2008 Regional Marine Monitoring Survey (Bight 08’) Quality Assurance Plan, July 2008.   Electronic Filename: Bight08_AQplan.pdf

Southern California Bight 2008 Regional Marine Monitoring Survey (Bight’08) Toxicology Laboratory Manual.  Electronic Filename: Bight08_ToxicologyLabManual_AppendixG.pdf

Weston Solutions Inc. Quality Assurance Program Plan/ Microbiology.  Electronic Filename: Appendix D- Micro lab QAPP.pdf.

Enclosed Electronic Data Files:

County of San Diego Southern California Bight 2008 Regional Marine Monitoring Survey (Bight’08) Benthic Invertebrate Data.  Electronic filename:  COSD Bight08 Benthic Inv_Data.xls

County of San Diego Southern California Bight 2008 Regional Marine Monitoring Survey (Bight’08) Chemistry Data.  Electronic filename:  COSD Bight08 Chemistry_Data.xls

County of San Diego Southern California Bight 2008 Regional Marine Monitoring Survey (Bight’08) Station Location Data.  Electronic filename:  COSD Bight08  Stations_Data.xls

County of San Diego Southern California Bight 2008 Regional Marine Monitoring Survey (Bight’08) Toxicity Data.  Electronic filename:  COSD Bight08 Toxicity_Data.xls

County of San Diego Southern California Bight 2008 Regional Marine Monitoring Survey (Bight’08) Microbiology and Total Suspended Solids (TSS) Data.  Electronic filename:  COSD Bight08 TSS Microbio_Data.xls