Project Title:  County of San Diego Co-Permittees Receiving Waters Urban Runoff Monitoring and Reporting Program (Order No. 2007-01) – Chemistry, Toxicology, Microbiology and Periphyton Data (2001-2008).

Originator:  County of San Diego National Pollution Discharge Elimination Program (NPDES)

Contact:  Jo Ann Weber; e-mail:  joann.weber@sdcounty.ca.gov; tel. 858-495-5317

Publication Info.:  This data are published annually as part of the San Diego County Municipal Copermittees Urban Runoff Monitoring Annual Reports as required by the San Diego County Municipal National Pollutant Discharge Elimination System (NPDES) Permit (RWQCB-Order No. R9-2007-0001).

Coordinate System:  WGS84

Project Background and Objectives:  The Receiving Waters and Urban Runoff Monitoring Program section of the Permit requires the Copermittees to collaborate with all other Copermittees to develop, conduct, and report on an annual watershed program.  The objectives of the program listed in Section I.A of the Permit are as follows:

- Assess compliance with the Permit.

- Measure and improve the effectiveness of the Copermittees’ urban runoff management programs.

- Assess the chemical, physical and biological impacts to receiving waters resulting from urban runoff discharges.

- Characterize urban runoff discharges.

- Identify sources of specific pollutants.

- Prioritize drainage and subdrainage areas that need management actions.

- Detect and eliminate illegal connections and illicit discharges (ICID) to the MS4.

- Assess the overall health of receiving waters.

Geographical Setting/ Sampling Locations:  Table 1 includes Mass Loading stations (MLS) and Temporary Watershed Assessment Stations (TWAS).  MLS stations were sampled from 2001-2008.  TWAS stations were sampled 2007-2008.  Table 2 includes stations sampled for the SWAMP periphyton assessment.

Table 1:  Receiving Waters and Urban Runoff Monitoring Stations

| Waterbodies | Station Code | Longitude | Latitude |
| --- | --- | --- | --- |
| Santa Margarita River | 902SMR-MLS | -117.3876351 | 33.23748559 |
| Santa Margarita River | 902SMR-MLS-2 | -117.2623041 | 33.39567144 |
| San Luis Rey River | 903SLR-MLS | -117.3582502 | 33.22064763 |
| San Luis Rey River | 903SLR-TWAS-1 | -117.2232925 | 33.28785873 |
| Agua Hedionda Creek | 904AHC-MLS | -117.2970815 | 33.14951951 |
| Agua Hedionda Creek | 904AHC-TWAS-1 | -117.2415829 | 33.15420869 |
| Loma Alta Creek | 904LAC-TWAS-1 | -117.361672 | 33.188217 |
| Buena Vista Creek | 904BVC-TWAS-1 | -117.3267 | 33.18088 |
| Escondido Creek | 904ESC-MLS | -117.2260318 | 33.04829013 |
| Escondido Creek | 904ESC-TWAS-1 | -117.13045 | 33.09904 |
| San Dieguito River | 905SDC-MLS | -117.205625 | 32.99908167 |
| San Dieguito River | 905SDC-TWAS-1 | -117.07538 | 33.0434 |
| Santa Ysabel Creek | 905SDC-TWAS-2 | -117.0316185 | 33.06154785 |
| Los Peñasquitos | 906LPC-MLS | -117.2226197 | 32.90459772 |
| Carroll Canyon Creek | 906LPC-TWAS-1 | -117.223349 | 32.900535 |
| Los Peñasquitos | 906LPC-TWAS-2 | -117.084042 | 32.94262 |
| Rose Creek | 906RC-TWAS-1 | -117.2236 | 32.81719 |
| Tecolote Creek | 906TC-MLS | -117.2030756 | 32.77293804 |
| San Diego River | 907SDR-MLS | -117.1686286 | 32.76524518 |
| Chollas | 908CC-SD8(1) | -117.1209628 | 32.70481788 |
| Sweetwater | 909SR-MLS | -117.0636041 | 32.65072453 |
| Otay River | 910OR-TWAS-1 | -117.084161 | 32.589961 |
| Tijuana River | 911TJR-MLS | -117.0840613 | 32.55131097 |

Table 2. Periphyton Monitoring Locations

| STATION | Station Code | Longitude | Latitude |
| --- | --- | --- | --- |
| Sandia Creek | 902REF-SC | -117.2490333 | 33.4247 |
| Santa Margarita River | 902SMR-TWAS-1 | -117.262370 | 33.396020 |
| Rainbow Creek | 902RC-WGR | -117.202050 | 33.407650 |
| San Luis Rey River | 903SLRR-BR | -117.360406 | 33.217109 |
| San Luis Rey River | 903SLR-TWAS-1 | -117.2232925 | 33.28785873 |
| Doane Creek | 903REF-DC | -116.893653 | 33.334049 |
| Agua Hedionda Creek | 904AHC-ECR | -117.297590 | 33.148840 |
| Agua Hedionda Creek | 904AHC-TWAS-1 | -117.2415829 | 33.15420869 |
| Buena Vista Creek | 904BVC-TWAS-1 | -117.3267 | 33.18088 |
| Escondido Creek | 904ESC-MLS | -117.2260318 | 33.04829013 |
| Escondido Creek | 904ESC-TWAS-1 | -117.13045 | 33.09904 |
| Loma Alta Creek | 904LAC-TWAS-1 | -117.361672 | 33.188217 |
| Green Valley Creek | 905GVC-WB | -117.075670 | 33.043600 |
| San Dieguito River | 905SDC-MLS | -117.205625 | 32.99908167 |
| Santa Ysabel Creek | 905SDC-TWAS-2 | -117.0316185 | 33.06154785 |
| Los Peñasquitos | 906LPC-MLS | -117.2226197 | 32.90459772 |
| Carroll Canyon Creek | 906LPC-TWAS-1 | -117.223349 | 32.900535 |
| Los Peñasquitos | 906LPC-TWAS-2 | -117.084042 | 32.94262 |
| Boulder Creek | 907REF-BC | -116.6621833 | 32.96378 |
| Chollas Creek | 908CC-FB | -117.0703167 | 32.72667 |

Project/ Task Description:  Monitoring was conducted at MLS stations for the wet weather season (October 1-April 30) over three storm events during 2001-2006.  Sampling was conducted during the first viable rainstorm of the monitoring season and during at least one storm after February 1st.

For the 2007 monitoring year, monitoring was conducted at MLS and TWAS stations during wet weather season and dry weather season.   Sampling was conducted during the first rainfall event and one event after February 1st for wet weather monitoring for a total of two events.  Dry weather monitoring events occurred during September /October and May/June of the monitoring year for a total of two events.

For the 2008 monitoring year, monitoring was conducted at MLS stations for the wet weather season during the first viable rainfall event of the monitoring season.

The flow weighted composite was analyzed for parameters found in Table 3.  The grab sample was analyzed for field measurements which included temperature, pH and specific conductance.  Laboratory measurements for the grab samples included Biological Oxygen Demand (BOD), oil and grease, total coliform, fecal coliform, and Enterococci.

Sediment samples were collected following the first flush rainfall event of the season for 2006-2008.  At each sample site, five sediment samples spaced equally apart from 3 monitoring reaches were collected and composited into one sample and were analyzed for synthetic pyrethroids found in Table 4.

Periphyton (algae) monitoring was conducted in 2007 in accordance with the EPA’s Rapid Bioassessment Protocols for Use in Streams and Wadeable Rivers (EPA-841-B-99-002, Section 6.2). Parameters sampled can be found below in Table 5.

Detailed descriptions of methods employed in this project can be found in the Regional Monitoring Workgroup - 2002/2003 Urban Runoff Monitoring Final Report Section 3: Storm Water Monitoring Method and Results (January, 2004), Regional Monitoring Workgroup - 2003/2004 Urban Runoff Monitoring Final Report (January, 2005) Section 3:  Methods, Regional Monitoring Workgroup - 2004/2005 Urban Runoff Monitoring Final Report (January, 2006) Section 3:  Methods,  Regional Monitoring Workgroup - 2005/2006 Urban Runoff Monitoring Final Report (January, 2007) Section 3:  Methods,  Regional Monitoring Workgroup - 2006/2007 Urban Runoff Monitoring Final Report (January, 2008) Section 3:  Methods,  Regional Monitoring Workgroup - 2007/2008 Urban Runoff Monitoring Final Report (January, 2009) Section 2:  Scope of Work and Appendix B: Methods, Regional Monitoring Workgroup - 2008/2009 Urban Runoff Monitoring Final Report (February, 2010) Section 2:  Scope of Work and Appendix B: Methods.  The documents are included with this submittal.

Table 3.  Parameters for Water Samples

| Parameter | Method | Unit | MDL | RL | No. Samples Analyzed |
| --- | --- | --- | --- | --- | --- |
| Allethrin | EPA 625m | Ng/L | 0.5-5 | 2-10 | 10 |
|  | EPA 625mNCI | Ng/L | 0.2-0.5 | 2-10 | 48 |
| Ammonia as N | EPA 350.1 | mg/L | 0.1 | Not Recorded | 1 |
|  | SM 4500-NH3 F | mg/L | 0.01-0.03 | 0.03-0.05 | 75 |
|  | SM 4500 NH3 B,C | mg/L | 0.02-0.5 | 0.1-1 | 152 |
|  | EPA 350.2 | mg/L | Not Recorded | 0.1 | 3 |
|  | SM 4500 NH3 | mg/L | 0.1-1 | Not Recorded | 30 |
|  | Not Recorded | mg/L | 4 | 4 | 2 |
| Antimony | EPA 200.8 | mg/L | 0.0002-0.005 | 0.002-0.006 | 246 |
|  | EPA 200.8m | µg/L | 0.1 | 0.5 | 150 |
|  | EPA 3020/6020 | mg/L | 0.002 | Not Recorded | 41 |
|  | EPA 6020 | mg/L | 0.002 | 0.002 | 85 |
|  | Not Recorded | mg/L | 0.002 | 0.006 | 4 |
| Arsenic | EPA 200.8 | mg/L | 0.0001-0.002 | 0.001-0.002 | 246 |
|  | EPA 200.8m | µg/L | 0.2 | 0.5 | 150 |
|  | EPA 3020/6020 | mg/L | 0.001 | Not Recorded | 41 |
|  | EPA 6020 | mg/L | 0.0001-0.001 | 0.001 | 85 |
|  | Not Recorded | mg/L | 0.002 | 0.05 | 4 |
| Bifenthrin | EPA 625m | Ng/L | 0.5-5 | 2-25 | 10 |
|  | EPA 625mNCI | Ng/L | 0.5 | 2 | 48 |
| BOD | EPA 405.1 | mg/L | 2 | 2 | 23 |
|  | SM 5210 B | mg/L | 0.58-40 | 1-40 | 222 |
|  | SM 2510 B | mg/L | 2 | 2 | 8 |
|  | Not Recorded | mg/L | 5 | 30 | 2 |
| Cadmium | EPA 200.8 | mg/L | 0.0002-0.001 | 0.001 | 246 |
|  | EPA 200.8m | µg/L | 0.2 | 0.4 | 150 |
|  | EPA 3020/6020 | mg/L | 0.001 | Not Recorded | 41 |
|  | EPA 6020 | mg/L | 0.0002-0.001 | 0.001 | 85 |
|  | Not Recorded | mg/L | 0.001 | 0.0046 | 4 |
| Chlorpyrifos | ELISA | µg/L | 0.03 | 0.03 | 62 |
|  | EPA 625 | ng/L | 5 | 10 | 30 |
|  | EPA 625m | ng/L | 1-5 | 2-20 | 136 |
|  | EPA 8141A | µg/L | 0.01 | Not Recorded | 1 |
|  | Not Recorded | µg/L | 3 | 3 | 2 |
|  |  | ng/L | 5 | 10 | 31 |
| Chromium | EPA 200.8 | mg/L | 0.0002-0.005 | 0.005 | 246 |
|  | EPA 200.8m | µg/L | 0.1 | 0.5 | 150 |
|  | EPA 3020/6020 | mg/L | 0.005 | Not Recorded | 41 |
|  | EPA 6020 | mg/L | 0.0002-0.005 | 0.005 | 85 |
|  | Not Recorded | mg/L | 0.002 | 0.002-0.016 | 4 |
| COD | EPA 410.4 | mg/L | 3.5-100 | 5-200 | 252 |
|  | SM 5220 D | mg/L | 30 | Not Recorded | 1 |
|  | Not Recorded | mg/L | 5 | 129 | 2 |
| Copper | EPA 200.8 | mg/L | 0.0002-0.005 | 0.002-0.01 | 246 |
|  | EPA 200.8m | µg/L | 0.4 | 0.8 | 146 |
|  | EPA 3020/6020 | mg/L | 0.005 | Not Recorded | 41 |
|  | EPA 6020 | mg/L | 0.0002-0.005 | 0.005 | 85 |
|  | Not Recorded | mg/L | 0.005 | 0.0135 | 4 |
| Cyfluthrin, total | EPA 625m | ng/L | 0.5-5 | 2-25 | 10 |
|  | EPA 625mNCI | ng/L | 0.5 | 2 | 48 |
| Cyhalothrin, lambda, total | EPA 625m | ng/L | 0.5-5 | 2-25 | 10 |
|  | EPA 625mNCI | ng/L | 0.5 | 2 | 48 |
| Cypermethrin, total | EPA 625m | ng/L | 0.5-5 | 2-25 | 10 |
|  | EPA 625mNCI | ng/L | 0.5 | 2 | 48 |
| Danitol | EPA 625m | ng/L | 0.5-5 | 2-25 | 10 |
|  | EPA 625mNCI | ng/L | 0.5 | 2 | 48 |
| Deltamethrin | EPA 625m | ng/L | 0.5-5 | 2-25 | 10 |
|  | EPA 625mNCI | ng/L | 0.5 | 2 | 48 |
| Diazinon | ELISA | µg/L | 0.03 | 0.03 | 62 |
|  | EPA 625 | ng/L | 5 | 10 | 30 |
|  | EPA 625m | ng/L | 2-5 | 4-20 | 136 |
|  | EPA 8141A | µg/L | 0.01 | Not Recorded | 1 |
|  | Not Recorded | µg/L | 6 | 6 | 2 |
|  |  | ng/L | 5 | 10 | 31 |
| Dissolved Solids | SM 2540 C | mg/L | 0.1-20 | 5-27 | 259 |
|  | EPA 160.1 | mg/L | Not Recorded | Not Recorded | 2 |
|  | Not Recorded | mg/L | Not Recorded | Not Recorded | 2 |
| Esfenvalerate | EPA 625m | ng/L | 0.5-5 | 2-25 | 10 |
|  | EPA 625mNCI | ng/L | 0.5 | 2 | 48 |
| Hardness as CaCO3 | EPA 200.7 | mg/L | 7-100 | 10-100 | 153 |
|  | SM 2340 B | mg/L | 1-10 | 5-10 | 106 |
|  | EPA 130.2 | mg/L | Not Recorded | Not Recorded | 2 |
|  | Not Recorded | mg/L | 7 | 7 | 2 |
| Lead | EPA 200.8 | mg/L | 0.0001-0.005 | 0.001-0.005 | 246 |
|  | EPA 200.8m | µg/L | 0.05 | 0.1 | 150 |
|  | EPA 3020/6020 | mg/L | 0.002 | Not Recorded | 41 |
|  | EPA 6020 | mg/L | 0.0001-0.002 | 0.002 | 85 |
|  | Not Recorded | mg/L | 0.005 | 0.082 | 4 |
| Malathion | EPA 625 | ng/L | 5 | 10 | 30 |
|  | EPA 625m | ng/L | 3-5 | 6-20 | 135 |
|  | Not Recorded | µg/L | 0.1 | 0.1 | 30 |
|  |  | ng/L | 5 | 10 | 31 |
| MBAS | EPA 425.1 | mg/L | 0.04 | Not Recorded | 3 |
|  | SM 5540 C | mg/L | 0.005-0.5 | 0.01-0.5 | 258 |
|  | Not Recorded | mg/L | 0.04 | 0.5 | 2 |
| Nickel | EPA 200.8 | mg/L | 0.0001-0.005 | 0.002-0.005 | 246 |
|  | EPA 200.8m | µg/L | 0.2 | 0.5 | 150 |
|  | EPA 3020/6020 | mg/L | 0.002 | Not Recorded | 41 |
|  | EPA 6020 | mg/L | 0.0002-0.002 | 0.002 | 85 |
|  | Not Recorded | mg/L | 0.005 | 0.005-0.1 | 4 |
| Nitrate as N | EPA 300.0 | mg/L | 0.01-0.1 | 0.05 | 89 |
|  | SW9056 | mg/L | Not Recorded | Not Recorded | 2 |
|  | SM 4500 NO3 E | mg/L | 0.009-0.5 | 0.05-1 | 152 |
|  | EPA 300 | mg/L | Not Recorded | 0.1 | 1 |
|  | SM 4500 NO3 | mg/L | 0.1-0.5 | Not Recorded | 30 |
|  | Not Recorded | mg/L | 0.1 | 45 | 2 |
| Nitrite as N | EPA 300.0 | mg/L | 0.01-0.1 | 0.05 | 63 |
|  | SW9056 | mg/L | Not Recorded | 0.5 | 2 |
|  | SM 4500 NO2 B | mg/L | 0.007-0.05 | 0.05-0.1 | 152 |
|  | EPA 300 | mg/L | Not Recorded | 0.1 | 1 |
|  | SM 4500 NO3 | mg/L | 0.05-0.1 | Not Recorded | 6 |
|  | SM 4500 NO2 | mg/L | 0.05 | Not Recorded | 24 |
|  | Not Recorded | mg/L | 0.1 | 1 | 2 |
| Nitrogen, Total Kjeldahl | EPA 351.2 | mg/L | 0.5 | Not Recorded | 1 |
|  | SM 4500 N Org B | mg/L | 0.46-7.3 | 0.5-8 | 71 |
|  | SM 4500 N C | mg/L | 0.3-2.5 | 0.5-2.5 | 181 |
|  | EPA 351.3 | mg/L | Not Recorded | 0.1 | 3 |
|  | SM N C | mg/L | 0.5 | Not Recorded | 1 |
|  | Not Recorded | mg/L | 0.5 | 0.5 | 2 |
| OilandGrease | EPA 1664 | mg/L | 5-10 | 1-5 | 34 |
|  | EPA 1664A | mg/L | 1 | 5 | 89 |
|  | EPA 413.1 | mg/L | 1 | 1 | 4 |
|  | EPA 413.2 | mg/L | 0.68-1 | 1 | 148 |
|  | Not Recorded | mg/L | 5 | 15 | 2 |
| Permethrin, total | EPA 625m | ng/L | 0.5-5 | 2-25 | 10 |
|  | EPA 625mNCI | ng/L | 0.5-5 | 2-25 | 36 |
| Permethrin-1 | EPA 625mNCI | ng/L | 5 | 25 | 12 |
| Permethrin-2 | EPA 625mNCI | ng/L | 5 | 25 | 12 |
| Phosphorus as P, Dissolved | EPA 365.2 | mg/L | 0.02 | 0.05 | 1 |
|  | SM 4500-P C | mg/L | 0.016 | 0.05 | 12 |
|  | SM 4500 P B, E | mg/L | 0.02-0.1 | 0.05-0.25 | 30 |
|  | SM 4500 P E | mg/L | 0.05-0.5 | Not Recorded | 93 |
|  | Not Recorded | mg/L | 0.02 | 2 | 2 |
| Phosphorus as P,Total | EPA 365.2 | mg/L | 0.02 | 0.05 | 3 |
|  | EPA 365.4 | mg/L | 0.2 | Not Recorded | 1 |
|  | SM 4500-P C | mg/L | 0.016 | 0.05 | 12 |
|  | SM 4500 P B, E | mg/L | 0.02-0.1 | 0.05-0.25 | 152 |
|  | SM 4500 P E | mg/L | 0.05-0.5 | Not Recorded | 93 |
|  | Not Recorded | mg/L | 0.02 | 2 | 2 |
| Piperonyl Butoxide | EPA 625m | ng/L | 5 | 20 | 30 |
| Prallethrin | EPA 625m | ng/L | 0.5-5 | 2-25 | 10 |
|  | EPA 625mNCI | ng/L | 0.5 | 2 | 48 |
| Selenium | EPA 200.8 | mg/L | 0.0005-0.005 | 0.004-0.02 | 246 |
|  | EPA 200.8m | µg/L | 0.2 | 0.5 | 150 |
|  | EPA 3020/6020 | mg/L | 0.002 | Not Recorded | 41 |
|  | EPA 6020 | mg/L | 0.002-0.004 | 0.004 | 85 |
|  | Not Recorded | mg/L | 0.005 | 0.02 | 4 |
| Suspended Solids | SM 2540 D | mg/L | 0.5-80 | 5-100 | 259 |
|  | EPA 160.2 | mg/L | Not Recorded | Not Recorded | 2 |
|  | Not Recorded | mg/L | 5 | 100 | 2 |
| Total Organic Carbon | EPA 415.1 | mg/L | 0.1-4.04 | 0.2-10 | 193 |
|  | SM 5310 B | mg/L | 0.1-2.02 | 0.2-5 | 35 |
| Turbidity | EPA 180.1 | NTU | 1 | 2 | 77 |
|  | SM 2130 B | NTU | 0.05-2.5 | 0.05-20 | 184 |
|  | Not Recorded | NTU | 0.5 | 20 | 2 |
| Zinc | EPA 200.8 | mg/L | 0.0003-0.05 | 0.02 | 246 |
|  | EPA 200.8m | µg/L | 0.1 | 0.5 | 150 |
|  | EPA 3020/6020 | mg/L | 0.02 | Not Recorded | 41 |
|  | EPA 6020 | mg/L | 0.0003-0.02 | 0.02 | 85 |
|  | Not Recorded | mg/L | 0.122 | 0.01 | 4 |

Table 4:  Parameters for Sediment Samples

| Parameter | Method | Unit | MDL | RL | No. Samples Analyzed |
| --- | --- | --- | --- | --- | --- |
| Allethrin | EPA 8270CmNCI | ng/g dw | 0.5 | 2 | 33 |
| Ash-Free Dry Weight | SM 10300 C D | g/m2 | 0.01 | 0.05 | 20 |
| Bifenthrin | EPA 8270CmNCI | ng/g dw | 0.5 | 2 | 33 |
| Chlorophyll-a | SM 10200 H | mg/m2 | 0.2 | 0.4 | 2 |
|  |  | mg/m3 | 5 | 10 | 18 |
| Cyfluthrin, total | EPA 8270CmNCI | ng/g dw | 0.5 | 2 | 33 |
| Cyhalothrin, lambda, total | EPA 8270CmNCI | ng/g dw | 0.5 | 2 | 33 |
| Cypermethrin, total | EPA 8270CmNCI | ng/g dw | 0.5 | 2 | 33 |
| Danitol | EPA 8270CmNCI | ng/g dw | 0.5 | 2 | 33 |
| Deltamethrin | EPA 8270CmNCI | ng/g dw | 0.5 | 2 | 33 |
| Esfenvalerate | EPA 8270CmNCI | ng/g dw | 0.5 | 2 | 33 |
| Permethrin, total | EPA 8270CmNCI | ng/g dw | 5 | 25 | 17 |
| Permethrin-1 | EPA 8270CmNCI | ng/g dw | 5 | 25 | 16 |
| Permethrin-2 | EPA 8270CmNCI | ng/g dw | 5 | 25 | 16 |
| Piperonyl Butoxide | EPA 8270Cm | ng/g dw | 5 | 20 | 17 |
| Prallethrin | EPA 8270CmNCI | ng/g dw | 0.5 | 2 | 33 |

Table 5.  Parameters for Periphyton Monitoring

| Parameter | Method | Unit | MDL | RL | No. Samples Analyzed |
| --- | --- | --- | --- | --- | --- |
| Ash-Free Dry Weight | SM 10300 C D | g/m2 | 0.01 | 0.05 | 20 |
| Chlorophyll-a | SM 10200 H | mg/m2 | 0.2 | 0.4 | 2 |
|  |  | mg/m3 | 5 | 10 | 18 |

Quality Control:  The quality control requirements employed in this project are those described in Weston Solutions Inc. Desk Reference for Quality Control of Work Products and Weston’s Quality Assurance Program Plan/ Microbiology (included with this submittal).  The documents include methods used for sample collection and handling, data management, validation, and record keeping (including proper chain of custody procedures), data quality objectives for laboratory measurements, and personnel training.

Enclosed Documents in Electronic Format:

Regional Monitoring Workgroup - 2002/2003 Urban Runoff Monitoring Final Report Section 3: Storm Water Monitoring Methods and Results (January, 2004).  Electronic Filename: Section 3 Methods & Results 02_03.pdf.

Regional Monitoring Workgroup - 2003/2004 Urban Runoff Monitoring Final Report (January, 2005) Section 3:  Methods.  Electronic Filename: Section 3 Methods 03_04.pdf.

Regional Monitoring Workgroup - 2004/2005 Urban Runoff Monitoring Final Report (January, 2006) Section 3:  Methods.  Electronic Filename: Section 3 Methods 04_05.pdf.

Regional Monitoring Workgroup - 2005/2006 Urban Runoff Monitoring Final Report (January, 2008) Section 3:  Methods.  Electronic Filename: Section 3 Methods 05_06.pdf.

Regional Monitoring Workgroup - 2006/2007 Urban Runoff Monitoring Final Report (January, 2009) Section 3:  Methods.  Electronic Filename: Section 3 Methods 06_07.pdf.

Regional Monitoring Workgroup - 2007/2008 Urban Runoff Monitoring Final Report (January, 2009) Section 2: 2007-2008  Scope of Work.   Electronic Filename: Section 2 SOW 07_08.pdf.

Regional Monitoring Workgroup - 2007/2008 Urban Runoff Monitoring Final Report (January, 2009) Appendix B: Program Monitoring and Data Analysis Methods.  Electronic Filename: Appendix B Methods 07_08.pdf.

Regional Monitoring Workgroup - 2008/2009 Urban Runoff Monitoring Final Report (February, 2010) Section 2:  2008-2009 Scope of Work.  Electronic Filename: Section 2 SOW 08_09.pdf.

Regional Monitoring Workgroup - 2008/2009 Urban Runoff Monitoring Final Report (February, 2010)  Appendix B: Program Monitoring and Data Analysis Methods.  Electronic Filename: Appendix B Methods 08_09.pdf.

Weston Solutions Inc. Desk Reference for Quality Control Of Work Products and Weston’s Quality Assurance Program Plan/ Microbiology.  Electronic Filename: QC of Work Products_Weston.doc.

Enclosed Electronic Data Files:

County of San Diego NPDS Co-Permittees Chemistry and Periphyton Data (2001-2008). Electronic filename:  COSD NPDS chem  periph_Data.xls

County of San Diego NPDS Co-Permittees Microbiology Data (2001-2008). Electronic filename:  COSD NPDS microbio_Data.xls

County of San Diego NPDS Co-Permittees Sampling Station Locations Data. Electronic filename:  COSD NPDS station_Data.xls

County of San Diego NPDS Co-Permittees Toxicity Testing Data (2001-2008). Electronic filename:  COSD NPDS toxicity_Data.xls