## 2009  data quality  report

Data quality was evaluated through a series of standards that measure the adequacy of sample collection and analysis methods. In general, certain performance criteria (data quality objectives) are established to ensure that the data are acceptable and usable. Three quantitative performance criteria were used in the Dry Weather Monitoring Program to evaluate the degree of certainty or usability of the data to users. These criteria included precision, accuracy, and completeness.  Precision is expressed as relative percent difference (RPD%) between two duplicate samples. Accuracy is expressed as the percent recovery (REC%) of an analyte from a sample of known analyte concentration. Completeness is defined as the percentage of actual measurements that are judged to be valid, over the planned overall measurements. Detailed information about the quality control program can be found in the Quality Assurance Project Plan For the County of San Diego’s Dry Weather Monitoring Program (County of San Diego, 2009, Revision 8) (QAPP).  A discussion of the analytical laboratory QA/QC results is provided below.

The laboratory analysis of grab samples was conducted by Truesdail Laboratory, Inc. (Truesdail) for all laboratory-tested constituents except metals.  All metal analysis (analysis for the dissolved cadmium, copper, lead, and zinc) were conducted by EnviroMatrix Analytical, Inc. (EnviroMatrix).  Data quality analysis was conducted using laboratory method blanks (LMB), blind field blanks (BFB), blind field duplicates (BFD), laboratory control samples (LCS), laboratory control sample duplicates (LCSDs), matrix spike samples (MSS), and matrix spike sample duplicates (MSSDs).   The number of LMBs, LCSs, LCSDups, MSSs and MSSDups varied with constituents measured.  Nine BFDs and 5 BFBs were included in the analysis.  Only those BFDs that had results exceeding the method detection limits for a given constituent were used.  For all pesticides and dissolved cadmium, lead, and zinc, none of the BFD samples met that criterion.

The summary of data quality analysis for analytical samples is presented in Table1.  Based on the laboratory control samples (LCS), proper accuracy was maintained within the specified data quality objectives for all analyzed constituents.  On the other hand, two of 16 dissolved zinc matrix spike samples exceeded the 120% DQO for percent recovery by 4%.  When blind field duplicates were considered, all dissolved metals, except for copper had results below method detection limits and could not be used to estimate precision.  For dissolved copper, one of the four blind field duplicate pairs showed an 80% exceedance of the ± 25% DQO for precision.  The duplicate pair had results of 1 μg/L and 2 μg/L.  Since both of these values reflect very low dissolved copper concentrations and were recorded with an accuracy of one significant figure, this was not used to reject any of the associated data.   For hardness, the precision DQO of ± 25% was exceeded by 2% in one blind field duplicate pair.   This was a small exceedance due to just one of the 9 blind field duplicates analyzed and was not considered sufficient to reject the data.

In addition to field duplicates, laboratory control samples and duplicates,  and matrix spike samples and duplicates, laboratory method blank samples and blind field blank samples were also processed for each analyte.  The results are presented in Table2.  For all blank samples, concentrations of all analytes fell below the method detection limits.  The same was true for the bacteria counts.

Table 1. Data Quality Objectives (DQO) and levels achieved for analytical parameters

| Parameter | Accuracy |  | Precision |  |  | Recovery |  | Completeness |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DQO | Achieved ( LCS ) | DQO | Achieved (Blind Field Duplicates) | Achieved (LCSDs and MSSDs ) | DQO | Achieved (MSS) | DQO | Achieved |
| Hardness | 80 – 120 % | 96 – 100% | ± 25% | ± 0 – 27% | ± 2% | 80 – 120 % | NM | 90% | 100% |
| MBAS | 80 – 120 % | NM | ± 25% | NM | NM | 80 – 120 % | NM | 90% | 100% |
| Oil and Grease | 80 – 120 % | 98 – 102% | ± 25% | NM | ± 0– 9% | 80 – 120 % | NM | 90% | 100% |
| Dissolved Cadmium | 80 – 120 % | 98 – 102% | ± 25% | All ND | ± 0– 7% | 80 – 120 % | 92 – 100% | 90% | 100% |
| Dissolved Copper | 80 – 120 % | 98 – 102% | ± 25% | ± 0 – 100% | ± 0– 4% | 80 – 120 % | 94 – 102% | 90% | 100% |
| Dissolved Zinc | 80 – 120 % | 98 – 104% | ± 25% | All ND | ± 0– 7% | 80 – 120 % | 93 – 124% | 90% | 100% |
| Dissolved Lead | 80 – 120 % | 95 – 103% | ± 25% | All ND | ± 0– 4% | 80 – 120 % | 97 – 105% | 90% | 100% |
| Diazinon, | 70 – 130 % | 100% | ± 25% | All ND | ± 0 – 16% | 50 – 150% | NM | 90% | 100% |
| Chlorpyrifos | 70 – 130 % | 100% | ± 25% | All ND | ± 1 – 13% | 50 – 150% | NM | 90% | 100% |
| Malathion | 80 – 120 % | 100% | ± 25% | All ND | ± 1 – 11% | 50 – 150% | NM | 90% | 100% |
| Enterococcus | Proper + or – response | Yes | 8.11 | 0 – 0.90 | NM | NA | NA | 90% | 100% |
| Fecal Coliform | Proper + or – response | Yes25 | 6.8326 | 0 – 1.26 | NM | NA | NA | 90% | 100% |
| Total Coliform | Proper + or – response | Yes25 | 8.3626 | 0 – 1.26 | NM | NA | NA | 90% | 100% |

Table 2. Results of the laboratory and blind field blank analysis

| Constituent | Laboratory Method Blanks |  | Blind Field Blanks |  |  |
| --- | --- | --- | --- | --- | --- |
|  | No. Processed | No. Results below MDL | No. Processed | No. Results below MDL | MDL |
| Hardness | 8 | 8 | 5 | 5 | 1.17 |
| MBAS | 2 | 2 | 0 | NA | 0.007 |
| Oil and Grease | 11 | 11 | 5 | 5 | 1.95 |
| Dissolved Cadmium | 4 | 4 | 5 | 5 | 0.119 |
| Dissolved Copper | 4 | 4 | 5 | 5 | 0.519 |
| Dissolved Zinc | 4 | 4 | 5 | 5 | 0.823 |
| Dissolved Lead | 4 | 4 | 5 | 5 | 0.119 |
| Diazinon, | 13 | 13 | 5 | 5 | 0.0072 |
| Chlorpyrifos | 13 | 13 | 5 | 5 | 0.0076 |
| Malathion | 13 | 13 | 5 | 5 | 0.0074 |
| Enterococcus | 0 | NM | 4 | 4 | 2 |
| Fecal Coliform | 0 | NM | 4 | 4 | 2 |
| Total Coliform | 0 | NM | 4 | 4 | 1 |

Results outside of DQO range are boldfaced

NM – Not Measured; NA – Not Applicable; All ND – All results Non-Detect

Based on blind field blanks submitted to the laboratory

Precision criterion is defined as Rlog within 3.27 x mean Rlog

NM – Not measured