## 2008 data quality report

Data quality is evaluated through a series of standards that measure the adequacy of sample collection and analysis methods. In general, certain performance criteria (data quality objectives) are established to ensure that the data are acceptable and usable. Three quantitative performance criteria were used in the Dry Weather Monitoring Program to evaluate the degree of certainty or usability of data. These criteria included precision, accuracy, and completeness.  Precision is expressed as relative percent difference (RPD%) between two duplicate samples. Accuracy is expressed as the percent recovery (REC%) of an analyte from a sample with known analyte concentration. Completeness is defined as the percentage of actual measurements that are judged to be valid, over the planned overall measurements. Detailed information about the quality control program can be found in the Quality Assurance Project Plan For the County of San Diego’s Dry Weather Monitoring Program (County of San Diego, 2008, Revision 6) (QAPP).  A discussion of the analytical laboratory QA/QC results is provided below.

The laboratory analysis of grab samples was conducted by Truesdail Laboratory, Inc. (Truesdail) for all laboratory-tested constituents except metals.  All metal analysis (analysis for the dissolved cadmium, copper, lead and zinc) were conducted by the EnviroMatrix Analytical, Inc. (EnviroMatrix).  Data quality analysis was conducted using laboratory method blanks (LMB), laboratory duplicates (LD), blind field blanks (BFB), blind field duplicates (BFD), laboratory control samples (LCS) and matrix spike samples (MSS).   The number of LMBs, LDs, LCSs and MSSs varied with constituents measured.  Nine BFDs and 5 FBs were included in the analysis.  Only those BFDs that had results exceeding the method detection limits for a given constituent were used.  For all pesticides and dissolved metals, none of the BFD samples met that criterion.

Mixed results were obtained for the grab samples (Table1).  Based on the laboratory control samples (LCS), proper accuracy was maintained within the specified data quality objectives for all but two analyzed constituents.  The two included malathion and dissolved cadmium.  For dissolved cadmium, accuracy exceeded the 120% upper limit by only 1% and for only on of the 7 LCSs.  Since this is only one and a very small exceedance, it was not considered disqualifying to the data.  For malathion, 4 of the 16 LCSs had results below the lower accuracy limit of 80% .  Since all malathion results in the field were below the detection limit, this is probably not a significant problem but will be addressed with the laboratory in the future.  Some of the matrix spike samples showed percent recovery values that were higher than the DQO of 120% for the dissolved lead (3 of 7 MSS) and dissolved zinc (2 of 7 MSS).  Additionally, one of 7 MSS for cadmium and one of 5 MSS for hardness showed results slightly below the lower 80% limit.  When blind field duplicates were considered, all dissolved metals results were below their detection limits and could not be analyzed for accuracy while hardness results fell within the DQOs.   The BFD results also showed one slight exceedance of the DQO for Enterococcus.  This was just one of the 9 sample pairs analyzed and is not considered sufficient to reject the data.

Table 1. Data Quality Objectives (DQO) and levels achieved for analytical parameters

| 1 Precision criterion is defined as Rlog within 3.27 x mean Rlog 2 Based on blind field blanks submitted to the laboratory Results outside of DQO range are boldfaced NM – Not Measured; NA – Not Applicable Parameter | Accuracy |  | Precision |  |  |  |  | Recovery |  | Completeness |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DQO | Achieved ( LCS ) |  | DQO | Achieved (Blind Field Duplicates) | Achieved (LD) | DQO |  | Achieved (MSS) | DQO | Achieved |
| Hardness | 80 – 120 % | 100 – 102% |  | ± 25% | ± 0 – 22% | ± 1– 2% | 80 – 120 % |  | 76 – 97% | 90% | 100% |
| MBAS | 80 – 120 % | 95 – 105% |  | ± 25% | NM | ± 3– 6% | 80 – 120 % |  | 98 – 106% | 90% | 100% |
| Oil and Grease | 80 – 120 % | 86 – 97% |  | ± 25% | All ND | ± 0– 2% | 80 – 120 % |  | NM | 90% | 100% |
| Dissolved Cadmium | 80 – 120 % | 93 – 121% |  | ± 25% | All ND | ± 0 – 15% | 80 – 120 % |  | 76 – 114% | 90% | 100% |
| Dissolved Copper | 80 – 120 % | 91 – 110% |  | ± 25% | All ND | ± 0 – 5% | 80 – 120 % |  | 85 – 110% | 90% | 100% |
| Dissolved Zinc | 80 – 120 % | 95 – 118% |  | ± 25% | All ND | ± 0 – 9% | 80 – 120 % |  | 93 - 131% | 90% | 100% |
| Dissolved Lead | 80 – 120 % | 94 -112% |  | ± 25% | All ND | ± 0 – 10% | 80 – 120 % |  | 94 - 126% | 90% | 100% |
| Diazinon, | 70 – 130 % | 87 - 110% |  | ± 25% | All ND | ± 2 – 12% | 50 – 150% |  | NM | 90% | 100% |
| Chlorpyrifos | 70 – 130 % | 97 - 117% |  | ± 25% | All ND | ± 1 – 17% | 50 – 150% |  | NM | 90% | 100% |
| Malathion | 80 – 120 % | 67 – 95% |  | ± 25% | All ND | ± 1 – 17% | 50 – 150% |  | NM | 90% | 100% |
| Enterococcus | Proper + or – response | Yes2 |  | 0.851 | 0 – 0.97 | NM | NA |  | NA | 90% | 100% |
| Fecal Coliform | Proper + or – response | Yes2 |  | 0.871 | 0 – 0.56 | NM | NA |  | NA | 90% | 100% |
| Total Coliform | Proper + or – response | Yes2 |  | 0.851 | 0 – 0.66 | NM | NA |  | NA | 90% | 100% |

In addition to field duplicates, laboratory duplicates, laboratory control samples and matrix spike samples, laboratory method blank samples and blind field blank samples were also processed for each analyte.  The results are presented in Table2.  For all but one laboratory method blank, concentrations of all analytes fell below the detection limits.  For all the blind field blanks all concentrations of all analytes fell below their corresponding detection limits.  These results meet the requirements of the QAPP.  Therefore none of the analytical laboratory data were rejected from the analysis.

Table 2. Results of the laboratory and blind field blank analysis

| Constituent | Laboratory Method Blanks |  | Blind Field Blanks |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | No. Processed | No. Results below MDL | No. Processed | Results | MDL | No. Results below MDL |
| Hardness | 17 | 16 | 5 | ND for all | 1.17 | 5 |
| MBAS | 3 | 3 | 0 | NA | 0.007 | NA |
| Oil and Grease | 16 | 16 | 5 | ND for all | 1.95 | 5 |
| Dissolved Cadmium | 7 | 7 | 5 | ND for all | 0.119 | 5 |
| Dissolved Copper | 7 | 7 | 5 | ND for all | 0.519 | 5 |
| Dissolved Zinc | 7 | 7 | 5 | ND for all | 0.823 | 5 |
| Dissolved Lead | 7 | 7 | 5 | ND for all | 0.119 | 5 |
| Diazinon, | 16 | 16 | 5 | ND for all | 0.0072 | 5 |
| Chlorpyrifos | 16 | 16 | 5 | ND for all | 0.0076 | 5 |
| Malathion | 16 | 16 | 5 | ND for all | 0.0074 | 5 |
| Enterococcus | 0 | NM | 5 | ND for all | 2 | 5 |
| Fecal Coliform | 0 | NM | 5 | ND for all | 2 | 5 |
| Total Coliform | 0 | NM | 5 | ND for all | 1 | 5 |

ND – constituent concentration below the detection level
NM – Not measured
All ND – All results Non-Detect