Project Title:  County of San Diego Dry Weather Monitoring Data 2003 through 2009

Originator:  County of San Diego Dept. of Public Works Watershed Protection Program

Contact:  Jo Ann Weber;  e-mail:  joann.weber@sdcounty.ca.gov; tel. 858-495-5317

Publication Info.: Published yearly as part of the Jurisdictional Urban Runoff Management Program Report.  Prepared for the California Regional Water Quality Control Board San Diego Region 9 by the County of San Diego.

Coordinate System:  NAD83

Project Background and Objectives:  San Diego Regional Water Quality Control Board (SDRWQCB) Order No. R9-2007-0001 requires that Dry Weather Analytical and Field Screening Monitoring (hereinafter Dry Weather Monitoring) be conducted.  Under Order No. R9-2007-0001, NPDES No. CAS0108758, Section D.4.c, each of the Copermittees, including the County of San Diego, must annually conduct dry weather field screening of discharges in its storm drain system.  This includes field observations, estimating flow, and other in-situ measurements at selected locations along the storm drain conveyance system during dry weather season (from May 1st through September 30th).  Copermittees are required to perform field screening at each site at least once per monitoring season and to conduct laboratory analysis for at least 25 percent of the sites with flowing or ponded water.  The County’s program has exceeded these minimum recommendations by doubling the number of annual site visits and increasing analytical sampling from 25 percent to 100 percent, if possible, of the sites with ponded or flowing water.

The primary purpose of the Dry Weather Monitoring Program is to identify and eliminate water quality problems that may be the result of non-storm water discharges to or from municipal separate storm sewer systems (MS4s).  The program is designed to address these illegal discharges and to minimize the negative impacts of human activities on receiving water bodies.

The program includes locations throughout all eight watersheds in the unincorporated County.  The streams and creeks in the unincorporated area often serve as conduits for urban runoff. In 2009, approximately 80 percent of the monitoring locations were located in natural or partially altered watercourses.  Given the Permit requirements and the lack of water quality data for the unincorporated areas, the County has used its dry weather monitoring data to evaluate water quality in the County’s watersheds.

This report includes the results of analytical testing for the receiving water bodies only from May 2003 through the end of September 2009.

Geographical Setting/ Sampling Locations:  The submitted data include only the receiving water sampling locations.  The locations are presented in Table 1 below.

Table 1: Receiving Water Dry Weather Monitoring Locations Employed Since 2001.

| Site ID | Site Description | Latitude | Longitude |
| --- | --- | --- | --- |
| 904CAR01 | San Elijo Creek @ La Granada | 33.02297 | -117.22729 |
| 904CAR02 | Escondido Creek @ East County Club Drive | 33.09901 | -117.13047 |
| 904CAR03 | Escondido Creek @ El Camino Del Norte | 33.04839 | -117.22716 |
| 904CAR04 | San Marcos Creek @ Discovery Street | 33.13046 | -117.20045 |
| 904CAR05 | Buena Creek @ Robelini Drive | 33.17239 | -117.20997 |
| 904CAR06 | San Marcos Creek @ Olive Street and Sycamore Drive | 33.17964 | -117.15211 |
| 904CAR08 | Reidy Canyon Creek @ Paseo Del Norte | 33.17810 | -117.09193 |
| 904CAR09 | San Elijo Creek @ El Camino Real | 33.01084 | -117.23985 |
| 904CAR10 | Tributary of San Elijo Creek @ San Elijo Road | 33.02585 | -117.21569 |
| 904CAR12 | Reidy Canyon Creek @ Bachelor Lane | 33.19801 | -117.08966 |
| 904CAR13 | Storm Drain Outfall to Lake San Marcos @ End of San Marino Drive | 33.12012 | -117.20997 |
| 904CAR14 | Tributary to Lake San Marcos @ End of El Chico Lane | 33.11896 | -117.20744 |
| 910OTY03 | Dulzura Creek @ Otay Lakes Valley Road | 32.63624 | -116.88456 |
| 910OTY04 | Olive Vista Creek @ Olive Vista Drive | 32.72168 | -116.85344 |
| 904SDG03 | Santa Maria Creek @ Rangeland Road | 33.03379 | -116.93608 |
| 904SDG04 | Hatfield Creek @ Magnolia Avenue | 33.05258 | -116.84492 |
| 904SDG05 | San Dieguito River @ El Apajo (end) | 32.99948 | -117.2055 |
| 904SDG07 | La Zanja Canyon Creek @ Rancho Santa Fe Farms Road | 32.97771 | -117.18116 |
| 904SDG08 | Green Valley Creek @ Rancho Bernardo Road | 33.01962 | -117.11974 |
| 904SDG09 | Felicita Creek @ Quiet Hills Farm Road | 33.07326 | -117.08373 |
| 904SDG12 | Storm Drain Channel @ 7th Street (below Collier County Park) | 33.04211 | -116.86165 |
| 907SDR01 | Chocolate Canyon Creek @ Arnold Way | 32.84127 | -116.8054 |
| 907SDR02 | Alpine Creek @ Tavern Road | 32.83192 | -116.77528 |
| 907SDR03 | Alpine Creek @ Midway Drive | 32.83879 | -116.79011 |
| 907SDR05 | Culvert @ Bradley Avenue and Graves Avenue | 32.81889 | -116.95928 |
| 907SDR07 | Forrester Creek @ Greenfield Drive | 32.80826 | -116.91151 |
| 907SDR08 | Los Coches Creek @ I-8 Business Route | 32.83599 | -116.9004 |
| 907SDR11 | San Diego River @ Channel Road | 32.86473 | -116.92755 |
| 907SDR13 | Eucalyptus Hills Creek @ Riverside Drive (Storm Drain Channel) | 32.86204 | -116.94466 |
| 907SDR15 | Lindo Lake Outfall @ Petite Lane | 32.85716 | -116.91278 |
| 907SDR16 | Los Coches Creek @ Los Coches Road and Ha Hana Road | 32.84004 | -116.91346 |
| 907SDR18 | Quail Creek Outfall @ Lakeshore Drive and Lindo Lake | 32.86030 | -116.9176 |
| 907SDR19 | Tributary of San Vicente Creek @ San Vicente Road | 33.00561 | -116.82115 |
| 907SDR20 | San Vicente Creek @ Wildcat Canyon Road | 32.99628 | -116.84387 |
| 907SDR21 | Oak Creek @ Olde Highway 80 | 32.84807 | -116.86946 |
| 907SDR22 | Tributary of Chocolate Canyon Creek @ Arnold Way | 32.84232 | -116.80839 |
| 907SDR25 | San Vicente Creek @ San Vicente Road | 33.00162 | -116.8016 |
| 907SDR34 | Tributary to Los Coches Creek @ 11962 Woodside Avenue | 32.85565 | -116.93548 |
| 903SLR01 | Moosa Canyon Creek @ Old River Road | 33.28369 | -117.21886 |
| 903SLR02 | Little Gopher Canyon Creek @ Old River Road | 33.26578 | -117.2332 |
| 903SLR04 | East Channel Creek @ Hutchinson Street and Hidden Lake Lane | 33.24084 | -117.24198 |
| 903SLR06 | Live Oak Creek @ Oak Cliff Drive | 33.33545 | -117.1883 |
| 903SLR08 | Moosa Canyon Creek @ Sunday Drive | 33.21497 | -117.03338 |
| 903SLR10 | Old 395 Creek @ Old Hwy 395 (below outfall pipe next to pole # P719838) | 33.20425 | -117.12894 |
| 903SLR11 | Old 395 Creek @ 29013 Champagne Blvd. (Old Hwy 395) | 33.24022 | -117.14607 |
| 903SLR12 | Green Canyon Creek @ Sycamore Drive | 33.33312 | -117.23551 |
| 903SLR14 | Ostrich Farm Creek @ Highway 76 | 33.29353 | -117.22373 |
| 903SLR15 | Moosa Canyon Creek @ End of Betsworth Road | 33.22763 | -117.08392 |
| 903SLR16 | San Luis Rey River @ Mission Road (the old bridge) | 33.26052 | -117.23836 |
| 903SLR17 | Keyes Creek @ Dunlin Road | 33.32384 | -117.15723 |
| 903SLR18 | San Luis Rey River @ Couser Canyon Pass | 33.34040 | -117.13124 |
| 903SLR21 | Pauma Creek @ Pala Road (Highway 76) | 33.32370 | -116.99665 |
| 903SLR27 | Live Oak Creek @ Highway 76 | 33.31514 | -117.19418 |
| 903SLR29 | Keys Creek @ Lilac Road | 33.28808 | -117.08333 |
| 903SLR30 | Couser Canyon Creek @ Couser Canyon Road | 33.33488 | -117.1312 |
| 902SMG07 | Sandia Creek @ Sandia Creek Drive (at USGS station) | 33.42460 | -117.24904 |
| 902SMG08 | De Luz Creek @ De Luz Road (Mile Marker 8 @ private driveway) | 33.42184 | -117.32179 |
| 902SMG09 | Santa Margarita River @ SDSU Ecological Reserve Entrance | 33.42839 | -117.19561 |
| 902SMG10 | Santa Margarita River @ Sandia Creek Drive (one-half mile east of De Luz Road) | 33.40750 | -117.25018 |
| 909SWT01 | Sweetwater River @ Willow Road | 32.65895 | -117.04231 |
| 909SWT02 | Long Canyon Creek @ Bonita Road near Acacia Ave. | 32.66558 | -117.02409 |
| 909SWT03 | Sweetwater River @ Plaza Bonita Road | 32.65069 | -117.06374 |
| 909SWT05 | San Miguel Creek @ Bonita Road | 32.66692 | -117.02325 |
| 909SWT07 | Spring Valley Creek @ Quarry Road | 32.70114 | -117.00927 |
| 909SWT08 | Casa de Oro Creek @ Valencia Street/Kings View Circle | 32.73330 | -117.00861 |
| 909SWT09 | Spring Valley Creek @ Valencia Street/Kings View Circle | 32.73335 | -117.0087 |
| 909SWT10 | Jamacha Creek @ Jamacha Road near Willow Glen Drive | 32.74445 | -116.93002 |
| 909SWT11 | Sweetwater River @ Steele Canyon Road | 32.74449 | -116.91693 |
| 909SWT12 | Sweetwater River @ Old Bridge | 32.73266 | -116.94029 |
| 909SWT13 | Tributary of Sweetwater River @ Millar Ranch Road south of Hwy 94 | 32.73028 | -116.93887 |
| 909SWT14 | Helix Street Drainage Next to Hwy 94 | 32.74968 | -117.00087 |
| 909SWT15 | Casa de Oro Creek @ Kenwood Drive and Barbic Court | 32.74544 | -116.99168 |
| 909SWT18 | Harbison Canyon Creek @ Collier Way near Harbison Canyon Road | 32.81511 | -116.83599 |
| 909SWT20 | Viejas Creek @ Via Viejas below Private Lake | 32.81894 | -116.75211 |
| 909SWT21 | North Fork of Sweetwater River @ Tavern Road and Hawk Vista Lane | 32.80879 | -116.78036 |
| 909SWT22 | Indian Springs Creek @ Highway 94 | 32.71966 | -116.87986 |
| 909SWT23 | Mexican Canyon Creek @ Jamul Road | 32.72929 | -116.87239 |
| 909SWT25 | Mexican Canyon Creek @ Campo Road near Jamacha Road | 32.73968 | -116.95245 |
| 911TIJ01 | Cottonwood Creek @ Old Highway 80 (Bridge Crossing) | 32.78844 | -116.49732 |
| 911TIJ02 | Pine Valley Creek @ Old Highway 80 and Pine Valley Road | 32.83776 | -116.53725 |
| 911TIJ04 | Campo Creek @ Highway 94 | 32.60939 | -116.47421 |

Project/ Task Description:  Analytical samples were collected at a least 25% of all sampling sites with flowing or standing water at least once every year.  The samples were analyzed for hardness, methylene blue active substances (MBAS), oil and grease, pesticides (diazinon, chlorpyrifos and malathion), dissolved metals (cadmium, copper, lead, and zinc) and fecal indicator bacteria (total coliform, fecal coliform, and Enterococcus).    Some samples were also analyzed for nutrients including ammonia as N, nitrate as N, nitrite as N, total Kjeldahl nitrogen, orthophosphate as P, and total phosphate as P.  Procedures for Dry Weather Monitoring Program closely followed the guidelines described in the Dry Weather Analytical and Field Screening Monitoring Procedures Manual included as Appendix 5 in the DWM QAPP enclosed.  In addition to the grab samples, physical water parameters of pH, electrical conductivity, turbidity, dissolved oxygen, temperature, and salinity were measured at all sampling locations with water using the Horiba U-10 multimeter probe. Other observations including flow were also recorded during each site visit.

Table 2: Constituents Monitored

| Constituent | Analysis Method | Unit | MDL | RL | No. Samples Analyzed |
| --- | --- | --- | --- | --- | --- |
| Ammonia as N | SM 4500 NH3 | mg/L | 0.1 | 0.1 | 82 |
| Nitrate as N | SM 4500 NO3 | mg/L | 0.009 | 0.05 | 48 |
| Nitrite as N | SM 4500 NO2 | mg/L | 0.05 | 0.05 | 106 |
| Total Kjeldahl Nitrogen | SM 4500 Norg | mg/L | 0.5 | 0.5 | 87 |
| Orthophosphate as P | SM 4500 PE | mg/L | 0.007 | 0.05 | 162 |
| Total Phosphate as P | SM4500-P | mg/L | 0.02 | 0.05 | 149 |
| Hardness as CaCO3 | EPA 130.2 | mg/L | 1.17 | 10 | 264 |
|  | EPA 200.7 | mg/L | 10 |  | 146 |
|  | SM 2340 | mg/L | 0.247 |  | 84 |
| Oil and Grease | EPA 1664 | mg/L | 1.95 | 5 | 329 |
|  | EPA 413 | mg/L | 1 |  | 181 |
| Methylene Blue Active Substances (MBAS) | SM 5540 C | mg/L | 0.1 | 0.5 | 178 |
|  | EPA 425.1 | mg/L | 0.007 |  | 226 |
| Diazinon | EPA 8141, EPA 8081 | g/L | 0.05, 0.0072 | 0.05 | 305 |
| Chlorpyrifos |  |  | 0.05, 0.0076 |  | 305 |
| Malathion |  |  | 0.05, 0.0074 |  | 304 |
| Cadmium (Dissolved) | EPA 6020, EPA 200.8 | g/L | 1, 0.1 | 5 | 469 |
| Copper (Dissolved) |  |  | 1, 0.2 | 5 | 470 |
| Lead (Dissolved) |  |  | 1, 0.1 | 5 | 469 |
| Zinc (Dissolved) |  |  | 10, 0.3 | 20 | 617 |
| Total Coliform | SM 9221, SM 9223 | MPN/ 100 ml | 2 | 20 | 479 |
| Fecal Coliform |  |  |  |  | 480 |
| Enterococcus | SM 9230 |  |  |  | 531 |

Quality Control: The quality control requirements employed in this project are those described in the Quality Assurance Project Plan for the County of San Diego’s Dry Weather Monitoring Program and MS4 Program (included with this submittal).  These include methods used for sample collection and handling, data management, validation, and record keeping (including proper chain of custody procedures), data quality objectives for laboratory measurements, and personnel training.  The results of data quality reviews, conducted annually, are provided in the enclosed annual data quality reports which are also published annually in the JURMP.  The enclosed Trusdail Laboratories, Inc. Quality Assurance and Quality Control Manual for Environmental Sample Analysis and EnviroMatrix Analytical, Inc. Quality Assurance Program Manual describe quality control procedures employed by the analytical laboratories that conducted sample analysis for this project.

Enclosed Documents in Electronic Format:

Dry Weather Monitoring Annual Data Quality Reports.  Electronic Folder Name:  DWM Annual Data Quality Reports.

EnviroMatrix Analytical, Inc. Quality Assurance Program Manual.  Electronic Filename: EMA-QAPM2007.pdf

Quality Assurance Project Plan for the County of San Diego’s Dry Weather Monitoring Program and MS4 Program. Electronic Filename: COSD QAPP DWM.doc

Trusdail Laboratories, Inc. Quality Assurance and Quality Control Manual for Environmental Sample Analysis.  Electronic Filename: Truesdail – QAQC manual 2005.pdf

Enclosed Electronic Data File:

County of San Diego Dry Weather Monitoring Data 2003 through 2009.  Electronic Filename:  COSD DWM Receiving Waters_Data.xls.