Project Title:  Rainbow Creek TMDL Monitoring 2004 through 2009

Originator:  County of San Diego Dept. of Public Works Watershed Protection Program

Contact:  Jo Ann Weber; e-mail:  joann.weber@sdcounty.ca.gov; tel. 858-495-5317

Publication Info: Published as part of the Santa Margarita River Watershed Urban Runoff Management Program Annual Reports.  Prepared for the California Regional Water Quality Control Board San Diego Region 9 by the County of San Diego.

Data from this monitoring program covering the time period from February 2007 through May 2009 have also been provided to the California Regional Water Quality Control Board as part of the CWA 319(h) Grant Agreement No. 05-194-559-0 San Diego Region Rainbow Creek Nutrient reduction TMDL.

Coordinate System:  NAD83

Project Background and Objectives:  The Rainbow Creek monitoring program has been developed to provide water quality data in support of the Total Maximum Daily Load (TMDL) for total nitrogen and total phosphorus in Rainbow Creek. The objective of this monitoring program is to characterize baseline conditions for nutrients in the Rainbow Creek watershed. In addition to nutrients, the samples have also been analyzed for the total dissolved solids, total iron and sulfate.   Founding for this program was in part provided through the CWA 319(h) Grant Agreement No. 05-194-559-0 San Diego Region Rainbow Creek Nutrient reduction TMDL.

Geographical Setting/ Sampling Locations:  Monitoring locations (Table 1) include those identified as “strategic nodes” by the San Diego Regional Water Quality Control Board during the development of the TMDL technical report.

Table 1: Rainbow Creek Monitoring Locations

| Site ID | Site Description | Latitude | Longitude |
| --- | --- | --- | --- |
| 902RBC01 | Rainbow Creek @ Jubilee Way | 33.42098 | -117.13286 |
| 902RBC02 | Rainbow Creek @ Huffstatler Road | 33.41544 | -117.15199 |
| 902RBC04 | Rainbow Creek @ Old Highway 395 | 33.41272 | -117.15853 |
| 902RBC05 | Rainbow Creek @ Oak Crest Trailer Park | 33.41212 | -117.16173 |
| 902RBC06 | Rainbow Creek @ 2219 Willow Glen Road | 33.40859 | -117.20523 |
| 902RBC10 | Rainbow Creek @ MWD Crossing | 33.40696 | -117.18344 |
| 902SMG05 | Rainbow Creek @ Willow Glen Road | 33.40757 | -117.20253 |
| 902SMG06 | Rainbow Creek @ Stage Coach Lane | 33.41056 | -117.21477 |
| 902RVT01 | Frontage Tributary @ Rainbow Creek Rd. (Resendiz Growers) | 33.42134 | -117.14340 |
| 902RVT02 | Chica tributary @ 1st Street | 33.42126 | -117.14983 |
| 902HST01 | Brow Ditch to Rainbow Creek @ Huffstatler Road | 33.41526 | -117.15204 |
| 902HST02 | Pipe from a nursery along Huffstatler Road (downstream of HST01) | 33.41174 | -117.15196 |
| 902MGT01 | Margarita Glen Tributary to Rainbow Creek | 33.40847 | -117.19877 |
| 902RGT01 | Rainbow Glen Tributary to Rainbow Creek | 33.41107 | -117.18569 |
| 902WGT01 | Willow Glen Tributary @ Willow Glen Road | 33.40784 | -117.20309 |
| 902VMT01 | Via Milpas Tributary to Rainbow Creek | 33.40957 | -117.21373 |

Project/ Task Description:  Grab samples were collected monthly during dry weather conditions i.e. at least 72 hours following any rain event with precipitation greater than or equal to 0.10 inches.  The samples were analyzed in the laboratory for nitrate-N, nitrite-N, total kjeldahl nitrogen (TKN), ammonia, ortho-phosphate-P, total phosphorus, total dissolved solids (TDS), total iron, and sulfate. Table 2 below provides a summary of the parameters and methods.

Table 2: Constituents Monitored

| Constituent | Analysis Method | Unit | MDL | RL | No. Samples Analyzed |
| --- | --- | --- | --- | --- | --- |
| Ammonia as N | EPA 350.2 | mg/L | 0.105 | 0.5 | 491 |
|  | SM 4500 NH3 |  | 0.009 | 0.05 | 114 |
| Nitrate as N | EPA 300.0 |  | 0.007 | 0.2 | 548 |
|  | SM 4500 NO3 |  | 0.007 | 0.05 | 58 |
| Nitrite as N | EPA 354.1 |  | 0.001 | 0.01 | 413 |
|  | SM4 500 NO2 |  | 0.001 | 0.01 | 219 |
| Total Nitrogen | By calculation |  | N/A | N/A | 501 |
| Total Kjeldahl Nitrogen | EPA 351.1 |  | 0.24 | 0.8 | 530 |
|  | SM 4500-N org C |  | 0.084 | 0.5 | 86 |
| OrthoPhosphate as P | EPA 365.2 |  | 0.004 | 0.02 | 501 |
|  | SM 4500 PE |  | 0.004 | 0.02 | 115 |
| Total Phosphate as P | EPA365.3 |  | 0.01 | 0.02 | 502 |
|  | SM4500-P |  | 0.01 | 0.02 | 112 |
| Total Dissolved Solids | EPA 160.1 |  | 1.28 | 75 | 511 |
|  | SM2540C |  | 0.007 | 50 | 96 |
| Total Iron | EPA 200.7 |  | 0.00095 | 0.02 | 300 |
| Sulfate | EPA 300.0 |  | 0.024 | 0.5 | 309 |
|  | SM 4500 SO4 E |  | 25 | 125 | 36 |

Quality Control: The quality control requirements employed in this project are those described in Quality Assurance Project Plan for CWA 319(h) Grant Agreement No. 05-194-559-0 San Diego Region Rainbow Creek Nutrient reduction TMDL Implementation Water Quality Monitoring (included with this submittal).  These include methods used for sample collection and handling, data management, validation, and record keeping (including proper chain of custody procedures), data quality objectives for laboratory measurements, and personnel training.  The enclosed Trusdail Laboratories, Inc. Quality Assurance and Quality Control Manual for Environmental Sample Analysis describes quality control procedures employed by the analytical laboratory that conducted sample analysis for this project.

Enclosed Documents in Electronic Format:

Quality Assurance Project Plan for CWA 319(h) Grant Agreement No. 05-194-559-0 San Diego Region Rainbow Creek Nutrient reduction TMDL Implementation Water Quality Monitoring. Electronic Filename: COSD RC TMDL Monitoring_QAPP.pdf

Quality Assurance Project Plan for CWA 319(h) Grant Agreement No. 05-194-559-0 San Diego Region Rainbow Creek Nutrient reduction TMDL Implementation Water Quality Monitoring – Signature Page. Electronic Filename: COSD RC TMDL Monitoring QAPP Sig Pg.pdf

Trusdail Laboratories, Inc. Quality Assurance and Quality Control Manual for Environmental Sample Analysis.  Electronic Filename: Truesdail – QAQC manual 2005.pdf

Enclosed Electronic Data File:

County of San Diego Rainbow Creek TMD Water Quality Monitoring Data 2004 through 2009.  Electronic Filename:  COSD RC TMDL Monitoring_Data.xls.