Project Title:  Lower San Luis Rey Water Quality Monitoring 2004 through 2009

Originator:  County of San Diego Dept. of Public Works Watershed Protection Program

Contact:  Jo Ann Weber; e-mail:  joann.weber@sdcounty.ca.gov; tel. 858-495-5317

Publication Info.: Published as part of the San Luis Rey Watershed Urban Runoff Management Program Annual Reports.  Prepared for the California Regional Water Quality Control Board San Diego Region 9 by the City of Oceanside, City of Vista and County of San Diego.

Coordinate System:  NAD83

### Project Background and Objectives:  The lower 19 miles of the San Luis Rey River are listed on the 2006 CWA 303(d) list of impaired waterbodies for total dissolved solids (TDS) and chloride. The list also includes the Pacific Ocean Shoreline at San Luis Rey River mouth for Enterococcus and Total Coliform bacteria.  The lower 19 miles of the San Luis Rey River are also being considered for listing on the 2008 CWA 303(d) list of impaired waterbodies for total nitrogen and phosphorus and for Fecal Coliform and Enterococcus bacteria.   Furthermore, the State Water Resources Control Board, Region 9 is developing a “Total Maximum Daily Loads for Indicator Bacteria Project I – Twenty Beaches and Creeks in the San Diego Region (Including Tecalote Creek)” that includes San Luis Rey HU (903.00).

The San Luis Rey River Watershed Urban Runoff Management Program (WURMP) group initiated this monitoring program in March 2004.   The purpose of the program was to:

- Determine which areas along main stem of San Luis Rey River and its tributaries exhibit the highest concentrations of TDS, chloride, nutrients, and bacteria.

- Determine the potential sources of some of those constituents

From March 2004 through June 2009, the County of San Diego Department of Public Works, Watershed Protection Program collected surface water samples in the lower San Luis Rey River and its tributaries.

Geographical Setting/ Sampling Locations:  The portion of the lower 19 miles of San Luis Rey River and its tributaries located within the County of San Diego jurisdictional area (excluding City of Oceanside).  The specific sampling locations are listed in Table 1 below.

Table 1.  Lower San Luis Rey Sampling Locations:

| Site ID | Site Description | Latitude | Longitude |
| --- | --- | --- | --- |
| 903SLR01 | Moosa Canyon Creek at Old River Road | 33.283600 | -117.218683 |
| 903SLR02 | Little Gopher Canyon Creek at Old River Road | 33.265683 | -117.233200 |
| 903SLR14 | Ostrich Farm Creek at Highway 76 | 33.29335 | -117.22396 |
| 903SLR16 | San Luis Rey River at Highway 76 (Old Bonsall Bridge) | 33.26052 | -117.23836 |
| 903SLR17 | Keys Creek at Dulin Road | 33.32363 | -117.15744 |
| 903SLR25 | San Luis Rey River at Olive Hill Road | 33.28838 | -117.22335 |
| 903SLR26 | Bonsall Creek at Highway 76 | 33.28959 | -117.22525 |
| 903SLR27 | Live Oak Creek at Highway 76 | 33.31514 | -117.19418 |
| 903SLR28 | San Luis Rey River at Pankey Road (Shearer Crossing) | 33.33281 | -117.14975 |
| 903SLR32 | Horse Ranch Creek | 33.33138 | -117.15067 |
| 903SLR34 | East Vista Way Tributary to SLR River | 33.25872 | -117.2393 |

Project/ Task Description:  Single grab samples were collected periodically since 2004 from the center of the stream at 11 locations listed in Table 1 above.  All samples were taken during dry weather conditions i.e. at least 72 hours following any rain event with precipitation greater than or equal to 0.10 inches.  Bacterial samples were collected using a 100 ml sterilized collection bottle supplied with sodium sulfite as a preservative.  All samples were stored at 4ºC and transported to the analytical laboratory on the same day.  The samples were analyzed in the laboratory for TDS, total suspended solids (TSS), chloride, magnesium, manganese, total iron, sulfate, potassium, sodium, calcium, alkalinity (hydroxide, carbonate and bicarbonate), fluoride, total boron, hardness, total coliform bacteria, fecal coliform bacteria, Enterococcus bacteria, ammonia-N, nitrate-N, nitrite-N, total Kjeldahl nitrogen (TKN), orthophosphate-P and total phosphorus (Table 2).

Table 2.  Constituents Monitored:

| Constituent | Analysis Method | Unit | MDL | RL | No. Samples Analyzed |
| --- | --- | --- | --- | --- | --- |
| Total Coliform | SM 9221 | MPN/ 100 ml | 1 | 20 | 151 |
|  | SM 9223 B |  | 1 | 20 | 3 |
| Fecal Coliform | SM 9221 |  | 1 | 20 | 151 |
|  | SM 9223 B |  | 1 | 20 | 3 |
| Enterococcus | SM 9230 |  | 2 | 20 | 158 |
| Alkalinity as CaC03 | EPA 310.1 | mg/L | 1.68 | 5 | 84 |
|  | SM 2320 B |  | 5 | 5 | 79 |
| Hydroxide Alkalinity | EPA 310.1 |  | Calc. |  | 41 |
|  | SM 2320 B |  |  |  | 47 |
| Bicarbonate Alkalinity | EPA 310.1 |  |  |  | 76 |
|  | SM 2320 B |  |  |  | 81 |
| Carbonate Alkalinity | EPA 310.1 |  |  |  | 73 |
|  | SM 2320B |  |  |  | 80 |
| Chloride | EPA 300.0 |  | 0.028 | 20 | 156 |
|  | SM4500 Cl C |  | 0.05 | 0.05 | 36 |
| Magnesium, Dissolved | EPA 200.7 |  | 0.00785 | 0.3 | 148 |
|  | EPA 6010 |  | 0.05 | 1 | 18 |
| Magnesium, Total | EPA 200.7 |  | 0.5 | 1 | 8 |
|  | EPA 6010 |  | 0.5 | 1 | 6 |
| Manganese, Total | EPA 200.7 |  | 0.03 | 0.3 | 8 |
|  | EPA 6010 |  | 0.03 | 0.3 | 24 |
| Iron, Total | EPA 200.7 |  | 0.00095 | 0.02 | 147 |
|  | EPA 200.8 |  | 0.00095 | 0.02 | 7 |
|  | EPA 6010 |  | 0.05 | 0.1 | 24 |
| Potassium, Dissolved | EPA 200.7 |  | 0.0019 | 0.2 | 145 |
|  | EPA 6010 |  | 1 | 2 | 1 |
| Potassium, Total | EPA 6010 |  | 1 | 2 | 19 |
| Sodium, Dissolved | EPA 200.7 |  | 0.00044 | 2 | 148 |
|  | EPA 6010 |  | 1 | 2 | 24 |
| Calcium, Dissolved | EPA 200.7 |  | 0.0047 | 2 | 149 |
|  | EPA 6010 |  | 1 | 2 | 24 |
| Sulfate | EPA 300.0 |  | 0.024 | 25 | 156 |
|  | SM 4500 |  | 25 | 125 | 24 |
| Hardness as CaCO3 | EPA 130.2 |  | 1.17 | 10 | 155 |
|  | EPA 200.7 |  | 10 | 10 | 24 |
| Boron, Total | EPA 200.7 |  | 0.0048 | 0.02 | 113 |
|  | EPA 200.8 |  | 0.00084 | 0.02 | 7 |
| Fluoride | EPA 300.0 |  | 0.005 | 0.1 | 153 |
| Ammonia as N | EPA 350.2 |  | 0.009 | 0.05 | 158 |
|  | SM 4500 NH3 |  | 0.009 | 0.05 | 47 |
| Nitrate as N | EPA 300.0 |  | 0.007 | 0.2 | 204 |
|  | SM 4500 NO3 E |  | 0.2 | 0.5 | 35 |
| Nitrite as N | EPA 354.1 |  | 0.001 | 0.01 | 15 |
|  | SM 4500 |  | 0.001 | 0.01 | 99 |
| Total Nitrogen | By calculation |  | Calc. |  | 64 |
| Total Kjeldahl Nitrogen | EPA 351.1 |  | 0.24 | 0.5 | 49 |
|  | SM 4500 Norg |  | 0.084 | 0.5 | 24 |
| Organic Nitrogen | By calculation |  | Calc. |  | 64 |
| Orthophosphate as P | EPA 365.2 |  | 0.004 | 0.02 | 130 |
|  | EPA 365.3 |  | 0.004 | 0.02 | 10 |
|  | SM 4500 PE |  | 0.004 | 0.02 | 65 |
| Total Phosphate as P | EPA365.3 |  | 0.01 | 0.02 | 148 |
|  | SM4500-P |  | 0.01 | 0.02 | 49 |
| Total Suspended Solids | EPA 160.2 |  | 0.217 | 2.5 | 135 |
|  | SM2540D |  | 0.307 | 2.5 | 46 |
| Total Dissolved Solids | EPA 160.1 |  | 2.28 | 75 | 149 |
|  | SM2540C |  | 0.07 | 50 | 36 |

Quality Control: The quality control requirements employed in this project followed those described in Quality Assurance Project Plan for the County of San Diego’s Dry Weather Monitoring and MS4 Program (included with this submittal).  These include methods used for sample collection and handling, data management, validation, and record keeping (including proper chain of custody procedures), data quality objectives for laboratory measurements, and personnel training.  The enclosed Trusdail Laboratories, Inc. Quality Assurance and Quality Control Manual for Environmental Sample Analysis describes quality control procedures employed by the analytical laboratory that conducted sample analysis for this project.

Enclosed Documents in Electronic Format:

Quality Assurance Project Plan for the County of San Diego’s Dry Weather Monitoring Program and MS4 Program. Electronic Filename: COSD QAPP DWM.doc

Trusdail Laboratories, Inc. Quality Assurance and Quality Control Manual for Environmental Sample Analysis.  Electronic Filename: Truesdail – QAQC manual 2005.pdf

Enclosed Electronic Data File:

County of San Diego Lower San Luis Rey Water Quality Monitoring Data 2004 through 2009.  Electronic Filename:  COSD Lower SLR_Data.xls.