Available recent sediment and fish tissue chlordane data for Santa Monica Bay were compiled to supplement results presented in the 2006 draft 303d factsheet. Results were subsequently compared against screening values to determine the magnitude and frequency of exceedences. All data used in this evaluation were collected between 1998 and 2004, and are the most recent chlordane data readily available for Santa Monica Bay.

Here is summary of the assessment results.  The sediment data had very few total chlordane values above the California 303(d) Policy screening value of 6.0 micrograms per kilogram (ug/kg).  Specifically, the Palos Verdes Shelf sediment data showed an exceedence frequency of 3.3%, while the Santa Monica Bay sediment datasets had exceedence frequencies of 0.0% and 0.5% (Table 1).  Combining the sediment datasets yields 4 out of 307 sediment samples exceeding the Policy screening value.  Two fish tissue datasets were also evaluated, and 0 of 434 total chlordane results were greater than the California Office of Environmental Health Hazard Assessment (OEHHA) tissue screening value of 30.00 ug/kg (Table 2). Detailed evaluation of these data indicates attainment of narrative water quality objectives for the sediment and tissue lines of evidence.

To ensure comparability between datasets, identical summation procedures were used to calculate total chlordane for each dataset.  These procedures were consistent with approaches described by NOAA (NOAA CPRD Standardized Sums) and SCCWRP (Chemistry Datasets Imputation Summation Procedures).  Specifically, detected values were summed or the highest non-detect value was used if all values were undetected.  In addition, if the highest non-detect value was greater than the sum of detected results, the non-detect value was included as the sum.  If total chlordane was provided in the dataset along with the raw data, the totals were recalculated using this methodology, where possible, to ensure consistency between datasets.

Raw and summed data are presented in the Excel spreadsheet for assessment. (We have also attached the Bight 98 results using these summation techniques for direct comparison and examination.)

## Table 1.  Chlordane results from sediment samples in Santa Monica Bay

# Santa Monica Bay Sediment Data

| Data Source | Number of Samples | Number of Non-Detects | Minimum (ug/kg) | Maximum (ug/kg) | Mean (ug/kg) | Number of Stations | Period of Record | Number Above Screening Value | Percent Above Screening Value | ##### Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LACSD: Palos Verdes Shelf data | 90 | 87 | 1.40 | 40.30 | 3.86 | 44 | 1998, 2000, 2002, 2004 | 3 | 3.3% | 1 |
| LACSD: Summary of Santa Monica Bay Bight 98 Data | 23 | 21 | 1.00 | 1.21 | 1.01 | 23 | 1998 | 0 | 0.0% | 2 |
| Hyperion WWTP: Santa Monica Bay data | 194 | 192 | 0.30 | 31.00 | 1.08 | 65 | 1998-2004 | 1 | 0.5% | 3 |
| Notes: - Four analytes were used to calculate total chlordane: cis-chlordane, trans-chlordane, oxychlordane, and trans-nonachlor. Total chlordane results were also provided, but were recalculated to replace the "ND" results with values as per the SCCWRP protocol. Data with MDLs > Screening Value were excluded from the analyses. - Total Chlordane data provided by LACSD were recalculated to follow the SCCWRP/NOAA summation procedures. Totals are based on chlordane-a and chlordane -g. - Number of analytes used for analyses varied (5, 7, or total provided in dataset). |  |  |  |  |  |  |  |  |  |  |

## Table 2.  Chlordane results from tissue samples in Santa Monica Bay

# Santa Monica Bay Tissue Data

| Data Source |  | Number of Samples | Number of Non-Detects | Minimum (ug/kg) | Maximum (ug/kg) | Mean (ug/kg) | Number of Stations | Period of Record | Number Above Screening Value | Percent Above Screening Value | ##### Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OEHHA/CFCP: Santa Monica Bay data |  | 9 | 5 | 2.00 | 7.66 | 2.66 | 3 | 1999 | 0 | 0.0% | 1, 2 |
| Hyperion WWTP: Santa Monica Bay data | Fillet | 219 | 208 | 0.09 | 3.63 | 1.10 | 7 | 2000-2004 | 0 | 0.0% | 3 |
|  | Whole Fish | 206 | 148 | 0.09 | 19.72 | 1.47 | 2 | 2000-2004 | 0 | 0.0% | 3 |
|  | Total | 425 | 356 | 0.09 | 19.72 | 1.28 | 7 | 2000-2004 | 0 | 0.0% | 3 |
| Notes: - Total chlordane is the sum of five analytes. - Stations include Santa Monica Pier (4110), Venice Pier (4120), and Party Boat to Malibu Kelp Beds (4140). - Total chlordane is the sum of five analytes or the total chlordane measurement provided in dataset. |  |  |  |  |  |  |  |  |  |  |  |