The Electronic

Deliverable Format

(EDF)

Version 1.2b

GUIDELINES & RESTRICTIONS

January 2001

Prepared by

ArsenaultLegg, Inc.

9600 Main Tree Drive

Anchorage, Alaska 99516

Phone: (907) 346-3827

Fax: (907) 346-1577

E-mail: information@arsenaultlegg.com

Web site: www.arsenaultlegg.com

Table of Contents

1	Introduction	1

1.1	Key Concepts	2

1.2	Document Conventions	3

1.3	Valid Values	5

2	Database Description	6

2.1	Sample Information	7

2.2	Test Information	7

2.3	Results Information	8

2.4	Quality Control Information	9

2.5	Control Limit Information	9

2.6	Narrative Information	10

3	Relational Files Format	11

3.1	EDFSAMP: The Sample Information File	11

3.2	EDFTEST: The Analysis (Test) Information File	14

3.3	EDFRES: The Results Information File	19

3.4	EDFQC: The QC Information File	24

3.5	EDFCL: The Quality Control Limit Information File	27

3.6	EDFNARR: The Narrative File	29

4	Flat File Format	30

4.1	EDFFLAT: The Flat File	30

5	File, Record, and Data Field Requirements	35

5.1	File and Record Requirements	35

5.2	Data Field Requirements	35

5.3	Diskette Submittal	36

Appendix A:	Summary of Data Elements	37

Appendix B:	Glossary of Terms	44

List of Tables

Table 1: [File Name]	4

Table 2: EDFSAMP (SAMPLE) Format	12

Table 3: EDFTEST (TEST) Format	16

Table 4: EDFRES (RESULTS) Format	21

Table 5: EDFQC (QC) Format	26

Table 6: EDFCL (CL) Format	28

Table 7: EDFFLAT Format	31

List of Figures

Figure 1: From Field to EDF	2

Figure 2: Example Figure Definition	3

Figure 3: Relational Database Structure of the EDF	6

Figure 4: One-to-Many Parent-Child Table Relationship	48

Figure 5: One Parent Record to Many Child Records	49

Figure 6: Primary Key	50

Acronyms

| ASCII | American Standard Code (for) Information Interchange |
| --- | --- |
| CAS | Chemical Abstract Service |
| CL | Control Limit |
| COC | Chain-of-Custody |
| COELT | U.S. Army Corps of Engineers Loading Tool |
| CSV | Comma Separated Values (AKA Comma/Quote Delimited) |
| EDCC | Electronic Deliverable Consistency Checker |
| EDD | Electronic Data Deliverable |
| EDF | Electronic Deliverable Format |
| EDMS2000 | enABL Data Management System, Version 2000 |
| FK | Foreign Key |
| LIMS | Laboratory Information Management System |
| NA | Not Applicable |
| NC | Non-Client |
| ND | Non-Detected |
| NPDL | North Pacific Division Laboratory |
| PK | Primary Key |
| QA | Quality Assurance |
| QC | Quality Control |
| RPD | Relative Percent Difference |
| VVL | Valid Value List |

# Introduction

The Electronic Deliverable Format (EDF), Version 1.2b, January 2001, is a comprehensive data standard for analytical laboratories, designed to facilitate the transfer of electronic data files from the laboratory to the end-user. Laboratories can produce their EDF using the U.S. Army Corps of Engineers Loading Tool (COELT) software, or may produce EDF with other programs outside of COELT.

The EDF data components include:

- Chain-of-Custody (COC) Information

- sample collection information

- administrative information

- preservatives added to the samples

- conditions of transport

- Laboratory Results Information

- tests performed

- parameters tested

- analytical results

- Quality Assurance (QA) Information (key to data verification)

- detection limits

- control limits for precision and accuracy

- narrative report explaining non-conformances

- Built-in Guidelines and Restrictions

- Valid Value Lists (VVLs)

The EDF may be used for the production of hard copy reports, electronic data review, or data summaries. The EDF is the absolute electronic reflection of the legally defensible hard copy laboratory report produced with COELT.

Figure 1: From Field to EDF

## Key Concepts

The benefits of using the EDF data standard include:

- Provides a comprehensive data standard for analytical laboratories, allowing different laboratories to provide consistent reporting parameters.

- Provides an efficient industry-wide, universal standard for electronic analytical data.

- Promotes the highest potential of data for transfer, review, and interpretation by multiple parties associated with current and future projects.

- Eliminates laborious and costly manual re-entry of hard copy laboratory data, which often results in transcription errors.

- May be produced by entering data manually, or by importing data directly from a Laboratory Information Management System (LIMS).

- Provides guidelines and restrictions that help reduce data entry errors and inconsistencies.

- Legally defensible hard copy reports can be generated directly from the electronic data in a standardized format.

- Presents quality assurance/quality control (QA/QC) information for each laboratory report, that is the key to data verification.

- Provides guardianship of catalogued VVLs, assuring universal consistency among users.

- Provides an electronic project archive of known quality, with historical data that are easily accessible and efficiently reviewed by different parties, for use in future environmental projects.

- Promotes dynamic growth of institutional knowledge between laboratories, consultants, their clients, and agencies.

## Document Conventions

This document presents the structure of the EDF and guidelines and restrictions for creating an EDF electronic data deliverable (EDD). Each data file is discussed in a level of detail that will allow a laboratory to create an EDF that meets the criteria of the data standard. Included is a discussion of guidelines and restrictions that apply to files and those that apply to individual fields. This is a very technical document. For a more narrative description of EDF, please refer to the EDF Overview document.

### Figure Representation of Files

Each file discussion begins with a figure representing the fields in the file. Refer to Figure 2 as an example. The fields are listed in the order in which they exist within the structure, and primary key fields are underlined. “Primary key” means a selected field (or fields in combination) that makes a record unique in a database. Refer to the Glossary in Appendix A for a technical definition of this and other terms. The order of the fields in the figure is the order expected for delivery.

Figure 2: Example Figure Definition

### Table Representation of EDF Files

The following table is a representation of the table defining each of the five relational files of the EDF fixed length format.

Table 1: [File Name]

| Field Name | Attrb | Start-End | PK | FK | VVL | REQ | Dscr. Name | Definition |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FIELD1 | C18 | 1-18 | Yes | Yes | Yes | Yes | Field 1 | Field 1 is a character field with 18 available positions. |
| FIELD2 | D8 | 19-26 | Yes | No | No | Yes | Field 2 | Field 2 is a date field with an expected format of YYYYMMDD. |
| FIELD3 | N5 | 27-31 | No | No | No | No | Field 3 | Field 3 is a numeric field with a total of 5 spaces available for numbers and decimals, with no restriction on the number of digits to the right of the decimal point other than the overall field size. |
| FIELD4 | L1 | 32-32 | No | No | No | Yes | Field 4 | Field 4 is a logic field with expected values of “T” (true) or “F” (false). |

The “Field Name” is the actual structural name of the field. All primary key fields are in bold type within these tables (e.g., FIELD2). All field names are italicized throughout this document. Fields are listed in their structural order within these tables.

“Attrb” describes the field attributes (type and size). For example:

- C8 is an 8-character field (alphanumeric).

- N5 is a numeric field with a total of 5 spaces available for numbers and decimals, with no restriction on the number of digits to the right of the decimal point other than the overall field size (e.g., 12345 or 123.4 or 1.234).

- D8 is a date field with an expected format of YYYYMMDD (i.e., 20010101).

- L1 is a logic field with expected values of “T” (true) or “F” (False).

- Time format is 4 digits using a 24-hour military clock without the colon (e.g., 1400 for 2:00 p.m.).

The “Start-End” column defines the starting and ending positions for the field within the data file.

“PK” further identifies with a “Yes” or “No” the primary key fields.

“FK” identifies with a “Yes” of “No” the foreign key fields. A “foreign key” is a primary key field in one file (a “parent file”) shared with a related file (“child file”) in a data file relationship. Refer to the Glossary in Appendix A for technical definitions of this and other terms.

The “VVL” column indicates with a “Yes” or “No” whether the data field requires a valid value code.

The “REQ” column indicates with a “Yes” or “No” whether entry into a field is required.

The “Dscr. Name” column gives the descriptive name of the field.

The “Definition” is a brief definition and/or explanation of the field and expectations for entry into the field.

### Conventions for Text

Throughout this document, file names are capitalized (e.g., the EDFSAMP file), and field names are capitalized and italicized (e.g., SAMPID). The words “file” and “table” are used interchangeably.

Each file discussion is organized into guidelines and restrictions for the file as a whole (“File Guidelines and Restrictions”), and guidelines and restrictions for entry into fields within the file (“Field Guidelines and Restrictions” and “Special Considerations”). File guidelines and restrictions include such information as whether the file must be populated and how it relates to another file in the structure.

Included in the field guidelines and restrictions are lists of which fields require VVLs, which fields require entry for submission, and the file’s primary and foreign keys. Any exceptions or special cases are listed under “Special Considerations.”

## Valid Values

Various data fields in the EDF require entry of valid values. Valid values are built-in codes that the format requires for certain fields, such as contractor names, matrices, and laboratories. The reason for using specific values for these fields is to standardize the data entry, to ensure data consistency and prevent errors. Freely entered data might contain extra spaces, commas, or dashes that would make meaningful data manipulation and thorough or accurate data searches impossible.

Most valid values are abbreviations of common or proper names; hence selecting the correct code is generally straightforward. However, some valid values are also used to link data properly (e.g., QCCODE is used to help link a laboratory replicate [“LR1”] to its original field sample [“CS”]). The EDF Data Dictionary provides lists of the valid value codes and their definitions for each valid value field in the EDF.

New valid value codes can be requested Monday through Friday between 9:00 a.m. and 6:00 p.m. Pacific Standard Time through the office of ArsenaultLegg, Inc., by phone (907) 346-3827, fax (907) 346-1577, or e-mail information@arsenaultlegg.com. Please allow 72 hours for code generation.

# Database Description

The EDF is a relational database consisting of five files, related to one another through common (key) fields. These data files are described as relational because the information in one file is related to information in other files, linked through a group of fields called the primary key. The primary key fields collectively make a record unique within a file. A record is a line of data (a row) in a table or file made up of distinct fields of information. The primary key fields in one file record must be identical to the same fields in the linking file record in order to “relate” the data records in both files.

Figure 3: Relational Database Structure of the EDF

## Sample Information

The EDFSAMP file (also referred to as the SAMPLE file) contains collection, location, and administrative information concerning field samples. Most of the information in this file should be available on the COC form. Only client samples appearing on the COC are to be entered into this file (i.e., no laboratory-generated samples should be entered into this file).

## Test Information

The EDFTEST file (also referred to as the TEST file), containing information regarding analytical tests performed on samples, is related to the SAMPLE file by sample collection information and field sample number. There is a one-to-many relationship between the SAMPLE and TEST files, meaning one record in the SAMPLE file can link to many TEST records.

One may envision that the sample collection information is unnecessary in the TEST file and that the field sample identification should be sufficient to link the SAMPLE file to the TEST file. However, not all consultants provide unique field sample numbers. It is conceivable that a sampling technician may assign sample numbers sequentially, starting over with the number “one” at each site. There are many instances of MW-1 (i.e., a sample from monitoring well #1) having been assigned to a variety of separate sites. Certainly, this does not represent a unique sample identifier. However, given the frequency of use, it would seem to have universal appeal. The additional sample collection information carried in the related fields in the TEST file will allow the EDF to distinguish among samples collected at different times, yet having been assigned the same sample number.

## Results Information

The EDFRES file (also referred to as the RESULTS file) contains information on results generated by the laboratory. The TEST file relates to the RESULTS file through the laboratory sample ID and analytical information. There is also a one-to-many relationship between the TEST and RESULTS files, as noted above (i.e., there can be more than one result generated for a single test). Each RESULTS record contains information about a specific analytical result.

## Quality Control Information

The EDFQC file (also referred to as the QC file) contains data related to laboratory quality control (QC) samples. Each QC sample is identified as belonging to a particular QC batch that serves to relate the QC and TEST files. However, the actual result for a QC sample and its related reference sample (i.e., the original sample of a duplicate or a spike) is stored in the RESULTS file.

## Control Limit Information

The EDFCL file (also referred to as the CL file) contains data associated with analytical control limits (CL). Each CL file record contains control limit information for a parameter analyzed by a particular analytical method. The CL and RESULTS files are related through the analytical method, parameter, and control limit revision date, collectively.

## Narrative Information

The EDFNARR file (also referred to as the NARRATIVE file) provides a means to transfer descriptive information about analyses that do not easily fit in a standardized format. This file does not require a specific format but should be delivered as an ASCII file.

# Relational Files Format

The following Chapter describes the fixed length relational files format, and guidelines and restrictions associated with each of the five relational data files of EDF.

## EDFSAMP: The Sample Information File

The purpose of the SAMPLE file is to track the administrative and field collection information associated with a sample. For every field-generated sample entering the laboratory, one record will be added to this file. Most of the information in this file should be available on the COC and is to be entered exactly as it appears on that form. Table 2, on page 12, presents the SAMPLE file structure and attributes.

### File Guidelines and Restrictions:

- LOGDATE, LOGTIME, LOGCODE, SAMPID, MATRIX, and LABCODE comprise the primary key.

- Non-Client (NC) and laboratory-generated QC samples (i.e., samples created in the laboratory) are not to be entered into this file. (“NC” samples are samples that do not originate from a client’s sites but are used to generate QC results for a client’s group of samples.)

### Field Guidelines and Restrictions:

- All fields except LOCID, req_method_grp, coc_matrix, dqo_id, LAB_METH_GRP, and meth_design_id require entry.

- LABCODE, LOGCODE, MATRIX, and COC_MATRIX require valid value entries. Refer to the EDF Data Dictionary for lists of valid value codes.

- LABCODE reflects the laboratory that received the sample and is responsible for generating the EDD.

Table 2: EDFSAMP (SAMPLE) Format

| Field Name | Attrb | Start-End | PK | FK | VVL | REQ | Dscr. Name | Definition |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| locid | C10 | 1-10 | No | No | No | No | Location ID | The unique identifier for the sample's location, as identified by the laboratory. |
| logdate | D8 | 11-18 | Yes | No | No | Yes | Collection Date | The date a field sample is collected. |
| logtime | C4 | 19-22 | Yes | No | No | Yes | Collection Time | The time that a field sample is collected, recorded using 24-hour military time. |
| logcode | C4 | 23-26 | Yes | No | Yes | Yes | Field Organization | The code identifying the company collecting the samples or performing field tests. |
| sampid | C25 | 27-51 | Yes | No | No | Yes | COC Sample ID | The unique identifier representing a sample, assigned by the consultant, as submitted to the laboratory on a chain-of-custody. |
| matrix | C2 | 52-53 | Yes | No | Yes | Yes | Matrix | The code identifying the sample matrix as determined by the laboratory (e.g., water, soil, etc.). |
| projname | C25 | 54-78 | No | No | No | Yes | Project Name | The identification assigned to the project by the organization performing the work. |
| npdlwo | C7 | 79-85 | No | No | No | Yes | Work Order Number | A delivery order number associated with the contract. |
| cntshnum | C12 | 86-97 | No | No | No | Yes | Control Sheet Number | The administratively-assigned identification used to track contracts. |
| labcode | C4 | 98-101 | Yes | No | Yes | Yes | Laboratory | The code identifying the laboratory that analyzes the sample. |
| req_method_grp | C25 | 102-126 | No | No | No | No | Requested Method Group | The unique identifier for the method or group of methods requested by the client for analysis of the sample. |
| coc_matrix | C2 | 127-128 | No | No | Yes | No | COC Matrix | The code identifying the sample matrix as noted on the chain-of-custody (e.g., water, soil, etc.). |
| dqo_id | C25 | 129-153 | Yes | No | No | No | Data Quality Objectives ID | The unique identifier representing the data quality objectives. |
| meth_design_id | C25 | 154-178 | Yes | No | No | No | Method Design ID | The unique identifier for the design of an analytical method. |
| lab_meth_grp | C25 | 179-203 | Yes | No | No | No | Lab Method Group | The unique identifier for a group of methods as defined by the laboratory. |

## EDFTEST: The Analysis (Test) Information File

The TEST file contains information concerning the analytical test associated with the sample. A test record is generated for each test performed that results in usable data. Five fields (LOGDATE, LOGTIME, LOGCODE, SAMPID, and LABCODE) from the SAMPLE file are carried over to the TEST file as foreign keys. Most of the information in the TEST file can be located at the top portion of a standard laboratory bench sheet. Table 3, on page 16, presents the TEST file structure and attributes.

### File Guidelines and Restrictions:

- MATRIX, LABCODE, LABSAMPID, QCCODE, ANMCODE, EXMCODE, ANADATE, EXTDATE, and RUN_NUMBER comprise the primary key.

- Each TEST record must have associated SAMPLE and RESULTS records.

- All sample types must be entered into this file (i.e., client samples, non-client samples, and all QC sample types).

### Field Guidelines and Restrictions:

- LABCODE, LOGCODE, MATRIX, QCCODE, ANMCODE, EXMCODE, BASIS, PRESCODE, SUB, and LNOTE require valid value entries. Refer to the EDF Data Dictionary for lists of valid value codes.

- MODPARLIST requires a “T” (true) entry if a parameter from the parameter list (refer to the actual method) is not reported. The parameter list is not considered modified if extra parameters are reported.

- LABSAMPID must be unique.

- RUN_NUMBER should have a value of one or greater.

- Multiple PRESCODEs may be used; commas without spaces separate the codes (e.g., “P08,P12”). If the no preservative was added, this field may be left blank.

- Multiple LNOTEs may be used; commas without spaces separate the codes (e.g., “AZ,B,CI”). If qualification is not require, this field may be left blank.

- LABLOTCTL must uniquely distinguish a group of samples that are prepared together.

- LABCODE reflects the laboratory that first receives the sample.

- Enter a LABCODE (other than “NA”) in the SUB field if the lab performing the analysis is not the laboratory that received the sample. “NA” must be entered into this field unless the test is subcontracted out.

- LOCID, LOGDATE, LOGTIME, SAMPID, LOGCODE, LAB_REPNO, REP_DATE, and COCNUM should be left blank for laboratory-generated and non-client samples (i.e., QCCODE is not “CS”).

- APPRVD should be left blank for non-client samples (i.e., QCCODE is “NC”).

- LAB_METH_GRP and METH_DESIGN_ID are optional fields.

Table 3: EDFTEST (TEST) Format

| Field Name | Attrb | Start-End | PK | FK | VVL | REQ | Dscr. Name | Definition |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| locid | C10 | 1-10 | No | No | No | No | Location ID | The unique identifier for the sample's location, as identified by the laboratory. |
| logdate | D8 | 11-18 | No | Yes | No | Yes | Collection Date | The date a field sample is collected. |
| logtime | C4 | 19-22 | No | Yes | No | Yes | Collection Time | The time that a field sample is collected, recorded using 24-hour military time. |
| logcode | C4 | 23-26 | No | Yes | Yes | Yes | Field Organization | The code identifying the company collecting the samples or performing field tests. |
| sampid | C25 | 27-51 | No | Yes | No | Yes | COC Sample ID | The unique identifier representing a sample, assigned by the consultant, as submitted to the laboratory on a chain-of-custody. |
| matrix | C2 | 52-53 | Yes | Yes | Yes | Yes | Matrix | The code identifying the sample matrix as determined by the laboratory (e.g., water, soil, etc.). |
| labcode | C4 | 54-57 | Yes | Yes | Yes | Yes | Laboratory | The code identifying the laboratory that analyzes the sample. |
| labsampid | C12 | 58-69 | Yes | No | No | Yes | Laboratory Sample ID | The unique identification number assigned to the sample by the laboratory. |
| qccode | C3 | 70-72 | Yes | No | Yes | Yes | QC Type | The code identifying the type of sample (e.g., laboratory-generated, environmental, etc.). |
| anmcode | C7 | 73-79 | Yes | No | Yes | Yes | Analytical Method | The code identifying the method of analysis. |
| modparlist | L1 | 80-80 | No | No | No | Yes | Modified Parameter List | A field indicating whether the parameter list of an analytical method has been modified. |
| exmcode | C7 | 81-87 | Yes | No | Yes | Yes | Preparation Method | The code identifying the method of preparation. |
| lablotctl | C10 | 88-97 | No | No | No | Yes | Preparation Batch Number | The unique identifier for a preparation and handling batch. |
| exlablot | C10 | 98-107 | No | No | No | No | OBSOLETE | OBSOLETE |
| anadate | D8 | 108-115 | Yes | No | No | Yes | Analysis Date | The date the sample (aliquot, extract, digest and/or leachate) is analyzed. |
| extdate | D8 | 116-123 | Yes | No | No | Yes | Preparation Date | The date that a sample is prepared for analysis. |
| run_number | N2 | 124-125 | Yes | No | No | Yes | Run Number | The numeric code distinguishing multiple or repeat analysis of a sample by the same method on the same day. |
| recdate | D8 | 126-133 | No | No | No | Yes | Received Date | The date the sample is received by the laboratory doing the analysis. |
| cocnum | C16 | 134-149 | No | No | No | No | Chain-of-Custody Number | The number assigned to the chain-of-custody. |
| basis | C1 | 150-150 | No | No | Yes | Yes | Basis | The code used to distinguish whether a sample is reported as dry or wet weight, filtered or not filtered. |
| prescode | C15 | 151-165 | No | No | Yes | No | Preservative | The code identifying the type of preservative added to the sample. |
| sub | C4 | 166-169 | No | No | Yes | Yes | Subcontracted Laboratory | The code identifying the subcontracted laboratory. |
| rep_date | D8 | 170-177 | No | No | No | No | Report Date | The date of the laboratory report. |
| lab_repno | C20 | 178-197 | No | No | No | No | Laboratory Report Number | The unique identifier for the laboratory report, assigned by the laboratory. |
| apprvd | C3 | 198-200 | No | No | No | No | Approved By | The initials of the individual approving the laboratory report. |
| lnote | C20 | 201-220 | No | No | Yes | No | Laboratory Test Notes | The code identifying notes pertaining to analytical performance irregularities that apply to the entire test. |
| lab_meth_grp | C25 | 221-245 | Yes | Yes | No | No | Lab Method Group | The unique identifier for a group of methods as defined by the laboratory. |
| meth_design_id | C25 | 246-270 | Yes | Yes | No | No | Method Design ID | The unique identifier for the design of an analytical method. |

## EDFRES: The Results Information File

The RESULTS file contains information concerning analytical results generated by the laboratory. Each record contains a parameter result. Parameter results are coded using the PVCCODE to distinguish whether they are primary results or supporting analytical data (i.e., second column confirmation). Results and detection limits are to be adjusted for dilution prior to data entry. Dilution adjustments are the only calculations necessary prior to entering values into the format. All other QC calculations will be performed in the database receiving the EDD. (NOTE: The exception to this is surrogates, which must be reported in “PERCENT” UNITS.) Table 4, on page 21, presents the RESULTS file structure and field attributes.

### File Guidelines and Restrictions:

- MATRIX, LABCODE, LABSAMPID, QCCODE, ANMCODE, EXMCODE, PVCCODE, ANADATE, RUN_NUMBER, and PARLABEL comprise the primary key.

- Each RESULTS record must have a corresponding TEST record.

- All sample types must be entered into this file (i.e., client samples, non-client samples, and all QC types).

### Field Guidelines and Restrictions:

- MATRIX, LABCODE, QCCODE, ANMCODE, EXMCODE, PVCCODE, PARLABEL, PARVQ, REPDLVQ, UNITS, SRM, and LNOTE require valid value entries. Refer to the EDF Data Dictionary for lists of valid value codes.

- LABCODE reflects the laboratory that receives the sample.

- RUN_NUMBER should have a value of one or greater.

- PARVALs less than REPDL must have a PARVQ of “ND.”

- Multiple LNOTEs may be used; commas without spaces separate the codes (e.g., “AZ,B,CI”). If qualification is not required, this field may be left blank.

- CLREVDATE should be blank for environmental samples (i.e., QCCODE is “CS” or “NC”), laboratory-generated blanks (i.e., QCCODE is “LB” or “RS”), and non-spiked parameter results, except for surrogate results (i.e., PARVQ is “SU”).

- LABDL and REPDL should be blank for parameters with UNITS of “PERCENT.”

- EXPECTED should be blank for all environmental sample results. For spiked samples, enter the AMOUNT OF THE SPIKE ADDED PLUS THE SAMPLE VALUE in this field. For non-spiked samples, enter the value expected into this field (i.e., for a distilled water blank, enter zero).

- CLREVDATE requires an entry when QCCODE is “MS/SD,” “BS/BD,” “RM/KD,” “LR,” “IC,” or “CC.”

- CLREVDATE requires an entry when PARVQ is “SU” or “IN.”

- PARVAL, LABDL, and REPDL should be adjusted for dilution (DILFAC).

- LAB_METH_GRP and METH_DESIGN_ID are optional fields.

#### Special Considerations for Surrogate Compounds:

- PARVQ must be entered as “SU.”

- UNITS must be entered as “PERCENT.”

- Expected must be entered as “100.”

- LABDL and REPDL should be blank. REPDLVQ and SRM should be “NA.”

#### Special Considerations for Tentatively Identified Compounds (TICs):

- PARVQ must be entered as “TI.”

- Chemical Abstract Service (CAS) numbers may be used (for TICs only) instead of PARLABELs to identify the parameter being reported. It is recommended that TICs without CAS numbers have PARLABEL valid values.

- LABDL and REPDL should be blank. REPDLVQ and SRM should be “NA.”

- RT is a recommended entry field for TIC results.

Table 4: EDFRES (RESULTS) Format

| Field Name | Attrb | Start-End | PK | FK | VVL | REQ | Dscr. Name | Definition |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| matrix | C2 | 1-2 | Yes | Yes | Yes | Yes | Matrix | The code identifying the sample matrix as determined by the laboratory (e.g., water, soil, etc.). |
| labcode | C4 | 3-6 | Yes | Yes | Yes | Yes | Laboratory | The code identifying the laboratory that analyzes the sample. |
| labsampid | C12 | 7-18 | Yes | Yes | No | Yes | Laboratory Sample ID | The unique identification number assigned to the sample by the laboratory. |
| qccode | C3 | 19-21 | Yes | Yes | Yes | Yes | QC Type | The code identifying the type of sample (e.g., laboratory-generated, environmental, etc.). |
| anmcode | C7 | 22-28 | Yes | Yes | Yes | Yes | Analytical Method | The code identifying the method of analysis. |
| exmcode | C7 | 29-35 | Yes | Yes | Yes | Yes | Preparation Method | The code identifying the method of preparation. |
| pvccode | C2 | 36-37 | Yes | No | Yes | Yes | Primary Value Type | The code identifying whether a sample result is a primary or a confirmatory value. |
| anadate | D8 | 38-45 | Yes | Yes | No | Yes | Analysis Date | The date the sample (aliquot, extract, digest and/or leachate) is analyzed. |
| run_number | N2 | 46-47 | Yes | Yes | No | Yes | Run Number | The numeric code distinguishing multiple or repeat analysis of a sample by the same method on the same day. |
| parlabel | C12 | 48-59 | Yes | No | Yes | Yes | Parameter | The code or CAS number identifying the analyte (parameter). |
| parval | N14 | 60-73 | No | No | No | Yes | Parameter Value | The analytical value for a compound, analyte, or physical parameter. (Formerly in the format N14,4 in EDF 1.2a.) |
| parvq | C2 | 74-75 | No | No | Yes | Yes | Parameter Value Qualifier | The code identifying the qualifier of an analytical result (e.g., greater than, equal to, etc.). |
| labdl | N9 | 76-84 | No | No | No | No | Method Detection Limit | The laboratory-established method detection limit. (Formerly in the format N9,4 in EDF 1.2a.) |
| repdl | N9 | 85-93 | No | No | No | No | Reporting Detection Limit | The laboratory-established method detection limit, adjusted for the particular sample preparation (e.g., weight, volume, or dilution). (Formerly in the format N9,4 in EDF 1.2a.) |
| repdlvq | C3 | 94-96 | No | No | Yes | Yes | Reporting Detection Limit Qualifier | The code identifying the type of reporting limit (e.g., practical quantitation limit, instrument detection limit, etc.). |
| parun | N12 | 97-108 | No | No | No | No | Parameter Uncertainty | The uncertainty of a measured value due to a measuring technique (expressed as plus or minus some value). (Formerly in the format N12,4 in EDF 1.2a.) |
| units | C10 | 109-118 | No | No | Yes | Yes | Units of Measure | The units for the parameter value measurement. |
| rt | N7 | 119-125 | No | No | No | No | Retention Time | The retention time of a tentatively identified compound (TIC), reported in minutes (min). (Formerly in the format N7,2 in EDF 1.2a.) |
| dilfac | N10 | 126-135 | No | No | No | Yes | Dilution Factor | The numeric factor indicating the level of sample dilution. (Formerly in the format N10,3 in EDF 1.2a.) |
| clrevdate | D8 | 136-143 | No | No | No | No | Control Limit Revision Date | The date a control limit is established. |
| srm | C12 | 144-155 | No | No | Yes | Yes | Standard Reference Material | The code identifying the standard reference material used in the analysis. |
| lnote | C20 | 156-175 | No | No | Yes | No | Laboratory Result Notes | The code identifying notes pertaining to analytical performance irregularities that apply to a single analyte. |
| lab_meth_grp | C25 | 176-200 | No | Yes | No | No | Lab Method Group | The unique identifier for a group of methods as defined by the laboratory. |
| meth_design_id | C25 | 201-225 | No | Yes | No | No | Method Design ID | The unique identifier for the design of an analytical method. |

## EDFQC: The QC Information File

The quality assurance information in the QC file is associated with an analytical result contained in the RESULTS file. The QC records will contain information on blanks, spikes, duplicates, and standard reference materials. No calculated results are required for this file. All QC calculations are performed by the database receiving the electronic deliverable.

QC samples are entered into the QC file based upon the QC batch (LABLOTCTL) with which they are associated. The LABLOTCTL allows the environmental samples to be grouped with their QC samples in order to evaluate the quality of the analytical results. The LABLOTCTL is an arbitrary number assigned by the laboratory to represent a group of samples prepared together, sharing the same QC samples. Table 5, on page 26, presents the QC file structure and field attributes.

### File Guidelines and Restrictions:

- MATRIX, LABCODE, LABLOTCTL, ANMCODE, PARLABEL, QCCODE, and LABQCID comprise the primary key.

- All spiked or split samples, and all laboratory-generated QC samples must be entered into this file.

- All QC data from subcontracted laboratories must be entered into this file.

### Field Guidelines and Restrictions:

- MATRIX, LABCODE, QCCODE, ANMCODE, PARLABEL, and UNITS require valid value entries. Refer to the EDF Data Dictionary for lists of valid value codes.

- The valid value entered into the QCCODE field is the QCCODE of the LABQCID sample.

- The EXPECTED value is the expected result of the LABQCID sample (i.e., the EXPECTED field result for a matrix spike is the value of the spike plus the value of the original sample, LABREFID).

- EXPECTED should be blank for laboratory-generated blanks (i.e., QCCODE is “LB” or “RS”).

- LABREFID should be blank for laboratory-generated blanks, reference materials, calibration standards, and spiked blanks (i.e., QCCODE is “LB,” “RS,” “RM/KD,” “IC,” “CC,” or “BS/BD”).

- LABCODE reflects the laboratory that receives the sample, even if the sample has been subcontracted out.

- LAB_METH_GRP and METH_DESIGN_ID are optional fields.

Table 5: EDFQC (QC) Format

| Field Name | Attrb | Start-End | PK | FK | VVL | REQ | Dscr. Name | Definition |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| matrix | C2 | 1-2 | Yes | Yes | Yes | Yes | Matrix | The code identifying the sample matrix as determined by the laboratory (e.g., water, soil, etc.). |
| labcode | C4 | 3-6 | Yes | Yes | Yes | Yes | Laboratory | The code identifying the laboratory that analyzes the sample. |
| lablotctl | C10 | 7-16 | Yes | Yes | No | Yes | Preparation Batch Number | The unique identifier for a preparation and handling batch. |
| anmcode | C7 | 17-23 | Yes | Yes | Yes | Yes | Analytical Method | The code identifying the method of analysis. |
| parlabel | C12 | 24-35 | Yes | Yes | Yes | Yes | Parameter | The code or CAS number identifying the analyte (parameter). |
| qccode | C3 | 36-39 | Yes | Yes | Yes | Yes | QC Type | The code identifying the type of sample (e.g., laboratory-generated, environmental, etc.). |
| labqcid | C12 | 39-50 | Yes | No | No | Yes | Laboratory QC Sample ID | The unique identification number assigned to the sample by the laboratory. |
| labrefid | C12 | 51-62 | No | No | No | No | Laboratory Reference ID | The laboratory sample ID of the quality control reference sample. |
| expected | N14 | 63-76 | No | No | No | No | Expected Parameter Value | The target result for a quality control sample or surrogate spike. (Formerly in the format N14,4 in EDF 1.2a.) |
| units | C10 | 77-86 | No | No | Yes | Yes | Units of Measure | The units for the parameter value measurement. |
| lab_meth_grp | C25 | 87-111 | No | Yes | No | No | Lab Method Group | The unique identifier for a group of methods as defined by the laboratory. |
| meth_design_id | C25 | 112-136 | No | Yes | No | No | Method Design ID | The unique identifier for the design of an analytical method. |

## EDFCL: The Quality Control Limit Information File

This file contains control limit information concerning the QC results. The file does not have to be revised unless new control charts are generated. However, for tracking purposes, it must be submitted with each digital deliverable. Table 6, on page 28, presents the CL file structure and field attributes.

### File Guidelines and Restrictions:

- MATRIX, LABCODE, ANMCODE, EXMCODE, PARLABEL, CLCODE, and CLREVDATE comprise the primary key.

- All results with associated CL criteria must have an associated entry in this file.

- When control limit entry is required, both accuracy and precision limits must be entered, except in the case of calibrations and lab replicates (i.e., QCCODE is “IC,” “CC,” or “LR”), which require only precision limits.

### Field Guidelines and Restrictions:

- MATRIX, LABCODE, CLCODE, ANMCODE, EXMCODE, and PARLABEL require valid value entries. Refer to the EDF Data Dictionary for lists of valid value codes.

- Use UPPERCL for relative percent difference (RPD) and upper accuracy recovery limit entries.

- LOWERCL should be zero for RPD (i.e., precision) entries.

- The LABCODE field reflects the laboratory that performed the analysis (i.e., if a subcontracted laboratory performed the analysis, the LABCODE would be the valid value for the subcontracted laboratory [SUB]).

- LAB_METH_GRP and METH_DESIGN_ID are optional fields.

Table 6: EDFCL (CL) Format

| Field Name | Attrb | Start-End | PK | FK | VVL | REQ | Dscr. Name | Definition |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| labcode | C4 | 1-4 | Yes | Yes | Yes | Yes | Laboratory | The code identifying the laboratory that analyzes the sample. |
| matrix | C2 | 5-6 | Yes | Yes | Yes | Yes | Matrix | The code identifying the sample matrix as determined by the laboratory (e.g., water, soil, etc.). |
| anmcode | C7 | 7-13 | Yes | Yes | Yes | Yes | Analytical Method | The code identifying the method of analysis. |
| exmcode | C7 | 14-20 | Yes | Yes | Yes | Yes | Preparation Method | The code identifying the method of preparation. |
| parlabel | C12 | 21-32 | Yes | Yes | Yes | Yes | Parameter | The code or CAS number identifying the analyte (parameter). |
| clrevdate | D8 | 33-40 | Yes | Yes | No | Yes | Control Limit Revision Date | The date a control limit is established. |
| clcode | C6 | 41-46 | Yes | No | Yes | Yes | Control Limit Type | The code identifying the type of quality control limit. |
| uppercl | N4 | 47-50 | No | No | No | Yes | Upper Control Limit | The upper control limit of a quality control criterion. |
| lowercl | N4 | 51-54 | No | No | No | No | Lower Control Limit | The lower control limit of a quality control criterion. |
| lab_meth_grp | C25 | 55-79 | No | Yes | No | No | Lab Method Group | The unique identifier for a group of methods as defined by the laboratory. |
| meth_design_id | C25 | 80-104 | No | Yes | No | No | Method Design ID | The unique identifier for the design of an analytical method. |

## EDFNARR: The Narrative File

The NARRATIVE file provides a means to transfer descriptive information about analyses that do not easily fit in a standardized format. This file does not require a specific format but should be delivered as an ASCII file.

It is recommended that a header record be included, containing the following information in comma/quote delimited format:

- Laboratory Report Number (LAB_REPNO)

- Laboratory (LABCODE)

- Laboratory Report Date (REP_DATE)

- EDD Version Number (EDD_VERSION) (e.g., EDF 1.2b)

An example NARRATIVE file might look like the following:

----------------------------------------------------------------------------------------------------------------

“LABREPORT#001”,“LAB1”, “01/11/2001”, “12B”

The following issues were encountered…

Signed By:

Title:

Date:

----------------------------------------------------------------------------------------------------------------

# Flat File Format

The following Chapter describes the flat file format of EDF, which includes one large file of data results (EDFFLAT) that links to the CL file described in Section 3.5 and Table 6.

## EDFFLAT: The Flat File

This file contains all of the data fields from the SAMPLE, TEST, RESULTS, and QC files of the relational format in one large “flat” file. This flat file links to the CL file through the same key fields with which the RESULTS file links to the CL file. The flat file may be in the fixed length, Excel *.xls, or CSV delimited formats as discussed in Chapter 3. For details on the CL file, please refer to Section 3.5.

Table 7: EDFFLAT Format

| Field Name | Attrb | Start-End | PK | FK | VVL | REQ | Dscr. Name | Definition |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| locid | C10 | 1-10 | No | No | No | No | Location ID | The unique identifier for the sample's location, as identified by the laboratory. |
| logdate | D8 | 11-18 | Yes | No | No | Yes | Collection Date | The date a field sample is collected. |
| logtime | C4 | 19-22 | Yes | No | No | Yes | Collection Time | The time that a field sample is collected, recorded using 24-hour military time. |
| logcode | C4 | 23-26 | Yes | No | Yes | Yes | Field Organization | The code identifying the company collecting the samples or performing field tests. |
| sampid | C25 | 27-51 | Yes | No | No | Yes | COC Sample ID | The unique identifier representing a sample, assigned by the consultant, as submitted to the laboratory on a chain-of-custody. |
| matrix | C2 | 52-53 | Yes | No | Yes | Yes | Matrix | The code identifying the sample matrix as determined by the laboratory (e.g., water, soil, etc.). |
| projname | C25 | 54-78 | No | No | No | Yes | Project Name | The identification assigned to the project by the organization performing the work. |
| npdlwo | C7 | 79-85 | No | No | No | Yes | Work Order Number | A delivery order number associated with the contract. |
| cntshnum | C12 | 86-97 | No | No | No | Yes | Control Sheet Number | The administratively-assigned identification used to track contracts. |
| labcode | C4 | 98-101 | Yes | No | Yes | Yes | Laboratory | The code identifying the laboratory that analyzes the sample. |
| labsampid | C12 | 102-113 | Yes | No | No | Yes | Laboratory Sample ID | The unique identification number assigned to the sample by the laboratory. |
| qccode | C3 | 114-116 | Yes | No | Yes | Yes | QC Type | The code identifying the type of sample (e.g., laboratory-generated, environmental, etc.). |
| anmcode | C7 | 117-123 | Yes | No | Yes | Yes | Analytical Method | The code identifying the method of analysis. |
| modparlist | L1 | 124-124 | No | No | No | Yes | Modified Parameter List | A field indicating whether the parameter list of an analytical method has been modified. |
| exmcode | C7 | 125-131 | Yes | No | Yes | Yes | Preparation Method | The code identifying the method of preparation. |
| lablotctl | C10 | 132-141 | Yes | No | No | Yes | Preparation Batch Number | The unique identifier for a preparation and handling batch. |
| exlablot | C10 | 142-151 | No | No | No | No | OBSOLETE | OBSOLETE |
| anadate | D8 | 152-159 | Yes | No | No | Yes | Analysis Date | The date the sample (aliquot, extract, digest and/or leachate) is analyzed. |
| extdate | D8 | 160-167 | Yes | No | No | Yes | Preparation Date | The date that a sample is prepared for analysis. |
| run_number | N2 | 168-169 | Yes | No | No | Yes | Run Number | The numeric code distinguishing multiple or repeat analysis of a sample by the same method on the same day. |
| recdate | D8 | 170-177 | No | No | No | Yes | Received Date | The date the sample is received by the laboratory doing the analysis. |
| cocnum | C16 | 178-193 | No | No | No | No | Chain-of-Custody Number | The number assigned to the chain-of-custody. |
| basis | C1 | 194-194 | No | No | Yes | Yes | Basis | The code used to distinguish whether a sample is reported as dry or wet weight, filtered or not filtered. |
| prescode | C15 | 195-209 | No | No | Yes | No | Preservative | The code identifying the type of preservative added to the sample. |
| sub | C4 | 210-213 | No | No | Yes | Yes | Subcontracted Laboratory | The code identifying the subcontracted laboratory. |
| rep_date | D8 | 214-221 | No | No | No | No | Report Date | The date of the laboratory report. |
| lab_repno | C20 | 222-241 | No | No | No | No | Laboratory Report Number | The unique identifier for the laboratory report, assigned by the laboratory. |
| apprvd | C3 | 242-244 | No | No | No | No | Approved By | The initials of the individual approving the laboratory report. |
| Tlnote | C20 | 245-264 | No | No | Yes | No | Laboratory Test Notes | The code identifying notes pertaining to analytical performance irregularities that apply to the entire test. |
| pvccode | C2 | 265-266 | Yes | Yes | Yes | Yes | Primary Value Type | The code identifying whether a sample result is a primary or a confirmatory value. |
| parlabel | C12 | 267-278 | Yes | No | Yes | Yes | Parameter | The code or CAS number identifying the analyte (parameter). |
| parval | N14 | 279-292 | No | No | No | Yes | Parameter Value | The analytical value for a compound, analyte, or physical parameter. (Formerly in the format N14,4 in EDF 1.2a.) |
| parvq | C2 | 293-294 | No | No | Yes | Yes | Parameter Value Qualifier | The code identifying the qualifier of an analytical result (e.g., greater than, equal to, etc.). |
| labdl | N9 | 295-303 | No | No | No | No | Method Detection Limit | The laboratory-established method detection limit. (Formerly in the format N9,4 in EDF 1.2a.) |
| repdl | N9 | 304-312 | No | No | No | No | Reporting Detection Limit | The laboratory-established method detection limit, adjusted for the particular sample preparation (e.g., weight, volume, or dilution). (Formerly in the format N9,4 in EDF 1.2a.) |
| repdlvq | C3 | 313-315 | No | No | Yes | Yes | Reporting Detection Limit Qualifier | The code identifying the type of reporting limit (e.g., practical quantitation limit, instrument detection limit, etc.). |
| parun | N12 | 316-327 | No | No | No | No | Parameter Uncertainty | The uncertainty of a measured value due to a measuring technique (expressed as plus or minus some value). (Formerly in the format N12,4 in EDF 1.2a.) |
| units | C10 | 328-337 | No | No | Yes | Yes | Units of Measure | The units for the parameter value measurement. |
| rt | N7 | 338-344 | No | No | No | No | Retention Time | The retention time of a tentatively identified compound (TIC), reported in minutes (min). (Formerly in the format N7,2 in EDF 1.2a.) |
| dilfac | N10 | 345-354 | No | No | No | Yes | Dilution Factor | The numeric factor indicating the level of sample dilution. (Formerly in the format N10,3 in EDF 1.2a.) |
| clrevdate | D8 | 355-362 | No | No | No | No | Control Limit Revision Date | The date a control limit is established. |
| srm | C12 | 363-374 | No | No | Yes | Yes | Standard Reference Material | The code identifying the standard reference material used in the analysis. |
| labrefid | C12 | 375-386 | No | No | No | No | Laboratory Reference ID | The laboratory sample ID of the quality control reference sample. |
| expected | N14 | 387-400 | No | No | No | No | Expected Parameter Value | The target result for a quality control sample or surrogate spike. (Formerly in the format N14,4 in EDF 1.2a.) |
| Rlnote | C20 | 401-420 | No | No | Yes | No | Laboratory Result Notes | The code identifying notes pertaining to analytical performance irregularities that apply to a single analyte. |
| req_method_grp | C25 | 421-445 | Yes | No | No | No | Requested Method Group | The unique identifier for the method or group of methods requested by the client for analysis of the sample. |
| coc_matrix | C2 | 446-447 | Yes | No | Yes | No | COC Matrix | The code identifying the sample matrix as noted on the chain-of-custody (e.g., water, soil, etc.). |
| dqo_id | C25 | 448-472 | Yes | No | No | No | Data Quality Objectives ID | The unique identifier representing the data quality objectives. |
| meth_design_id | C25 | 473-497 | Yes | No | No | No | Method Design ID | The unique identifier for the design of an analytical method. |
| lab_meth_grp | C25 | 498-522 | Yes | No | No | No | Lab Method Group | The unique identifier for a group of methods as defined by the laboratory. |

# File, Record, and Data Field Requirements

File, record, and data field requirements identified below must be strictly followed in order to generate acceptable EDDs.

## File and Record Requirements

An EDD may be submitted as an ASCII fixed length *.txt file, as a Microsoft Excel tab delimited *.xls file, or as a comma separated value (CSV) delimited ASCII *.txt file (also known as “comma/quote delimited”).

Each line of data is equivalent to a single record in the data submission. Each record is made up of distinct fields of information. A record must not be dependent on another record or field for data (i.e., each data record must be autonomous of other data records). Valid data must be entered in each record. Listed below are the ASCII file and record specifications for entering each record of data in its specified file.

- Do not enter the column heading or field name in the ASCII file. This information is not part of the file. Only authorized codes from the valid value list should be keyed into fields requiring valid values.

- Do not create left margins. In each file, every record starts in the farthest left position of “position number 1.” If entering the data via a spreadsheet, set the left margin at zero and the right margin at the end position of the last field of the record. The first record or row in the file, and every subsequent record or row must contain valid data. Blank or empty rows (lines) or records are not allowed in ASCII files.

- Every record within a file must be unique. If, for each key field, a record's data appears exactly the same in another record, these two records are considered to be duplicate records. Do not enter data that refers to another record.

## Data Field Requirements

When producing the fixed or tab delimited formats, data formats (attributes) must be strictly followed. Valid data must always be entered for every field. Do not add, delete, or otherwise omit any field in any format.

In the fixed length format, data fields in a file are limited to a certain number of spaces and the data must be in a specific position. Character data must be left justified within a field. Numeric data must be right justified within a field. If the information to be entered is shorter than the field width, insert blank spaces in the field's remaining positions. If the data to be entered is longer than the allowed field width, the data must be shortened to a unique identifier or significant value.

The start- and end-position numbers indicate the exact character locations where the applicable data must be placed in the file. There are some cases where the field is a single character wide. It, therefore, will have the same start- and end-position number. The single character of data must be put in that position of the record.

For the CSV delimited format, field length is still important in that data cannot exceed the length of the field, but blank spaces do not need to be entered when a value is shorter than the field’s length. For example, when entering a LABSAMPID, which is a C12 field, if the value to be entered is only C5, in the CSV delimited format it would look like:

“12345”,“next field entry”

In the fixed length format, it would look like:

12345…….next field entry

(where the dots represents 7 blank spaces before the next field).

## Diskette Submittal

Data disks are submitted on a per laboratory report basis. Hence, as a laboratory report is completed and converted into the EDF, it then must be processed for submittal. The submittal process is outlined below:

- Copy files onto an MS-DOS formatted disk or CD.

- Check the consistency of the file formats by loading them into the Electronic Deliverable Consistency Checker (EDCC). The EDCC is a stand-alone software program that checks each data submission for the proper EDF format, warns the user of potential formatting problems, and reports the results of the consistency check.

- An EDF EDD that does not pass the EDCC will not be accepted.

- Each of the five files and the NARRATIVE file of the relational format must be named exactly as specified in this document (i.e., edfsamp.txt, edftest.txt, edfres.txt, edfqc.txt, edfcl.txt, and edfnarr.txt). The files of the flat file format must be named edfflat.txt and edfcl.txt.

- Try to place all five files associated with one laboratory report on a single diskette. If the files are too large, compress the files with some version of Winzip® and attempt to place the compressed file onto one diskette.

- Note, compressed files  must be delivered with a “*.zip” file extension and given the name of the LAB_REPNO as convention (e.g., “mylabreport.zip”).

- Use multiple diskettes only if the compressed file will not fit on a single diskette.

- Each diskette must be externally labeled with the laboratory name, date, the Report Number,  and the names of the files supplied on that specific diskette.

- Write-protect all disks before submittal.

- Provide a hard copy of the laboratory report printed directly from the electronic data.

- Include an EDCC Error Report with each submittal.

# Summary of Data Elements

| Field Name | In Table(s) | Attrb | Null Allowed | VVL | Descr. Name | Definition | Guidelines & Restrictions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| anadate | TEST RESULTS | D8 |  |  | Analysis Date | The date the sample (aliquot, extract, digest and/or leachate) is analyzed. | Must be in the format YYYYMMDD. Must be later than or equal to EXTDATE, RECDATE, LOGDATE, and earlier than or equal to REP_DATE. |
| anmcode | TEST RESULTS QC CL | C7 |  | x | Analytical Method | The code identifying the method of analysis. | Must contain a valid value. |
| apprvd | TEST | C3 | x |  | Approved By | The initials of the individual approving the laboratory report. | No entry for laboratory-generated QC and non-client samples. |
| basis | TEST | C1 |  | x | Basis | The code used to distinguish whether a sample is reported as dry or wet weight, filtered or not filtered. | Must contain a valid value. Valid values for soil samples are "W" or "D" or leachate codes; for water samples "F," "L," or "N." |
| clcode | CL | C6 |  | x | Control Limit Type | The code identifying the type of quality control limit. | Must contain a valid value. |
| clrevdate | RESULTS CL | D8 | x |  | Control Limit Revision Date | The date a control limit is established. | Must be in the format YYYYMMDD. No entry when QCCODE is "CS," "NC," "LB," or "RS," and non-spiked parameters (except when PARVQ is "SU" or "IN"). |
| cntshnum | SAMPLE | C12 |  |  | Control Sheet Number | The administratively-assigned identification used to track contracts. | Entry of "NA" is acceptable. |
| coc_matrix | SAMPLE | C2 | x | x | COC Matrix | The code identifying the sample matrix as noted on the chain-of-custody (e.g., water, soil, etc.). | Optional. This field provides a link with the COC EDD from EDMS2000. It represents the sample matrix as identified by the field organization, and must contain a valid value. |
| cocnum | SAMPLE | C16 | x |  | Chain-of-Custody Number | The number assigned to the chain-of-custody. | No entry for laboratory-generated QC and non-client samples. |
| dilfac | RESULTS | N10 |  |  | Dilution Factor | The numeric factor indicating the level of sample dilution. | Must be greater than zero. (Formerly in the format N10,3 in EDF 1.2a.) |
| dqo_id | SAMPLE | C25 | x |  | Data Quality Objectives ID | The unique identifier representing the data quality objectives. | Optional. This field provides a link with the COC EDD from EDMS2000. |
| exlablot | TEST | C10 | x |  | OBSOLETE | OBSOLETE | OBSOLETE |
| exmcode | TEST RESULTS CL | C7 |  | x | Preparation Method | The code identifying the method of preparation. | Must contain a valid value. If no preparation performed enter "NONE;" if preparation method is included in analysis method enter "METHOD." |
| expected | QC | N14 | x |  | Expected Parameter Value | The target result for a quality control sample or surrogate spike. | No entry when QCCODE is "CS," "NC," "LB," or "RS." For matrix spikes, this value is the amount spiked plus the reference sample PARVAL. Enter "100" when UNITS are "PERCENT." (Formerly in the format N14,4 in EDF 1.2a.) |
| extdate | TEST RESULTS | D8 |  |  | Preparation Date | The date that a sample is prepared for analysis. | Must be in the format YYYYMMDD. If no preparation performed, enter ANADATE. |
| lab_meth_grp | TEST RESULTS QC CL | C25 | x |  | Lab Method Group | The unique identifier for a group of methods as defined by the laboratory. | Optional. This field provides a link with the EDMS2000. |
| lab_repno | TEST | C20 | x |  | Laboratory Report Number | The unique identifier for the laboratory report, assigned by the laboratory. | No entry for laboratory-generated QC and non-client samples. |
| labcode | SAMPLE TEST RESULTS QC CL | C4 |  | x | Laboratory | The code identifying the laboratory that analyzes the sample. | This field represents the laboratory that received the sample and is responsible for producing the electronic deliverable, and must contain a valid value. |
| labdl | RESULTS | N9 |  |  | Method Detection Limit | The laboratory-established method detection limit. | Enter zero when UNITS is "PERCENT" or PARVQ is "TI." Must be adjusted for dilution. Must be greater than or equal to zero. (Formerly in the format N9,4 in EDF 1.2a.) |
| lablotctl | TEST QC | C10 |  |  | Preparation Batch Number | The unique identifier for a preparation and handling batch. | Must uniquely define a group of samples prepared together. |
| labqcid | QC | C12 |  |  | Laboratory QC Sample ID | The unique identification number assigned to the sample by the laboratory. | This is equivalent to the LABSAMPID. |
| labrefid | QC | C12 | x |  | Laboratory Reference ID | The laboratory sample ID of the quality control reference sample. | This is the LABSAMPID of the reference sample. No entry unless QCCODE is "MS/SD" or "LR." |
| labsampid | TEST RESULTS | C12 |  |  | Laboratory Sample ID | The unique identification number assigned to the sample by the laboratory. | Must be unique. |
| locid | SAMPLE TEST | C10 | x |  | Location ID | The unique identifier for the sample's location, as identified by the laboratory. | No entry for laboratory-generated QC and non-client samples. |
| logcode | SAMPLE TEST | C4 | x | x | Field Organization | The code identifying the company collecting the samples or performing field tests. | Must contain a valid value. No entry for laboratory-generated QC and non-client samples. |
| logdate | SAMPLE TEST | D8 | x |  | Collection Date | The date a field sample is collected. | Must be in the format YYYYMMDD. No entry for laboratory-generated QC and non-client samples. Must be earlier than RECDATE, EXTDATE, ANADATE, and REP_DATE. |
| logtime | SAMPLE TEST | C4 | x |  | Collection Time | The time that a field sample is collected, recorded using 24-hour military time. | Must be a valid time between 0000 and 2359. No entry for laboratory-generated QC and non-client samples. |
| lowercl | CL | N4 |  |  | Lower Control Limit | The lower control limit of a quality control criterion. | Must be an integer greater than or equal to zero and less than UPPERCL. Enter zero for precision limit. |
| matrix | SAMPLE TEST RESULTS QC CL | C2 |  | x | Matrix | The code identifying the sample matrix as determined by the laboratory (e.g., water, soil, etc.). | This field represents the sample matrix as identified by the laboratory, and must contain a valid value. |
| meth_design_id | SAMPLE TEST RESULTS QC CL | C25 | x |  | Method Design ID | The unique identifier for the design of an analytical method. | Optional. This field provides a link with the COC EDD from EDMS2000. |
| modparlist | TEST | L1 |  |  | Modified Parameter List | A field indicating whether the parameter list of an analytical method has been modified. | Must enter "T" (true) or "F" (false) if a parameter from the method parameter list is not reported. The parameter list is not considered modified if extra parameters are reported. |
| npdlwo | SAMPLE | C7 |  |  | NPDL Work Order Number | A delivery order number associated with the contract. | Entry of "NA" is acceptable. |
| parlabel | RESULTS QC CL | C12 |  | x | Parameter | The code or CAS number identifying the analyte (parameter). | Must contain a valid value. |
| parun | RESULTS | N12 | x |  | Parameter Uncertainty | The uncertainty of a measured value due to a measuring technique (expressed as plus or minus some value). | No entry necessary for non-radiochemical results. If entered, must be greater than or equal to zero. (Formerly in the format N12,4 in EDF 1.2a.) |
| parval | RESULTS | N14 |  |  | Parameter Value | The analytical value for a compound, analyte, or physical parameter. | (Formerly in the format N14,4 in EDF 1.2a.) |
| parvq | RESULTS | C2 |  | x | Parameter Value Qualifier | The code identifying the qualifier of an analytical result (e.g., greater than, equal to, etc.). | Must contain a valid value. |
| prescode | TEST | C15 | x | x | Preservative | The code identifying the type of preservative added to the sample. | Must contain a valid value. Multiple codes may be entered, separated by commas (no spaces between values). |
| projname | SAMPLE | C25 | x |  | Project Name | The identification assigned to the project by the organization performing the work. | No entry for laboratory-generated QC and non-client samples. |
| pvccode | RESULTS | C2 |  | x | Primary Value Type | The code identifying whether a sample result is a primary or a confirmatory value. | Must contain a valid value. There may be only one "PR" result per LABSAMPID, ANMCODE, EXMCODE, and PARLABEL. |
| qccode | TEST RESULTS QC | C3 |  | x | QC Type | The code identifying the type of sample (e.g., laboratory-generated, environmental, etc.). | Must contain a valid value. |
| recdate | TEST | D8 |  |  | Received Date | The date the sample is received by the laboratory doing the analysis. | Must be in the format YYYYMMDD. For laboratory-generated QC samples enter date sample was created (e.g., EXTDATE). |
| rep_date | TEST | D8 | x |  | Report Date | The date of the laboratory report. | Must be in the format YYYYMMDD. No entry for laboratory-generated QC and non-client samples. |
| repdl | RESULTS | N9 |  |  | Reporting Detection Limit | The laboratory-established method detection limit, adjusted for the particular sample preparation (e.g., weight, volume, or dilution). | Enter zero when UNITS is "PERCENT" or PARVQ is "TI." Must be adjusted for dilution. Must be greater than or equal to zero. (Formerly in the format N9,4 in EDF 1.2a.) |
| repdlvq | RESULTS | C3 |  | x | Reporting Detection Limit Qualifier | The code identifying the type of reporting limit (e.g., practical quantitation limit, instrument detection limit, etc.). | Must contain a valid value. Enter "NA" when UNITS is "PERCENT" or PARVQ is "TI." |
| req_method_grp | SAMPLE | C25 | x |  | Requested Method Group | The unique identifier for the method or group of methods requested by the client for analysis of the sample. | Optional. This field provides a link with the COC EDD from EDMS2000. |
| rlnote | RESULTS | C20 | x | x | Laboratory Result Notes | The code identifying notes pertaining to analytical performance irregularities that apply to a single analyte. | Must contain a valid value. Multiple codes may be entered, separated by commas (no spaces between values). |
| rt | RESULTS | N7 | x |  | Retention Time | The retention time of a tentatively identified compound (TIC), reported in minutes (min). | No entry necessary except when PARVQ is "TI." If entered must be greater than or equal to zero. (Formerly in the format N7,2 in EDF 1.2a.) |
| run_number | TEST RESULTS | N2 |  |  | Run Number | The numeric code distinguishing multiple or repeat analysis of a sample by the same method on the same day. | Must be an integer greater than or equal to 1. |
| sampid | SAMPLE TEST | C25 | x |  | COC Sample ID | The unique identifier representing a sample, assigned by the consultant, as submitted to the laboratory on a chain-of-custody. | This field represents the sample ID as it appears on the COC. No entry for laboratory-generated QC and non-client samples. |
| srm | RESULTS | C12 |  | x | Standard Reference Material | The code identifying the standard reference material used in the analysis. | Must contain a valid value. Enter "NA" if no reference material. |
| sub | TEST | C4 |  | x | Subcontracted Laboratory | The code identifying the subcontracted laboratory. | Must contain a valid value. Enter "NA" if no analyses are subcontracted. |
| tlnote | TEST | C20 | x | x | Laboratory Test Notes | The code identifying notes pertaining to analytical performance irregularities that apply to the entire test. | Must contain a valid value. Multiple codes may be entered, separated by commas (no spaces between values). |
| units | RESULTS QC | C10 |  | x | Units of Measure | The units for the parameter value measurement. | Must contain a valid value. |
| uppercl | CL | N4 |  |  | Upper Control Limit | The upper control limit of a quality control criterion. | Must be an integer greater than or equal to one and greater than LOWERCL. |

# Glossary of Terms

ANADATE (Analysis Date) - The date a sample or extract is analyzed. The date format for this field is YYYYMMDD. (D8)

ANMCODE (Analytical Method) - The code identifying the method of analysis by which the sample was analyzed. Refer to the EDF Data Dictionary for a list of valid values. (C7)

APPRVD (Approved By) - Initials of the individual approving the laboratory report. (C3)

Attributes - The format and size attributes of a database field. A field type of C8 is a field that can hold up to eight alphanumeric characters. An N5 field type has a total of 5 spaces available for numbers and decimals, with no restriction on the number of digits to the right of the decimal point other than the overall field size (e.g., 12345 or 123.4 or 1.234). A D8 field type is a date field, usually formatted as YYYYMMDD ([year][month][day]). An L1 field type is a logic field with expected values of T (true) or F (false).

BASIS (Basis) - Identifies the basis (W = wet, D = dry, F = field filtered, L = lab filtered, or N = not filtered) on which analytical results are reported for all matrices. This field is also used to indicate leaching procedures. Refer to the EDF Data Dictionary for a list of valid values. (C1)

Blank Spike - A laboratory-generated quality control sample with a known amount of spiked compound, prepared using the same glassware, reagents, solvents, etc., as the associated environmental samples. Blank spikes are used to monitor the laboratory’s method accuracy (i.e., how close their result is to a known true value).

CLCODE (Control Limit Type) - The code identifying the type of quality control limits. Clcodes are assigned based upon the type of quality assurance sample as well as the source of validation criteria. Refer to the EDF Data Dictionary for a list of valid values. (C6)

CLREVDATE (Control Limit Revision Date) - The date that the control limit is established. The format of this field is YYYYMMDD. (D8)

CNTSHNUM (Control Sheet Number) - The administratively-assigned identification used to track contracts.  (C12)

COC_MATRIX (Chain-of-Custody Matrix) - The code identifying the sample matrix as noted on the chain-of-custody (e.g., water, soil, etc.). This field links to the chain-of-custody tables in the EDMS2000. Refer to the EDF Data Dictionary for a list of valid values. (C2)

COC (Chain-of-Custody) - A form used to track sample custody from sample collection to receipt by the laboratory. Also includes request for analyses and other instructions to the laboratory. The COC is included in the container used to transport samples from the field to the laboratory.

COCNUM (Chain-of-Custody Number) - The number assigned to the chain-of-custody. (C16)

COELT (U.S. Army Corps of Engineers Loading Tool) - A software tool designed for data entry, data export, data verification, and data reporting, used by analytical laboratories to generate EDF deliverables. The current version is 1.2a, and is available to anyone, free of charge.

Database - A collection of information arranged into records (rows) and fields (columns) for ease of sorting and manipulation within a table or related tables.

Deliverable - A report, data, etc., that is “delivered” to another party, either electronically, or in hard copy format.

DILFAC (Dilution Factor) - Numeric factor indicating level of sample dilution. (N10) (Formerly in the format N10,3 in EDF 1.2a.)

DQO_ID (Data Quality Objectives ID) - The unique identifier representing the data quality objectives. This field links to the chain-of-custody tables in the EDMS2000. (C25)

EDCC (Electronic Deliverable Consistency Check) - A software tool designed to verify EDF (Lab EDD) deliverables for compliance to the EDF guidelines and restrictions (refer to the Electronic Deliverable Format, Version 1.2a, May 1997 document for details on the EDF structure). The current version is 1.2a, and is available to anyone, free of charge.

EDD (Electronic Data Deliverable) - Information stored in a defined format, accessible via a computer (e.g., stored on diskette, internal hard drive, CD ROM, magnetic tape, etc.).

EDF (Electronic Deliverable Format) - An analytical laboratory data format consisting of 5 related text files in ASCII format. The current version is 1.2b.  EDF deliverables can be generated using the COELT software, or other database software.

EDMS2000 (enABL Data Management System, Version 2000) - A comprehensive Web-accessible environmental data management system developed by ArsenaultLegg, Inc., driven by the EDMS2000 database.

EXLABLOT (Extraction QC Lot Number) - An obsolete field into which spaces need to be entered. (C10)

EXMCODE (Preparation Method) - A code showing the method that was used to extract or prepare a sample for analysis. Refer to the EDF Data Dictionary for a list of valid values. (C7)

EXTDATE (Preparation Date) - The date a sample is extracted or prepared for analysis. The format of this field is YYYYMMDD. (D8)

EXPECTED - (Expected Parameter Value) - The target result for a quality control sample. Samples that are reported in units of percent have expected values of 100. (N14) (Formerly in the format N14,4 in EDF 1.2a.)

Field - An area of a table (a column) that contains a particular piece of information. One or more fields make a record.  Fields are defined by the attributes of format and size. Refer to Figure 6.

File - A named group of electronic data in a defined format.

Foreign Key - Primary key field of a parent table shared with a child table in a data table relationship.

Guidelines and Restrictions - Information provided to the user regarding data entry, data performance, and data delivery expectations.

Hard Copy Report - The laboratory’s written, signed report of analytical results for a group of samples in a project.

LABCODE (Laboratory) - A code identifying the analytical laboratory. Refer to the EDF Data Dictionary for a list of valid values. (C4)

LABDL (Method Detection Limit) - The laboratory-established method detection limit (i.e., the minimum detectable concentration of an analyte that can be measured and reported with 99% confidence that the analyte concentration is different from a blank for a given matrix). This limit must be adjusted for dilution. The LABDL field may or may not contain the same value as the REPDL field, depending on the reporting format of the individual laboratory. Regardless, the laboratory must enter a value into LABDL unless the parameter is a tentatively identified compound, or has units of PERCENT. (N9) (Formerly in the format N9,4 in EDF 1.2a.)

LABLOTCTL (Preparation Batch Number) - A unique number identifying an autonomous batch or group of environmental samples prepared together, and sharing the same quality control within the same time period. This group is equivalent to the EPA SW-846 concept of a “Quality Assurance Batch.” (C10)

LAB_METH_GRP (Lab Method Group) - The unique identifier for a group of methods as defined by the laboratory. This field links to the chain-of-custody tables in the EDMS2000. (C25)

LABQCID (Laboratory QC Sample Identification) - The laboratory-assigned QC sample ID number. All quality assurance samples are entered into this field, including laboratory-generated samples (blanks and laboratory control samples), as well as environmental samples that have been altered by the laboratory (matrix spike). This field requires unique laboratory-assigned sample identifiers. (C12)

LABREFID (Laboratory Reference Sample Identification) - The reference sample is the sample upon which the quality control sample is referenced in order to calculate the quality assurance result. A reference sample is used in conjunction with a quality control sample (LABQCID) to determine precision and accuracy. (C12)

LAB_REPNO (Laboratory Report Number) - Laboratory-assigned number uniquely identifying the hard copy report. (C20)

LABSAMPID (Laboratory Sample Identification) - The unique identification number assigned to a sample by the laboratory doing the testing. (C12)

LNOTE (Laboratory Notes) - These are data qualifiers describing various observations and difficulties with the analysis associated with a test or analyte. Multiple data qualifiers may be entered into this field separated by commas without spaces. For laboratory data without qualifiers, spaces may be entered into this field. Refer to the EDF Data Dictionary for a list of valid values. (C20)

Location - A permanent, unique identifier assigned to the physical spot from where a field sample is collected, or measurements are taken for a project.

LOCID (Location Identification) - This is a unique identifier assigned to a specific point (location) where measurements or samples are taken. (C10)

LOGCODE (Field Organization) - A code identifying the company responsible for the collection of samples or the performing of field tests (environmental sampling information). Refer to the EDF Data Dictionary for a list of valid values. (C4)

LOGDATE (Collection Date) - This is the date that a sample is collected. The format of this field is YYYYMMDD. (D8)

LOGTIME (Collection Time) - The time that an environmental sample is collected. The format of this field is a 24-hour military clock HHMM. (C4)

LOWERCL (Lower Control Limit) - The lower limit of a quality control acceptance criterion. Enter spaces into this field for relative percent difference. (N4)

MATRIX (Matrix) - A code identifying a sample's medium or makeup (e.g., soil, water, air, etc.). Refer to the EDF Data Dictionary for a list of valid values. (C2)

Matrix Spike - A laboratory-generated quality control sample made up of the same matrix as the environmental sample, plus a known quantity of a known substance (spike). Matrix spikes are used to assess matrix interference effects on method accuracy.

METH_DESIGN_ID (Method Design ID) - The unique identifier for the design of an analytical method. This field links to the chain-of-custody tables in the EDMS2000. (C25)

MODPARLIST (Modified Parameter List) - A field indicating whether the compound list of a method has been amended. If the parameter list is modified, enter T (true) into this field. A modification indicates the deletion of compounds analyzed within a method, as listed in SW-846. (L1)

NPDLWO (Work Order Number) - A delivery order number associated with the contract. (C7)

Parent-to-Child Records - In a relational database, the relationships between tables can be one-to-many (i.e., one record in the first table is related to many records in the second table), or one-to-one (i.e., one record in the first table relates to one record in the second table).

In a one-to-many table, the table on the “one” end is called the  parent table, and the table on the “many” end is called the child table. A parent may have many child tables, but each child table has only one parent table. This relationship is called a one-to-many, or parent-to-child, relationship, as shown in Figure 4.

A&A, Inc., 1997

Figure 4: One-to-Many Parent-Child Table Relationship

A parent table also contains parent records that relate to many child records. Therefore, many child records within one child table will have one parent record in the parent table. For example, one well location, MW-01, may relate to many samples taken at that location, as indicated in Figure 5. Parent records may also have only one child record, or a one-to-one relationship.

A&A, Inc., 1997

Figure 5: One Parent Record to Many Child Records

PARLABEL (Parameter) - The parameter label is the code assigned to a measurement parameter. The code is generally a common acronym representing the parameter or analyte. The PARLABEL is used in the database instead of the full analyte name to reduce the error inherent in transferring large names with numbers, commas, and spaces. Refer to the EDF Data Dictionary for a list of valid values. (C12)

PARUN (Parameter Uncertainty) - The analytical uncertainty associated with a laboratory result. This field is present only for radiochemical results. For all other analytes enter a zero. (N12) (Formerly in the format N12,4 in EDF 1.2a.)

PARVAL (Parameter Value) - This field represents the actual analytical value for a compound or analyte. It is the result generated after a sample has been analyzed or a test performed. For parameter results not calculated due to multiple runs, or if the analyte is below the LABDL, enter a zero into this field. (N14) (Formerly in the format N14,4 in EDF 1.2a.)

PARVQ (Parameter Value Qualifier) - A code qualifying the analytical result. The parameter value qualifier is designed to describe to what the analytical value is equivalent, (i.e., not detected, equals to, or not reported). These codes also identify TICs and surrogates. Refer to the EDF Data Dictionary for a list of valid values. (C2)

PRESCODE (Preservative Added) - This is the code identifying the type of chemical preservative added to the sample. This code only applies to the chemical additives--holding temperature and container selection is assumed to be within EPA guidelines, unless otherwise identified in the LNOTE field. More than one PRESCODE may be entered into this field, separated by commas without spaces. Refer to the EDF Data Dictionary for a list of valid values. (C15)

Primary Key - A field or set of fields that uniquely identify a record within a table. Key fields within a table define the primary key. Each database record can be uniquely identified using the combination of data fields that make up the primary key, as illustrated in Figure 6. Each table within the EDMS contains a primary key.

A&A, Inc., 1997

Figure 6: Primary Key

PROJNAME (Project Name) - The identification assigned to the project by the organization performing the work. (C25)

PVCCODE (Primary Value Type) - This allows the coding of Gas Chromatography or Gas Chromatography/Mass Spectroscopy results to show whether the reported result was obtained from a primary or a confirmatory analysis. Methods or analytes not requiring confirmation and requiring only one analysis run, should be reported with the PVCCODE of PR. Refer to the EDF Data Dictionary for a list of valid values. (C2)

QCCODE (Quality Control Type) - A code identifying the sample type, i.e., field samples or laboratory-generated quality control samples. Refer to the EDF Data Dictionary for a list of valid values. (C3)

RECDATE (Received Date) - Date that the laboratory physically takes custody of the sample. The format of this field is YYYYMMDD. (D8)

Record - A line of data (a row) in a table or file made up of distinct fields of information.  Refer to Figure 6.

REP_DATE (Report Date) - Date that the laboratory generates the hard copy report. The format for this field is YYYYMMDD. (D8)

REPDL (Reported Detection Limit) - The detection limit reported by the laboratory to determine whether a parameter is detectable. (N9) (Formerly in the format N9,4 in EDF 1.2a.)

REPDLVQ (Reported Detection Limit Qualifier) - A qualifier used to define the type of detection limit that the laboratory is reporting, (i.e., practical quantitation limits, instrument detection limits, etc.). Refer to the EDF Data Dictionary for a list of valid values. (C3)

REQ_METHOD_GRP (Requested Method Group) - The unique identifier for the method or group of methods requested by the client for analysis of the sample. This field links to the chain-of-custody tables in the EDMS2000. (C25)

RT (Retention Time) - Retention time of a TIC. It is reported in minutes. (N7) (Formerly in the format N7,2 in EDF 1.2a.)

RUN_NUMBER (Run Number) - This field permits the numerical coding of multiple or repeat analyses of a sample (one LABSAMPID) by the same analytical method. (N2)

SAMPID (Field-Assigned Sample Identification) - The number assigned during sample collection in the field. (C25)

SRM (Standard Reference Material) - Code identifying source of reference material for calibration standard confirmation. Refer to the EDF Data Dictionary for a list of valid values. (C12)

SUB (Subcontracted Laboratory) - Field identifying the subcontracted laboratory. Refer to the EDF Data Dictionary for a list of valid values. (C4)

Surrogate - A compound that is similar to the target analyte(s) in chemical composition, extraction, chromatography, and behavior in the analytical process, but that is not normally found in environmental samples. Samples are spiked with known amounts of surrogates as a check on method procedure accuracy. Percent recoveries are calculated for each surrogate and are an indication of the percent recovery of the analytes in the sample.

Table - A format for data that allows for data manipulation within a database.  Tables are organized with columns and rows of information.  (Refer to Figure 6.)

UPPERCL (Upper Control Limit) - The upper limit of a quality control acceptance criterion. Enter relative percent difference and percent difference limits into the UPPERCL. (N4)

UNITS (Units of Measure) - The units of measure used to report a result (e.g., for soil or for water). Refer to the EDF Data Dictionary for a list of valid values. (C10)

Valid Value - Specially-assigned, standardized coded value designating an approved “valid” value for entry into a field in the database.