# Summary of Data Elements

| Field Name | In Table(s) | Attrb | Null Allowed | VVL | Descr. Name | Definition | Guidelines & Restrictions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| anadate | TEST RESULTS | D8 |  |  | Analysis Date | The date the sample (aliquot, extract, digest and/or leachate) is analyzed. | Must be in the format YYYYMMDD. Must be later than or equal to EXTDATE, RECDATE, LOGDATE, and earlier than or equal to REP_DATE. |
| anmcode | TEST RESULTS QC CL | C7 |  | x | Analytical Method | The code identifying the method of analysis. | Must contain a valid value. |
| apprvd | TEST | C3 | x |  | Approved By | The initials of the individual approving the laboratory report. | No entry for laboratory-generated QC and non-client samples. |
| basis | TEST | C1 |  | x | Basis | The code used to distinguish whether a sample is reported as dry or wet weight, filtered or not filtered. | Must contain a valid value. Valid values for soil samples are "W" or "D" or leachate codes; for water samples "F," "L," or "N." |
| clcode | CL | C6 |  | x | Control Limit Type | The code identifying the type of quality control limit. | Must contain a valid value. |
| clrevdate | RESULTS CL | D8 | x |  | Control Limit Revision Date | The date a control limit is established. | Must be in the format YYYYMMDD. No entry when QCCODE is "CS," "NC," "LB," or "RS," and non-spiked parameters (except when PARVQ is "SU" or "IN"). |
| cntshnum | SAMPLE | C12 |  |  | Control Sheet Number | The administratively-assigned identification used to track contracts. | Entry of "NA" is acceptable. |
| coc_matrix | SAMPLE | C2 | x | x | COC Matrix | The code identifying the sample matrix as noted on the chain-of-custody (e.g., water, soil, etc.). | Optional. This field provides a link with the COC EDD from EDMS2000. It represents the sample matrix as identified by the field organization, and must contain a valid value. |
| cocnum | SAMPLE | C16 | x |  | Chain-of-Custody Number | The number assigned to the chain-of-custody. | No entry for laboratory-generated QC and non-client samples. |
| dilfac | RESULTS | N10 |  |  | Dilution Factor | The numeric factor indicating the level of sample dilution. | Must be greater than zero. (Formerly in the format N10,3 in EDF 1.2a.) |
| dqo_id | SAMPLE | C25 | x |  | Data Quality Objectives ID | The unique identifier representing the data quality objectives. | Optional. This field provides a link with the COC EDD from EDMS2000. |
| exlablot | TEST | C10 | x |  | OBSOLETE | OBSOLETE | OBSOLETE |
| exmcode | TEST RESULTS CL | C7 |  | x | Preparation Method | The code identifying the method of preparation. | Must contain a valid value. If no preparation performed enter "NONE;" if preparation method is included in analysis method enter "METHOD." |
| expected | QC | N14 | x |  | Expected Parameter Value | The target result for a quality control sample or surrogate spike. | No entry when QCCODE is "CS," "NC," "LB," or "RS." For matrix spikes, this value is the amount spiked plus the reference sample PARVAL. Enter "100" when UNITS are "PERCENT." (Formerly in the format N14,4 in EDF 1.2a.) |
| extdate | TEST RESULTS | D8 |  |  | Preparation Date | The date that a sample is prepared for analysis. | Must be in the format YYYYMMDD. If no preparation performed, enter ANADATE. |
| lab_meth_grp | TEST RESULTS QC CL | C25 | x |  | Lab Method Group | The unique identifier for a group of methods as defined by the laboratory. | Optional. This field provides a link with the EDMS2000. |
| lab_repno | TEST | C20 | x |  | Laboratory Report Number | The unique identifier for the laboratory report, assigned by the laboratory. | No entry for laboratory-generated QC and non-client samples. |
| labcode | SAMPLE TEST RESULTS QC CL | C4 |  | x | Laboratory | The code identifying the laboratory that analyzes the sample. | This field represents the laboratory that received the sample and is responsible for producing the electronic deliverable, and must contain a valid value. |
| labdl | RESULTS | N9 |  |  | Method Detection Limit | The laboratory-established method detection limit. | Enter zero when UNITS is "PERCENT" or PARVQ is "TI." Must be adjusted for dilution. Must be greater than or equal to zero. (Formerly in the format N9,4 in EDF 1.2a.) |
| lablotctl | TEST QC | C10 |  |  | Preparation Batch Number | The unique identifier for a preparation and handling batch. | Must uniquely define a group of samples prepared together. |
| labqcid | QC | C12 |  |  | Laboratory QC Sample ID | The unique identification number assigned to the sample by the laboratory. | This is equivalent to the LABSAMPID. |
| labrefid | QC | C12 | x |  | Laboratory Reference ID | The laboratory sample ID of the quality control reference sample. | This is the LABSAMPID of the reference sample. No entry unless QCCODE is "MS/SD" or "LR." |
| labsampid | TEST RESULTS | C12 |  |  | Laboratory Sample ID | The unique identification number assigned to the sample by the laboratory. | Must be unique. |
| locid | SAMPLE TEST | C10 | x |  | Location ID | The unique identifier for the sample's location, as identified by the laboratory. | No entry for laboratory-generated QC and non-client samples. |
| logcode | SAMPLE TEST | C4 | x | x | Field Organization | The code identifying the company collecting the samples or performing field tests. | Must contain a valid value. No entry for laboratory-generated QC and non-client samples. |
| logdate | SAMPLE TEST | D8 | x |  | Collection Date | The date a field sample is collected. | Must be in the format YYYYMMDD. No entry for laboratory-generated QC and non-client samples. Must be earlier than RECDATE, EXTDATE, ANADATE, and REP_DATE. |
| logtime | SAMPLE TEST | C4 | x |  | Collection Time | The time that a field sample is collected, recorded using 24-hour military time. | Must be a valid time between 0000 and 2359. No entry for laboratory-generated QC and non-client samples. |
| lowercl | CL | N4 |  |  | Lower Control Limit | The lower control limit of a quality control criterion. | Must be an integer greater than or equal to zero and less than UPPERCL. Enter zero for precision limit. |
| matrix | SAMPLE TEST RESULTS QC CL | C2 |  | x | Matrix | The code identifying the sample matrix as determined by the laboratory (e.g., water, soil, etc.). | This field represents the sample matrix as identified by the laboratory, and must contain a valid value. |
| meth_design_id | SAMPLE TEST RESULTS QC CL | C25 | x |  | Method Design ID | The unique identifier for the design of an analytical method. | Optional. This field provides a link with the COC EDD from EDMS2000. |
| modparlist | TEST | L1 |  |  | Modified Parameter List | A field indicating whether the parameter list of an analytical method has been modified. | Must enter "T" (true) or "F" (false) if a parameter from the method parameter list is not reported. The parameter list is not considered modified if extra parameters are reported. |
| npdlwo | SAMPLE | C7 |  |  | NPDL Work Order Number | A delivery order number associated with the contract. | Entry of "NA" is acceptable. |
| parlabel | RESULTS QC CL | C12 |  | x | Parameter | The code or CAS number identifying the analyte (parameter). | Must contain a valid value. |
| parun | RESULTS | N12 | x |  | Parameter Uncertainty | The uncertainty of a measured value due to a measuring technique (expressed as plus or minus some value). | No entry necessary for non-radiochemical results. If entered, must be greater than or equal to zero. (Formerly in the format N12,4 in EDF 1.2a.) |
| parval | RESULTS | N14 |  |  | Parameter Value | The analytical value for a compound, analyte, or physical parameter. | (Formerly in the format N14,4 in EDF 1.2a.) |
| parvq | RESULTS | C2 |  | x | Parameter Value Qualifier | The code identifying the qualifier of an analytical result (e.g., greater than, equal to, etc.). | Must contain a valid value. |
| prescode | TEST | C15 | x | x | Preservative | The code identifying the type of preservative added to the sample. | Must contain a valid value. Multiple codes may be entered, separated by commas (no spaces between values). |
| projname | SAMPLE | C25 | x |  | Project Name | The identification assigned to the project by the organization performing the work. | No entry for laboratory-generated QC and non-client samples. |
| pvccode | RESULTS | C2 |  | x | Primary Value Type | The code identifying whether a sample result is a primary or a confirmatory value. | Must contain a valid value. There may be only one "PR" result per LABSAMPID, ANMCODE, EXMCODE, and PARLABEL. |
| qccode | TEST RESULTS QC | C3 |  | x | QC Type | The code identifying the type of sample (e.g., laboratory-generated, environmental, etc.). | Must contain a valid value. |
| recdate | TEST | D8 |  |  | Received Date | The date the sample is received by the laboratory doing the analysis. | Must be in the format YYYYMMDD. For laboratory-generated QC samples enter date sample was created (e.g., EXTDATE). |
| rep_date | TEST | D8 | x |  | Report Date | The date of the laboratory report. | Must be in the format YYYYMMDD. No entry for laboratory-generated QC and non-client samples. |
| repdl | RESULTS | N9 |  |  | Reporting Detection Limit | The laboratory-established method detection limit, adjusted for the particular sample preparation (e.g., weight, volume, or dilution). | Enter zero when UNITS is "PERCENT" or PARVQ is "TI." Must be adjusted for dilution. Must be greater than or equal to zero. (Formerly in the format N9,4 in EDF 1.2a.) |
| repdlvq | RESULTS | C3 |  | x | Reporting Detection Limit Qualifier | The code identifying the type of reporting limit (e.g., practical quantitation limit, instrument detection limit, etc.). | Must contain a valid value. Enter "NA" when UNITS is "PERCENT" or PARVQ is "TI." |
| req_method_grp | SAMPLE | C25 | x |  | Requested Method Group | The unique identifier for the method or group of methods requested by the client for analysis of the sample. | Optional. This field provides a link with the COC EDD from EDMS2000. |
| rlnote | RESULTS | C20 | x | x | Laboratory Result Notes | The code identifying notes pertaining to analytical performance irregularities that apply to a single analyte. | Must contain a valid value. Multiple codes may be entered, separated by commas (no spaces between values). |
| rt | RESULTS | N7 | x |  | Retention Time | The retention time of a tentatively identified compound (TIC), reported in minutes (min). | No entry necessary except when PARVQ is "TI." If entered must be greater than or equal to zero. (Formerly in the format N7,2 in EDF 1.2a.) |
| run_number | TEST RESULTS | N2 |  |  | Run Number | The numeric code distinguishing multiple or repeat analysis of a sample by the same method on the same day. | Must be an integer greater than or equal to 1. |
| sampid | SAMPLE TEST | C25 | x |  | COC Sample ID | The unique identifier representing a sample, assigned by the consultant, as submitted to the laboratory on a chain-of-custody. | This field represents the sample ID as it appears on the COC. No entry for laboratory-generated QC and non-client samples. |
| srm | RESULTS | C12 |  | x | Standard Reference Material | The code identifying the standard reference material used in the analysis. | Must contain a valid value. Enter "NA" if no reference material. |
| sub | TEST | C4 |  | x | Subcontracted Laboratory | The code identifying the subcontracted laboratory. | Must contain a valid value. Enter "NA" if no analyses are subcontracted. |
| tlnote | TEST | C20 | x | x | Laboratory Test Notes | The code identifying notes pertaining to analytical performance irregularities that apply to the entire test. | Must contain a valid value. Multiple codes may be entered, separated by commas (no spaces between values). |
| units | RESULTS QC | C10 |  | x | Units of Measure | The units for the parameter value measurement. | Must contain a valid value. |
| uppercl | CL | N4 |  |  | Upper Control Limit | The upper control limit of a quality control criterion. | Must be an integer greater than or equal to one and greater than LOWERCL. |