Tip#1: Entry of FIELD_PT_NAME and SAMPID fields in EDF for AB2886 reporting:

* The Field Point Names uploaded to GeoTracker must be the same as the well

names and borehole names used in the hard copy report submitted to the lead

agency caseworker.  The SWRCB will reject EDFs where the field point names

do not match well names and borehole names in the hard copy reports.

The SAMPID field is the sample identification as listed on the COC. In many

cases this will be identical to the entry in FIELD_PT_NAME.

* Depth Specific Sampling

The SWRCB will be providing an expanded reporting format for dealing with

depth specific sampling information when final regulations are enacted.

Presently, for soil samples, please upload a single Field Point Name for

each borehole (e.g. B-1) and use the SAMPID field to identify the sampling

depth in feet (e.g. B-1d5.0, B-1d10.0, etc.).

For nested wells, each "well" should be entered with a separate Field Point

Name (i.e. MW-1A, MW-1B, MW-1C).

If depth discrete sampling is performed within a single groundwater well,

please upload a single Field Point Name for the well (e.g. MW-1) and use the

SAMPID field to identify the sampling depth in feet (e.g. MW-1d15.0,

MW-1d20.0…).

Tip#2: Entry of FIELD_PT_NAME for field QC samples:

* Field QC Samples:

The SWRCB will be providing a permanent solution for dealing with field QC

information when final regulations are enacted.  Presently, for entry of

field QC data in EDF, enter Field Point Names of QCTB for a Trip Blank, QCEB

for an Equipment Blank, and QCAB for an Ambient Blank.

To associate field QC samples with a specific well or borehole, one may

append the Field Point Name to "QCEB" for a total of 10 characters in

length. For example, if an equipment blank is taken at monitoring well MW-1,

the Field Point Name for that equipment blank could be identified as

"QCEBMW-1."