| PAUL R. MINASIAN, INC. WILLIAM H. SPRUANCE, INC. WILLIAM H. BABER III, INC. JEFFREY A. MEITH M. ANTHONY SOARES MICHAEL V. SEXTON MICHELLE DECASTRO | (A PARTNERSHIP INCLUDING PROFESSIONAL CORPORATIONS) 1681 BIRD STREET P.O. BOX 1679 OROVILLE, CALIFORNIA 95965-1679 TELEPHONE (530) 533-2885 FACSIMILE (530) 533-0197 | PAUL JACKSON MINASIAN, 1933-1981 ─────── DAVID H. MINASIAN, RET. 1989 |
| --- | --- | --- |

March 30, 1998

Victoria Whitney

State Water Resources Control Board

P.O. Box 2000

Sacramento, California 95812-2000

Re:Draft Environmental Impact Report on the Implementation of the 1995 Water Quality Control Plan for the San Francisco Bay/Sacramento-San Joaquin Delta Estuary

Dear Ms. Whitney:

These comments, and those attached to this letter, are submitted on behalf of the San Joaquin River Exchange Contractors Water Authority and its members Central California Irrigation District, San Luis Canal Company, Firebaugh Canal Water District and Columbia Canal Company (the Exchange Contractors).

Comments Regarding Increased Groundwater Pumping

The comments in Attachment 1 are those of Kenneth D. Schmidt and Associates, Groundwater Quality Consultants.    The curriculum vitae of Kenneth D. Schmidt was submitted to the Water Board along with the Exchange Contractors notice of intent to appear and witness list. His comments address impacts    related to increased groundwater pumping and the apparent contradiction between concentration-based water quality goals in the San Joaquin River and water conservation goals.

Comments Regarding the Groundwater Flow System in Relation to Drain Flow and Water Quality

The comments in Attachment 2 are those of Steven J. Deverel, Ph.D., Principal Hydrologist, Hydrofocus, Inc.    The curriculum vitae of Steven J. Deverel was submitted to the Water Board along with the Exchange Contractors notice of intent to appear and witness list.    His comments address the deficiency in the EIRs conceptualization of the hydrologic system of the San Joaquin Valley and the analysis of the alternatives.    Of particular concern is the manner in which the EIR fails to account for the regional nature of the groundwater flow system in relation to drain flow and water quality.

Comments Regarding Vernalis Calculations for Flow Alternative 4

In the event that the Exchange Contractors were to return to the river, the calculation of water quantity is no longer based upon the Exchange Contract monthly quantities but on the quantities established by the Contract for Purchase of Miller & Lux Water Rights made July 27, 1939 (Purchase Contract).    The Purchase Contract, when read along with Exchange Contract, explains that if the Exchange Contractors return to the San Joaquin River for their water diversions, they are entitled to divert water according to Schedule 1 found on page 4 of the Purchase Contract, a copy of which is attached as Exhibit C for your convenience.    You can see from Schedule 1 that there is not a monthly maximum quantity computed in acre feet; rather, month by month there is a schedule of 24 hour mean flows in cubic feet per second.    Multiplying out the 24 hour mean flow by month results in a much higher quantity of water than the 840,000 acre feet established under the Exchange Contract.    Consequently, it is erroneous for the DWR SIM Model for this alternative to be based on the Exchange Contract quantity rather than the purchase contract Schedule 1 flow schedule.

Additionally, you will note that the Schedule 1 flow schedule is measured at the Whitehouse gauging station which is found just upstream of Gravelly Ford.    Releases from Friant in order to meet the flow schedule at Whitehouse gauging station would have to be adjusted for losses.    There are minimal losses between the Whitehouse gauging station and the headworks for the Exchange Contractors diversions at Mendota Pool.

The assumptions made to calculate responsibility to achieve the Vernalis Flow Objectives under Alternative 4 are flawed (page IV-19 et. seq.).    The responsibility calculation is based on DWR SIM output files which assume that the Exchange Contractors are taking their water from the San Joaquin River; however, the flaw in the assumption is that the Exchange Contractors are limited to 840,000 acre feet  the quantity of water to which they are entitled under the Exchange Contract.    In the EIR page IV-19, the statement is made that the Exchange Contractors have riparian and pre-1914 appropriated water rights on the upper San Joaquin River, but they executed a contract with the CVP to receive water from the Delta in exchange for their San Joaquin River water.    This is not totally correct.    The Exchange Contractors did execute the Exchange Contract with the Bureau of Reclamation to receive water form the Delta; however, there was never a giving up of their San Joaquin River entitlement.    The Exchange Contract merely recites that the Exchange Contractors agree not to exercise their rights upon the San Joaquin River so long as they receive a substitute water supply delivered by the Bureau from the Delta or other sources.

Comments Regarding Application of Watershed Protection Statutes

The DEIR at page IV-19 discusses assumptions made to achieve flow objectives under Alternative 4.    The Exchange Contractors note, parenthetically, that if a person or entity seeks to assert area of origin or similar rights then they ought to be required to participate in a proceeding aimed at meeting water quality, public trust and similar obligations imposed on the watershed and/or its river systems.

Other Comments on the DEIR Incorporated

In addition to these comments, the Exchange Contractors join in the separate comments submitted by the San Joaquin River Group Authority, and the separate comments submitted by this law firm on behalf of its clients.

We note that the Water Board is supplementing chapters V, VI and XIII and we may submit

additional comments to address that supplement.

Very truly yours,

MINASIAN, SPRUANCE, BABER,

MEITH, SOARES & SEXTON, LLP

By

MICHAEL V. SEXTON

MVS/adv

Enclosures

/tmp/in/input.doc

March 16, 1998

Mr. Mike Sexton

Minasian Law Firm

P.O. Box 1679

Oroville, California 95965-1679

Re:  Draft EIR-Implementation of

1995 Bay/Delta WQCP (November 1997)

Dear Mike:

Following are my review comments on the Draft EIR for the 1995 Bay/Delta WQCP.  Most of my comments are on Chapters VI and VIII.  Overall, the document contains a large amount of useful information, and a number of serious impacts are identified, many of which are related to increased groundwater pumping.  Probably the greatest concern is the apparent contradiction between concentration-based water quality goals in the San Joaquin River and water conservation goals.  Following are my review comments on specific parts of the Draft EIR and associated documents.

Page III-73.  Some of the referenced west side streams have good quality water.  It isnt true that water from all of them (i.e. Los Banos Creek) is often highly mineralized.  In addition, the amount of flow in a number of these is highly important as a source of recharge to groundwater in local areas.

Page III-75.  The major sources of recharge to the lower aquifer on the west side of the valley werent discussed.  Groundwater inflow from the east, from what has traditionally been referred to as the  forebay area, is significant.  Also, downward leakage of groundwater from the upper aquifer is important.

Page III-123.  Part C.  The comment about the Corcoran Clay horizon being below well depths should be deleted. This is pure speculation, based primarily on surface geophysical surveys.  It would be better to say that there has been a lot of controversy over whether or not the Corcoran Clay is present in much of Kern County.  It is generally now agreed that it is not present in most of the Kern Fan.  Also, other less regionally extensive clay layers are more important confining beds in some parts of the valley.

Page VI-78.  Part B.  The statement about lack of constraints on groundwater pumping is somewhat overstated.  For example, if the pumping is unreasonable and causes significant damage to others, existing factors such as litigation could be a constraint.

The comment about mitigation is somewhat unclear.  Does it  only apply to fossil fuels, or to all of the items listed above (including groundwater pumping)?  The order of the headings used in the report is hard to follow.  The comment that these effects are not mitigable, and are the inevitable consequence are somewhat astonishing to be in a Draft EIR, particularly if they refer to groundwater pumping.

Page VI-116. Second full paragraph.  The statement about subsidence infers that subsidence is rather instantaneous, and stops as soon as pumping during droughts stops.  However, compaction recorder records indicate that all of the subsidence doesnt occur just when the pumping is occurring (i.e. there is a significant lag time).  Also, there presently is a concern about subsidence in some areas that are outside of the ones that were delineated by the U.S. Geological Survey several decades ago.  This is because of the drilling of new wells and pumpage of confined groundwater in other areas during the past decade or two.  Examples are north of Los Banos in and near the Central California Irrigation District (CCID), and at some locations of concentrated wells pumping into the Delta-Mendota Canal.

Page VI-117.  It should be mentioned somewhere in the EIR that the  subsidence impacts are essentially irreversible.  During the recent drought, subsidence in local areas was exascerbated by pumping from many concentrated wells into canals or other water bodies, as part of water transfer or exchange programs.

Page VI-118.  The comment that groundwater overdraft occurs throughout the San Joaquin River and Tulare Basins is technically incorrect and misleading.  Careful examination of hundreds of water-level hydrographs for wells in the valley indicates a very different conclusion.  The greatest overdraft is generally in undistricted areas with no surface water supplies, and in districts with inadequate surface water supplies.  Recently, overdraft has also resumed in some areas which have had their surface water supplies curtailed.  Water-level hydrographs indicate that there was no overdraft in significant parts of both basins, before surface water supplies were curtailed.  Also, in some areas, such as much of the west side, water levels in some strata have been rising over the long term.  This proposed project has the potential not only to worsen overdraft, but to create overdraft in areas where overdraft wasnt previously occurring.

Page VI-119.  First paragraph, second line.  Replace could with would, since it is a certainty, if the project is undertaken.

Page VI-120.  I strongly concur with the serious impacts identified for the Madera and Chowchilla Basins.

Page VI-121.  From the technical standpoint, its hard to believe that a project would be seriously proposed that would increase overdraft in the Madera Basin, given the existing situation.

Page VI-122.  Item C.  The comment about loss of usable storage due to groundwater quality degradation should be clarified and expanded upon.  For example, exactly where was this accounting practice done in the San Joaquin Valley?  Does this imply that the SWRCB is writing off the groundwater degradation (accepting it) in certain areas?

Page VI-123.  Second paragraph.  Besides overdrafting in some areas east of the valley trough, increased water deliveries and decreased pumpage in some parts of the west side of the valley have been a contributing factor.  There is a well developed cone of depression and overdrafting in the Raisin City area.  Thus the Fresno and other areas should also be mentioned.

Page VIII-1.  As mentioned previously, the comments on west side streamflow should be revised.

Page VIII-7.  The period 1985-94 was used, but this doesnt appear to be a representative hydrologic base period, because the first part was a wet period and the last part was primarily a drought.  The present tense is used in much of the discussion, as if these conditions are representative of long-term hydrologic conditions.

Page VIII-11.  Above Item 2.  The comment on salt accretion and excretion in the basin need to be clarified for the reader.  Is the comment referring to surface water, groundwater, soils, geologic materials, some of these, or all of these?  Also, are there places in the basin where salts are precipitated and removed from the surface water-groundwater system?  Such processes happen in nature in playas.  It may also be occurring with respect to carbonate and sulfate salts as part of the normal irrigation process.

Page VIII-14.  Last paragraph.  I highly concur with the comments on the valley-wide drain.

Page VIII-15.  Measuring the electrical conductivity, which is a measure of salt concentration, and applying water quality standards based on this, appear to be in direct conflict with water conservation goals.  This is because water conservation practices normally concentrate salts in the deep percolation water.  The surface water and groundwater are substantially related hydraulically, and directly connected at some locations, such as along the lower part of the San Joaquin River.  Thus it appears that both systems, and both the water quantity and salt concentrations all need to be inter-related more than has been done in the EIR and related documents.  For example, taking surface water out of one place and putting in another, when the hydrologic system is interconnected, may not be a good long-term solution.

Part VIII-16.  Water conservation may reduce the salt load, but can increase the salt concentrations in deep percolation.  Salt concentrations depend on the amounts of both water and salt.  For example, if surface water with a TDS of 300 mg/l is used for irrigation at an irrigation efficiency of two-thirds, the deep percolation would have a TDS of about 900 mg/l.  If groundwater with a TDS of 900 mg/l was substituted and used for irrigation at the same irrigation efficiency, the deep percolation would have a TDS of about 2,700 mg/l, which is an increase of about 1,800 mg/l.  The resulting increased concentration in the deep percolation water could have a substantial impact if the recharged groundwater is pumped and used or flows into a stream.

Page VIII-17.  Last paragraph.  The principal best management practice of water conservation appears contradictory to water- quality goals based on concentration of salts.

Page VIII-19.  In terms of land retirement, how was the total acreage to be funded determined?  How much of the gained water can be used to mitigate this project?

Page VIII-23.  The value of 19,145 acre-feet should be rounded off, as it isnt accurate to five significant figures.  This comment is applicable to other parts of the EIR and related documents.

Page VIII-30.  The units for the values without percentages werent provided.  This is a very difficult table to understand.

Page VIII-31.  Part 4.  The title construction related effects should be revised, as it doesnt clearly describe what is discussed in this part.

Other Documents Referenced

I also reviewed several documents that were referenced in the Draft EIR.  One was the Draft Technical Appendix on Groundwater for the CVPIA PEIS (September 1997).  This document is extremely difficult to read, partly due to its organization.  For example, even though the alternatives arent explained or discussed until Page III-31, numerous tables are presented before this page (i.e. Table III-5 on Page III-26) that apparently contain the results of evaluations of the alternatives.  This poor organization of the report makes it very difficult to read through, because one has to continuously flip back and forth from one section to another to try to understand it.

For the last paragraph of the Introduction on Page I-1, an explanation for why this was done should be added.  One of the greatest limitations of the evaluation is presented in the second paragraph of page II-1, as follows:

Detailed site-specific information on all groundwater basins and sub-basins potentially affected by CVPIA is not included in this chapter (II).  In fact, such an evaluation, which is absolutely necessary to understand the impacts of implementation of CVPIA, is not presented in any of the documents.  Impacts on individual water districts and parts of large districts, such as the CCID, werent presented.  Rather, a general, regional, broad-brush evaluation was attempted.  The value of this is unclear.

The references on groundwater quality cited in the second paragraph of Page II-3 are grossly inadequate for use in evaluating impacts.  These were intended for other purposes, and not for use in an evaluation of such a highly significant undertaking.

The compositing of different aquifers and large depth intervals of aquifers, such as discussed in the second paragraph on Page II-14, over-simplifies the groundwater system.  Maps (i.e. Figure II-8) where the TDS in the San Joaquin Valley groundwater is presented in a space of about four inches by five inches are way too general to be meaningful.  In addition, the relatively small maps are difficult to use for specifics, as numerous important geographical features, such as the San Joaquin River werent shown.

The discussions of water levels and overdraft is confusing (i.e. Pages II-24 to II-78).  For example, Figure II-13 doesnt show a long-term hydrologic base period representing average surface water supplies.  Thus the value of it is hard to understand.  Documentation of the overdraft values wasnt provided.  The errors in items such as pumpage, canal seepage, and deep percolation, which arent measured in most of the San Joaquin Valley, could render the conclusions invalid.

The generality comments about high nitrates (Page II-30 last paragraph) are inaccurate.  The Cities of Turlock, Merced, and Madera havent had significant problems with nitrate to my knowledge.  The comments on DBCP on Page II-31 and the associated map (Figure II-16) are highly misleading.  For example, no DBCP is shown in groundwater in Kern County or in the Modesto or Lodi areas, although there has been extensive litigation over this.  Thus this map greatly underestimates the extent of DBCP in shallow groundwater in the San Joaquin Valley.

Some of the illustrations, such as Figure II-17, have shading patterns that arent discernable (i.e. the 5 to 10 feet symbol looks the same as the 10 to 20 feet symbol).

On Page II-40, above the heading Groundwater Levels, the 1990 conditions (pumping vs. perennial yield) are of little value, unless 1990 represents an average hydrologic base period.  Using records for one year in a prolonged drought to represent typical groundwater conditions is of little value.

It would be advisable to tell the reader what the alternatives are earlier in the report (i.e. at least on Page III-1).

On Page III-4, compositing of the water levels for Layer 1 and 2 (which werent explained) is again of questionable value, even though the report indicates this provides a reasonable representation.

The value of maps such as Figures III-6b and III-6c is very questionable.  For example, no depression cone is shown in the Fresno urban area (the largest municipal pumping center in the valley) or in the area southeast of Madera, which has little surface water and a huge cone of depression.  Why these maps were presented is difficult to understand.

On Page III-33, why was no water-level change shown in most of the Exchange Contractors service areas?  This looks incorrect.

On Page III-39 the subsidence shown (1 to 10 feet) in part of the Exchange Contractors service area would be highly significant.  However, additional subsidence could occur farther north than shown (north of Highway 152).

Please call me if you have any questions.

Sincerely yours,

Kenneth D. Schmidt

KDS/bbs

COMMENTS ON DRAFT ENVIRONMENTAL IMPACT REPORT FOR IMPLEMENTATION OF THE 1995 BAY/DELTA WATER QUALITY CONTROL PLAN

1.0 General Comments

- The primary deficiency of the report is the lack of sufficient analysis and detail to adequately meet goal 6 stated on page II-1, Equitably distribute the responsibility of meeting the objectives contained in the 1995 Bay/Delta Plan, for two reasons.    First, the modeling results are not quantitative enough and the data is not available to insure that the implementation of the alternatives will have the intended effects.    Second, reoperating drainage systems as is described in the Environmental Impact Report (EIR) will cause more drainage water to be stored in downslope areas than upslope areas.    Additional analysis and data collection is needed to equitably allocate the responsibility for meeting flow and salinity objectives in the San Joaquin River.

- The implementation of the proposed reoperation of drainage systems will result in higher groundwater and soil concentrations of selenium, boron, molybdenum and salt.    This is due to increased evapoconcentration from the shallow water table during storage of shallow groundwater in the summer when there is high evapotranspiration. Additional analysis and data is needed to assess the impacts of reoperation of drainage systems on groundwater and soil salinity.

- The proposed reoperation of drainage systems seems to be based on an inaccurate conceptualization of the hydrologic system.    This conceptualization holds that there is a perched water table and that the regional groundwater flow system has little or no influence on drainflow.

- The implementation of reoperation of the drainage systems or other methods to equitably reduce salt loading to the San Joaquin River needs to based on a regional analysis of the groundwater flow system because of the upslope hydraulic effects on downslope drainflow.

- Under the proposed drainage system reoperation, the upslope hydraulic influences will cause water logging and salt accumulation to occur in drained areas closer to the valley axis which will prevent future viable agricultural operations.

- Future iterations of the EIR should consider the use of groundwater pumping for reduction of drainage loads during critical times.

- The estimated effects of meeting the flow and salinity standards in the San Joaquin River are preliminary and require further qualification and quantification.    Additional work is needed to validate and determine the accuracy of the model results.    Questions need to be answered about the meaning of the estimated effects relative to actual flows and the confidence level of the results.

- The EIR does not address all of water quality issues in the San Joaquin River. Specifically, selenium, boron and molybdenum are constituents of concern in the San Joaquin River and Mud and Salt Sloughs that are not addressed by the EIR.    Water quality objectives for these constituents are specified in the Water Quality Control Plan for the Sacramento River and San Joaquin River Basins.    The question as to how the concentrations of these constituents in the San Joaquin River will be affected by the implementation of the proposed alternatives needs to be answered.

- All of the water quality concerns should be addressed simultaneously.    Addressing only salinity now will lead to redundancies and inefficiencies in addressing all of the water-quality issues in the San Joaquin River.    Specific comments follow.

2.0 Specific Comments

2.1 Shallow groundwater and drainage

On page VIII-3, the report states that local mounds of shallow, salty ground water can form above dense clay lenses implying that the primary source of the drainage problem in the western San Joaquin Valley is a perched water table. The U.S. Geological Survey has shown using results from geologic borings, water levels in observation wells, chemical tracers and modeling studies that the shallow water table is in direct hydraulic communication with deeper aquifers above and below the Corcoran Clay.    The drainage problem in the western San Joaquin Valley, is due to increasing groundwater storage throughout the semiconfined zone above the Corcoran Clay (see Belitz and Heimes (1990) and Belitz and others (1992) for further explanation of the groundwater flow system).    Perched groundwater is limited in regional extent.    The regional nature of the groundwater flow system needs to be considered in further analysis of the proposed alternatives for meeting the salinity objectives in the San Joaquin River.    The consequences of not doing this are described below.

Page VIII-18.    The report states that subsurface storage of drainage water may have to be closely monitored.    There is insufficient evidence to support the thesis that subsurface storage of drainage water will work as a long-term strategy for managing drainage loads to the San Joaquin River.    Close monitoring will be required to evaluate the long-term changes in groundwater and soil chemical composition.

Page VIII-27.    Controlled timing of drainage releases as proposed in the report cannot be reasonably implemented under the current analytical strategy in which exceedance of the salinity standard in any month requires storage of drainwater for three months.    Real time data collection and analysis to determine releases is essential if this program is to be implemented.    This is due to the need for minimizing the time that drainwater will be stored in the field.    Under the modeling scenarios presented in the EIR, exceedance of the water quality objectives in any given month requires storage for the entire 3-month period.    In the case of the January through March period, this could often mean that drain water would be unnecessarily stored during February and March.

2.2    Reducing the need for dilution releases

The report states that the focus of the analysis of environmental impacts is to determine whether the need for dilution water releases can be significantly reduced by implementing salinity control measures.    However, the report does not quantify the level of significance. Further comments follow relative to this issue.

As an example, on page VIII-30, table VIII-7 compares the dilution release requirements for elimination of tile and wetland discharges with the reference alternative 1.    The average dilution requirement difference for the 73-year period between alternative 1 and the elimination of tile discharges is 35 thousand- acre feet (TAF) (a decrease from 68 TAF to 33 TAF).    Future analysis needs to determine the error associated with these estimates and then determine if the implementation of alternatives makes sense given the range of predicted values.

For example, if the estimation error in the modeling estimates for dilution flow releases from New Melones is plus or minus 30 % (based on conversations with Regional Board staff), the range of differences is 5 (0.70*68 - 1.30*33 = 4.7) to 65 TAF (1.3*68  0.7*33 = 65.30).    This means that the average difference between dilution flow releases between alternative 1 and the case where tile discharges are eliminated ranges from 5 to 65 TAF.    Future analysis would determine if the possibility of a long-term difference in dilution requirements as low as 5 TAF warrants implementation.    Future consideration of alternatives should present an analysis of the level of significance that is based on modeling and measurement error and apply it to long term averages and hydrologic extremes.

Similarly, on page VIII-32 in figure VIII-11, the predicted average differences in EC at Vernalis after New Melones Reservoir Releases are small, ranging from 0 to 146 mhos/cm. Measurement error of EC can be plus or minus 2 to 5 percent (Hem, 1985) but can be a high as 10 percent.    For the values in table VIII-11, 2 to 5 percent corresponds to 8 to 34 mhos/cm.    This range encompasses most of the differences in the table. Therefore, the estimated differences in EC as the result of implementation are similar in magnitude to the measurement error most of the year.    Future analysis should determine the level of significance associated with these estimates and ask the question: Given the range of estimates, should this alternative be implemented?    The answer to this question appears to be that the uncertainty is too high to insure a reasonable level of success of the alternative.

2.3    Controlled drainage release

Starting on page VIII-31, the report describes the implementation of controlled tile drainage release.    There are two problems with the proposed regional implementation of this management practice and the current level of analysis.    First, the effects on groundwater and soil salinity have not been sufficiently evaluated.    The study of drainage system operation by Ayars and others (1996) cited in the EIR assessed the changes in groundwater salinity during 1 year in a single field in the Grasslands Subarea and concluded that the quality of the drain water from these sites would not change significantly as a result of the imposed management (controlled drainage).    One year is insufficient to assess changes in groundwater salinity.    Also, the salt balance and effects on groundwater quality need to be examined in a regional context.

Second, the proposed methodology for drainage system reoperation does not account for the regional nature of the groundwater flow system.    Implementation of the controlled drainage methodology assumes that the hydraulic controls on groundwater and changes in groundwater quality in any field are solely the result of activities in that field.    Studies conducted at the field and regional scale clearly demonstrate that this is not the case (e.g. Fio (1994), Deverel and Fio (1991), Fio and Deverel (1991)).    Data from these studies demonstrate that 1) groundwater flow to drains is influenced by regional hydraulic controls and 2) the most saline groundwater that enters drains is at depths greater than the water table and can flow to drains from upgradient fields.    This means that fields in areas closest to the valley axis will be required to store more drainage water than upslope areas.

Controlling drainage as is described in the EIR will change the regional groundwater hydraulics such that evapotranspiration of the shallow groundwater will become the    predominant mechanism for groundwater discharge instead of agricultural drainage as is currently the case.    This will cause additional evapoconcentration of the shallow groundwater, a condition the drainage systems were designed to prevent, and will lead to higher groundwater and soil concentrations of selenium, boron, molybdenum and salt over time.    Because there will be less dilution with irrigation water and less outlet for drainage water during times of high ET, there will be upward movement of deep saline groundwater towards the root zone.    This will lead to the salinization and water logging of drained areas closest to the valley axis which will eventually reduce the viability of agriculture.

Controlling drainage in the manner described in the report does not contribute to the equitable distribution of the responsibility (objective 6 of the EIR) of meeting the water-quality objectives contained in the 1995 Bay/Delta Plan because some fields will store more than their share of drainage water.    In the case of areas closest to the valley axis such as Firebaugh Canal Water District (FCWD), there is saline groundwater moving across the district boundary from upslope districts, some of which moves into drainage systems.    Water moving from upslope and hydraulic forces originating upslope without sufficient outlet through drainage systems in these areas, will cause water logging and excessive salinization.    The EIR states that pilot studies will have to be completed before full implementation of this alternative and that successful implementation requires careful monitoring and management.    The EIR should, as is required under CEQA, fully describe mitigation measures.    This should include monitoring and mitigation of groundwater and soil salinization that may result from implementation of this alternative.

2.4 Groundwater pumping

Future versions of the EIR should consider groundwater pumping as a strategy for reducing drain loads to the San Joaquin River during critical periods as there is substantial evidence that groundwater pumping can reduce drainage volumes.    The San Joaquin Valley Drainage Program (1990) recommended groundwater pumping from the semiconfined zone as a strategy for lowering the water table and reducing drainage volumes and loads in the western San Joaquin Valley.    The results of using a regional groundwater flow model described in Belitz and Phillips (1992) indicate that drainage volumes can be reduced substantially by pumping from the semiconfined and confined aquifers in the western San Joaquin Valley.    Pumping from the shallow (within 50 feet of land surface) semiconfined zone and/or from deeper zones will lower the water table and reduce drainage volumes.    Discharge of pumped groundwater to the San Joaquin River during times of high assimilative capacity could lower the water table sufficiently to create groundwater storage for periods when there is little or no assimilative capacity in the river.    This will reduce drainage volumes and loads to the river during periods when there is little or no assimilative capacity in the San Joaquin River.    Implementation of this strategy will require real time monitoring of flow and salt concentrations in the San Joaquin River and quantitative regional analysis of the groundwater flow system.

3.0 Summary

The EIR is deficient in its conceptualization of the hydrologic system and the analysis of the alternatives.    It does not fully address water quality issues in the San Joaquin River and it is unclear how the implementation of the proposed alternatives relates to meeting water-quality objectives besides salinity. The proposed alternative for holding drainage water does not account for the regional nature of the groundwater flow system in relation to drain flow and water quality.    Implementation of this alternative can lead to salinization and waterlogging of drained areas closest to the valley axis.

Even though the EIR states that the focus of the analysis is to determine whether the need for dilution water releases can be significantly reduced by implementing salinity control measures, the significance level is not quantified.    Future analysis needs to quantify the uncertainty in the estimated effects and incorporate the uncertainty into the decision making process for implementing alternatives for meeting water quality and flow objectives.

4.0    References

Ayars, J.E., Schoneman, R., Mead, R., Soppe, R., Dale, F. and Meso, B., 1996, Shallow groundwater management project final report, USDA-ARS, Fresno, California.

Belitz, Kenneth and Heimes, F.J., 1990, Character and evolution of the groundwater flow system in the central part of the western San Joaquin Valley, California:    U.S. Geological Survey Water Supply Paper 2348

Belitz, Kenneth, Phillips, S.P., and Gronberg, J.M., 1992, Numerical simulation of ground-water flow in the central part of the western San Joaquin Valley, California: U.S. Geological Survey Water Supply Paper 2396

Belitz, Kenneth, Phillips, S.P.,    1992, Simulation of water-table response alternatives, central part of the western San Joaquin Valley, California, U.S. Geological Survey Water-Resources Investigation Report 91-4193.

Deverel, S.J. and Fio, J.L., 1991, Groundwater flow and solute movement to drain laterals, western San Joaquin Valley, California. I. Geochemical assessment: Water Resources Research, 27, 2233-2246.

Fio, J.L and Deverel, S.J., 1991, Groundwater flow and solute movement to drain laterals, western San Joaquin Valley, California.    II. Quantitative hydrologic assessment:        Water Resources Research, 27, 2247-2257.

Fio, J.L., 1994, Calculation of a water budget and delineation of contributing sources to drainflows in the western San Joaquin Valley, California.    U.S. Geological Survey Open File Report 94-45.

Hem, J.D., 1985, Study and interpretation of the chemical characteristics of natural water, U.S. Geological Survey Water-Supply Paper 2254.

San Joaquin Valley Drainage Program, 1990, A management plan for agricultural subsurface drainage and related problems on the westside San Joaquin Valley

ATTACHMENT:

UNITED STATES

DEPARTMENT OF THE INTERIOR

BUREAU OF RECLAMATION

Central Valley Project, California

Contract for Purchase of Miller & Lux Water Rights

See File Copy