Victoria Whitney, Chief

Bay/Delta Unit

Division of Water Rights

State Water Resources Control Board

P.O. Box 2000

Sacramento, CA 95812-2000

Re:	City of Stockton’s Comments on DEIR for Implementation

of the 1995 Bay/Delta Water Quality Control Plan, Volume 4.

Dear Ms. Whitney:

The following comments on Volume 4 of the Draft Environmental Impact Report, containing revised Chapters V, VI, and XIII, are submitted on behalf of the City of Stockton.

GENERAL COMMENTS

The City incorporates by reference the general comments contained on pages 1 through 4 of its comment letter of March 31, 1998.    We do note that Tables V-1 and V-2 have been improved to show the deliveries that would be made in the base case (Alternative 1), which responds to our previous comment regarding Alternative 1.

The revised chapters do not adequately discuss the effects of proposed actions or alternatives on water quality in the San Joaquin River.    Flow, circulation, water temperature, and other variables affected by the alternatives all have direct and indirect impacts on water quality in the San Joaquin River.    Water quality issues of concern are not limited to salinity, and may include conventional pollutants or toxics.    Stockton has specifically provided the Board with information on the effects of flow, temperature, and circulation patterns on dissolved oxygen (D.O.) concentrations.    There are D.O. objectives that apply to the San Joaquin River year around.    Operation of facilities pursuant to the 1995 water quality control plan will result in certain D.O. conditions.    Changes due to any of the proposed actions will, in turn, result in changes in D.O. levels.    This potential impact should be disclosed and considered.    In addition, the proposed San Joaquin River Agreement itself does not even address D.O., effects on D.O., or the means by which the participation of its signatories would contribute to meeting D.O. objectives.

SPECIFIC COMMENTS

Chapter V.

Page V-1.    Flow alternative 8 does not mention the D.O. standard.    Does the modeling show that this standard will be met if alternative 8 is implemented?

Page V-2.    Alternative 5 appears to have included flow “requirements” from each river that generally were higher than necessary (“significantly exceeds the Delta flow objectives”).    Does this imply that the simulated flows at Vernalis are higher than would actually occur under this alternative if actually implemented to satisfy the 1995 WQCP objectives?    How much lower would the actual flows likely be?

Page V-2.    The statement is made that the other alternatives would have smaller 73-year period average delivery reductions than Alternative 6 if they also included combined use of points of diversion.    Please explain how the combined point of diversion can have such a substantial effect.    What would be the results if the other alternatives also had the combined point of diversion?    If Alternative 6 didn’t have it?

Page V-2.    Not enough details are provided to examine the possible advantages of Alternative 6.    What were the specified Vernalis flows and simulated DMC releases (monthly and annual values)?    It appears that this alternative could be used to satisfy the Vernalis salinity objectives rather than rely on New Melones releases.    Could this alternative also be used to increase flows at Vernalis when upstream water quality was poor (high nutrients and algae concentrations) or when dissolved oxygen concentrations were approaching the DO objectives?    Did the modeling of Alternative 6 take these possibilities into account?

Page V-18.    As stated in our earlier comments, if Friant is considered an export project, it is a violation of the area of origin statute, Water Code section 11460, to place responsibility for Friant’s obligations on New Melones, whose contractors are within the area of origin.    This has the effect of depriving the City of Stockton, which is also entitled to protection as a municipality partially within the Delta and entirely adjacent to it.

Page V-19.  The Draft EIR describes the effects of Flow Alternative 6 only briefly.    Stockton is very interested in the increased Vernalis flows that were obtained using the DMC recirculation alternative.    What were the specified Vernalis flows and simulated DMC releases (monthly and annual values)?    This alternative could be used to satisfy the Vernalis salinity objectives (rather than relying on New Melones releases).    Could this alternative also be used to increase flows during periods when San Joaquin River water quality was poor (high nutrients and algae) or when D.O. concentrations were approaching the D.O. objective?

Chapter VI.

Figure VI-14.    This figure shows average monthly EC at Vernalis for 1987-1992 for each flow alternative.    Why are there different simulated EC (i.e. flows) for different alternatives, when all are supposed to satisfy the same 1995 WQCP Vernalis salinity and Vernalis flow objectives?

Pages VI-8.    It is suggested here that the differences in CCC chloride concentrations predicted by DWRSIM and DWRDSM indicate the need for “carriage water” to support exports during winter months (December and January).    However, another possible explanation is that DWRDSM includes the effects of San Joaquin River salinity and Delta agricultural drainage water salinity, whereas DWRSIM only accounts for the effects of Delta outflow on CCC chloride.    The simulated chloride concentrations that exceed the WQCP objectives during this period indicate that the chloride objectives may require higher outflows than the specified WQCP outflow objectives for these months.    Why are the October chlorides lower than the January chlorides, even though the outflow requirements are lower in October than January?    Why are dry year chlorides higher than critical year chlorides?    The actual relationships between Delta flows and salinity should be more clearly identified rather than described as “modeling mysteries” that defy explanation.

Page VI-72.    The DEIR simply assumes that reduced water deliveries would not have growth-restricting effects in the City of Stockton.    With regard to California water, one cannot simply conclude that historic patterns indicate future trends.    Major changes in the availability of alternative supplies of water render that assumption insupportable.    Because of the overdrafted East San Joaquin County groundwater basin and the water quality problems resulting from saline intrusion, replacing lost surface water supplies with groundwater is an environmentally harmful option and there is no evidence that the other measures mentioned would be sufficient to replace the lost supply.    See also comment to pages VI-123-124.

Page VI-120.    The 70,000 acre foot overdraft listed for San Joaquin County understates the problem.    More than 200,000 acre feet/year would be required to permit recovery of the basin and to put a halt to saline intrusion.

Page VI-123-124.    A reduction in CVP deliveries in San Joaquin County will cause a significant increase in the groundwater overdraft and eastward movement of the saline groundwater intrusion in the vicinity of the City of Stockton.    This is a significant effect which threatens a municipal water supply.    There is no demonstration in the DEIR that the mitigation measures listed in these pages are feasible, would be implemented, or would be effective to prevent that significant adverse impact.

Chapter XIII

The selected joint POD alternatives are extremely confusing because they are only slightly different in most cases and the POD alternatives do not correspond with the flow alternatives (e.g. only flow Alternative 6 included joint POD).    As presented, the water supply benefits and potential salinity effects from the joint POD cannot be easily understood, and should not be considered to be independent of the flow alternatives.

Thank you for the opportunity to comment.    If you have any questions regarding these comments, please contact Morris Allen, Director of Municipal Utilities, at (209) 937-8700.

Very truly yours,

Virginia A. Cahill

Attorneys for City of Stockton

VAC:map

cc:	Morris Allen

Bart Thiltgen, Esq.