July 29, 2026

Victoria Whitney, Chief

Bay/Delta Unit

Division of Water Rights

State Water Resources Control Board

P.O. Box 2000

Sacramento, CA 95812-2000

Subject:    Comments on Bay-Delta WQCP Draft EIR

Dear Ms. Whitney:

You will find enclosed the comments of the United States Bureau of Reclamation and Fish and Wildlife Service on the draft environmental impact report (DEIR) relating to implementation of the Boards 1995 water quality control plan (WQCP) for the San Francisco Bay/Sacramento-San Joaquin Delta Estuary.

The enclosed document reflects only technical comments on the environmental analysis and conclusions contained in the DEIR.    Considering that the DEIR did not identify a preferred alternative, we deferred comments of a policy nature as to a preferred alternative or the direction the Board should take in implementing the 1995 WQCP.    We reserve the right to submit further comments and testimony during future workshop and hearings.

We look forward to working closely with the Board and its staff in implementing the water quality objectives for the Bay-Delta Estuary.    If you have any questions, please contact me directly.

Sincerely,

Alf W. Brandt

Assistant Regional Solicitor

TECHNICAL COMMENTS

on

Draft Environmental Impact Report for Implementation of

the 1995 Bay-Delta Water Quality Control Plan

Submitted by:    	The Bureau of Reclamation (Reclamation) and

The Fish and Wildlife Service (Service)

of the United States Department of the Interior

General Comments

The Services review of the DEIR indicated some limitations relative to fish population discussions.    For example, the fish section provides uneven coverage of the species and drainages, with some species and drainages discussed extensively (Yuba River), others much less extensively (Feather River), and others not at all (any tributary of the lower Sacramento River region other than Butte Creek).    Also,    the DEIR does not evaluate some of the key anadromous fish species such as sturgeon, American shad, and steelhead.    Revision of the DEIR should review the Anadromous Fish Restoration Program (AFRP) Working Paper to obtain further information to broaden the fish discussion, including flow and juvenile American shad index, and flow related to sturgeon abundance.    Unfortunately, that information may not be sufficiently quantitative to allow for use in comparing alternatives.

The discussions on the effects of the various project alternatives do not correlate well to the species mentioned in the Environmental Setting section of the DEIR.    For example, species are listed in tables within the Environmental Setting section that are not mentioned in the effects sections.    Insufficient information has been provided to adequately evaluate the effects of the projects on the sensitive species. The discussion of the environmental effects of the various Suisun Marsh alternatives on sensitive plant species is deficient. The Service recommends that the DEIR be revised to include additional information needed to evaluate the effects and include a summary table of all the alternatives and their effects to facilitate review of the document.

It should be noted that the use of annual monthly averages in the various hydrological tables of flow, export, etc., limits our ability to see potential significant changes caused by the flow and joint points of diversion (JPOD) alternatives.    Anadromous fish life histories reflect specific environmental needs, such as temperature limits and critical migration periods.    Monthly averages may appear to be acceptable to the fish requirements, but daily levels may reflect harsh conditions that could cause high mortalities not reflected in the DEIR.

Another major concern is that once a flow alternative is chosen, operational adjustments will be made in the Delta by the SWP and CVP to keep the Delta in balance during controlled conditions.    This will influence the real world flows that determine Delta hydraulic parameters important to anadromous fish that are not represented in the DEIR.

New salmon smolt survival models may be a better assessment tool for alternative comparisons than the one used in the DEIR, and should be considered when revising the DEIR.    A new salmon survival model (1997) for the Sacramento River portion of the Delta has been developed by John Rice and Ken Newman.    Service staff in Stockton have provided a paper by Rice and Newman to the Board staff that describes the model.    It would be useful to use the new model to see if it provides a similar conclusion relative to that of the earlier 1995 smolt survival model used to prepare the DEIR.

Additional coded-wire tagged (CWT) salmon smolt survival data from recent years is available for the central Delta where smolt survival down the Sacramento River and in Georgiana Slough has been evaluated at different flows and export levels.    While this data alone does not lend itself to quantifying the relationships between survival, flow and exports for alternative comparisons, it may provide useful information to help verify smolt survival model output.    It should be noted that recent analysis by Service staff on smolt survival data in the San Joaquin Delta have provided statistically significant relationships between flow at Stockton and smolt survival.    This relationship is available upon request.    The original smolt survival model for the San Joaquin Delta developed by Service biologists in the early 1990's was very limited and should not be used to compare alternatives.    Finally, Carl Mesick (consultant) has    recently developed significant relationships between smolt survival in the San Joaquin Delta and flow, percent of inflow diverted to the export facilities, and water temperatures.    The Service will make the above assessment tools available upon request for future use in any DEIR revisions.

SPECIFIC COMMENTS

Chapter II - Project Description

Page II-2    -      Chloride concentrations at the Contra Costa Canal at Pumping Plant # 1 are now affected by the operation of Contra Costa Water Districts new Los Vaqueros facilities.    With these new facilities, pumping at Pumping Plant # 1 is now reduced and this can result in stagnation within Rock Slough near the pumping plant and higher chloride concentrations than what are actually present in Old River at the confluence with Rock Slough.    Because the analysis in the DEIR does not reflect operation of the Los Vaqueros facilities, the impact to meeting the municipal and industrial water quality objective at Pumping Plant # 1 is not addressed.    Ultimately, the compliance location identified in the Plan must be changed to a location that can be directly influenced by CVP and SWP operations without reliance on operations by Contra Costa Water District and that truly reflects protection of the beneficial use at issue here.

Page II-14, Footnote 1 Reclamation believes that the use of the DWRSIM model to analyze releases from New Melones for salinity control in the Southern Delta will understate the quantity of water required.    As a consequence of understating the water needed for salinity control, the reservoir appears to have a better capability to meet its other obligations.      Use of the SJRIO model will better model salinity control in the San Joaquin River.

Page II-15, 3rd full Paragraph: It is very doubtful that meeting other objectives of the

Bay-Delta Plan will be sufficient to meet the narrative salmon objective of the Bay-Delta

Plan. The Service suggests revision of this paragraph and a larger discussion of what the

narrative salmon objective means. It should be noted that Interior, in    November 20,

1997    water announcement, identified actions pursuant to the Anadromous Fish

Restoration Program (AFRP) and the Central Valley Project Improvement Act (CVPIA)

that will contribute toward meeting the Boards narrative salmon objective.

Page II-15    - Under all of the flow objectives alternatives, the USBR continues to be exclusively responsible for the release of water to meet the salinity objectives at Vernalis.    This responsibility is based on the language in D-1422 and on the observation that the construction of the CVP has substantially increased salinity loads and reduced flows in the San Joaquin River. (WPRS 1980, Grober 1996)

The USBRs exclusive responsibility is due solely to the fact that D-1422 has imposed that responsibility on the USBR and on no one else.    Grobers observation is simply that, an observation, and it does not and cannot impose any responsibility.    The second part of this statement therefore should be deleted.

Page II-17    - Under this alternative the CVP deliveries to the Tulare Lake basin and the Kern River watershed are assumed to be inbasin deliveries.

Treating Tulare Lake and Kern River basin deliveries as in-basin deliveries is not just an assumption.    It has the support of statute, including Water Code Sections 11460 (cited), 11463, and others.

Page II-18    - While the discussion of Term 91 is basically accurate, the discussion needs to clarify that in-basin water right holders, whether junior or senior to the CVP or the SWP, do not have a right to storage releases from the projects.

Page II-19    - Table II-5    The utility of this table (and Table II-6) could be significantly expanded if the source of water (e.g. American River, Stony Creek, etc.) for each of the water rights were added to the table.    The following corrections need be made to this table concerning CVP water rights.

1.    Application A014858B - the File Date should be the same as the priority date specified in the order permitting this application. The order associated with this application specifies a priority date of 6/16/52.

2.    Application A009364 - storage season should be 10/1 to 6/30

3.    Application A005645A - licensed for 14,800 af storage and not 14,400 af.

4.    Application A002212 - license is for Stony Gorge reservoir and not Black Butte

Page II-19, Table II-5 Tables shown in the DEIR listing potentially impacted water rights

appear to have some inconsistencies and omissions.    The impacts analysis provided in

the DEIR only includes water right holders who have consumptive, post-1914

appropriative water rights with a cumulative face value in excess of 5,000 acre-feet per

year.    This cutoff precludes analysis of water rights held by wildlife refuges, whose loss of water may have a significant impact on the refuges wetlands and other habitats and the species that depend on those habitats.

A number of the National Wildlife Refuges (NWR) authorized to receive water through the Central Valley Project Improvement Act (CVPIA) also have state-based appropriative water rights which were identified in Table II-5 of the DEIR.    However, it is not clear in the DEIR what the potential impacts will be on the following refuges, which are not authorized to receive water pursuant to the CVPIA:    Butte Sink NWR, Modoc NWR, Sacramento River NWR, San Joaquin River NWR, Stone Lakes NWR, Arena Plains Unit of the Grassland Wildlife Management Area.    The Service recommends that, if the Board intends to apportion responsibility for the Delta to the refuges water rights, any future DEIR revisions include an evaluation of the potential impacts on the above refuges because their lack of access to water may have a disproportionately large effect on the refuges wetlands and the ability of managers to maintain adequate habitat to meet resource requirements in the refuges, including listed species habitat.

Page II-28    - Table II-6 The title of this table leads to some confusion.    Though the rights listed are indeed major San Joaquin Basin water rights, they are all water rights upstream of Vernalis.    The title should be modified.    Figure III-11 also leads to some confusion. The title, San Joaquin River Region, is correct but this region covers the watershed of streams (Consumnes, Mokelumne and Calaveras) usually referred to as East-side Streams.    The following corrections need be made to this table concerning CVP water rights.

1.    Application A014858B - The priority date should be 6/16/52 according to the permit.

2.    Application A000234 - total storage should be 500,000 af and not 5,000,000.

Page II-32, Table II-7 It is not clear why the 40-30-30 index is applied to the San Joaquin River Basin.    This may give an incorrect measure of the wetness and thus water availability of the San Joaquin Basin.

Page II-32      f.    Flow Alternative 6 (re-circulation): The DEIR presents very little

information to show to what extent this alternative has actually been explored and shown

to be a viable alternative.    Very little supporting information is presented here.    A couple of    concerns are: 1) The DEIR should compare the export pumping required to implement this alternative to the export pumping that would occur at the same time period for the other flow alternatives; and 2) the DEIR should evaluate the extent water re-circulation would change the basic water composition of the San Joaquin River and what sort of problems or impacts might occur to anadromous fish or other aquatic resources during this time period.    For instance, during drier water conditions this alternative would potentially result in a large percentage of San Joaquin flow that would be from re-circulated and drain water.    This appears to be the only flow alternative where joint-point-of-diversion between the State Water Project (SWP) and Central Valley Project (CVP) is required, and at a level only limited by the physical capacities of the pumping plants.    This feature does not appear to be accounted for specifically in the analysis of the flow objectives section.

Page II-38, 1st paragraph, 3rd sentence:    The Interim South Delta Program (ISDP) tracks through this document as an alternative action that is considered by the DEIR to resolve salinity, oxygen, etc. problems.    In fact, all of the South Delta Salinity Alternatives assume continued operation of barriers.    The Service recommends that the DEIR evaluate other alternatives to determine whether flow objectives and source control in the San Joaquin Valley would provide    adequate salinity controls without barriers.      See our general comments under South Delta Salinity Alternatives (ISDP).

Page II-38, Southern Delta Salinity Alternatives a-c:    In these alternatives, salinity criteria are met through flow objectives combined with installation of barriers.    As part of each alternative, the DEIR should evaluate the changes to south Delta exports at the SWP and CVP pumping plants that would help meet these criteria. Additionally, part of the southern Delta salinity originates from San Joaquin Valley drain water.    As part of each alternative, the DEIR should evaluate source control of salinity in agricultural drain water.    The San Joaquin Valley Drainage Program produced the following plan: A Management Plan for Agricultural Subsurface Drainage and Related Problems on the Westside San Joaquin Valley, by U.S. Department of the Interior and California Resources Agency (September 1990), which discusses some of the measures to control drain water salinity, that should be incorporated in the DEIR analyses.    The Central Valley Regional Water Quality Control Board also has a work plan to address salinity and boron in the San Joaquin River.

Page II-39, Dissolved Oxygen Alternatives a-d:    The DEIR should evaluate an alternative where additional flows (without ISDP) are triggered by a critically low BOD and ammonia level at the Stockton treatment plant discharge.    The evaluation should model the flows being provided by curtailment of exports at the CVP and SWP and/or by San Joaquin side reservoir releases.

Page II-40, Joint Point of Diversion Alternative 4:    It should be noted, that even though it may not be apparent in the monthly modeling, one benefit of a joint point of diversion is the ability to switch diversions at the CVP or SWP to reduce entrainment of Delta smelt.    It would be appropriate for the DEIR to discuss or evaluate this benefit.

Chapter III - Environmental Setting

Page III-2, Figure III-1,(Also    III-2, III-3 and VIII-1) These maps incorrectly locate the Chowchilla River.    They also seem to have labeled the Bear/Merced Creek as the Chowchilla River.    We suggest that all maps show Fresno Slough as in Figure VIII-1, and that the Chowchilla and Eastside Bypasses be shown as well.

Page III-5    -    It is more accurate to say that the CVP is owned by the United States and operated by the USBR.

Page III-6    - Figure III-3.    This figure should include the Red Bluff Diversion Dam and Nimbus Dam.    While Los Banos and Little Panoche Detention Dams have been included, they probably are not necessary.

Page III-7    - ... receive their supply from natural flow and storage regulated at Shasta Dam. should read ... receive their supply from natural flow, storage regulated at Shasta Dam, and Trinity Basin imports.

Page III-7    - ... long-standing historical rights ...    The Exchange Contractors have riparian and pre-1914 rights.    It is not clear why a new category of long-standing historical rights should be established.

Page III-9    - Table III-1.    It should be noted that the CVP contract with the Colusa Drain Mutual Water Company is a replacement water contract, and no water is delivered directly to CDMWC by the CVP.    This contract enables them to divert water from the Colusa Basin Drain when their rights are deficient.    The CVP then makes up the impact of that diversion to downstream senior right holders.    Both Table III-1 and III-2 do not show the units of the data displayed (acre-feet per year).    The time period used for the averaged data should be noted.

Page III-11    - The sentence on the temperature control device should read: A temperature control device was recently installed on Shasta Dam which is intended to allow all releases at Shasta to pass through generation facilities when the system is being operated to meet a temperature standard for fishery enhancement/protection on the upper Sacramento River.

Page III-11    - Keswick Reservoir serves as an afterbay to regulate releases...

Page III-12    - Lake Natoma regulates the releases... (not re-regulates).

Page III-13    - ... CVP and exchange contractors on the west side of the San Joaquin Valley

Page III-13    - ... replace the rivers natural flow which is diverted at Friant Dam...    It should be noted that the inflow to Friant Dam is generally highly regulated by the extensive storage and

generation system of PG&E and SCE.    The water flowing above Millerton Lake therefore is not natural flow in the accepted meaning of the term.

Page III-13 - . . . However, only a portion of the drain was constructed, terminating at Kesterson National Wildlife Refuge.    The discovery of accumulations of selenium in the drainage water and sediments at Kesterson forced the closure of the refuge and the drain after 1985.    The above information is incorrect.    It should be stated that: only a portion of the drain was constructed, terminating at Kesterson Reservoir.    Kesterson Reservoir was incorporated into Kesterson National Wildlife Refuge.    The discovery of accumulations of selenium in the drainage water and sediments at Kesterson Reservoir forced the closure of the reservoir and the drain after 1985.

Page III-14    - Additional water supplies are provided to the Friant-Kern Canal...    It should be noted that such water supplies are not associated with the CVP and the CVP merely facilitates exchanges or wheeling for CVP contractors if such actions do not affect the ability of the CVP to deliver contractual supplies.

Page III-14    -    Eastside Division.    Please note that, ordinarily, the only facility that can contribute to flow and water quality requirements in the Sacramento-San Joaquin Delta is New Melones.    Hidden and Buchanan reservoirs have hydraulic continuity with San Joaquin River and Delta only during flood conditions.

Page III-14      Friant Division.    Water supply is made available in part through an exchange agreement and from purchase of water rights.

Page III-15    - Black Butte Reservoir is not part of the Orland Project.    Black Butte is technically integrated with the CVP.    The Orland Project is not part of the CVP.    The operations of Black Butte Reservoir are coordinated with the Orland Project so as not to affect the rights of the Orland Project and to allow water rights to be developed on the Stony Creek watershed under the watershed protection statute principles with respect to the water rights at Black Butte Reservoir.

Page III-15    - It should be noted that the Army Corps of Engineers (COE)    reservoirs on east-side tributaries to the Tulare Lake Basin do not have federally-held water rights associated with them.    All water rights are held by local water users.

Page III-21    - Major Diversions.    It should be noted that some of the major diverters listed in this section are covered by water right settlement contracts with the CVP or SWP.

Page III-22    - ...1,800 local diversions within the Delta, many of which are unregulated...    It is not clear what is meant by the term unregulated.

Page III-24    - Table III-5.    The following corrections need be made to this table concerning CVP water rights.

-    Folsom Dam, A013370 and A013371: Priority should be OCT 1949 and not Nov 1949.

-    New Melones Dam, A014858B priority date Jun 52

-    Shasta & Keswick Dams, A009364, Storage season should be 10/1 - 6/30

Page III-26, third complete paragraph, 1st sentence:    Change this sentence as follows: "Delta smelt, Sacramento splittail, and longfin smelt are species of special concern because of their declining numbers in the Delta and federal status as threatened (Delta smelt), proposed threatened (Sacramento splittail), and species of concern (longfin smelt) under the ESA."

Page III-30, last complete sentence: Change this sentence as follows: "The USFWS listed the Delta smelt as threatened on March    5, 1993 and issued a formal biological opinion for SWP and CVP operations on May 26,    1993."

Page III-32, g. Sacramento Splittail.    Add a sentence at the end of this section: "The Sacramento splittail was proposed as threatened by USFWS on January 6, 1994."

Pages III-40-50    It is unclear whether Sacramento River Region is equivalent to Sacramento River Basin. These terms appear to be used interchangeably, yet most of the plants included in Table III-8 are plants found at low elevations.    Table III-8 is incomplete.    The table does not match all the habitats mentioned in the text on pages III-48 and III-49, nor the map on page III-40.    The following federally listed plant species should be included in the table.

Loch Lomond coyote-thistle, Eryngium constancei, endangered (E)

few-flowered navarretia, Navarretia leucocephala ssp. pauciflora,(E)

many-flowered navarretia, Navarretia leucocephala ssp. plieantha,(E)

Lake County stonecrop, Parvisedum leiocarpum,(E)

Stebbins morning-glory, Calystegia stebbinsii,(E)

Pine Hill ceanothus, Ceanothus roderickii,(E)

Pine Hill flannelbush, Fremontodendron californicum ssp. decumbens,(E)

El Dorado bedstraw, Galium californicum ssp. sierrae,(E)

Laynes butterweed, Senecio layneae, threatened

Page III-51    Table III-10.    The Service recommends that steelhead, currently listed as a threatened species under the Federal Endangered Species Act (ESA), and fall-run and late fall-run chinook salmon and green sturgeon be added to this list of Sacramento River Basin sensitive fish species.    Refer to the Delta Native Fishes Recovery Plan or Moyle, et als  Fish Species of Special Concern in California when assembling a list of sensitive fish species.    Each of the fish species included in this sensitive fish species table should also be included in Table III-9 for fish species found in the Sacramento River and tributaries.

Page III-53 second paragraph, fourth sentence: The Service questions listing Sacramento splittail as a coldwater game fish. Splittail tolerate much warmer temperatures than do rainbow and

brown trout (their cohorts in the coldwater game fish category) and are not normally considered game fish. These categories contribute little to the DEIR, and should be dropped.

Page III-54 Third full paragraph, first sentence: The Service questions the conclusion that fall-run chinook salmon are the most important anadromous fish in the Yuba River. Abundance and importance should not be seen as synonymous.    The DEIR should be revised to clarify    this issue.    Further, the Yuba River supports a self-sustaining population of steelhead and is essentially the only wild steelhead fishery remaining in the Central Valley as documented in the California Department of Fish and Games February 1996 report, Steelhead Restoration And Management Plan For California. The Service recommends that steelhead be included in the Yuba River discussion.

Page III-54 Fourth full paragraph, first sentence:    The statement that The original spring-run population had disappeared from the Yuba River by 1959" is not consistent with a similarly worded, but decidedly different statement, in the California Department of Fish and Game report Status of Actions to Restore Central Valley Spring Run Chinook Salmon (February 1, 1996). CDFGs report reads ...virtually disappeared... rather than ...disappeared... and throughout their discussion leaves open the possibility that some or all of the spring run population presently in the river may be endemic.    The Service recommends that the DEIR be revised consistent with CDFGs report.

Page III-55    First full paragraph:    To say that aquatic habitat in the American River ...includes a meandering streambed in a broad floodplain isolated from surrounding urban areas by 30-foot levees appears to be a contradictory statement.    It is our understanding that the current configuration of levees through most of the lower American River allows very little meandering of the streambed.    Without the levees, the American River, as it approaches the Sacramento River, would likely meander in a more classical manner.    We acknowledge that some meandering occurs at low flows, but at high flows it continues to be constrained by the levees.    The DEIR should revise the statement to avoid any misunderstanding.

Page III-55    Second full paragraph, second sentence, (d): The Service questions the statement that ...about 60 percent (of chinook salmon escaping to the American River from 1969 to 1981) were produced from salmon spawning naturally in the river and 40 percent from hatcheries.    Such statements have been made in the past, but have not stood up to scrutiny.    Others have proposed different ratios, but the data is insufficient to support any specific ratio.    The DEIR should reflect that there is not a good estimate of the ratio, but future fractional marking of hatchery fish should help refine the estimate.

Page III-72, a. Surface Water Hydrology.  It should be noted that prior to construction of Friant Dam, there was no flow in the San Joaquin River below Sack Dam on numerous occasions.

The Chowchilla and Fresno Rivers are not major tributaries to the San Joaquin.    These rivers have foothill watersheds, not mountain watersheds.    Their flow is predominately based on rainfall, not snowpack.

Page III-73 -    It should be noted that the dissolved oxygen problems on the Stanislaus River have

been corrected through removal and treatment of the point source wastewater and the contribution of flows from New Melones when required to maintain the dissolved oxygen standard.

Page III-79, Table III-13.    Sensitive Plant Species in the San Joaquin River Basin: Please add the diamond petaled poppy (Eschscholzia rhombipetala) to the list of sensitive plant species found in the San Joaquin River Basin.      It was thought to be extinct until 1997 when a single population was found at the Lawrence Livermore National Laboratory in Alameda County.

Page III-80    Second full paragraph, last sentence states that:    ...the Anadromous Fish Restoration Program (AFRP), under which the Service will produce a final plan that will likely propose minimum flows as a necessary element of the achievement of the goal of doubling anadromous fish populations in the Central Valley.    The statement is inacurate.    It should be noted that the Service proposed flows for only CVP-controlled streams in the 1997 Revised Draft Restoration Plan for the Anadromous Fish Restoration Program. It did not propose minimum flows in non-CVP streams.    Also, the AFRP goal is to make all reasonable efforts to at least double natural production of anadromous fish.    (See the Restoration Plan for a more complete statement of this goal). The Service recommends that the above sentence in the DEIR be revised consistent with the above information.

Also, for your information, the AFRP has produced Draft Guidelines for Allocation of Water Acquired Pursuant to Section 3406(b)(3) of the CVPIA dated October 22, 1996, to assist in the allocation of water that is acquired under authority of CVPIA Section 3406(b)(3) for the benefit of anadromous fish in non-CVP streams.    The above comments should help to accurately describe the AFRP as related to the proposal of minimum flows for anadromous fish restoration in the Central Valley.

Page III-80 -    In the discussion of the Delta smelt biological opinion, the proper references to the CVP and SWP need to be made.

Page III-82, Table III-15, Sensitive Wildlife Species in the San Joaquin River Basin: Add the following bat species: Small-footed myotis (Myotis ciliolabrum), Long-eared myotis (Myotis evotis), Long-legged myotis (Myotis volans), and Yuma myotis (Myotis yumanensis).    Additionally, the scientific name of the Townsend's big-eared bat has changed: Corynorhinus (formerly Plecotus townsendii).    California greater mastiff    bat is Eumops perotis ssp. californicus. Also, it should be noted that both the riparian woodrat and riparian brush rabbit have been proposed as federally listed endangered species.

Page III-101, Table III-17.    Sensitive Fish Species in the Sacramento-San Joaquin Delta, Sacramento perch:    It should be noted that the Sacramento perch has been extirpated from the Delta.    It still exists, however    in scattered ponds throughout the Central Valley.

Section F, Pages III-104-106:    A table of the sensitive species that occur in Suisun Marsh should be included in section F.    Besides the species included in paragraph three, the sensitive plant species listed in Table VII-11 also should be mentioned in Section F.

Page III-117, Table III-21.    Sensitive Fish Species in the San Francisco Bay Estuary, Tidewater goby:    It should be noted that tidewater goby has been extirpated from most of its historic range in the San Francisco Bay estuary.    Historically, it was found in both Corte Madera and Novato creeks.

Page III-122      The last major overflow of Tulare Lake Basin to San Joaquin River occurred in 1983."    The term major is, of course, subjective but there have been significant (again a subjective term) flows more recently.

Page III-155, Southern California, 7.    Fish:    Include tidewater goby as a southern California coastal fish species.    Tidewater goby is distributed in coastal California from the Oregon border to the Mexican border.

Chapter IV - Analytical Methods

Page IV-2      DWRSIM - Monthly time step.    As this section explains the monthly time step does not model some hydrologic phenomena such as storm flows. It should be added that other short-term aspects of project operation, such as daily pumping rates, changes and associated environmental effects cannot be modeled in the monthly time step model.

Page IV-4    Reclamation has limited flow releases below Goodwin Dam to 1500 cfs during non-flood operations to protect agriculture.    The use of 8000 cfs capacity therefore overstates the ability of New Melones to provide water to meet downstream requirements.

The dissolved oxygen requirement on the Stanislaus River is shown as an instream flow requirement, which is not correct.    The values incorporated are actually an estimate of the water required to meet the DO requirement based on 3 years of historical record.

Page IV-7    CVP Delta export demands.    It should be noted that Exchange Contractor demands from the Delta-Mendota Canal are reduced when either San Joaquin River or James By-pass flows are available at Mendota Pool.

Page IV-12    par. 3 last sentence: Add the words transported and western, to read as follows: ...through the export salvage facilities and then been transported and released into the western Delta.

Page IV-12 par. 5:    It should be noted that the effects of many other factors potentially influencing survival were recently modeled and evaluated by statistician Ken Newman. As discussed under general comments, the Smolt Survival Model, Newman and Rice, June 1997, is available for use when revising the DEIR.

Chapter V - Water Supply Impacts

Chapter V-    Much of the analysis in this chapter depends upon the Delta requirement or the Delta outflow requirement.    There needs to be a discussion either here or in Chapter IV of how the required Delta flows are determined and if they are different between alternatives.

Page V-1      It should be noted that while this document uses the critical period of 1928-1934 for the entire Central Valley, this is not the true critical period for the San Joaquin River Basin.    The true critical period for San Joaquin River Basin is the most recent drought, 1987-1992.    Therefore impacts for the 1928-34 critical period are an understatement of what the impacts of the most adverse historical period would be.

Page V-2,    Alternative 3    - The DEIR makes the following statement regarding Alternatives 3 and 4: For modeling purposes, the assumption is made that a water right holder in the Sacramento Basin will contract for supplemental supply while a water right holder in the San Joaquin Basin will pump groundwater.

The DEIR should note that Congressional authorization would be required in order for Reclamation to provide such a supplemental water supply to water right holders (or, to say it differently, to allow water right holders to continue to divert flows when their water rights have been determined to be deficient) .    The existing Reclamation contracting programs for right holders on the Sacramento River and the Colusa Basin Drain provide an example of how such a program could work.    This program, if pursued, would require that limitations on diversion quantities (or irrigated acreage) be determined by year type or some other method to reflect the availability of water that will be equitable for the project (including the SWP if they are involved in this program) and project users, non-project water right holders, and the environmental objectives of the WQCP.

Page V-2    The assumption that a supplemental water supply in the San Joaquin Basin will be obtained by pumping ground water is a gross over-simplification.    Ground water is only available in limited quantities and locations.    Without surface water deliveries the historic conjunctive use will not be supportable because of reduced recharge to the groundwater basins.

Page V-3, Table V-3 and V-4  -    The storage levels shown for the reservoirs in the San Joaquin Basin are overstated because of choice of critical period to examine.

Page V-4    -     It is unclear what    optimization studies are being referenced.    Do you mean operation studies?

Page V-4,    Tables V-3 and 4.      It is unclear why Alternatives 3 and 4 should result in higher carryover storage for Eastman Reservoir.    Supposedly this reservoir would not perform differently under Alternatives 1, 2, 3, or 4 because neither it, Friant, or Hensley would have any changes in deliveries.    Hensley should not have changed either but its change is at least in the range of round-off errors over a period of years.

Page V-6, Transfer Capacity    - In the portrayal of transfer capacity in the summary of impacts in Chapter V, the analyses apparently assume the full capacity of the project pumping plants is available.    The results of these analyses should be qualified to note that while full capacity may be technically available, as a practical matter it is unlikely that the full capacity would be used or that water in the quantities shown is available for transfer.

Page V-14, paragraphs 3 and 4:    The DEIR should analyze whether alternative 5 may have different outcomes depending on how the tributary distribution of responsibility is allocated.

Page V-18 - It would be informative to have an explanation of how Don Pedro and McClure reservoirs are reoperated so that they have no delivery reductions.    Perhaps other reservoirs can use this strategy.

Page V-18 (and Page II-29) - We suggest that adding a table listing the unimpaired flows used to determine the obligations.

Page V-18 and II-29 -    Alternative 5    - As indicated on Page V-2, this alternative significantly exceeds the Delta flow objectives, but further refinement would result in water supply impacts closer to the other alternatives.    This caveat should be more widely stated in the DEIR.    This problem means that the relatively high environmental values of this alternative should be viewed with caution and that it is probable that if this alternative were refined, as the DEIR suggests, the environmental values would be closer to those of the other alternatives.    This, of course, reflects the fact that the standards are what drive the environmental analysis and if an alternative, inadvertently or by choice, exceeds the standards, higher environmental values will result.

Reclamation believes the analysis of Alternative 5 is flawed for several reasons.

1.    Water losses on the upper San Joaquin River have not been considered in the analysis.    This leads to an overstatement of the water reaching the lower San Joaquin and the Delta.

2.    The losses on the upper San Joaquin River range from 15 percent to 100 percent depending upon flow and antecedent conditions.    Under certain conditions, the proposed releases will not get to the Merced River and therefore will not contribute to flow at Vernalis.    If this is corrected in the modeling and    analysis of this alternative, either more water would have to be released from Friant to meet the requirement in the Delta (with corresponding greater impacts on the Friant Unit) or less would have to be shown reaching the Delta (with corresponding lower environmental benefits).    It should be noted that, historically, the San Joaquin River had extended periods when the natural river flow at Friant Dam was reduced to zero.

3.    The DWRSIM model oversimplifies the hydrologic and structural/geometric conditions in the reach from Friant to the Merced River.    Actual conditions on the upper San Joaquin River, Fresno River, and Chowchilla River are such that they are not in hydraulic continuity with the middle reach of the San Joaquin River, except in times of high flows or what would reasonably be described as flood conditions.

4.    Currently even under such flood conditions, the reach of the San Joaquin River between Sand Slough Control Structure and the Mariposa Bypass (approximately 21 river miles) does not carry flow, because flows are diverted into the by-passes.    Under non-flood conditions, the reach of the San Joaquin River from Sack Dam to Bear Creek (approximately 67 river miles) does not carry flow.

5.    The DEIR proposes flow releases from reservoirs and rivers that have little or no hydraulic continuity with the Delta.    Releases from these sources would suffer significant losses to reach the Delta or the main-stem of Sacramento or San Joaquin River --    e.g.,    requiring releases from the Fresno and Chowchilla Rivers (Hidden and Buchanan Dams, as noted above),    from Stony Creek (Black Butte Dam), and from Consumnes River (Sly Park Dam) in order to provide flows in the Delta.

6.    Why the Board included or excluded certain reservoirs for Bay-Delta responsibility is not explained.    For example, under Alternative 5, Placer County Water Agencys Middle Fork Project does not have any Delta responsibility because its consumptive use is less than 30,000 af.    In the future, this project may have a consumptive use of 120,000 af.    Present consumptive use may not be an appropriate criteria.

Page V-19    Alternative 6  -    There are a number of technical issues associated with Alternative 6 that the DEIR has not addressed.    These issues may be beyond the scope of the DEIR at this point in time. They, however, will need to be addressed at some point.

1.    The use of Newman Wasteway raises several issues, including:

a.    Sediment contamination and water quality in the wasteway.

b.    Wasteway flow velocity and potential for scour.

c.    Flooding potential near wasteway outlet.

d.    Structural modifications, which may be necessary for recirculation use (dredging, bridge work, etc.).

2. Salmon smolt imprinting on Delta-Mendota Canal water may create problems.

3.    High pumping during the pulse flow period may cause entrainment and other fishery impacts.

4.    Legal and institutional constraints and considerations associated with recirculation have not been addressed.

5.    Complete analysis requires modeling at the half-monthly flow step (e.g., April 15-30 and May 1-15) rather than the monthly time-step level.    When Reclamation made a similar flow study using a monthly model to simulate the half-monthly flow, it was determined that averaging across the month effectively gave the model two extra weeks to meet the average monthly flow, which in turn allowed the model to deliver more Delta-Mendota Canal water than a half-month period would actually allow.    Under these circumstances, the following may occur in the study results:

a.    Less New Melones water would be released than would actually be required.

b.    Delta-Mendota Canal recirculation contribution may be overstated.

c.    April and May Stanislaus River flows may be understated.

d.    Summertime Stanislaus River flows may be overstated.

6.    Based on Reclamation's studies, Alternative 6 may have a limited ability to save water on the Stanislaus.    The times when Alternative 6 could potentially save some water in New Melones would be when the fishery requirement calls for 300 cfs during Apr/May and when the flow requirement at Vernalis is not being met.    In Reclamation studies that have considered the recirculation proposal, we have assumed that during April and May, New Melones will be operated to 1500 cfs to meet instream flow requirements a majority of the time.    Because of that limitation, Reclamation would not be able to release additional water to attempt to meet the flow requirements at Vernalis.

7.    Alternative 6 assumes the joint point of diversion is in use to mitigate water supply impacts to the CVP.    This prevents a level comparison of Alternative 6 with the other alternatives.    This means that the tables comparing the flow alternatives create the false impression that the recirculation alternative has lower impacts than the non-recirculation alternatives, when the addition of joint point to the other alternatives might show a much different picture.

8.    Table VI-23 - This shows a "savings" of water in New Melones during the winter months under Alternative 6.    It is not clear how this savings was achieved.    The only winter-time releases we can identify are for instream flows, flood control spills, and

possibly (on rare occasions) some water quality releases.    None of these releases can be 	reduced to save water for summer releases so it is unclear how the savings were achieved.

Page V-19 and II-33    Alternative 7    - The Board staff has announced that this alternative will be replaced by a further refinement of an agreement for flows on the San Joaquin.    This is referred to as the San Joaquin River Agreement, or the VAMP (Vernalis Adaptive Management Plan) agreement.    The basis for Alternative 7 needs to be clearly stated.

Chapter VI - Environmental Effects

Page VI-2:    2nd sentence: The phrase eggs, smelts, and larvae should read eggs, smolts, and larvae.

Page VI-23, Habitat Loss section, last sentence:    It should be noted that wetted channel periphery is related to water elevation and is controlled by the amount of water in a channel and thus a function of flow.    Wetted channel periphery is important to certain species of fish, including Sacramento splittail, which depend on newly flooded    areas to provide spawning habitat.    Additionally, wetted channel periphery is important to riparian vegetation for adequate soil water to allow evapotranspiration/photosynthesis to occur and which then affects dependent aquatic and terrestrial organisms.    The timing of higher water elevation is critical in the life-cycle of some fish, including Sacramento splittail, and riparian vegetation and associated aquatic and terrestrial organisms.    Thus, the DEIR should note that    the various flow alternatives may have a significant effect on wetted channel periphery and a resulting significant effect on fish, wildlife, and plants.    This may translate into a requirement for mitigation.

Page VI-24 par. 7:    It should be noted that this is a conclusionary statement, made without supporting information. Its possible that the flow alternatives could have some effect on contaminant concentrations. In general, given a fixed input of contaminant, the higher the flow, the more dilute the contaminant. The Service recommends that this paragraph be revised.

Page VI-25 par. 6 sent. 3:    Most salmon juveniles salvaged in the spring at the Delta pumps are from the San Joaquin basin.    It should be noted that although positive proof is elusive, Sacramento River runs are much larger and under some conditions fish from the Sacramento

River system make up a greater proportion of the fish salvaged at the pumps than those from the San Joaquin River basin.

Page VI-26. Paragraph 2, line 7: Should be revised to read: difference in smolt survival indices among the other alternatives for each salmon run.

Page VI-29, Delta smelt, 4th complete paragraph:    The analysis of the benefits to Delta smelt from the various flow alternatives are based on differences in exports and Delta outflow.    The Service recognizes that the models used for the analysis dont account for behavioral cues.      However, it should be noted that a key benefit of flows to Delta smelt is movement of larvae and juvenile fish from upstream areas to suitable rearing habitat in Suisun Bay.    This movement results from physical transport of larvae and providing behavioral cues to free-swimming juvenile fish.    Since Delta smelt may spawn in different geographic areas of the Delta and in different months on a yearly basis, the location and timing of flows to move larval and juvenile fish is critical.    The DEIR should analyze and compare the ability of each Alternative to allow for flows from different tributaries in different months of the year.    Contact the Service for suggested analysis techniques.

Page VI-31, Sacramento splittail, 2nd complete paragraph:    The DEIR should analyze the water elevation height and its effects to wetted channel periphery so that a comparison can be made between alternatives on Sacramento splittail spawning habitat availability.    Contact the Service for suggested analysis techniques.

Page VI-35, Neomysis, 4th sentence:    Add another factor that reduces abundance of N. mercedis in upstream areas:      "(3) higher concentrations of contaminants in upstream areas that cause direct losses through acute effects and indirect losses through effects to food organisms".

Page VI-38, Vegetation and Wildlife, 3rd complete paragraph:    It should be noted that the position of X2 is one measure of salinity which is    determined by Delta outflow.    However, agricultural drainage from the San Joaquin river and tributaries contributes to salinity-related issues.    Thus, changes to flows on the San Joaquin River and its tributaries may have localized effects on vegetation and associated wildlife.    Furthermore, if contaminants such as selenium are included in the analysis of changes to flows on the San Joaquin River and its tributaries, then again vegetation and wildlife may be affected by different flow alternatives.    Additionally, the    magnitude of flows may affect erosion which will affect in-channel islands and channel peripheries in general.

Page VI-46    Last paragraph:    The reference to the Working Paper as plan needs to be revised as it could lead to confusion between flows that are presented in the Working Paper versus what is presented in the 1997 AFRP Revised Draft Restoration Plan, which is the most recent plan.

Page VI-49, third full paragraph:    The summary analysis of the effects of the alternative on flows in the Sacramento River below Keswick Dam should include discussion of the effects of the alternatives on the ability of the CVP to meet the winter-run chinook salmon biological opinion.    Specifically, the DEIR should analyze the impacts of each alternative on the water temperatures provided in the Sacramento River for winter-run chinook salmon.

Flow Alternatives VI-46 to VI-55:    It is difficult to discern how Delta exports would be affected by each of the different flow alternatives. A set of tables comparing the monthly export levels by alternatives would be useful.    Also, we remain interested as to: (1) what happens to flow levels in each stream outside of the April thru June time period, especially in the fall and winter on all streams and through the summer on steelhead and winter-run and spring-run chinook salmon streams (including temperature modeling output); (2) the effects on the Bear, Mokelumne, and Calaveras rivers; (3) assumptions made to model American River flows; and (4) how share was determined.    The revised DEIR should address the issues above relative to the various flow alternatives.

Alternative 6 (re-circulation): The DEIR presents very little supporting information to show the extent this alternative has actually been explored and shown to be a viable alternative. A couple of concerns are: 1) the DEIR should compare the export pumping required to implement this alternative to the export pumping that would occur at the same time period for the other flow alternatives; and 2) the DEIR should evaluate the extent water re-circulation would change the basic water composition of the San Joaquin River and what sort of problems or impacts might occur to anadromous fish and other aquatic resources during this time period.    For instance, during drier water conditions this alternative would potentially result in a large percentage of San Joaquin flow that would be from re-circulated and drain water.    Also, it is unclear how this alternative will effect water temperatures.    This appears to be the only flow alternative where

joint-point-of-diversion between the SWP and CVP is required, and at a level only limited by the physical capacities of the pumping plants.    This feature does not appear to be accounted for specifically in the analysis of the flow objectives section.    Alternative 6 seems to be the best one for striped bass, although there is no explanation why.    In some cases it is unclear why the model performs as it does under the various alternatives.    Some analysis and explanation in the DEIR of the above concerns would be helpful.

Alternative 7:    It is unclear to what extent Alternative 7, and the conditions described in the Letter of Intent reflect the conditions that the Vernalis Adaptive Management Program (VAMP) may create.

Page VI-61, Vegetation and Wildlife, 2nd paragraph, 4th sentence: Changes to river stage throughout the drainage will affect wetted channel periphery and thus affect riparian vegetation and associated wildlife.    If contaminants such as selenium are included in the analysis of changes to flows on the San Joaquin River and its tributaries, then again vegetation and wildlife may be affected by different alternatives. Additionally,    magnitude of flows may affect erosion which will affect in-channel islands and channel peripheries in general.

VI-78, Mitigation    - Some of these impacts could be mitigated by undertaking actions completely unassociated with water such as constructing a mass transit system so that fewer pollutants are produced.    This may be beyond the scope of this DEIR.

Page VI-118 -    It is unlikely that water transfers will be viable alternatives for groundwater use, unless water use changes dramatically in another area.    Also, a map with the counties and the groundwater basins would be helpful to this discussion.

Page VI-122    -    Although the groundwater overdraft may be mitigated by measures 1 and 2, the groundwater will not be available to mitigate the effects of reduced surface water supplies.    Measure 3 seems to ignore the current conjunctive use taking place in the Friant service area and elsewhere in the San Joaquin Basin. Measure 4 can reduce the need for groundwater but can also reduce the deep percolation to the groundwater from the application of water in the area.    Measure 5 has a number of inherent limitations that affect its viability.

VI-133, Effects in Friant Service Area. -    It should be noted that conjunctive use is already an important element of water use in this area and that increased groundwater use, as a long-term measure, is of limited usefulness, as the DEIR concludes.

Page VI-135    - The reference to Figure VI-37 should be to Figure VI-33.

VI-137,    Mitigation.    -    Some analysis of the feasibility of and consequences of the mitigation measures would be helpful and informative.

Pages VI-72 to VI-77, Energy.    -    This section does not fully discuss CVP load requirements and the magnitude of the effects on the CVP load from the proposed alternatives.    It would be helpful to see tables or graphs depicting Project-use requirement from the base case and the alternatives.

As of the date of the report, the DWRSIM model does not have the capability to adequately simulate and calculate CVP load.      Accordingly, results shown on Table VI-53 and Figure VI-29 may not fully represent the proposed alternatives' impacts on the CVP energy and capacity available for sale (generation less Project-use).    (Same comment for Table VI-54 and Figure VI-30.)

Chapter VII - Suisun Marsh Salinity Objectives

The following comments are presented by the Service.

General Comments

The Board has not fulfilled its pledge to implement a standard to prevent reduction of tidal salt marsh habitat in the Suisun Marsh.    When the Service issued the biological opinion to the EPA on November 2, 1994, for the promulgation of Federal water quality standards (Service File No. 1-1-93-F-61) and subsequently concurred that the States standards provided equivalent protection to the Federal standards, the Service agreed to a narrative criterion which stated there would be no reduction of tidal salt marsh habitats as a result of implementing the standards.    We based our decisions upon commitments to develop and implement scientifically-based numerical standards to protect tidal marsh ecosystems in Suisun Marsh.    Although the Suisun Ecological Workgroup (the SEW) has been established to develop these standards and it has been working for over a year now, no such standards have been developed.    The DEIR now specifically states that the Board will not implement the narrative standard at this time and that they will defer establishment and implementation of any numerical standards until the next triennial review.

The Boards intended delay in implementing the narrative standard fails to resolve a number of serious issues related to the health of the marsh and is contrary to our understanding of previous commitments.    The Service believes that there are fundamental flaws in the analysis for the alternatives portrayed.    All of the proposed alternatives perpetuate implementation of the original D-1485 standards with deficiency standards provided in Order WR 95-6.    The Service believes that these standards have adversely affected listed and proposed species living in the marsh, particularly the California clapper rail and the salt marsh harvest mouse.    Their continued implementation would continue these adverse effects and could thwart recovery objectives for listed and proposed species in the Marsh in the future.    The Board should take this opportunity to bring the extent and quality of existing tidal marsh habitats and restoration of additional tidal marsh areas to meet the recovery objectives of listed and proposed species into the analysis of alternatives.

There are fundamental flaws in the D-1485 standards as written.    These standards are based on studies that analyzed stomach content of waterfowl and are directed at promoting a single plant species, alkali bulrush, as a waterfowl food plant in the managed wetlands within the marsh.    These studies are scientifically questionable.    The channel water salinities therefore have been controlled and reduced, thereby limiting the size and quality of channel tidal and brackish marsh habitats dependent on more saline soil and water conditions.      Additionally, the methods employed to manage diked wetlands in the Marsh have prevented the establishment of habitat conditions for salt marsh harvest mice inland from the levees.    These effects should be fully analyzed in the DEIR so that the narrative standard may be implemented and numeric standards developed.

It does not appear that the effects to salt marsh species have been addressed by the DEIR. For example, actions proposed in Marsh Alternative 5, such as the Green Valley Creek flow augmentation to meet western marsh salinity objectives, could freshen and reduce the quality of, if not directly eliminate, tidal marsh in the upper reaches of marsh channels.    In these upper reaches, salts would normally be expected to precipitate out, providing habitat to California clapper rails and salt marsh harvest mice.    Implementation of Marsh Alternative 5 also would continue maintaining low water salinities throughout the Marsh, including channels which support tidal and brackish marshes, through operation of the Montezuma Slough Salinity Control Gates.

The Service proposes analysis of another alternative, which would implement the narrative standard by applying benchmarks for tidal marsh.    Although the SEW has not developed scientifically based, numeric standards for the Marsh, the Service has participated in several SEW subcommittees that are addressing the effects of water salinity standards on listed plant and animal species.    In those subcommittees, the Service has recommended that water salinity levels throughout the Marsh would be determined indirectly by the attainment of the X2 Delta Outflow standards.    Under this scenario, all compliance stations currently within the Marsh would become monitoring stations for water salinities.    Furthermore, California clapper rail and salt marsh harvest mouse habitat and populations at these and other locations throughout the Marsh could be monitored as recommended in the Services biological opinion for the promulgation of Federal water quality standards to assess the effects on these species.    This approach would require that the Montezuma Slough Salinity Control Gates not be operated, except possibly in individual or consecutive years of extreme drought conditions.    The Service believes that alternative management activities for the managed wetlands are available and could be implemented to offset any increases in channel water salinities and thus still provide for preferred waterfowl food plants within these managed areas.    While not completely providing for historic salinity conditions within the Marsh, the Service believes this approach would: (1) improve habitat conditions for listed plants and animals within existing tidal marshes in the Marsh, (2) facilitate improved management of salt marsh harvest mouse mitigation lands in diked wetlands, and (3) better accommodate tidal restoration of diked wetlands in the Marsh to meet the recovery objectives of federally listed and proposed species.    This alternative should be analyzed in the DEIR.

Specific Comments

The DEIR must analyze the specific effects of the alternatives on the federally listed plant species that occur within Suisun Marsh.    The Service is very concerned about the adverse effects that the alternatives could have upon two federally listed plant species that occur within Suisun Marsh, soft birds-beak (Cordylanthus mollis ssp. mollis) and Suisun thistle (Cirsium hydrophilum ssp. hydrophilum).

Page VII-21, Suisun Marsh Alternatives 1-6:

All the Alternatives should include an analysis of salinity regimes so that restoration of "historic" patterns of vegetation and species will occur. These patterns generally have both north-south and east-west distributions.    It should be noted that such habitats as pickleweed with associated salt marsh harvest mouse rely on brackish water salinity regimes that are different than the management done for waterfowl habitat.

Chapter VIII - San Joaquin River Basin Salinity Control

General Comments

Alternatives 2 and 4 call for the use of water authorized under CVPIA for full habitat development on Central Valley Refuge and Wetland Areas [Section 3406(d)(2)] or Level 4 to provide a flow through system to avoid impacts resulting from shifting wetland releases to February.      The DEIR indicates that the impacts are said to occur to waterfowl food production capabilities of the wetlands.    It should be noted that impacts also will occur to wetland-dependent wildlife reproduction which occurs from early March into June.    In addition, the use of Level 4 water supplies is to enhance wetland management capabilities to improve habitat conditions.    The use of 20,000AF in March and April could impact effective use of these water supplies for the full benefit of wetlands and wildlife resources.    The DEIR should evaluate these potential impacts.

It should be noted that the most desirable timing for moist soil food production calls for de-watering wetland units beginning in March.    This coincides with germination of the most desirable plant species.    This drawdown is then followed by an irrigation 30-45 days later to stimulate growth and seed production.    By maximizing releases in February, germination of either less desirable or undesirable plants will reduce food availability for fall and winter waterfowl and other migratory bird populations, and increase maintenance requirements and possibly water use on the impacted wetland areas.    In summary, the alternatives and their impacts need to be fully addressed in the DEIR.

Specific Comments

Page VIII-7    - It is unclear how the historical wetland contribution of salts to the San Joaquin River of eight percent was estimated.    This should be clarified.

Page VIII-19    -    The CVPIA authorization for land retirement is misstated.    The land retirement authorization in CVPIA Section 3408 (h) does not specify a dollar amount to be spent, an amount of acreage to be retired, or the fate of water associated with retired lands.

Page VIII-15    -    The logic for the actions chosen for discussion is unclear.    The subject is salinity management in the San Joaquin River yet relevant ongoing programs such as the San Joaquin River Management Program, the Grassland Bypass Project, wetland water supplies, DWRs real time water quality management program, and other San Joaquin River watershed activities are not addressed.    Conversely, programs discussed such as out of valley disposal, evaporation ponds, land retirement, and others, while relevant to a portion of the San Joaquin River watershed, have historically and currently been primarily focused on areas that do not discharge salts to the River.    This discussion should, at least, include the four ongoing programs mentioned above.

Page VIII-21    -    The alternative scenarios chosen of controlled wetland and agricultural discharges could have been formulated differently and may have produced different conclusions if other, perhaps more realistic, assumptions had been chosen.    Hence, if the SWRCB elects to direct the Central Valley RWQCB to evaluate controlled releases in more detail, it should be with the widest latitude possible for the RWQCB to formulate reasonable salinity management methods.

Page VIII-25-26, Salinity Control Alternative 2 (and Table VIII-5).    -    This scenario apparently shifts all but 10,000 AF of the March and April wetland release to the month of February and assumes that this release has a TDS concentration of 1900 mg/l.    It also assumes that the CVPIA water has a TDS concentration of 960 mg/l and additional CVPIA water has a concentration of 600 mg/l.    The distinction between these two terms and the basis for the TDS figures remains unclear.    Although the scenario is simplistic, perhaps to demonstrate the intent of the action more clearly, the 1900 mg/l wetland drainage salt concentration should not be applied to the reoperated wetland discharges of 43,000 AF.    February salinity concentrations in wetland discharges may be less than 50% of the average April salinity concentrations.    Reclamation suggests that this scenario be more completely explained.

VIII-26, Table VIII-5:    It is not clear where the numbers used in table VIII-5 originated.    Also, the rationale for the 10,000 AF in each month (March and April) is unclear.    The DEIR should clarify the source and rationale for these numbers.

Page VIII-27, C.    Environmental Impacts of Implementing Salinity Control Alternatives: A complete analysis of vegetation and wildlife effects should be done for the various alternatives described in this section.    Land retirement and tile construction will have fairly large effects on plants and animals.

Page VIII-29    -    This table validates the previous statement about the ability of New Melones to actually meet the Vernalis quality requirements and consequently its ability to meet the flow requirements.

Chapter IX - South Delta Salinity Alternatives

General Comments

The DEIR relies on implementation of the barriers program as described in the Interim South Delta Program (ISDP) DEIR/DEIS, or the temporary barriers program.    The Service strongly opposes such assumptions.    The Service has commented extensively in the past that the ISDP DEIR/DEIS has not sufficiently analyzed alternatives to barriers in the South Delta.    To assume their placement and operation in this water rights DEIR is a serious prejudgement of an environmental process that has not reached its conclusion.

Operation of tidal barriers has significant adverse effects on the hydrologic processes that occur in the Delta.    These processes are critical for the survival and recovery of Delta native fishes such as the Delta smelt.    Hydrologic changes, such as changes in net flow in Delta channels and increased scour, result in minimizing the benefits of habitat and transport flows associated with the rest of the flow alternatives in the plan.    Additionally, the barriers likely result in increased exposure to in-Delta agricultural diversions, diminished water quality, delays in-migration, and increased salvage at the State and Federal export facilities.

The Service believes that the Board should evaluate whether the flow objectives and source control in the San Joaquin Valley would provide the needed salinity reductions for South Delta agricultural interests.    If such an analysis shows that those two programs are ineffective, then other structural alternatives may be explored.

Chapter XI    - Economics

Reclamation has the following general comments on the economic studies associated with this DEIR.

1.    Much of the discussion regarding economics in the DEIR and especially in Chapter XI, offers little by way of explanation and appears to expect the reader to take a lot on faith.

2.    Reclamation economists are familiar with Central Valley Production Model (CVPM), which is referenced in both Chapters VI and XI, and assume it was utilized correctly.    However, it is not absolutely clear whether the results of the CVPM analysis concerning falling ground-water tables and increasing pumping costs as presented in Chapter VI were carried forward into the quantification of economic impacts in Chapter XI.    If the analysis was done correctly, the effects of increased ground-water pumping costs should be included in the Chapter XI analysis.    Unfortunately, the textual treatment in Chapter XI is so limited that we cannot know for sure.

3.    The economic effects of the degradation of ground-water quality as water tables fall was not addressed in Chapter XI, either qualitatively or quantitatively.    It should have been.    It could result in changes in cropping patterns with all its attendant changes and/or increases in leaching costs to compensate for poorer quality water.    One could also view ground water (and its quality) as a common-property resource which has economic value to society.    The text should then discuss the implications for future use of this resource and the cost to society from its continued degradation.    If its quality is degraded, the range of future uses would be limited or treatment costs incurred.    Imposing restrictions of ground-water pumping would obviously have its own set of    economic impacts, and it seems that they should be addressed.    The text ignores these important issues.

4.    On page XI-9, the text makes reference to the costs of urban water shortages as developed by Larry Dale Associates.    Reclamation economists find the values cited to be exceptionally high.    It is not shown how the values were developed and the reader would not know the mix of urban uses - residential, commercial, or industrial - they represent.    The DEIR also fails to identify the geographic area for which these values were developed.    In essence, this is a black box, and the text should explain more.    (The reference may in fact explain these questions.)

5.    The economic issues associated with CVP/SWP contracts for replacement supplies or groundwater pumping assumed in Alternatives 3 and 4 do not appear to have been addressed.

Chapter XII - Mandatory Findings

Page XII-3: The first bullet states that Under dry year conditions, the dedication is reduced to 600 TAF.    It should be noted that this is not necessarily consistent with CVPIA Section 3406(b)(2)(C), as interpreted in Interiors November 20, 1997, Announcement.    The November

20 Announcement mentions that a reduction of the 800 TAF could occur in critically dry years,    and that the reduction may be up to 25% but could also be something less depending on    reduction in agricultural deliveries.

Volume 2 - Chapter XIII

Joint Point of Diversion Alternatives

General Comments

This chapter was prepared by the Bureau of Reclamation for the State Board.    As this is the State Boards document, it had editorial control over the final draft that appears in this document.    Reclamation has found several errors and omissions in this chapter.    Reclamation has not made any comments on other aspects of this chapter because of our role in preparing this chapter.    Reclamation expects to work with State Board staff to revise this chapter in response to comments that other parties will be making.      Please note that in preparing the impact analysis of these alternatives, Reclamation used the same analysis procedures and models that were used for the analysis of the Flow Alternatives to the extent possible.    The following comments come primarily from the Service.

It should be noted that the DEIR does not integrate the flow alternatives to implement the Bay-Delta Plan with the JPOD alternatives, which could potentially bias the impact analysis.    The Service recommends that the revised DEIR impact analysis integrate the flow alternatives to implement the Bay-Delta Plan with the JPOD alternatives.

The DEIRs no-action alternative for the JPOD also does not include any analysis of the actions that Interior will take pursuant to CVPIA Section 3406(b)(2), as reflected in Interiors November 20 Announcement. These actions will have potential impacts and benefits to anadromous fish that contribute toward meeting the Boards narrative salmon objective, which should be considered.

The model used in the DEIR to assess JPOD alternatives apparently utilizes CVP and

SWP    export and canal capacity as soon as it is available.    This forces most of the JPOD

use earlier    in the year (for example October-January rather than spread out into

February-March) when    greater potential impacts to the same species (i.e. Spring-run

salmon) may occur than if the    JPOD use was spread out.    Reevaluation of the use of the

JPOD alternatives over a wider time frame would be useful.    The revised DEIR should also include a table to reflect when fish (by key species) entrainment occurs over the year to enable comparisons of JPOD use to fish presence.

The DEIR does not provide data on the potential changes in export to inflow (E/I) ratio caused by JPOD.    Providing Delta E/I ratios for all months for each of the flow and JPOD alternatives would assist in the comparisons.

The figure comparing JPOD alternatives for striped bass production reflects a lot of lumping.    Disclosing the intermediate modeling data reflecting life stage abundance values for larvae and young-of-year would add useful information to the DEIR.

The DEIR does not specifically attempt to evaluate the impact of JPOD alternatives on other key anadromous species such as steelhead, green and white sturgeon and American shad.    See our general comments at the beginning of this document for the Services suggestions on revising the DEIR.

Specific Comments

Pages XIII - 4 to 6:    The definition of the JPOD alternatives is unclear and confusing.    Each should be reviewed and revised.    The descriptions on Page II-39 and 40 are not consistent with Page XIII-4 to 6.

Tables XIII-6 to 13:    Alternative 1 in the 73-year period is the same as the critical period.    This is not possible.    The DEIR also should clarify in the text what the negative values mean.

Table XIII-14      Again, the DEIR should clarify what the negative values mean.

Page XIII-15    - In Tables XIII-6 through XIII-13 and XIII- 6 through XIII-25, the first line of data in all odd numbered tables (monthly averages for the critical period) is not correct.    It will be corrected for the Final EIR.

Page XIII-33, end of last paragraph: The Service does not believe that the timing of emigration of winter and spring-run chinook salmon, can be separated to the extent that the DEIR does.    This should be revised by qualifying the existing statement.

Table XIII - 15:    The table of Q west values was a helpful part of the DEIR.    Positive and negative flows in the south Delta channels (lower Old and middle Rivers) would also be useful to see for the JPOD alternatives.

Table XIII-26 to 31      It would be useful to see how the JPOD alternatives compare for all months

not just April-June, and to explain the negative values in the text.